| 3GPP TR 37.717-41-11 V0.6.0 (2021-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Networks; Dual Connectivity (DC) of 4 bands LTE inter-band CA (4DL/1UL) and 1 NR band (1DL/1UL) (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	7

1	Scope	9

2	References	9

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	10

4	Background	10

4.1	TR Maintenance	10

5	DC of 4 LTE band (4DL/1UL) + 1 NR band: Specific Band Combination Part	10

5.1	Inter-band EN-DC	10

5.1.1	DC_a-b-c-d_ne	10

5.1.1.1	Configuration for EN-DC	10

5.1.1.2	∆TIB and ∆RIB values	10

5.1.1.3	Reference sensitivity exceptions	11

5.1.2	DC_1-7-20-32_n28	11

5.1.2.1	Configuration for EN-DC	11

5.1.2.2	∆TIB and ∆RIB values	11

5.1.2.3	Reference sensitivity exceptions	12

5.1.3	DC_1-7-20-32_n78	12

5.1.3.1	Configuration for EN-DC	12

5.1.3.2	∆TIB and ∆RIB values	12

5.1.3.3	Reference sensitivity exceptions	12

5.1.4	DC_3-7-20-32_n78	12

5.1.4.1	Configuration for EN-DC	12

5.1.4.2	∆TIB and ∆RIB values	13

5.1.4.3	Reference sensitivity exceptions	13

5.1.4	 DC_2-7-28-66_n66	13

5.1.4.1 	Configuration for EN-DC	13

5.1.4.2	 ∆TIB and ∆RIB values	13

5.1.5	DC_2-5-7-66_n66	14

5.1.5.1	Configurations for EN-DC	14

5.1.5.2	∆TIB and ∆RIB values	14

5.1.5.3	REFSENS requirements	14

5.1.6	DC_1-3-7-40_n78	15

5.1.6.1	Configuration for EN-DC	15

5.1.6.2	∆TIB and ∆RIB values	15

5.1.6.3	Reference sensitivity exceptions	15

5.1.7	DC_1-3-8-40_n78	15

5.1.7.1	Configuration for EN-DC	15

5.1.7.2	∆TIB and ∆RIB values	16

5.1.7.3	Reference sensitivity exceptions	16

5.1.8	DC_1-7-8-40_n78	16

5.1.8.1	Configuration for EN-DC	16

5.1.8.2	∆TIB and ∆RIB values	16

5.1.8.3	Reference sensitivity exceptions	17

5.1.9	DC_3-7-8-40_n78	17

5.1.9.1	Configuration for EN-DC	17

5.1.9.2	∆TIB and ∆RIB values	17

5.1.9.3	Reference sensitivity exceptions	18

5.1.10	DC_2-7-28-66_n7	18

5.1.10.1	 Configurations for EN-DC	18

5.1.10.2	 ∆TIB and ∆RIB values	18

5.1.10.3	 Reference sensitivity exceptions	18

5.1.11	DC_2-5-7-66_n7/ DC_2-5-7-66-66_n7	19

5.1.11.1	 Configurations for EN-DC	19

5.1.11.2	 ∆TIB and ∆RIB values	19

5.1.11.3	 Reference sensitivity exceptions	19

5.1.12	DC_1-3-7-8_n28	19

5.1.12.1	 Configurations for EN-DC	19

5.1.12.2	 ∆TIB and ∆RIB values	20

5.1.12.3	 Reference sensitivity exceptions	20

5.1.13	DC_3-7-8-40_n1	20

5.1.13.1	 Configurations for EN-DC	20

5.1.13.2	 ∆TIB and ∆RIB values	20

5.1.13.3	 Reference sensitivity exceptions	21

5.1.14	DC_1-3-20-40_n78	21

5.1.14.1	Configuration for EN-DC	21

5.1.14.2	∆TIB and ∆RIB values	21

5.1.14.3	Reference sensitivity exceptions	22

5.1.15	DC_1-3-8-11_n28	22

5.1.15.1	Configurations for EN-DC	22

5.1.15.2	∆TIB and ∆RIB values	22

5.1.15.3	Reference sensitivity exceptions	22

5.1.16	DC_1-3-8-11_n77	23

5.1.16.1	Configurations for EN-DC	23

5.1.16.2	∆TIB and ∆RIB values	23

5.1.16.3	Reference sensitivity exceptions	23

5.1.17	DC_1-7-8-20_n78	24

5.1.17.1	Configuration for EN-DC	24

5.1.17.2	∆TIB and ∆RIB values	24

5.1.17.3	Reference sensitivity exceptions	24

5.1.18	DC_2-7-12-66_n78	24

5.1.18.1	Configuration for EN-DC	24

5.1.18.2	∆TIB and ∆RIB values	25

5.1.18.3	Reference sensitivity exceptions	25

5.1.19	DC_2-7-66-71_n78	25

5.1.19.1	Configuration for EN-DC	25

5.1.19.2	∆TIB and ∆RIB values	25

5.1.19.3	Reference sensitivity exceptions	26

5.1.20	DC_2-5-7-66_n2	26

5.1.20.1	Configuration for EN-DC	26

5.1.20.2	∆TIB and ∆RIB values	26

5.1.20.3	Reference sensitivity exceptions	26

5.1.21	DC_2-7-66-71_n2	27

5.1.21.1	Configuration for EN-DC	27

5.1.21.2	∆TIB and ∆RIB values	27

5.1.21.3	Reference sensitivity exceptions	27

5.1.22	DC_2-7-12-66_n2	27

5.1.22.1	Configuration for EN-DC	27

5.1.22.2	∆TIB and ∆RIB values	28

5.1.22.3	Reference sensitivity exceptions	28

5.1.23	 DC_1-3-28-40_n78	28

5.1.23.1	Configuration for EN-DC	28

5.1.23.2	∆TIB and ∆RIB values	28

5.1.23.3	REFSENS requirements	29

5.1.24 	DC_1-3-7-38_n28	29

5.1.24.1	 Configurations for EN-DC	29

5.1.24.2	 ∆TIB and ∆RIB values	29

5.1.24.3	 Reference sensitivity exceptions	29

5.1.25	DC_1-3-7-28_n3	30

5.1.25.1	Configuration for EN-DC	30

5.1.25.2	∆TIB and ∆RIB values	30

5.1.25.3	Reference sensitivity exceptions	30

5.1.26	DC_2-5-30-66_n2	31

5.1.26.1	Configuration for EN-DC	31

5.1.26.2	∆TIB and ∆RIB values	31

5.1.26.3	Reference sensitivity exceptions	31

5.1.27	DC_2-5-30-66_n66	32

5.1.27.1	Configuration for EN-DC	32

5.1.27.2	∆TIB and ∆RIB values	32

5.1.27.3	Reference sensitivity exceptions	32

5.1.28	DC_2-14-30-66_n2	33

5.1.28.1	Configuration for EN-DC	33

5.1.28.2	∆TIB and ∆RIB values	33

5.1.28.3	Reference sensitivity exceptions	33

5.1.29	DC_2-14-30-66_n66	34

5.1.29.1	Configuration for EN-DC	34

5.1.29.2	∆TIB and ∆RIB values	34

5.1.29.3	Reference sensitivity exceptions	34

5.1.30	DC_2-29-30-66_n66	34

5.1.30.1	Configuration for EN-DC	34

5.1.30.2	∆TIB and ∆RIB values	35

5.1.30.3	Reference sensitivity exceptions	35

5.1.31	DC_3-7-20-28_n1	35

5.1.31.1	 Configurations for EN-DC	35

5.1.31.2	 ∆TIB and ∆RIB values	35

5.1.31.3	 Reference sensitivity exceptions	36

5.1.32	DC_1-3-5-7_n77	36

5.1.32.1	Configuration for EN-DC	36

5.1.32.2	∆TIB and ∆RIB values	36

5.1.32.3	REFSENS requirements	36

5.1.33	DC_1-7-20-38_n3	37

5.1.33.1	Configuration for EN-DC	37

5.1.33.2	∆TIB and ∆RIB values	37

5.1.33.3	Reference sensitivity exceptions	37

5.1.34	DC_1-7-8-20 _n3	37

5.1.34.1	Configuration for EN-DC	37

5.1.34.2	∆TIB and ∆RIB values	38

5.1.34.3	Reference sensitivity exceptions	38

5.1.35	DC_1-7-20-28 _n3	38

5.1.35.1	Configuration for EN-DC	38

5.1.35.2	∆TIB and ∆RIB values	38

5.1.35.3	Reference sensitivity exceptions	39

5.1.36	DC_1-7-20-32_n3	39

5.1.36.1	Configuration for EN-DC	39

5.1.36.2	∆TIB and ∆RIB values	39

5.1.36.3	Reference sensitivity exceptions	39

5.1.37	DC_1-7-20-32_n8	39

5.1.37.1	Configuration for EN-DC	39

5.1.37.2	∆TIB and ∆RIB values	40

5.1.37.3	Reference sensitivity exceptions	40

5.1.39	DC_1-7-28-32_n3	40

5.1.39.1	Configuration for EN-DC	40

5.1.39.2	∆TIB and ∆RIB values	40

5.1.39.3	Reference sensitivity exceptions	40

5.1.40	DC_1-20-28-32_n3	41

5.1.40.1	Configuration for EN-DC	41

5.1.40.2	∆TIB and ∆RIB values	41

5.1.40.3	Reference sensitivity exceptions	41

5.1.41	DC_3-7-8-20_n1	41

5.1.41.1	Configuration for EN-DC	41

5.1.41.2	∆TIB and ∆RIB values	42

5.1.41.3	Reference sensitivity exceptions	42

5.1.42	DC_3-7-20-32_n1	42

5.1.42.1	Configuration for EN-DC	42

5.1.42.2	∆TIB and ∆RIB values	42

5.1.42.3	Reference sensitivity exceptions	43

5.1.43	DC_7-8-20-32 _n1	43

5.1.43.1	Configuration for EN-DC	43

5.1.43.2	∆TIB and ∆RIB values	43

5.1.43.3	Reference sensitivity exceptions	43

5.1.44	DC_7-20-28-32_n1	43

5.1.44.1	Configuration for EN-DC	43

5.1.44.2	∆TIB and ∆RIB values	44

5.1.44.3	Reference sensitivity exceptions	44

5.1.45	DC_7-20-28-32_n3	44

5.1.45.1	Configuration for EN-DC	44

5.1.45.2	∆TIB and ∆RIB values	44

5.1.45.3	Reference sensitivity exceptions	45

5.1.46	DC_7-20-32-38 _n1	45

5.1.46.1	Configuration for EN-DC	45

5.1.46.2	∆TIB and ∆RIB values	45

5.1.46.3	Reference sensitivity exceptions	45

5.2	Inter-band NE-DC	46

5.2.1	DC_na_b-c-d-e	46

5.2.1.1	Configuration for NE-DC	46

5.2.1.2	∆TIB and ∆RIB values	46

Annex A - Change history	47

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is a technical report for Dual Connectivity (DC) of 4 LTE bands (4DL/1UL) and 1 NR band (1DL/1UL) under Rel-17 time frame. The purpose is to gather the relevant background information and studies in order to address Dual Connectivity (DC) of 4 LTE band (4DL/1UL) and 1 NR band (1DL/1UL) for the Rel-17 band combinations. The co-existence analysis and RF front end requirements such as ΔRIB,c and ΔTIB,c are described based on the band combination basis since such information have no difference between the DC configurations consisting with the same E-UTRA band and the same NR band. The actual requirements are added to the corresponding technical specification.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

It is preferred that the reference to 21.905 be the first in the list.

# 3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

# 4	Background

The present document is a technical report for Dual Connectivity (DC) of 4 bands LTE inter-band CA and 1 NR band under Rel-17 timeframe. The document covers each band combination specific issues (i.e. one sub-clause defined per band combination)

## 4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

# 5	DC of 4 LTE band (4DL/1UL) + 1 NR band: Specific Band Combination Part

<Editor’s note: The requirements for specific band combinations shall be described according to the same manner as specified in TS38.101-3.>

## 5.1	Inter-band EN-DC

### 5.1.1	DC_a-b-c-d_ne

<Editor’s note: This example section will be voided in final TR>

#### 5.1.1.1	Configuration for EN-DC

<Editor’s note: If you need a note use same note numbering as in TS 38-101-3>

Table 5.2B.4.4-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_aA-bA-cA-dA_neA | DC_aA_neA DC_bA_neA DC_cA_neA DC_dA_neA |

#### 5.1.1.2	∆TIB and ∆RIB values

<Editor’s note: rows in ΔTIB,c andΔRIB tables shall be deleted if ΔTIB,c andΔRIB values are zero. >

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_a-b-c-d_ne | a |  |
|  | b |  |
|  | c |  |
|  | d |  |
|  | ne |  |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_a-b-c-d_ne | a |  |
|  | b |  |
|  | c |  |
|  | d |  |
|  | ne |  |

#### 5.1.1.3	Reference sensitivity exceptions

<Editor’s note: Unless specific sensitivity exceptions for intermodulation interference is needed due to dual uplink operation for DC in NR FR1 this section shall be omitted. >

### 5.1.2	DC_1-7-20-32_n28

#### 5.1.2.1	Configuration for EN-DC

Table 5.1.2.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-7A-20A-32A_n28A | DC_1A_n28A DC_7A_n28A DC_20A_n28A |

#### 5.1.2.2	∆TIB and ∆RIB values

Table 5.1.2.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-7A-20A-32A_n28A | 1 | 0.5 |
|  | 7 | 0.6 |
|  | 20 | 0.6 |
|  | n28 | 0.7 |

Table 5.1.2.2.-2: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1A-7A-20A-32A_n28A | 1 | 0 |
|  | 7 | 0 |
|  | 20 | 0.2 |
|  | 32 | 0 |
|  | n28 | 0.2 |

#### 5.1.2.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

### 5.1.3	DC_1-7-20-32_n78

#### 5.1.3.1	Configuration for EN-DC

Table 5.1.3.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-7A-20A-32A_n78A | DC_1A_n78A DC_7A_n78A DC_20A_n78A |

#### 5.1.3.2	∆TIB and ∆RIB values

Table 5.1.3.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-7A-20A-32A_n78A | 1 | 0.6 |
|  | 7 | 0.7 |
|  | 20 | 0.4 |
|  | n78 | 0.8 |

Table 5.1.3.2.-2: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1A-7A-20A-32A_n78A | 1 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0.2 |
|  | 32 | 0 |
|  | n78 | 0.5 |

#### 5.1.3.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

### 5.1.4	DC_3-7-20-32_n78

#### 5.1.4.1	Configuration for EN-DC

Table 5.1.4.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_3A-7A-20A-32A_n78A | DC_3A_n78A DC_7A_n78A DC_20A_n78A |

#### 5.1.4.2	∆TIB and ∆RIB values

Table 5.1.4.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-20-32 _n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |

Table 5.1.4.2.-2: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-7-20-32 _n78 | 3 | 0 |
|  | 7 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | n78 | 0 |

#### 5.1.4.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

### 5.1.4		DC_2-7-28-66_n66

#### 5.1.4.1 	Configuration for EN-DC

Table 5.2B.4.4-1:  Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_2A-7A-28A-66A_n66A DC_2A-7C-28A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_28A_n66A DC_66A_n66A4 |
| NOTE 4: Only single switched UL is supported |  |

#### 5.1.4.2	 ∆TIB and ∆RIB values

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-28-66_n66 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 28 | 0.6 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7-28-66_n66 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 28 | 0.2 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |

### 5.1.5	DC_2-5-7-66_n66

#### 5.1.5.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (five bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-5A-7A-66A_n66A DC_2A-5A-7C-66A_n66A | DC_2A_n66A DC_5A_n66A DC_7A_n66A DC_66A_n66A4 |
| NOTE 4: Only single switched UL is supported |  |

#### 5.1.5.2	∆TIB and ∆RIB values

For DC_2-5-7-66_n66, the TIB,c and RIB,c values are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-7-66_n66 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |

Table 7.3B.3.3.4-1: ΔRIB due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5-7-66_n66 | 2 | 0.3 |
|  | 5 | 0 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |

#### 5.1.5.3	REFSENS requirements

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

### 5.1.6	DC_1-3-7-40_n78

#### 5.1.6.1	Configuration for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-3A-7A-40A_n78A DC_1A-3A-7A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_40A_n78A |

#### 5.1.6.2	∆TIB and ∆RIB values

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A-7A-40A_n78A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.5 |
|  | 40 | 0.35 |
|  | n78 | 0.85 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 7.3B.3.3.4-1: ΔRIB due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1A-3A-7A-40A_n78A | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0 |
|  | 40 | 0.45 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

#### 5.1.6.3	Reference sensitivity exceptions

In addition to its fallbacks, there is no particular MSD requirement needed for this band combination.

### 5.1.7	DC_1-3-8-40_n78

#### 5.1.7.1	Configuration for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-3A-8A-40A_n78A DC_1A-3A-8A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_40A_n78A |

#### 5.1.7.2	∆TIB and ∆RIB values

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-8-40_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 8 | 0.6 |
|  | 40 | 0.35 |
|  | n78 | 0.85 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 7.3B.3.3.4-1: ΔRIB due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-8-40_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 8 | 0.2 |
|  | 40 | 0.45 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

#### 5.1.7.3	Reference sensitivity exceptions

In addition to its fallbacks, there is no particular MSD requirement needed for this band combination.

### 5.1.8	DC_1-7-8-40_n78

#### 5.1.8.1	Configuration for EN-DC

Table 5.1.x.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-7A-8A-40A_n78A DC_1A-7A-8A-40C_n78A | DC_1A_n78A DC_7A_n78A DC_8A_n78A DC_40A_n78A |

#### 5.1.8.2	∆TIB and ∆RIB values

Table 5.1.x.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-8-40_n78 | 1 | 0.6 |
|  | 7 | 0.5 |
|  | 8 | 0.6 |
|  | 40 | 0.35 |
|  | n78 | 0.85 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 5.1.x.2.-2: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-7-8-40_n78 | 1 | 0.2 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | 40 | 0.45 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

#### 5.1.8.3	Reference sensitivity exceptions

In addition to its fallbacks, there is no particular MSD requirement needed for this band combination.

### 5.1.9	DC_3-7-8-40_n78

#### 5.1.9.1	Configuration for EN-DC

Table 5.1.x.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_3A-7A-8A-40A_n78A DC_3A-7A-8A-40C_n78A | DC_3A_n78A DC_7A_n78A DC_8A_n78A DC_40A_n78A |

#### 5.1.9.2	∆TIB and ∆RIB values

Table 5.1.x.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-8-40_n78 | 3 | 0.6 |
|  | 7 | 0.5 |
|  | 8 | 0.6 |
|  | 40 | 0.35 |
|  | n78 | 0.85 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 5.1.x.2.-2: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-7-8-40_n78 | 3 | 0.2 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | 40 | 0.45 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

#### 5.1.9.3	Reference sensitivity exceptions

In addition to its fallbacks, there is no particular MSD requirement needed for this band combination.

### 5.1.10	DC_2-7-28-66_n7

#### 5.1.10.1	 Configurations for EN-DC

Table 5.1.x.1-1: Band combinations EN-DC (five bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-7A-28A-66A_n7A | DC_2A_n7A DC_7A_n7A4 DC_28A_n7A DC_66A_n7A |
| NOTE 4: Only single switched UL is supported |  |

#### 5.1.10.2	 ∆TIB and ∆RIB values

Table 5.1.x.2-1: ΔTIB,c due to EN-DC(five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-28-66_n7 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 28 | 0.6 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

Table 5.1.x.2-2: ΔRIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7-28-66_n7 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 28 | 0.2 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

#### 5.1.10.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

### 5.1.11	DC_2-5-7-66_n7/ DC_2-5-7-66-66_n7

#### 5.1.11.1	 Configurations for EN-DC

Table 5.1.x.1-1: Band combinations EN-DC (five bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-5A-7A-66A_n7A DC_2A-5A-7A-66A-66A_n7A | DC_2A_n7A DC_5A_n7A DC_7A_n7A4 DC_66A_n7A |
| NOTE 4: Only single switched UL is supported |  |

#### 5.1.11.2	 ∆TIB and ∆RIB values

Table 5.1.x.2-1: ΔTIB,c due to EN-DC(five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-7-66_n7 DC_2-5-7-66-66_n7 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

Table 5.1.x.2-2: ΔRIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5-7-66_n7 DC_2-5-7-66-66_n7 | 2 | 0.3 |
|  | 5 | 0.2 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

#### 5.1.11.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

### 5.1.12	DC_1-3-7-8_n28

#### 5.1.12.1	 Configurations for EN-DC

Table 5.1.x.1-1: Band combinations EN-DC (five bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-3A-7A-8A_n28A | DC_1A_n28A DC_3A_n28A DC_7A_n28A DC_8A_n28A |

#### 5.1.12.2	 ∆TIB and ∆RIB values

Table 5.1.x.2-1: ΔTIB,c due to EN-DC(five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7-8_n28 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
|  | n28 | 0.6 |

Table 5.1.x.2-2: ΔRIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-7-8_n28 | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | n28 | 0.2 |

#### 5.1.12.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

### 5.1.13	DC_3-7-8-40_n1

#### 5.1.13.1	 Configurations for EN-DC

Table 5.1.x.1-1: Band combinations EN-DC (five bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-7A-8A-40A_n1A DC_3A-7A-8A-40C_n1A | DC_3A_n1A DC_7A_n1A DC_8A_n1A DC_40A_n1A |

#### 5.1.13.2	 ∆TIB and ∆RIB values

Table 5.1.x.2-1: ΔTIB,c due to EN-DC(five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-8-40_n1 | 3 | 0.5 |
|  | 7 | 0.8 |
|  | 8 | 0.6 |
|  | 40 | 0.9 |
|  | n1 | 0.6 |

Table 5.1.x.2-2: ΔRIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-8-40_n1 | 3 | 0 |
|  | 7 | 0.3 |
|  | 8 | 0.2 |
|  | 40 | 0.8 |
|  | n1 | 0.1 |

#### 5.1.13.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

### 5.1.14	DC_1-3-20-40_n78

#### 5.1.14.1	Configuration for EN-DC

Table 5.1.14.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-3A-20A-40A_n78A DC_1A-3A-20A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A DC_40A_n78A |

#### 5.1.14.2	∆TIB and ∆RIB values

It is proposed to re-use relaxation values from DC_1-3-20_n41-n78 which is very similar combination.

Table 5.1.14.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-20-40_n78 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 20 | 0.3 |
|  | 40 | 0.55 |
|  | n78 | 0.85 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 5.1.14.2.-2: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-20-40_n78 | 1 | 0 |
|  | 3 | 0 |
|  | 20 | 0 |
|  | 40 | 05 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

#### 5.1.14.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

### 5.1.15	DC_1-3-8-11_n28

#### 5.1.15.1	Configurations for EN-DC

5.2B.4.4-1: Inter-band EN-DC configurations within FR1 (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-3A-8A-11A_n28A | DC_1A_n28A DC_3A_n28A DC_8A_n28A DC_11A_n28A |

#### 5.1.15.2	∆TIB and ∆RIB values

For DC_1-3-8-11_n28, the TIB,c and RIB,c values are given in the tables below.

6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-8-11_n28 | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 8 | 0.6 |
|  | 11 | 0.9 |
|  | n28 | 0.6 |

7.3B.3.3.4-1: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-8-11_n28 | 1 | 0 |
|  | 3 | 0.3 |
|  | 8 | 0.2 |
|  | 11 | 0.5 |
|  | n28 | 0.2 |

#### 5.1.15.3	Reference sensitivity exceptions

Co-existence study for DC_1-3-8-11_n28 was covered by the studies for the fallback modes of DC_1-3-8_n28, DC_1-3-11_n28, DC_1-8-11_n28 and DC_3-8-11_n28.

No additional MSD requirement need to be defined for this dual connectivity configuration.

### 5.1.16	DC_1-3-8-11_n77

#### 5.1.16.1	Configurations for EN-DC

5.2B.4.4-1: Inter-band EN-DC configurations within FR1 (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-3A-8A-11A_n77A | DC_1A_n77A DC_3A_n77A DC_8A_n77A DC_11A_n77A |
| DC_1A-3A-8A-11A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_8A_n77A DC_11A_n77A |

#### 5.1.16.2	∆TIB and ∆RIB values

For DC_1-3-8-11_n77, the TIB,c and RIB,c values are given in the tables below.

6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-8-11_n77 |  |  |
|  | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 8 | 0.6 |
|  | 11 | 0.9 |
|  | n77 | 0.8 |

7.3B.3.3.4-1: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-8-11_n77 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 8 | 0.2 |
|  | 11 | 0.5 |
|  | n77 | 0.5 |

#### 5.1.16.3	Reference sensitivity exceptions

Co-existence study for DC_1-3-8-11_n77 was covered by the studies for the fallback modes of DC_1-3-8_n77, DC_1-3-11_n77, DC_1-8-11_n77 and DC_3-8-11_n77.

No additional MSD requirement need to be defined for this dual connectivity configuration.

### 5.1.17	DC_1-7-8-20_n78

#### 5.1.17.1	Configuration for EN-DC

Table 5.1.17.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-7A-8A-20A_n78A | DC_1A_n78A DC_7A_n78A DC_8A_n78A DC_20A_n78A |

#### 5.1.17.2	∆TIB and ∆RIB values

Table 5.1.17.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-7A-8A-20A_n78A | 1 | 0.6 |
|  | 7 | 0.7 |
|  | 8 | 0.6 |
|  | 20 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.17.2.-1: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1A-7A-8A-20A_n78A | 1 | 0.2 |
|  | 7 | 0.2 |
|  | 8 | 0.2 |
|  | 20 | 0.2 |
|  | n78 | 0.5 |

#### 5.1.17.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

### 5.1.18	DC_2-7-12-66_n78

#### 5.1.18.1	Configuration for EN-DC

Table 5.1.18.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_2A-7A-12A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_12A_n78A DC_66A_n78A |
| DC_2A-2A-7A-12A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_12A_n78A DC_66A_n78A |

#### 5.1.18.2	∆TIB and ∆RIB values

Based on values for DC_1-3-7-20_n78 in 38.101-3.

Table 5.1.18.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A-7A-12A-66A_n78A | 2 | 0.6 |
|  | 7 | 0.6 |
|  | 12 | 0.6 |
|  | 66 | 0.6 |
|  | n78 | 0.6 |

Table 5.1.18.2.-2: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2A-7A-12A-66A_n78A | 2 | 0.2 |
|  | 7 | 0.2 |
|  | 12 | 0 |
|  | 66 | 0.2 |
|  | n78 | 0.5 |

#### 5.1.18.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

### 5.1.19	DC_2-7-66-71_n78

#### 5.1.19.1	Configuration for EN-DC

Based on values for DC_1-3-7-20_n78 in 38.101-3.

Table 5.1.19.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_2A-7A-66A-71A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A DC_71A_n78A |
| DC_2A-2A-7A-66A-71A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A DC_71A_n78A |

#### 5.1.19.2	∆TIB and ∆RIB values

Table 5.1.19.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A-7A-66A-71A_n78A | 2 | 0.6 |
|  | 7 | 0.6 |
|  | 66 | 0.6 |
|  | 71 | 0.6 |
|  | n78 | 0.6 |

Table 5.1.19.2.-2: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2A-7A-66A-71A_n78A | 2 | 0.2 |
|  | 7 | 0.2 |
|  | 66 | 0.2 |
|  | 71 | 0 |
|  | n78 | 0.5 |

#### 5.1.19.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

### 5.1.20	DC_2-5-7-66_n2

#### 5.1.20.1	Configuration for EN-DC

Table 5.1.20.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_2A-5A-7A-66A_n2A | DC_5A_n2A DC_7A_n2A DC_66A_n2A |

#### 5.1.20.2	∆TIB and ∆RIB values

Based on values for CA_2-7-13-66 in 36.101.

Table 5.1.20.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A-5A-7A-66A_n2A | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |

Table 5.1.20.2.-2: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2A-5A-7A-66A_n2A | 2 | 0.3 |
|  | 5 | 0 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n2 | 0.3 |

#### 5.1.20.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

### 5.1.21	DC_2-7-66-71_n2

#### 5.1.21.1	Configuration for EN-DC

Based on values for DC_2-7-66_n71.

Table 5.1.21.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_2A-7A-66A-71A_n2A | DC_7A_n2A DC_66A_n2A DC_71A_n2A |

#### 5.1.21.2	∆TIB and ∆RIB values

Table 5.1.21.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A-7A-66A-71A_n2A | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | 71 | 0.3 |
|  | n2 | 0.5 |

Table 5.1.21.2.-2: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2A-7A-66A-71A_n2A | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | 71 | 0 |
|  | n2 | 0.3 |

#### 5.1.21.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

### 5.1.22	DC_2-7-12-66_n2

#### 5.1.22.1	Configuration for EN-DC

Based on values for CA_2-4-7-12 in 36.101.

Table 5.1.22.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_2A-7A-12A-66A _n2A | DC_7A_n2A DC_12A_n2A DC_66A_n2A |

#### 5.1.22.2	∆TIB and ∆RIB values

Table 5.1.22.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A-7A-12A-66A _n2A | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 12 | 0.8 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |

Table 5.1.22.2.-2: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2A-7A-12A-66A _n2A | 2 | 0.3 |
|  | 7 | 0.3 |
|  | 12 | 0.5 |
|  | 66 | 0.5 |
|  | n2 | 0.3 |

#### 5.1.22.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

### 5.1.23		DC_1-3-28-40_n78

#### 5.1.23.1	Configuration for EN-DC

Table 5.1.23.1-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-3A-28A-40A_n78A DC_1A-3A-28A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_28A_n78A DC_40A_n78A |

#### 5.1.23.2	∆TIB and ∆RIB values

E-UTRA CA_1-3-28-40 has been used as a basis for proposed relaxations.

Table 5.1.23.2-1: ΔTIB,c due to EN-DC(five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-28-40_n78 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 28 | 0.6 |
|  | 40 | 0.5 |
|  | n78 | 0.8 |

Table 5.1.23.2-1: ΔRIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-28-40_n78 | 1 | 0 |
|  | 3 | 0 |
|  | 28 | 0.2 |
|  | 40 | 0 |
|  | n78 | 0.5 |

#### 5.1.23.3	REFSENS requirements

No additional MSD requirement need to be defined for this EN-DC configuration.

### 5.1.24 	DC_1-3-7-38_n28

#### 5.1.24.1	 Configurations for EN-DC

Table 5.1.24.1-1: Band combinations EN-DC (five bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-3A-7A-38A_n28A5 DC_1A-3C-7A-38A_n28A5 | DC_1A_n28A DC_3A_n28A |
| NOTE 5: Band 7 and Band 38 are restricted as DL Scell. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. |  |

#### 5.1.24.2	 ∆TIB and ∆RIB values

Table 5.1.24.2-1: ΔTIB,c due to EN-DC(five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7-38_n28 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0 |
|  | 38 | 0 |
|  | n28 | 0.5 |

Table 5.1.24.2-2: ΔRIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-7-38_n28 | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
|  | 38 | 0.2 |
|  | n28 | 0.2 |

#### 5.1.24.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

### 5.1.25	DC_1-3-7-28_n3

#### 5.1.25.1	Configuration for EN-DC

Table 5.1.25.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-3A-7A-28A_n3A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A DC_28A_n3A |
| DC_1A-3A-7C-28A_n3A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_28A_n3A |
| NOTE 4: Only single switched UL is supported |  |

#### 5.1.25.2	∆TIB and ∆RIB values

Based on values for CA_1-3-7-28 in 36.101.

Table 5.1.25.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7-28_n3 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
|  | n3 | 0.6 |

Table 5.1.25.2.-2: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-7-28_n3 | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
|  | 28 | 0.2 |
|  | n3 | 0 |

#### 5.1.25.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

### 5.1.26	DC_2-5-30-66_n2

#### 5.1.26.1	Configuration for EN-DC

Table 5.1.26.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_2A-5A-30A-66A_n2A | DC_2A_n2A1 DC_5A_n2A DC_30A_n2A DC_66A_n2A |
| NOTE 4: Only single switched UL is supported |  |

#### 5.1.26.2	∆TIB and ∆RIB values

Based on values for CA_2-5-30-66 in 36.101.

Table 5.1.26.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-30-66_n2 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |

Table 5.1.26.2.-2: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-5-30-66_n2 | 2 | 0.4 |
|  | 5 | 0 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n2 | 0.4 |

#### 5.1.26.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

### 5.1.27	DC_2-5-30-66_n66

#### 5.1.27.1	Configuration for EN-DC

Table 5.1.27.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_2A-5A-30A-66A_n66A | DC_2A_n66A DC_5A_n66A DC_30A_n66A DC_66A_n66A4 |
| NOTE 4: Only single switched UL is supported |  |

#### 5.1.27.2	∆TIB and ∆RIB values

Based on values for CA_2-5-30-66 in 36.101.

Table 5.1.27.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-30-66_n66 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |

Table 5.1.27.2.-2: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-5-30-66_n66 | 2 | 0.4 |
|  | 5 | 0 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n66 | 0.4 |

#### 5.1.27.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

### 5.1.28	DC_2-14-30-66_n2

#### 5.1.28.1	Configuration for EN-DC

Table 5.1.28.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_2A-14A-30A-66A_n2A | DC_2A_n2A4 DC_14A_n2A DC_30A_n2A DC_66A_n2A |
| NOTE 4: Only single switched UL is supported |  |

#### 5.1.28.2	∆TIB and ∆RIB values

Based on values for CA_2-14-30-66 in 36.101.

Table 5.1.28.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-14-30-66_n2 | 2 | 0.5 |
|  | 14 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |

Table 5.1.28.2.-2: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-14-30-66_n2 | 2 | 0.4 |
|  | 14 | 0 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n2 | 0.4 |

#### 5.1.28.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

### 5.1.29	DC_2-14-30-66_n66

#### 5.1.29.1	Configuration for EN-DC

Table 5.1.29.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_2A-14A-30A-66A_n66A | DC_2A_n66A DC_14A_n66A DC_30A_n66A DC_66A_n66A4 |
| NOTE 4: Only single switched UL is supported |  |

#### 5.1.29.2	∆TIB and ∆RIB values

Based on values for CA_2-14-30-66 in 36.101.

Table 5.1.29.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-14-30-66_n66 | 2 | 0.5 |
|  | 14 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |

Table 5.1.29.2.-2: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-14-30-66_n66 | 2 | 0.4 |
|  | 14 | 0 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n66 | 0.4 |

#### 5.1.29.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

### 5.1.30	DC_2-29-30-66_n66

#### 5.1.30.1	Configuration for EN-DC

Table 5.1.30.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_2A-29A-30A-66A_n66A | DC_2A_n66A DC_30A_n66A DC_66A_n66A4 |
| NOTE 4: Only single switched UL is supported |  |

#### 5.1.30.2	∆TIB and ∆RIB values

Based on values for CA_2-29-30-66 in 36.101.

Table 5.1.30.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-29-30-66_n66 | 2 | 0.5 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |

Table 5.1.30.2.-2: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-29-30-66_n66 | 2 | 0.4 |
|  | 29 | 0 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n66 | 0.4 |

#### 5.1.30.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

### 5.1.31	DC_3-7-20-28_n1

#### 5.1.31.1	 Configurations for EN-DC

Table 5.1.31.1-1: Band combinations EN-DC (five bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-7A-20A-28A_n1A | DC_3A_n1A DC_7A_n1A DC_20A_n1A DC_28A_n1A |

#### 5.1.31.2	 ∆TIB and ∆RIB values

Table 5.1.31.2-1: ΔTIB,c due to EN-DC(five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-20-28_n1 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |
|  | n1 | 0.6 |

Table 5.1.31.2-2: ΔRIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-20-28_n1 | 3 | 0 |
|  | 7 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |
|  | n1 | 0 |

#### 5.1.31.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

### 5.1.32	DC_1-3-5-7_n77

#### 5.1.32.1	Configuration for EN-DC

Table 5.1.32.1-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-3A-5A-7A_n77A DC_1A-3A-5A-7A_n77(2A) DC_1A-3A-5A-7A-7A_n77A DC_1A-3A-5A-7A-7A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_5A_n77A DC_7A_n77A |

#### 5.1.32.2	∆TIB and ∆RIB values

The TIB,c and RIB,c values are specified below:

Table 5.1.32.2-1: ΔTIB,c due to EN-DC(five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-5-7_n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n77 | 0.8 |

Table 5.1.32.2-2: ΔRIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-5-7_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n77 | 0.5 |

#### 5.1.32.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

### 5.1.33	DC_1-7-20-38_n3

#### 5.1.33.1	Configuration for EN-DC

Table 5.1.33.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-7A-20A-38A_n3A | DC_1A_n3A DC_20A_n3A |

#### 5.1.33.2	∆TIB and ∆RIB values

Table 5.1.33.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-20-38_n3 | 1 | 0.6 |
|  | 7 | 0.5 |
|  | 20 | 0.5 |
|  | 38 | 0.5 |
|  | n3 | 0.6 |

Table 5.1.33.2.-2: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-7-20-38_n3 | 1 | 0 |
|  | 7 | 0 |
|  | 20 | 0 |
|  | 38 | 0.2 |
|  | n3 | 0 |

#### 5.1.33.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

### 5.1.34	DC_1-7-8-20 _n3

#### 5.1.34.1	Configuration for EN-DC

Table 5.1.34.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-7A-8A-20A _n3A | DC_1A_n3A DC_7A_n3A DC_8A_n3A DC_20A_n3A |

#### 5.1.34.2	∆TIB and ∆RIB values

Table 5.1.34.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-8-20 _n3 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
|  | 20 | 0.6 |
|  | n3 | 0.6 |

Table 5.1.34.2.-1: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-7-8-20 _n3 | 1 | 0 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | 20 | 0.2 |
|  | n3 | 0 |

#### 5.1.34.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

### 5.1.35	DC_1-7-20-28 _n3

#### 5.1.35.1	Configuration for EN-DC

Table 5.1.35.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-7A-20A-28A _n3A | DC_1A_n3A DC_7A_n3A DC_20A_n3A DC_28A_n3A |

#### 5.1.35.2	∆TIB and ∆RIB values

Table 5.1.35.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-20-28 _n3 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |
|  | n3 | 0.6 |

Table 5.1.35.2.-1: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-7-20-28_n3 | 1 | 0 |
|  | 7 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |
|  | n3 | 0 |

#### 5.1.35.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination

### 5.1.36	DC_1-7-20-32_n3

#### 5.1.36.1	Configuration for EN-DC

Table 5.1.36.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-7A-20A-32A_n3A | DC_1A_n3A DC_7A_n3A DC_20A_n3A |

#### 5.1.36.2	∆TIB and ∆RIB values

Table 5.1.36.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-20-32_n3 | 1 | 0.7 |
|  | 7 | 0.7 |
|  | 20 | 0.3 |
|  | n3 | 0.7 |

Table 5.1.36.2.-1: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-7-20-32_n3 | 1 | 0 |
|  | 7 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | n3 | 0 |

#### 5.1.36.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination

### 5.1.37	DC_1-7-20-32_n8

#### 5.1.37.1	Configuration for EN-DC

Table 5.1.37.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-7A-20A-32A_n8A | DC_1A_n8A DC_7A_n8A DC_20A_n8A |

#### 5.1.37.2	∆TIB and ∆RIB values

Table 5.1.37.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-20-32_n8 | 1 | 0.7 |
|  | 7 | 0.7 |
|  | 20 | 0.6 |
|  | n8 | 0.6 |

Table 5.1.37.2.-1: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-7-20-32_n8 | 1 | 0 |
|  | 7 | 0 |
|  | 20 | 0.2 |
|  | 32 | 0 |
|  | n8 | 0.2 |

#### 5.1.37.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination

### 5.1.39	DC_1-7-28-32_n3

#### 5.1.39.1	Configuration for EN-DC

Table 5.1.39.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-7A-28A-32A_n3A | DC_1A_n3A DC_7A_n3A DC_28A_n3A |

#### 5.1.39.2	∆TIB and ∆RIB values

Table 5.1.39.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-28-32_n3 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
|  | n3 | 0.6 |

Table 5.1.39.2.-1: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-7-28-32_n3 | 1 | 0 |
|  | 7 | 0 |
|  | 28 | 0.2 |
|  | 32 | 0 |
|  | n3 | 0 |

#### 5.1.39.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination

### 5.1.40	DC_1-20-28-32_n3

#### 5.1.40.1	Configuration for EN-DC

Table 5.1.40.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-20A-28A-32A_n3A | DC_1A_n3A DC_20A_n3A DC_28A_n3A |

#### 5.1.40.2	∆TIB and ∆RIB values

Table 5.1.40.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-20-28-32_n3 | 1 | 0.5 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |
|  | n3 | 0.5 |

Table 5.1.40.2.-1: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-20-28-32_n3 | 1 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |
|  | 32 | 0 |
|  | n3 | 0 |

#### 5.1.40.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination

### 5.1.41	DC_3-7-8-20_n1

#### 5.1.41.1	Configuration for EN-DC

Table 5.1.41.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_3A-7A-8A-20A_n1A | DC_3A_n1A DC_7A_n1A DC_8A_n1A DC_20A_n1A |

#### 5.1.41.2	∆TIB and ∆RIB values

Table 5.1.41.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-8-20_n1 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
|  | 20 | 0.6 |
|  | n1 | 0.6 |

Table 5.1.41.2.-1: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-7-8-20_n1 | 3 | 0 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | 20 | 0.2 |
|  | n1 | 0 |

#### 5.1.41.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

### 5.1.42	DC_3-7-20-32_n1

#### 5.1.42.1	Configuration for EN-DC

Table 5.1.42.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_3A-7A-20A-32A_n1A | DC_3A_n1A DC_7A_n1A DC_20A_n1A |

#### 5.1.42.2	∆TIB and ∆RIB values

Table 5.1.42.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-20-32_n1 | 3 | 0.7 |
|  | 7 | 0.7 |
|  | 20 | 0.3 |
|  | n1 | 0.7 |

Table 5.1.42.2.-1: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-7-20-32_n1 | 3 | 0 |
|  | 7 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | n1 | 0 |

#### 5.1.42.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

### 5.1.43	DC_7-8-20-32 _n1

#### 5.1.43.1	Configuration for EN-DC

Table 5.1.43.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-8A-20A-32A _n1A | DC_7A_n1A DC_8A_n1A DC_20A_n1A |

#### 5.1.43.2	∆TIB and ∆RIB values

Table 5.1.43.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-8-20-32_n1 | 7 | 0.7 |
|  | 8 | 0.6 |
|  | 20 | 0.7 |
|  | 32 | N/A |
|  | n1 | 0.5 |

Table 5.1.43.2.-1: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7-8-20-32_n1 | 7 | 0 |
|  | 8 | 0.2 |
|  | 20 | 0.2 |
|  | 32 | 0 |
|  | n1 | 0 |

#### 5.1.43.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

### 5.1.44	DC_7-20-28-32_n1

#### 5.1.44.1	Configuration for EN-DC

Table 5.1.44.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-20A-28A-32A_n1A | DC_7A_n1A DC_20A_n1A DC_28A_n1A |

#### 5.1.44.2	∆TIB and ∆RIB values

Table 5.1.44.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-20-28-32_n1 | 7 | 0.7 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |
|  | n1 | 0.7 |

Table 5.1.44.2.-1: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7-20-28-32_n1 | 7 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |
|  | 32 | 0 |
|  | n1 | 0 |

#### 5.1.44.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination

### 5.1.45	DC_7-20-28-32_n3

#### 5.1.45.1	Configuration for EN-DC

Table 5.1.45.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-20A-28A-32A_n3A | DC_7A_n3A DC_20A_n3A DC_28A_n3A |

#### 5.1.45.2	∆TIB and ∆RIB values

Table 5.1.45.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-20-28-32_n3 | 7 | 0.7 |
|  | 20 | 0.6 |
|  | 28 | 0.5 |
|  | n3 | 0.7 |

Table 5.1.45.2.-1: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7-20-28-32_n3 | 7 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.1 |
|  | 32 | 0 |
|  | n3 | 0 |

#### 5.1.45.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination

### 5.1.46	DC_7-20-32-38 _n1

#### 5.1.46.1	Configuration for EN-DC

Table 5.1.46.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-20A-32A-38A _n1A | DC_20A_n1A |

#### 5.1.46.2	∆TIB and ∆RIB values

Table 5.1.46.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-20-32-38_n1 | 20 | 0.3 |
|  | n1 | 0.7 |

Table 5.1.46.2.-1: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7-20-32-38_n1 | 7 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | 38 | 0.2 |
|  | n1 | 0 |

#### 5.1.46.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

## 5.2	Inter-band NE-DC

### 5.2.1	DC_na_b-c-d-e

<Editor’s note: This example section will be voided in final TR>

#### 5.2.1.1	Configuration for NE-DC

<Editor’s note: If you need a note use same note numbering as in TS 38-101-3>

Table 5.5B.4a.4-1: Band combinations NE-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_naA_bA-cA-ndA-eA | DC_naA_bA DC_naA_cA DC_naA_dA DC_naA_eA |

#### 5.2.1.2	∆TIB and ∆RIB values

<Editor’s note: Unless ΔTIB,c andΔRIB for specific reason need to be specified different than the correspondingly specified EN-DC configuration this section shall be omitted. >

# Annex A - Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-08 | 3GPP RAN4#96-e | R4-2010399 |  |  |  | TR skeleton | 0.0.1 |
| 2020-08 | 3GPP RAN4#96-e | R4-2010400 R4-2009860 R4-2009861 R4-2009862 R4-2011624 |  |  |  | Addition of TPs from RAN4#96-e: TP for TR 37.717-41-11: DC_1-7-20-32_n28 TP for TR 37.717-41-11: DC_1-7-20-32_n78 TP for TR 37.717-41-11: DC_3-7-20-32_n78 TP for TR 37.716-41-11 DC_2A-7A-28A 66A_n66A / DC_2A-7C-28A-66A _n66A | 0.1.0 |
| 2020-11 | 3GPP RAN4#97-e | R4-2015216 R4-2016657 R4-2016672 R4-2016673 R4-2016674 R4-2016675 R4-2015416 R4-2015417 R4-2015418 R4-2016677 |  |  |  | Addition of TPs from RAN4#97-e: TP for 37.717-41-11 for DC_2-5-7-66_n66 TP to TR 37.717-41-11 DC_1A-3A-7A-40C_n78A TP to TR 37.717-41-11 DC_1A-3A-8A-40C_n78A TP to TR 37.717-41-11 DC_1A-7A-8A-40C_n78A TP to TR 37.717-41-11 DC_3A-7A-8A-40C_n78A TP for TR 37.717-41-11: DC_2A-7A-28A-66A_n7A TP for TR 37.717-41-11:DC_2A-5A-7A-66A_n7A/DC_2A-5A-7A-66A-66A_n7A TP for TR 37.717-41-11:DC_1A-3A-7A-8A_n28A TP for TR 37.717-41-11:DC_3A-7A-8A-40A_n1A/DC_3A-7A-8A-40C_n1A | 0.2.0 |
| 2021-01 | 3GPP RAN4#98-e | R4-2101926 R4-2103008 R4-2100672 R4-2100673 R4-2101573 R4-2102038 R4-2102039 R4-2102040 R4-2102041 R4-2102042 |  |  |  | Addition of TPs from RAN4#98-e: TP to TR 37.717-41-11: DC_1-3-20-40_n78 TP for TR 37.717-41-11: EN-DC_1-3-8-11_n28 TP for TR 37.717-41-11: EN-DC_1-3-8-11_n77 TP for TR 37.717-41-11: DC_1-7-8-20_n78 TP to TR 37.717-41-11 to include 2A-7A-12A-66A_n78A, 2A-2A-7A-12A-66A_n78A TP to TR 37.717-41-11 to include 2A-7A-66A-71A_n78A, 2A-2A-7A-66A-71A_n78A TP to TR 37.717-41-11 to include 2A-5A-7A-66A_n2A TP to TR 37.717-41-11 to include 2A-7A-66A-71A_n2A TP to TR 37.717-41-11 to include 2A-7A-66A-71A_n2A | 0.3.0 |
| 2021-04 | 3GPP RAN4#98bis-e | R4-2107188 R4-2105284 R4-2106646 R4-2107067 R4-2107068 R4-2107069 R4-2107070 R4-2107071 R4-2107072 |  |  |  | Addition of TPs from RAN4#98-e: TP to TR 37.717-41-11: DC_1-3-28-40_n78 TP for TR 37.717-41-11: DC_1A-3A-7A-38A_n28A/DC_1A-3C-7A-38A_n28A TP for TR 37 717-41-11 to include DC_1A-3A-7A-28A_n3A TP for TR 37 717-41-11 to include DC_2A-5A-30A-66A_n2A TP for TR 37 717-41-11 to include DC_2A-5A-30A-66A_n66A TP for TR 37 717-41-11 to include DC_2A-14A-30A-66A_n2A TP for TR 37 717-41-11 to include DC_2A-14A-30A-66A_n66A TP for TR 37 717-41-11 to include DC_2A-29A-30A-66A_n66A | 0.4.0 |
| 2021-05 | 3GPP RAN4#99-e | R4-2110715 R4-2110250 |  |  |  | Addition of TPs from RAN4#99-e TP for TR 37.717-41-11: DC_3A-7A-20A-28A_n1A | 0.5.0 |
| 2021-08 | 3GPP RAN4#100-e | R4-2113683 R4-2112460 R4-2112937 R4-2113613 R4-2113614 R4-2113615 R4-2113616 R4-2113618 R4-2113619 R4-2113627 R4-2113645 R4-2113646 R4-2113647 R4-2113648 R4-2113649 |  |  |  | Addition of TPs from RAN4#100-e TP for TR 37.717-41-11 DC_1-3-5-7_n77 TP for TR 37.717-41-11: DC_1A-7A-20A-38A_n3A TP for TR 37.717-41-11: DC_1-7-8-20_n3 TP for TR 37.717-41-11: DC_1-7-20-28_n3 TP for TR 37.717-41-11: DC_1-7-20-32_n3 TP for TR 37.717-41-11: DC_1-7-20-32_n8 TP for TR 37.717-41-11: DC_1-7-28-32_n3 TP for TR 37.717-41-11: DC_1-20-28-32_n3 TP for TR 37.717-41-11: DC_3-7-8-20_n1 TP for TR 37.717-41-11: DC_3-7-20-32_n1 TP for TR 37.717-41-11: DC_7-8-20-32_n1 TP for TR 37.717-41-11: DC_7-20-28-32_n1 TP for TR 37.717-41-11: DC_7-20-28-32_n3 TP for TR 37.717-41-11: DC_7-20-32-38_n1 | 0.6.0 |