3GPP TSG-RAN WG4 Meeting #100-e	R4-2113750

# Electronic meeting, August 16th-27th, 2021

Source:		Ericsson

Title:			900MHz RMR band – UE RF

Agenda item:		8.4.2

Document for:		Approval

# Introduction

The 900MHz NR band for European RMR WI ([1]) has been revised in RAN#92e meeting to introduce a new 900MHz band for Railway in Europe.

After some initial discussion  in last RAN4#99-e meeting, this contribution is further addressing the UE RF aspects for this new band.

# Discussion

## ECC Decision(20)02

The technical conditions for RMR terminals other than cab-radios are given in Annex A2.3 of [2]:

For radio access technologies other than GSM-R, the following parameters apply:

Maximum output power: 23 dBm;

ACLR: 30 dB minimum;

Uplink power control is mandatory and shall be activated.

According to TS 38.101-1, those requirements are already covered by PC3 NR UE.

Noting that cab-radio is not in the scope of the RMR 900MHz WI ([1]), the impacts in TS 38.101-1 are including the new band in the list of operating bands, adding its system parameters and the usual limits when introducing a new band, when relevant (e.g. REFSENS, …).

Proposal: When specifying the new 900 MHz RMR band, no additional requirement to that band should be introduced in TS 38.101-1, only the usual limits should be updated accordingly when relevant (e.g. REFSENS, …).

# Conclusion

In this contribution, we studied the impacts of introducing the 900MHz RMR band in TS 38.101-1, based on ECC Decision (20)02. We made the following proposal:

Proposal: When specifying the new 900 MHz RMR band, no additional requirement to that band should be introduced in TS 38.101-1, only the usual limits should be updated accordingly when relevant (e.g. REFSENS, …).

# References

- RP-211495, Introduction of 900MHz NR band for Europe for Rail Mobile Radio (RMR), UIC

- ECC Decision(20)02,  Harmonised use of the paired frequency bands 874.4-880.0 MHz and 919.4-925.0 MHz and of the unpaired frequency band 1900-1910 MHz for Railway Mobile Radio (RMR), 20 November 2020

- ECC Report 313, Technical study for co-existence between RMR in the 900 MHz range and other applications in adjacent bands

- ECC Report 318, Compatibility between RMR and MFCN in the 900 MHz range, the 1900-1920 MHz band and the 2290-2300 MHz band