3GPP TSG-RAN WG4 Meeting #100e	 R4-2113785

# Electronic Meeting, August 16th – 27th, 2021

Title: 	Simulation results for PUSCH demodulation requirements for FR1 256QAM

Source: 	Huawei, HiSilicon

Agenda item:	9.12.3.1

Document for:	Discussion

Simulation assumptions

In RAN4 #99-e meeting, the WF [1] for PUSCH 256QAM was agreed. Following simulation results were agreed:

| Parameter | Value |
| --- | --- |
| Transform precoding | Disabled |
| CP | Normal CP |
| Number of Tx | 1 |
| Number of Rx | 2, 4, 8 |
| Number of layers | 1 |
| TDD pattern | 15kHz SCS: 3D1S1U, S=10D:2G:2U 30kHz SCS: 7D1S2U, S=6D:4G:4U |
| DM-RS sequence generation | NID=0, nSCID=0 |
| DMRS type | Type 1 with single-symbol DM-RS |
| Number of DMRS symbols | 1+1 |
| Number of DM-RS CDM groups without data | 2 |
| Ratio of PUSCH EPRE to DM-RS EPRE | -3dB |
| symbols length | 14 |
| start symbol index | 0 |
| Time domain resource allocation type | type A and B |
| Frequency domain resource | Full applicable test bandwidth |
| MCS index | [20, 21, 22, 24] |
| Carrier frequency (GHz) | 4 |
| Propagation condition | TDLA30-10 Low |
| SCS and BW | 15kHz: 5MHz, 10MHz, [20MHz]; 30kHz: 10MHz, [20MHz], 40MHz, [100MHz] |
| SRS | Not configured |
| PTRS | Not configured |
| Phase noise modelling | No explicit PN modelling |
| Tx EVM | Option 1: 3.5%; Option 1: 0 |
| Timing offset, Frequency offset | 0, 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled |
| Number of HARQ transmissions and RV sequence | 4, {0,2,3,1} |
| Testing metric | SNR @70% of maximum throughput |

In this paper, we share our views on those left open issues.

Simulation results

1T2R

| PUSCH mapping type | Propagation condition | CBW/SCS | MCS | Results w/o Tx EVM (dB) | Results w 3.5% Tx EVM (dB) | Difference |
| --- | --- | --- | --- | --- | --- | --- |
| PUSCH mapping type A | TDLA30-10 Low | 15kHz, 5MHz | MCS20 | 17.80 | 18.34 | 0.54 |
|  |  |  | MCS21 | 18.80 | 18.94 | 0.14 |
|  |  |  | MCS22 | 19.80 | 19.91 | 0.11 |
|  |  |  | MCS24 | 21.80 | 22.80 | 1 |
|  |  | 15kHz, 10MHz | MCS20 | 18.41 | 19.09 | 0.68 |
|  |  |  | MCS21 | 19.40 | 19.69 | 0.29 |
|  |  |  | MCS22 | 20.39 | 20.51 | 0.12 |
|  |  |  | MCS24 | 22.40 | 23.38 | 0.98 |
|  |  | 30kHz, 10MHz | MCS20 | 18.40 | 18.59 | 0.19 |
|  |  |  | MCS21 | 18.66 | 19.40 | 0.74 |
|  |  |  | MCS22 | 20.37 | 20.52 | 0.15 |
|  |  |  | MCS24 | 21.79 | 22.68 | 0.89 |
|  |  | 30kHz, 40MHz | MCS20 | 18.40 | 18.51 | 0.11 |
|  |  |  | MCS21 | 18.43 | 19.40 | 0.97 |
|  |  |  | MCS22 | 19.44 | 20.40 | 0.96 |
|  |  |  | MCS24 | 21.45 | 22.56 | 1.11 |

Observation 1: For 1T2R, larger than 20dB SNR results for MCS 24 and almost 1dB difference with and without explicitly modelling 3.5% Tx EVM in the simulation.

Observation 2: For 1T2R, almost 20dB SNR results for MCS 22, larger than 20dB results with additional impairment margin consideration.

Observation 3: For 1T2R, negligible difference between with and without explicit modelling 3.5% Tx EVM for MCS 20/21/22 cases except for MCS21/22 with 40MHz/30kHz SCS.

1T4R

| PUSCH mapping type | Propagation condition | CBW/SCS | MCS | Results w/o Tx EVM (dB) | Results w 3.5% Tx EVM (dB) | Difference between w and w/o |
| --- | --- | --- | --- | --- | --- | --- |
| PUSCH mapping type A | TDLA30-10 Low | 15kHz, 5MHz | MCS20 | 13.68 | 13.80 | 0.12 |
|  |  |  | MCS21 | 13.86 | 14.79 | 0.93 |
|  |  |  | MCS22 | 14.80 | 15.79 | 0.99 |
|  |  |  | MCS24 | 16.82 | 17.82 | 1 |
|  |  | 15kHz, 10MHz | MCS20 | 13.37 | 13.47 | 0.1 |
|  |  |  | MCS21 | 13.95 | 14.43 | 0.48 |
|  |  |  | MCS22 | 14.80 | 15.44 | 0.64 |
|  |  |  | MCS24 | 16.81 | 17.81 | 1 |
|  |  | 30kHz, 10MHz | MCS20 | 13.48 | 13.80 | 0.32 |
|  |  |  | MCS21 | 13.80 | 14.16 | 0.36 |
|  |  |  | MCS22 | 14.87 | 15.79 | 0.92 |
|  |  |  | MCS24 | 16.80 | 17.79 | 0.99 |
|  |  | 30kHz, 40MHz | MCS20 | 13.40 | 13.47 | 0.07 |
|  |  |  | MCS21 | 13.45 | 14.39 | 0.94 |
|  |  |  | MCS22 | 14.50 | 15.40 | 0.9 |
|  |  |  | MCS24 | 16.44 | 17.49 | 1.05 |

Observation 4: For 1T4R, SNR is below 20dB for all MCS cases.

Observation 5: For 1T4R, almost 1dB difference for MCS 22/24 with and without explicitly modelling 3.5% Tx EVM in the simulation except MCS22 with 10MHz/15kHz SCS.

Observation 6: For 1T4R, negligible difference between with and without explicit modelling 3.5% Tx EVM for MCS 20 cases.

1T8R

| PUSCH mapping type | Propagation condition | CBW/SCS | MCS | Results w/o Tx EVM (dB) | Results w 3.5% Tx EVM (dB) | Difference between w and w/o |
| --- | --- | --- | --- | --- | --- | --- |
| PUSCH mapping type A | TDLA30-10 Low | 15kHz, 5MHz | MCS20 | 10.80 | 10.81 | 0.01 |
|  |  |  | MCS21 | 11.75 | 11.80 | 0.05 |
|  |  |  | MCS22 | 11.96 | 12.80 | 0.84 |
|  |  |  | MCS24 | 13.88 | 14.83 | 0.95 |
|  |  | 15kHz, 10MHz | MCS20 | 10.40 | 10.63 | 0.23 |
|  |  |  | MCS21 | 11.01 | 11.49 | 0.48 |
|  |  |  | MCS22 | 11.80 | 12.48 | 0.68 |
|  |  |  | MCS24 | 13.80 | 14.80 | 1 |
|  |  | 30kHz, 10MHz | MCS20 | 10.80 | 10.80 | 0 |
|  |  |  | MCS21 | 10.80 | 11.62 | 0.82 |
|  |  |  | MCS22 | 11.86 | 12.80 | 0.94 |
|  |  |  | MCS24 | 13.80 | 14.79 | 0.99 |
|  |  | 30kHz, 40MHz | MCS20 | 10.40 | 10.42 | 0.02 |
|  |  |  | MCS21 | 10.42 | 11.39 | 0.97 |
|  |  |  | MCS22 | 11.56 | 12.40 | 0.84 |
|  |  |  | MCS24 | 13.44 | 14.44 | 1 |

Observation 7: For 1T8R, SNR is far below 20dB for all MCS cases.

Observation 8: For 1T8R, almost 1dB difference for MCS 22/24 with and without explicitly modelling 3.5% Tx EVM in the simulation.

Observation 9: For 1T8R, negligible difference between with and without explicit modelling 3.5% Tx EVM for MCS 20 cases.

Summary

In this paper, we provide our simulation results for PUSCH performance requirements for FR1 UL 256QAM, and our observations are summarized as below:

For 1T2R:

Observation 1: For 1T2R, larger than 20dB SNR results for MCS 24 and almost 1dB difference with and without explicitly modelling 3.5% Tx EVM in the simulation.

Observation 2: For 1T2R, almost 20dB SNR results for MCS 22, larger than 20dB results with additional impairment margin consideration.

Observation 3: For 1T2R, negligible difference between with and without explicit modelling 3.5% Tx EVM for MCS 20/21/22 cases except for MCS21/22 with 40MHz/30kHz SCS.

For 1T4R:

Observation 4: For 1T4R, SNR is below 20dB for all MCS cases.

Observation 5: For 1T4R, almost 1dB difference for MCS 22/24 with and without explicitly modelling 3.5% Tx EVM in the simulation except MCS22 with 10MHz/15kHz SCS.

Observation 6: For 1T4R, negligible difference between with and without explicit modelling 3.5% Tx EVM for MCS 20 cases.

For 1T8R:

Observation 7: For 1T8R, SNR is far below 20dB for all MCS cases.

Observation 8: For 1T8R, almost 1dB difference for MCS 22/24 with and without explicitly modelling 3.5% Tx EVM in the simulation.

Observation 9: For 1T8R, negligible difference between with and without explicit modelling 3.5% Tx EVM for MCS 20 cases.

Reference

[1] R4-2108667, Way forward for FR1 PUSCH 256QAM performance requirements, Huawei, HiSilicon, RAN4#99-e, August, 2021