3GPP TSG-RAN WG4 Meeting #100-e	R4-2113795

# Electronic Meeting, 16th - 27th Aug, 2021

Title: 	Discussion on channel modeling for NR FR2 HST

Source: 	Huawei, HiSilicon

Agenda item:	9.9.2.3

Document for:	Discussion

Background

During RAN#99-e meeting, Way Forward [1] on Channel modelling for FR2 HST was approved. In this contribution, we share our views about the channel modelling for NR FR2 HST.

Discussion

Channel model for Uni-directional RRH deployment

- For uni-directional deployment, one channel model (either toward to serving beam or away from serving beam) is applied for demodulation requirement even if UE can travel in two directions in practice. - Use the HST-DPS channel model below as a starting point for FR2 HST Uni-Directional RRH Deployment: - UE is moving towards serving beam - The cosine of angle θ(t) used in Doppler shift fst=fd cosθt is provided as - cosθt=Ds_offset +Ds-vtDmin2+Ds_offset+Ds-vt2, 0<t≤Dsv (eq. 1) - cosθt=cosθt modDsv, t>Ds/v , (eq. 2) - where 0 ≤Ds_offset<Ds (eq. 3) - The Ds_offset value for introducing performance requirements can be further discussed and decided based on typical values from Deployment scenarios analysis. - The value has no restriction on deployment. - Ds_offset value only used for demodulation requirements - FFS the starting point of t=0 by considering simulation efforts

For the channel model for Uni-directional RRH deployment, the main issue is the value of Ds_offset that has impact on Doppler and delay trajectory. Here we derived the Doppler and delay trajectory for both Options for Scenario A and Scenario B for Uni-directional RRH deployment.

Figure 2.1-1 Doppler and Delay trajectory for Uni-directional, Scenario A

Table 2.1-1 Maximum Doppler and Delay jump for Uni-directional, Scenario A

| Ds_offset[m] | 0 | 10 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Maximum Doppler jump[Hz] | 9627 | 2813 | 1017 | 495 | 288 | 187 | 131 | 97 | 74 | 59 | 47 |
| Maximum delay jump [us] | 2.3 | 2.32 | 2.33 | 2.33 | 2.33 | 2.33 | 2.33 | 2.33 | 2.33 | 2.33 | 2.33 |

Figure 2.1-2 Doppler and Delay trajectory for Uni-directional, Scenario B

Table 2.1-2 Maximum Doppler and Delay jump for Uni-directional, Scenario B

| Ds_offset[m] | 0 | 100 | 200 | 300 | 400 | 500 | 600 | 700 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Maximum Doppler jump[Hz] | 9500 | 4159 | 1811 | 918 | 530 | 335 | 226 | 160 |
| Maximum delay jump [us] | 1.89 | 2.11 | 2.21 | 2.25 | 2.28 | 2.29 | 2.3 | 2.31 |

We can see that the maximum Doppler and delay jump can be very different for different Ds_offset. For our understanding, the test purpose for the demodulation requirements under HST scenario is to verify the performance whether UE can adjust the large frequency offset and propagation delay, so the Ds_offset, Doppler and delay should be considered for the channel model for Uni-directional RRH deployment. For the detail value for Ds_offset, we can use the input from the deployment scenario discussion.

The maximum Doppler and delay jump can be very different for different Ds_offset.

The Ds_offset, Doppler and delay should be considered for the channel model for Uni-directional RRH deployment.

Using the following channel model for Uni-directional RRH deployment.

- The cosine of angle θt used in Doppler shift fst=fd cosθt and the relative delay Tt for the signal from the RRH is provided as

cosθt=Ds_offset +Ds-vtDmin2+Ds_offset+Ds-vt2,  0<t≤Dsv    (eq. 1)

cosθt=cosθt modDsv,  t>Ds/v  ,   (eq. 2)

Tt=Dmin2+Ds_offset+Ds-vt2,  0<t≤Dsv    (eq. 3)

Tt=Tt modDsv,  t>Ds/v ,    (eq. 4)

- Where

0≤Ds_offset<Ds   (eq. 5)

For the detail value for Ds_offset, use the input from the deployment scenario discussion.

Channel model for Bi-directional RRH deployment

- GTW agreement: Option 2 with DPS based channel model as starting point. - Option 2(a): To match Bi-directional deployment Scheme-1: UE connect to 2nd-nearest RRH). - cosθt=Ds-vtDmin2+Ds-vt2, 0<t≤(0.5*Ds)/v - cosθt=-vtDmin2+vt2, (0.5*Ds)/v<t≤Ds/v - cosθt modDsv, t>Ds/v - Option 2(b): based on Scheme-2 for Bidirectional RRH Deployment: - cosθt=-（Ds+vt）Dmin2+Ds+vt2, 0≤t<Ds_offsetv, - cosθt=-vtDmin2+-vt2, Ds_offsetv≤t<0.5*Dsv, - cosθt=Ds-vtDmin2+Ds-vt2, 0.5*Dsv≤t<(Ds-Ds_offset)v - cosθt=-vtDmin2+-vt2, (Ds-Ds_offset)v≤t<Dsv - cosθt= cosθt mod(Dsv), t>Dsv. - Option 2(c): based on Scheme-3 for Bidirectional RRH Deployment: - cosθt=Ds-vtDmin2+Ds-vt2, 0<t≤Aoffset/v - cosθt=-vtDmin2+vt2, Aoffset/v<t≤(0.5*Ds)/v - cosθt=Ds-vtDmin2+Ds-vt2, (0.5*Ds)/v <t≤Boffset/v - cosθt=-vtDmin2+vt2, Boffset/v<t≤(Ds)/v - cosθt modDsv, t>Ds/v - Aoffset<(0.5*Ds)<Boffset <Ds - Option 2(e): based on DPS scheme in which UE switch serving RRH at Ds/2, and no beam configured from neighboring RRH for coverage hole around RRH site - cosθt=Ds2-vtDmin2+Ds2-vt2, 0≤t≤Dsv, - cosθt= cosθt mod(Dsv), t>Dsv.

Here we derived the Doppler and delay trajectory for both Options for Scenario A and Scenario B for Bi-directional RRH deployment, using the typical value of Aoffset, Boffset and Ds_offset as per our contribution [2] and [3].

Figure 2.2-1 Doppler and Delay trajectory for Bi-directional, Scenario A

Table 2.2-1 Maximum Doppler and Delay jump for Bi-directional, Scenario A

| Ds_offset[m] | Scheme 1 | Scheme 2 | Scheme 3 |
| --- | --- | --- | --- |
| Maximum Doppler jump[Hz] | 19437 | 19437 | 19437 |
| Maximum delay jump [us] | 0.00 | 2.33 | 2.00 |

Figure 2.2-2 Doppler and Delay trajectory for Bi-directional, Scenario B

Table 2.2-2 Maximum Doppler and Delay jump for Bi-directional, Scenario B

| Ds_offset[m] | Scheme 1 | Scheme 2 | Scheme 3 |
| --- | --- | --- | --- |
| Maximum Doppler jump[Hz] | 17872 | 17872 | 17872 |
| Maximum delay jump [us] | 0.00 | 2.22 | 0.73 |

Same as discussion for Uni-directional RRH deployment, the switching point, Doppler and delay should be considered for the channel model for Bi-directional RRH deployment. As per our contribution [2] and [3], Scheme 3 is prefered considering less delay jump comparing to other schemes.

The switching point, Doppler and delay should be considered for the channel model for Bi-directional RRH deployment.

Using the following channel model for Bi-directional RRH deployment.

- The cosine of angle θt used in Doppler shift fst=fd cosθt and the relative delay Tt for the signal from the RRH is provided as

cosθt=Ds-vtDmin2+Ds-vt2,  0≤t<230/v, 350/v≤t<470/v

cosθt=-vtDmin2+vt2,  230/v≤t<350/v, 470/v≤t<Ds/v

cosθt modDsv,    t≥Ds/v

T(t)=Dmin2+Ds-vt2,  0≤t<230/v, 350/v≤t<470/v

T(t)=Dmin2+vt2,  230/v≤t<350/v, 470/v≤t<Ds/v

Tt=Tt modDsv,  t>Ds/v ,    (eq. 4)

Proposals

In this contribution, we discuss on channel modelling for NR FR2 HST. Our observations and proposals are:

- The maximum Doppler and delay jump can be very different for different Ds_offset.

- The Ds_offset, Doppler and delay should be considered for the channel model for Uni-directional RRH deployment.

Using the following channel model for Uni-directional RRH deployment.

- The cosine of angle θt used in Doppler shift fst=fd cosθt and the relative delay Tt for the signal from the RRH is provided as

cosθt=Ds_offset +Ds-vtDmin2+Ds_offset+Ds-vt2,  0<t≤Dsv    (eq. 1)

cosθt=cosθt modDsv,  t>Ds/v  ,   (eq. 2)

Tt=Dmin2+Ds_offset+Ds-vt2,  0<t≤Dsv    (eq. 3)

Tt=Tt modDsv,  t>Ds/v ,    (eq. 4)

- Where

0≤Ds_offset<Ds   (eq. 5)

For the detail value for Ds_offset, use the input from the deployment scenario discussion.

The switching point, Doppler and delay should be considered for the channel model for Bi-directional RRH deployment.

Using the following channel model for Bi-directional RRH deployment.

- The cosine of angle θt used in Doppler shift fst=fd cosθt and the relative delay Tt for the signal from the RRH is provided as

cosθt=Ds-vtDmin2+Ds-vt2,  0≤t<230/v, 350/v≤t<470/v

cosθt=-vtDmin2+vt2,  230/v≤t<350/v, 470/v≤t<Ds/v

cosθt modDsv,    t≥Ds/v

T(t)=Dmin2+Ds-vt2,  0≤t<230/v, 350/v≤t<470/v

T(t)=Dmin2+vt2,  230/v≤t<350/v, 470/v≤t<Ds/v

Tt=Tt modDsv,  t>Ds/v ,    (eq. 4)

Reference

R4-2108661, Way Forward on Channel Modelling for FR2 HST, RAN4#99-e, Nokia, Nokia Shanghai Bell

R4-2113793, Discussion on NR FR2 HST deployment Scenario-A, RAN4#100-e, Huawei, HiSilicon

R4-2113794, Discussion on NR FR2 HST deployment Scenario-B, RAN4#100-e, Huawei, HiSilicon