3GPP TSG-RAN WG4 Meeting # 100-e	R4-2113866

Electronic Meeting, August 16-27, 2021

Source:	ZTE Corporation

Title:	On complexity reduction for RedCap UEs

Agenda item:	9.20.3.2

Document for:	Discussion

# 1	Introduction

In RAN4 99-e, there were discussions on the RRM impacts by the new feature of reduced capability UEs. Some of the WF [1] are copied below.

- This paper discusses the issues listed above and also provide our view on the other related issues.

2	Discussions

- Reduced capability UEs are user devices which have less antenna ports and also support less MIMO layers. Such UEs are anticipated to be of more affordable prices and can be used as sensors, IoT devices and wearables. One of the intention to introduce such devices is to reduce the UE capability in order to prolong the standby time with a small battery.

As can be imagined, reduced UE capability on the Tx / Rx branches and MIMO layers would have impact on some of the RRM impacts. During the last meeting, the impact for RLM , LR, BWP switch are marked as FFS.

In TR 38.875 [2], there is a dedicated chapter for UE complexity reduction. The reduction on UE complexity has many impact on RRM requirements, for instance the number of Rx branches is decreased, the UE processing capability is also decreased, resulting in a longer processing time and so on. Specifically, it is mentioned in the TR that the value of N1 and N2 are relaxed:

7.5.5 Analysis of specification impacts A new UE processing time capability needs to be defined if relaxed UE processing time in terms of N1 and N2 is introduced. New values of N1 and N2, as well as how the PDSCH processing time and PUSCH preparation time are determined by N1 and N2, need to be defined. Depending on the degree of relaxation of the N1 and N2 values, specification details on scheduling timing related to the default TDRA tables and HARQ-ACK timing range may also need to be updated.

whereas N1 and N2 are defined in 38.214 [3]:

5.3 UE PDSCH processing procedure time If the first uplink symbol of the PUCCH which carries the HARQ-ACK information, as defined by the assigned HARQ-ACK timing K1 and the PUCCH resource to be used and including the effect of the timing advance, starts no earlier than at symbol L1, where L1 is defined as the next uplink symbol with its CP starting after after the end of the last symbol of the PDSCH carrying the TB being acknowledged, then the UE shall provide a valid HARQ-ACK message. - N1 is based on µ of table 5.3-1 and table 5.3-2 for UE processing capability 1 and 2 respectively, where µ corresponds to the one of (µPDCCH, µPDSCH, µUL) resulting with the largest Tproc,1, where the µPDCCH corresponds to the subcarrier spacing of the PDCCH scheduling the PDSCH, the µPDSCH corresponds to the subcarrier spacing of the scheduled PDSCH, and µUL corresponds to the subcarrier spacing of the uplink channel with which the HARQ-ACK is to be transmitted, and κ is defined in subclause 4.1 of [4, TS 38.211].

From the above specifications, it’s clear that RRM requirements related to PDSCH processing time, PUSCH preparation time and HARQ-ACK transmission delay will be impacted.

In the study, for the purpose of evaluation, relaxed CSI computation time was also considered, assuming doubled Z and Z' compared to the values defined in TS 38.214 clause 5.4.

From the above content in the TR, it’s proposed that requirements related to CSI report shall also be re-visited.

RRM requirements related to PDSCH processing time, PUSCH preparation time, HARQ-ACK transmission delay and CSI reporting shall be modified.

For some RRM requirements such as initial access, the allocated resources for the UE may depend on whether the network identifies the UE type (RedCap or legacy UEs). Thus, for some RRM requirements for instance the random access, there may need to be at least two sets of requirements for RedCap UEs depending on whether early indication of the UR type is supported / successful.

For some RRM requirements for instance the random access, there may need to be at least two sets of requirements for RedCap UEs depending on whether early indication of the UR type is supported / successful.

The other aspect of UE capacity reduction is the interruption caused by RF retuning. The current interruption requirements defined in TS 38.133 [3] cannot be directly re-used for RedCap UEs due to the difference in minimum UE capability. This, it’s proposed to also re-visit interruption requirements for RedCap UEs.

RRM requirements related to interruptions shall be modified for redCap UEs.

3	Conclusion

Proposal 1: RRM requirements related to PDSCH processing time, PUSCH preparation time, HARQ-ACK transmission delay and CSI reporting shall be modified.

Proposal 2: For some RRM requirements for instance the random access, there may need to be at least two sets of requirements for RedCap UEs depending on whether early indication of the UR type is supported / successful.

Proposal 3: RRM requirements related to interruptions shall be modified for redCap UEs

References

- R4-2108359

- TR 38.875

- TS 38.133