3GPP TSG-RAN WG4 Meeting # 100-e 	R4-2113947

Electronic Meeting, August 16th-27th, 2021

Agenda Item: 9.20.3.1

Source: MediaTek Inc.

Title: General and RRM requirements impacts

Document for: Discussion and decision

# Introduction

In this contribution paper, we discuss the general and RRM requirement impacts for RedCap. The WID has the following objectives [1]:

- Specify support for the following UE complexity reduction features [RAN1, RAN2, RAN4]: - Reduced maximum UE bandwidth: - Maximum bandwidth of an FR1 RedCap UE during and after initial access is 20 MHz. - Maximum bandwidth of an FR2 RedCap UE during and after initial access is 100 MHz. - Reduced minimum number of Rx branches: - For frequency bands where a legacy NR UE is required to be equipped with a minimum of 2 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands. - For frequency bands where a legacy NR UE (other than 2-Rx vehicular UE) is required to be equipped with a minimum of 4 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands. - A means shall be specified by which the gNB can know the number of Rx branches of the UE. - Maximum number of DL MIMO layers: - For a RedCap UE with 1 Rx branch, 1 DL MIMO layer is supported. - For a RedCap UE with 2 Rx branches, 2 DL MIMO layers are supported. - Relaxed maximum modulation order: - Support of 256QAM in DL is optional (instead of mandatory) for an FR1 RedCap UE. - No other relaxations of maximum modulation order are specified for a RedCap UE. - Duplex operation: - HD-FDD type A with the minimum specification impact (Note that FD-FDD and TDD are also supported.)

Also, from the way forward of the previous meeting [2]:

- Inter-frequency mobility - FFS whether to define inter-frequency RRM requirements for RedCap UE in Rel-17. - Inter-RAT mobility - Option 1: - Do not define inter-RAT RRM requirements for RedCap UE in Rel-17. - Option 2: - Do not define inter-RAT RRM requirements on 2G/3G for RedCap UE in Rel-17. - FFS: whether define inter-RAT RRM requirements on LTE for RedCap UE in Rel-17.

# Discussion on RRM impacts

This section provides discussion on aspects related to the UE complexity reduction features in 5G NR RedCap devices.

# Reduced maximum UE bandwidth

Reducing the bandwidth (BW) down to 20 MHz for FR1 and 100 MHz for FR2 may have impact on CSI-RS measurement requirements, e.g. CSI-RS based RLM, CSI-RS based L1-RSRP, and L3 CSI-RS, where BW of 48 PRBs was assumed in some requirements. For example, for CSI-RS based RLM, the BW assumed in PDCCH transmission parameters is 48 PRBs, which is higher than the BW limit for RedCap devices. Therefore, either the bandwidth assumed in PDCCH transmission parameters for CSI-RS based RLM should be reduced or SCSs = 60 KHz should not be in the requirements for RedCap.

- The measurement bandwidth for SSB based RLM is lower than the maximum bandwidth agreed for RedCap UE in FR1 and FR2, hence no changes are needed.

- The maximum bandwidth for CSI-RS based RLM is higher than the maximum bandwidth agreed for RedCap UE in FR1. To address the issue, either reducing the maximum BW for CSI-RS based RLM or exclude the SCSs 60 KHz from RedCap UEs requirements.

- Support further study the bandwidth parameters for CSI-RS based RLM to accommodate the reduced maximum bandwidth in RedCap for FR1.

# Reduced minimum number of Rx branches

Reducing the minimum number of Rx branches has potential impact on the test setup (performance) requirements, as well as PDCCH transmission parameters in RLM core requirements. As we agreed in the previous meeting (RAN4#99-e meeting) to delay the discussion on performance requirements, hence in this section we will discuss only the impact on core requirements.

The existing requirements for hypothetical PDCCH parameters of 5G NR RLM are specified with the minimum number of antennas equal to two. Besides, reducing the number of antenna to one antenna can result in reducing the received power down to half compared to using two antennas. Therefore, the hypothetical PDCCH parameters shall be extended to cover the case of using a single antenna with a 3 dB higher compared to the case of using two antennas.

- For RLM in RedCap, the PDCCH requirements for ratio of hypothetical PDCCH RE energy to average SSS/CSI-RS RE energy for out-of-sync and in-sync are for minimum of two antennas.

- Support modification on PDCCH transmission parameters for RLM requirements with the single antenna port in RedCap devices, including the ratio of hypothetical PDCCH RE energy to average SSS/CSI-RS RE energy for out-of-sync and in-sync with single antenna port to be with [3] dB higher compared to the existing two antenna requirements.

# Duplex operation

The duplex operation could have impact on the following agendas:

# Evaluation period for RLM measurements

The configuration of the evaluation period requirements in 5G NR, such as in Table 8.1.2.2-1 in [3], is based on the FD-FDD and the TDD duplex modes, however, in RedCap devices the HD-FDD type A is introduced and hence this may have impact on the existing requirements. Looking back at LTE Cat-M1 RLM evaluation period requirements for HD-FDD, we can recall a condition that: ‘at least 1 DL subframe per radio frame of PCell is available at the UE during Qin_CatM1 and Qout_CatM1 evaluation periods’ [4]. Now, in 5G NR specification in Section 5 in [5], the physical layer in the UE assesses once per indication period, where the UE determines the indication period as the maximum between the shortest periodicity for radio link monitoring resources and 10 msec. Therefore, the condition in 5G NR in RedCap devices for HD-FDD should be: ‘At least 1 RLM-RS must fall with DL occasion within an indication period. The UE determines the indication period as the maximum between the shortest periodicity for radio link monitoring resources and 10 msec’.

- In LTE RLM requirements, there is a condition for the evaluation period of HD-FDD to ensure that at least one DL subframe per radio frame is available during the evaluation period.

- Support adding the following condition for the evaluation period of 5G NR RLM requirement for HD-FDD: At least 1 RLM-RS must fall with DL occasion within an indication period. The UE determines the indication period as the maximum between the shortest periodicity for radio link monitoring resources and 10 msec.

Furthermore, the lower bound of the evaluation period for RLM requirements was increased at lease by two in LTE Cat-M RLM requirements compared to the general LTE RLM requirements [4]. Given that the LTE Cat-M is for services that costs less, long battery life and cheaper devices, which are similar motivation for using the RedCap devices. Therefore, the lower bound in the evaluation period of the 5G NR RedCap RLM requirements shall be doubled.

- For the evaluation period of LTE Cat-M RLM requirements the lower bound was doubled compared to the evaluation period of the general RLM requirements in LTE.

- Support extending the lower bound of the evaluation period of 5G NR RedCap RLM requirement by two compared to the existing general 5G NR RLM requirements.

# Scheduling availability

In the existing 5G NR requirement for scheduling availability there are scheduling restriction for a UE performing measurement in TDD bands for FR1 [3]. Given that the RedCap devices operates in HD-FDD type A and that the time domain scheduling for HD-FDD bands is similar to that of TDD bands, therefore, scheduling restriction shall be introduced for the HD-FDD bands in RedCap.

- In 5G NR scheduling availability requirements there are scheduling restriction for a UE performing measurement in TDD bands.

- Support introducing scheduling availability restriction on 5G NR RedCap UEs performing measurements in HD-FDD bands.

# Discussion on inter-frequency and inter-RAT

This section provides discussion on the inter-frequency and the inter-RAT features for 5G NR RedCap.

# Inter-frequency

Wearable use case is one of the use cases for RedCap, which means the UE is expected to have mobility. Now, consider a scenario where two neighbouring cities are using two different carrier frequencies in their networks and assume a UE is moving from one city toward the other, and hence the UE shall utilise the inter-frequency feature to switch its carrier. Therefore, inter-frequency shall be considered with equal priority to intra-frequency.

- Support inter-frequency mobility for 5G NR RedCap devices in Rel-17.

# Inter-RAT

The current 5G NR network coverage is not very good compared to the LTE network. Also, given that the wearable use case is one of the use cases in RedCap devices, hence the LTE network can be utilized to guarantee the RedCap devices with a good connection especially during UE mobility. Nevertheless, this could increase the workload in RAN4, hence we suggest to prioritize defining the LTE inter-RAT mobility requirements in RedCap for IDLE mode, after that, we can examine the LTE inter-RAT mobility CONNECTED mode.

- Support LTE inter-RAT mobility for 5G NR RedCap devices in Rel-17 at least in IDLE mode. FFS CONNECTED mode.

# Summary

In this contribution, discussion on general and RRM requirement in RedCap UEs are provided and we have the following observations:

Observation 1: The measurement bandwidth for SSB based RLM is lower than the maximum bandwidth agreed for RedCap UE in FR1 and FR2, hence no changes are needed.

Observation 2: The maximum bandwidth for CSI-RS based RLM is higher than the maximum bandwidth agreed for RedCap UE in FR1. To address the issue, either reducing the maximum BW for CSI-RS based RLM or exclude the SCSs 60 KHz from RedCap UEs requirements.

Observation 3: For RLM in RedCap, the PDCCH requirements for ratio of hypothetical PDCCH RE energy to average SSS/CSI-RS RE energy for out-of-sync and in-sync are for minimum of two antennas.

Observation 4: In LTE RLM requirements, there is a condition for the evaluation period of HD-FDD to ensure that at least one DL subframe per radio frame is available during the evaluation period.

Observation 5: For the evaluation period of LTE Cat-M RLM requirements the lower bound was doubled compared to the evaluation period of the general RLM requirements in LTE.

Observation 6: In 5G NR scheduling availability requirements there are scheduling restriction for a UE performing measurement in TDD bands.

Also, we have the following proposals:

Proposal 1: Support further study the bandwidth parameters for CSI-RS based RLM to accommodate the reduced maximum bandwidth in RedCap for FR1.

Proposal 2: Support modification on PDCCH transmission parameters for RLM requirements with the single antenna port in RedCap devices, including the ratio of hypothetical PDCCH RE energy to average SSS/CSI-RS RE energy for out-of-sync and in-sync with single antenna port to be with [3] dB higher compared to the existing two antenna requirements.

Proposal 3: Support adding the following condition for the evaluation period of 5G NR RLM requirement for HD-FDD: At least 1 RLM-RS must fall with DL occasion within an indication period. The UE determines the indication period as the maximum between the shortest periodicity for radio link monitoring resources and 10 msec.

Proposal 4: Support extending the lower bound of the evaluation period of 5G NR RedCap RLM requirement by two compared to the existing general 5G NR RLM requirements.

Proposal 5: Support introducing scheduling availability restriction on 5G NR RedCap UEs performing measurements in HD-FDD bands.

Proposal 6: Support inter-frequency mobility for 5G NR RedCap devices in Rel-17.

Proposal 7: Support LTE inter-RAT mobility for 5G NR RedCap devices in Rel-17 at least in IDLE mode. FFS CONNECTED mode.

# References

- RP-211574, “Revised WID on support of reduced capability NR devices”.

- R4-2108359, “WF on RedCap RRM requirements”.

- 3GPP TS 38.133, “NR; Requirements for support of radio resource management (Release 17)”, V17.2.0.

- 3GPP TS 36.133, “(E-UTRA); Requirements for support of radio resource management (Rel-16)”, V16.9.0.

- 3GPP TS 38.213, “NR; Physical layer procedures for control (Release 16)” V16.1.0.