3GPP TSG-RAN WG4 Meeting #100-e                         	         R4-2114026

# Electronic meeting, 16th August – 27th August, 2021

Agenda item:	9.2.2

Source:	Huawei, HiSilicon

Title:	on FR1 SA TRP and TRS test methodology

Document for:	Discussion

# Introduction

In RAN#92e meeting, the new R17 work item for NR FR1 UE TRP and TRS was revised and agreed [1]. WF [2] was also agreed in RAN4 #99e. This contribution discusses aspects related to SA test methodology.

# Discussion

Section 6.2 of TS 38.521-1 [3] has developed configurations and parameters for maximum power tests, while section 7.3 of TS 38.521-1 [3] contains receive sensitivity test configurations. The configurations in both section 6.2 and 7.3 could be adopted for OTA tests, with the exception to limit the TRP tests to QPSK modulation only because BPSK was also defined in section 6.2 of TS 38.521-1.

Proposal 1: reuse the configurations defined in section 6.2 and 7.3 of TS 38.521-1, but only test TRP and TRS under QPSK modulation.

The scope of the work item [1] does not include CA for SA. This means SA TRP tests should be limited to single carrier only.

Proposal 2: SA TRP tests should be limited to single carrier only.

To save measurement time, multiple probes could be used for TRP measurement. The radiation patterns of sub-6GHz antennas typically cover 360 degrees because the aim is to achieve omni directional patterns. In other words, it may not be necessary to optimize the probe positions, but rather to follow simple arrangement such as a line, which could represent a plane cut in  in rotational measurement systems. Multiple probes arranged over an area can also be used. However, rotational movement and interpolation among measurement points need to be carefully considered.

Similar multiple probe approach could also be used for TRS measurement with probes transmitting in sequence. The time saving in this case would come from the removal of positioning movement required in single probe systems.

Proposal 3: Multiple probe approach could be considered for measurement time reduction.

# Conclusions

This contribution presents the following proposals for SA TRP and TRS tests.

Proposal 1: Reuse the configurations defined in section 6.2 and 7.3 of TS 38.521-1, but only test TRP and TRS under QPSK modulation.

Proposal 2: SA TRP tests should be limited to single carrier only.

Proposal 3: Multiple probe approach could be considered for measurement time reduction

# References

- RP-211158, New WID: Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC), RAN#92-e, June 2021

- R4-2108620, WF on FR1 TRP TRS, vivo, TSG-RAN WG4 Meeting #99e, May 2021

- TS 38.521-1 v16.5.0