3GPP TSG-RAN WG4 #100-e	R4-2114041

Electronic Meeting, August 16-27, 2021

Agenda item:	9.12.2.3

Source:	MediaTek Inc.

Title:	Discussion on PDSCH requirements for CRS-IM

Document for:	Endorsement

- Introduction

In RAN4#99-e, a way forward for CRS interference handling was agreed [1]. Based on the agreed simulation assumptions, we provide simulation results and our views in this contribution.

- Discussion

Simulation results based on the agreed assumptions

For the topic of CRS interference handling, RAN4 agreed to evaluate the performance for both scenarios in Figure 1.

| Scenario 1: Serving and interference cells are both operating in DSS (NR+LTE) mode | Scenario 2: Serving cell is operating in NR mode and interference cell is operating in LTE mode |
| --- | --- |
| Figure 1. Scenarios for CRS interference in overlapping spectrum for LTE and NR |  |

Based on simulation assumptions and cases for evaluation [2], we provide simulation results to compare the performance of different CRS interference handling schemes, including interference cancellation (IC), rate matching (RM) and LLR weighting. It is noted that our results are based on S = 3, L = 11 for all CRS interference handling schemes, which is a more fair comparison for the performance of RM and IC.

| Figure 2. Simulation results based on the assumptions in [2], but with S = 3, L = 11. |  |
| --- | --- |

Observation 1: Based on the simulation results with agreed simulation assumptions, the performance of IC outperforms that of RM.

Simulation results for UE located near cell center and at the sector edge

Except the simulation assumptions adopted in [2], we also provide some possible scenarios for evaluation. Considering UE located near the cell center and at the sector edge, UE will experience high INR and also high SNR. Some simulations results are provided based on Table 1 below.

Table 1. Simulation assumptions for UE located near the cell center and at the sector edge

| Case Number | Scenario | MIMO | MCS index | CBW/ SCS | Duplex Mode | Interference Cell Number and Interference Power | PDSCH loading level | PDSCH Start Symbol and Symbol Length |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | NR+LTE | 4x4 Low | 16QAM MCS13 (1 layer) | 10MHz/15kHz | FDD synchronized network | 1 interference cell with INR = 20 dB | 20% | S = 3, L = 11 |
| B |  |  | 256QAM MCS21 (1 layer) |  |  |  |  |  |
| C |  |  | 256QAM MCS23 (2 layer) |  |  | 1 interference cells with INR = 25 dB |  |  |
| D |  |  | 256QAM MCS25 (4 layer) |  |  |  |  |  |

| Figure 2. Simulation results based on the assumptions in Table 1. |  |
| --- | --- |

For these four cases in Table 1, it can be shown that the throughput of R16-RM is higher than that of IC. Hence, we think RAN4 should consider more scenarios for the performance evaluation of different CRS interference handling schemes.

Observation 2: Considering the scenario of UE located near the cell center and at the sector edge, the performance of RM is better than that of IC.

Observation 3: From the results of Figure 1 and Figure 2, the performance of IC is not always outperform RM.

Proposal 1: RAN4 needs to consider more scenarios for the performance evaluation of different CRS interference handling schemes.

Also, we would like to mention that CRS-IC receiver is a successive processing of detection, decoding, re-encoding and cancellation. It reconstructs CRS interference signal and further cancels the interference from the received signal, followed by the demodulation and decoding of the desired data. Compared with CRS rate matching, it will increase the receiver complexity with the interference detection, interference estimation and interference cancellation. This may increase the UE processing time and may make it difficult to fulfill the requirement for response timing between DL data and DL HARQ. However, for RM schemes, there is no any further increase for the complexity of UE, where UE just needs to know the rate matching pattern for reception.

Observation 4: The receiver complexity of CRS-IC receiver is much higher than that of RM.

Proposal 2: Send LS to request RAN1 for evaluating the UE processing timeline due to CRS-IC.

- Conclusion

The observations and proposals are summarized as below:

Observation 1: Based on the previous agreed assumptions, the performance of IC outperforms that of RM.

Observation 2: Considering the scenario of UE located near the cell center and at the sector edge, the performance of RM is better than that of IC.

Observation 3: From the results of Figure 1 and Figure 2, the performance of IC is not always outperform RM.

Proposal 1: RAN4 needs to consider more scenarios for the performance evaluation of different CRS interference handling schemes.

Observation 4: The receiver complexity of CRS-IC receiver is much higher than that of RM.

Proposal 2: Send LS to request RAN1 for evaluating the UE processing timeline due to CRS-IC.

- Reference

- R4-2108662, “WF on CRS interference handling in scenarios with overlapping spectrum for LTE and NR”, China Telecom

- R4-2108663, “Template for simulation result collection for CRS interference handling”, China Telecom