3GPP TSG-RAN4 Meeting #100-e	R4-2114136

# Electronic Meeting, 16th May – 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | Draft | rev | - | Current version: | 15.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of RMC for NB TDD RLM test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI_15, NB_IOTenh2 |  |  |  |  |  | Date: |  |  | 2021-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The antenna configuration is 2x1 in the NB-TDD RLM test cases. However the RMC R.14, R.18 and R.22 used in the test cases are defined for 1x1. Correction is needed to align the antenna configuration and RMC. In test case A.7.3.93, the required rate of correct events is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use 2x1 RMCs in the test cases to align with the antenna configuration. Adding the required rate of correct events as 90% in A.7.3.93. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test Case configuration would be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7.3.88 – A.7.3.95 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

A.7.3.88	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage

A.7.3.88.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD category NB1 UE properly detects the out of sync for the purpose of monitoring downlink radio link quality of the NB-IoT Cell. This test will partly verify the NB-IoT TDD radio link monitoring requirements in clause 7.23.

The test parameters are given in Tables A.7.3.88.1-1, A.7.3.88.1-2, A.7.3.88.1-2A, A.7.3.88.1-3 and A.7.3.88.1-4. nCell 1 is the active NB-IoT cell in the test. The test consists of four successive time periods with time duration of T1, T2, T3 and T4 respectively, excluding the transition time duration dT, where the SNR increases or decreases gradually in small steps. Figure A.7.3.88.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync state with the following testing procedure:

-	Prior to the start of the time duration T1, the UE shall be fully synchronized to nCell1

-	Starting at point A, the SNR is decreased in small steps from SNR1 to SNR2 within dT

-	At the start of the time duration T2, the UE is provided with a UL grant with NPDCCH

Note:	The UE is expected to decode the NPDCCH and complete the UL transmission during T2 according to the UL grant.

-	Starting at point B, the SNR is decreased in small steps from SNR2 to SNR3 within dT

-	During T3, the SNR is kept as SNR3

Note:	The UE is expected to detect OOS and declare RLF during T3.

-	Starting at point C, the SNR is increased in small steps from SNR3 to SNR1 with dT

-	At the start of the time period T4, the UE will be provided with another UL grant with NPDCCH

Note:	The UE is not expected to decode the UL grant and conduct any UL transmission during T4, since the UE is expected to declare RLF during T3.

In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode the NPDCCH and complete the UL transmission when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.7.3.88.1-1: General test parameters for TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | nCell 1 |  |
| Neighbour cell |  | eCell 1 |  |
| CP length |  | Normal |  |
| Deployment Mode |  | In-band |  |
| NPDCCH transmission parameters Rmax |  | 8 | Other NPDCCH parameters are defined in “ out-of-sync” column in Table 7.23.2-1 |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211 [16] |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211 [16] |
| DRX cycle | ms | 256 | See Table A.7.3.88.1-3 |
| Layer 3 filtering Note 2,3 |  | Enabled | Counters: N310 = 1 N311 = 1 |
| T310 timer Note 2,3 | ms | 0 | T310 is disabled |
| T311 timer Note 2,3 | ms | 1000 | T311 is enabled |
| T1 | s | 5.12 |  |
| dT | S | 0.8 |  |
| T2 | s | 10.24 |  |
| dT | S | 0.7 |  |
| T3 | s | 5.12 |  |
| dT | S | 1.4 |  |
| T4 | s | 5.12 |  |
| Note 1: NPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: N310, N311, T310 and T311 are defined in TS 36.331 [2]. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. |  |  |  |

Table A.7.3.88.1-2: nCell specific test parameters for TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage

| Parameter | Unit | nCell 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | dT | T2 | dT | T3 | dT | T4 |
| BWchannel | kHz | 180 |  |  |  |  |  |  |
| PRB location within eCell |  | eCell 1 BWchannel 10MHz: 30 |  |  |  |  |  |  |
| NPDCCH parameters as defined in A.3.1.5.7 |  | R.15 NB-TDD |  |  |  |  |  |  |
| NPBCH_RA | dB | -3 |  |  |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |
|  | dBm/15 kHz | Specified in Table A.7.3.88.1-2A |  |  |  |  |  |  |
| SNR Note 4, 5 | dB | -3.1 | Note 6 | -9.1 | Note 6 | -14.1 | Note 6 | -3.1 |
| Propagation condition |  | AWGN |  |  |  |  |  |  |
| Configuration |  | 2x1 |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: Void Note 3: Void Note 4: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 5: The SNRs in time periods T1, T2, T3 and T4 are denoted as SNR1, SNR2, SNR3 and SNR1 respectively in figure A.7.3.88.1-1. Note 6: The Test system shall reduce its transmit power in steps of ((SNR2-SNR1) / (10*dT)) dB every 100ms until SNR2 is achieved at the end of dT. Note 7: The Test system shall reduce its transmit power in steps of ((SNR3-SNR2) / (10*dT)) dB every 100ms until SNR3 is achieved at the end of dT. Note 8: The Test system shall increase its transmit power in steps of ((SNR1-SNR3) / (10*dT)) dB every 100ms until SNR1 is achieved at the end of dT. |  |  |  |  |  |  |  |  |

Table A.7.3.88.1-2A: eCell 1 specific test parameters for TDD Out-of-sync radio link monitoring test in DRX for UE category NB1 In-band mode in normal coverage

| Parameter | Unit | eCell 1 |
| --- | --- | --- |
|  |  | T1-T4 |
| BWchannel | MHz | 10 |
| NOCNG Pattern | - | BWchannel 10MHz: NOP.1 TDD |
| PBCH_RA | dB | -3 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| OCNG_RANote 1 | dB |  |
| OCNG_RBNote 1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
|  | dB | -12 |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 2x1 |
| Note 1: OCNG shall be used such that the eCell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |

Table A.7.3.88.1-3: DRX-Configuration for TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | pp1 | As specified in clause 6.7.3 in TS 36.331 [2] |
| drx-InactivityTimer | pp0 |  |
| drx-RetransmissionTimer | pp0 |  |
| drx-StartOffset | 0 |  |

Table A.7.3.88.1-4: TimeAlignmentTimer -Configuration for NB-IoT TDD out-of-sync testing for UE category NB1 In-band mode in normal coverage

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 [2] |

Figure A.7.3.88.1-1: SNR variation for out-of-sync testing in DRX for NB-IoT TDD out-of-sync testing for UE category NB1 In-band mode in normal coverage

A.7.3.88.2	Test Requirements

The UE behaviors in each test shall be as follows:

-	The UE shall complete the NPUSCH transmission during T2 according to the received UL grant;

-	The UE shall not conduct any NPUSCH transmission during T4

A correct event is defined as UE behaves correctly in all above steps. The correct events observed during repeated tests shall be at least 90%.

A.7.3.89	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage

A.7.3.89.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD category NB1 UE properly detects the out of sync for the purpose of monitoring downlink radio link quality of the NB-IoT Cell. This test will partly verify the NB-IoT TDD radio link monitoring requirements in clause 7.23.

The test parameters are given in Tables A.7.3.89.1-1, A.7.3.89.1-2, A.7.3.89.1-2A, A.7.3.89.1-3 and A.7.3.89.1-4. nCell 1 is the active NB-IoT cell in the test. The test consists of four successive time periods with time duration of T1, T2, T3 and T4 respectively, excluding the transition time duration dT, where the SNR increases or decreases gradually in small steps. Figure A.7.3.89.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync state with the following testing procedure:

-	Prior to the start of the time duration T1, the UE shall be fully synchronized to nCell1

-	Starting at point A, the SNR is decreased in small steps from SNR1 to SNR2 within dT

-	At the start of the time duration T2, the UE is provided with a UL grant with NPDCCH

Note:	The UE is expected to decode the NPDCCH and complete the UL transmission during T2 according to the UL grant.

-	Starting at point B, the SNR is decreased in small steps from SNR2 to SNR3 within dT

-	During T3, the SNR is kept as SNR3

Note:	The UE is expected to detect OOS and declare RLF during T3.

-	Starting at point C, the SNR is increased in small steps from SNR3 to SNR1 with dT

-	At the start of the time period T4, the UE will be provided with another UL grant with NPDCCH

Note:	The UE is not expected to decode the UL grant and conduct any UL transmission during T4, since the UE is expected to declare RLF during T3.

In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode the NPDCCH and complete the UL transmission when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.7.3.89.1-1: General test parameters for TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | nCell 1 |  |
| Neighbour cell |  | eCell 1 |  |
| CP length |  | Normal |  |
| Deployment Mode |  | In-band |  |
| NPDCCH transmission parameters Rmax |  | 16 | Other NPDCCH parameters are defined in “ out-of-sync” column in Table 7.23.2-1 |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211 [16] |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211 [16] |
| DRX cycle | ms | 256 | See Table A.7.3.61.1-3 |
| Layer 3 filtering Note 2,3 |  | Enabled | Counters: N310 = 1 N311 = 1 |
| T310 timer Note 2,3 | ms | 0 | T310 is disabled |
| T311 timer Note 2,3 | ms | 1000 | T311 is enabled |
| T1 | s | 5.12 |  |
| dT | s | 0.7 |  |
| T2 | s | 10.24 |  |
| dT | s | 0.8 |  |
| T3 | s | 5.12 |  |
| dT | s | 1.4 |  |
| T4 | s | 5.12 |  |
| Note 1: NPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: N310, N311, T310 and T311 are defined in TS 36.331 [2]. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. |  |  |  |

Table A.7.3.89.1-2: nCell specific test parameters for TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage

| Parameter | Unit | nCell 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | dT | T2 | dT | T3 | dT | T4 |
| BWchannel | kHz | 180 |  |  |  |  |  |  |
| PRB location within eCell |  | eCell 1 BWchannel 10MHz: 30 |  |  |  |  |  |  |
| NPDCCH parameters as defined in A.3.1.5.7 |  | R.15 NB-TDD |  |  |  |  |  |  |
| NPBCH_RA | dB | -3 |  |  |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |
|  | dBm/15 kHz | Specified in Table A.7.3.89.1-2A |  |  |  |  |  |  |
| SNR Note 4, 5 | dB | -6.3 | Note 6 | -11.4 | Note 7 | -17.4 | Note 8 | -6.3 |
| Propagation condition |  | AWGN |  |  |  |  |  |  |
| Antenna Configuration |  | 2x1 |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: Void Note 3: Void Note 4: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 5: The SNRs in time periods T1, T2, T3 and T4 are denoted as SNR1, SNR2, SNR3 and SNR1 respectively in figure A.7.3.89.1-1. Note 6: The Test system shall reduce its transmit power in steps of ((SNR2-SNR1) / (10*dT)) dB every 100ms until SNR2 is achieved at the end of dT. Note 7: The Test system shall reduce its transmit power in steps of ((SNR3-SNR2) / (10*dT)) dB every 100ms until SNR3 is achieved at the end of dT. Note 8: The Test system shall increase its transmit power in steps of ((SNR1-SNR3) / (10*dT)) dB every 100ms until SNR1 is achieved at the end of dT. |  |  |  |  |  |  |  |  |

Table A.7.3.89.1-2A: eCell 1 specific test parameters for TDD Out-of-sync radio link monitoring test in DRX for UE category NB1 In-band mode in enhanced coverage

| Parameter | Unit | eCell 1 |
| --- | --- | --- |
|  |  | T1-T4 |
| BWchannel | MHz | 10 |
| NOCNG Pattern | - | BWchannel 10MHz: NOP.1 TDD |
| PBCH_RA | dB | -3 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| OCNG_RANote 1 | dB |  |
| OCNG_RBNote 1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
|  | dB | -12 |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 2x1 |
| Note 1: OCNG shall be used such that the eCell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |

Table A.7.3.89.1-3: DRX-Configuration for TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | pp1 | As specified in clause 6.7.3 in TS 36.331 [2] |
| drx-InactivityTimer | pp0 |  |
| drx-RetransmissionTimer | pp0 |  |
| drx-StartOffset | 0 |  |

Table A.7.3.89.1-4: TimeAlignmentTimer -Configuration for TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 [2] |

Figure A.7.3.89.1-1: SNR variation for TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage

A.7.3.89.2	Test Requirements

The UE behaviors in each test shall be as follows:

-	The UE shall complete the NPUSCH transmission during T2 according to the received UL grant;

-	The UE shall not conduct any NPUSCH transmission during T4.

A correct event is defined as UE behaves correctly in all above steps. The correct events observed during repeated tests shall be at least 90%.

A.7.3.90	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage

A.7.3.90.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD category NB1 UE configured in normal coverage properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the NB-IoT Cell when DRX is used. This test will partly verify the TDD radio link monitoring requirements in clause 7.23.

The test parameters are given in Tables A.7.3.90.1-1, A.7.3.90.1-2, A.7.3.90.1-3, A.7.3.90.1-4 and A.7.3.90.1-5. nCell 1 is the active cell in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively, excluding the transition time duration dT, where the SNR increases or decreases gradually in small steps. Figure A.7.3.90.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states.

Prior to the start of the time duration T1, the UE shall be fully be synchronized to nCell 1. The UE is scheduled in every uplink subframe to transmit NPUSCH, which is received by the test equipment. By measuring the reception of the NPUSCH, detection of out of sync and in sync requirements can be measured. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode NPDCCH and to send NPUSCH during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

The test setup in each test during time durations T1, T2 and T3 are as follows:

-	During the period from time point A to time point B, the SNR is decreasing linearly from SNR1 to SNR2.

-	During the period from time point C to time point D, the SNR is increasing linearly from SNR2 to SNR3.

-	During the period T3, the test system shall send the UE a grant to transmit in uplink. UE under test is expected to decode the uplink grant and switch to uplink and complete the uplink transmission.

-	Thereafter UE switches back to downlink.

In each run of the test, the test equipment selects NPDCCH repetition level, and sends the RRC configuration to the UE. NPDCCH repetition level is determined by RRC parameter npdcch-NumRepetitions [2]. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.90.1-1: General test parameters for TDD in-sync test with DRX for UE category NB1 In-Band mode in normal coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IoT operational mode |  |  | In-band |  |
| Active cell |  |  | nCell 1 | NB-IOT cell nCell1 is within E-UTRA cell eCell1 |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | Format N1 | As defined in TS 36.212 [21] |
|  | Number of OFDM symbols for legacy control channels |  | 3 | In sync threshold Qin_NB-IoT and the corresponding hypothetical NPDCCH transmission parameters are as specified in clause 7.23.2 and Table 7.23.2-1 respectively. |
|  | NPDCCH aggregation level | eCCE | 2 |  |
|  | NPDCCH repetition level |  | 2 |  |
|  | Ratio of NPDSCH to NRS EPRE |  | 0 |  |
|  | Ratio of NPDCCH to NRS EPRE |  | 0 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | Format N1 | As defined in TS 36.212 [21] |
|  | Number of OFDM symbols for legacy control channels |  | 3 | Out of sync threshold Qout_NB-IoT and the corresponding hypothetical NPDCCH transmission parameters are as specified in clause 7.23.2 and Table 7.23.2-1 respectively. |
|  | NPDCCH aggregation level | eCCE | 2 |  |
|  | NPDCCH repetition level |  | 8 |  |
|  | Ratio of NPDSCH to NRS EPRE |  | 0 |  |
|  | Ratio of NPDCCH to RS EPRE | dB | 0 |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211 [16] |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-1 in TS 36.211 [16] |
| DRX cycle |  | ms | 256 | See Table A.7.3.63.1-4 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 4000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T1 |  | s | 4 |  |
| dT |  | s | 1.4 |  |
| T2 |  | s | 2.12 |  |
| dT |  | s | 1.4 |  |
| T3 |  | s | 4 |  |
| Note 1: NPDCCH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.90.1-2: nCell 1 specific test parameters for TDD in-sync radio link monitoring test with DRX for UE category NB1 In-Band mode in normal coverage

| Parameter | Unit | nCell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | dT | T2 | dT | T3 |
| BWchannel | kHz | 180 |  |  |  |  |
| PRB location within eCell | - | eCell 1 BWchannel 10MHz: 30 |  |  |  |  |
| NPDCCH parameters defined in A.3.1.5.7 |  | R.15 NB-TDD |  |  |  |  |
| Ratio of NPDSCH to NRS EPRE |  | -3 |  |  |  |  |
| NPDCCH_RA | dB | -3 |  |  |  |  |
| NPDCCH_RB | dB | -3 |  |  |  |  |
| NPBCH_RA | dB | -3 |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |
| NOCNG_RANote1 | dB |  |  |  |  |  |
| NOCNG_RBNote1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | Specified in Table A.7.3.90.1-3 |  |  |  |  |
| SNR Note 5, Note 6 | dB | -3.1 | Note 7 | -14.1 | Note 8 | -3.1 |
| Propagation condition |  | AWGN |  |  |  |  |
| Antenna Configuration |  | 2x1 |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in ncell1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains NPDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2, and SNR1 respectively in figure A.7.3.90.1-1. Note 7: The Test system shall reduce its transmit power in steps of (((SNR2-SNR1) / (10*dT)) dB every 100ms till SNR2 is achieved at the end of dT. Note 8: The Test system shall increase its transmit power in steps of (((SNR1-SNR2) / (10*dT)) dB every 100ms till SNR1 is achieved at the end of dT. |  |  |  |  |  |  |

Table A.7.3.90.1-3: eCell 1 specific test parameters for TDD in-sync radio link monitoring test with DRX for UE category NB1 In-Band mode in normal coverage

| Parameter | Unit | eCell 1 |
| --- | --- | --- |
|  |  | T1-T3 |
| BWchannel | MHz | 10 |
| NOCNG Pattern | - | BWchannel 10MHz: NOP.1 TDD |
| PBCH_RA | dB | -3 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| OCNG_RANote 1 | dB |  |
| OCNG_RBNote 1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
|  | dB | 4 |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 2x1 |
| Note 1: OCNG shall be used such that the eCell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |

Table A.7.3.90.1-4: DRX-Configuration for E-UTRAN TDD in-sync tests with DRX for UE category NB1 In-Band mode in normal coverage

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | pp1 | As specified in clause 6.3.2 in TS 36.331 [2] |
| drx-InactivityTimer | pp0 |  |
| drx-RetransmissionTimer | pp0 |  |
| longDRX-CycleStartOffset | Sf256 |  |
| shortDRX | disable |  |

Table A.7.3.90.1-5: TimeAlignmentTimer -Configuration for E-UTRAN TDD in-sync tests with DRX for UE category NB1 In-Band mode in normal coverage

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 [2] |

Figure A.7.3.90.1-1: SNR variation for in-sync testing with DRX

A.7.3.90.2	Test Requirements

The UE behaviour in each test shall be as follows:

During the period T3, the UE under test is expected to decode the uplink grant and switch to uplink and complete the uplink transmission. This is considered a correct event.

The rate of correct events observed during repeated tests shall be at least 90%.

A.7.3.91	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage

A.7.3.91.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD category NB1 UE configured in enhanced coverage properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the NB-IoT Cell when DRX is used. This test will partly verify the TDD radio link monitoring requirements in clause 7.23.

The test parameters are given in Tables A.7.3.91.1-1, A.7.3.91.1-2, A.7.3.91.1-3, A.7.3.91.1-4 and A.7.3.91.1-5. nCell 1 is the active cell in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively, excluding the transition time duration dT, where the SNR increases or decreases gradually in small steps.  Figure A.7.3.91.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states.

Prior to the start of the time duration T1, the UE shall be fully be synchronized to nCell 1. The UE is scheduled in every uplink subframe to transmit NPUSCH, which is received by the test equipment. By measuring the reception of the NPUSCH, detection of out of sync and in sync requirements can be measured. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode NPDCCH and to send NPUSCH during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

The test setup in each test during time durations T1, T2 and T3 shall be as follows:

-	During the period from time point A to time point B, the SNR is decreasing linearly from SNR1 to SNR2.

-	During the period from time point C to time point D, the SNR is increasing linearly from SNR2 to SNR3.

-	During the period T3, the test system shall send the UE a grant to transmit in uplink. UE under test is expected to decode the uplink grant and switch to uplink and complete the uplink transmission.

-	Thereafter UE switches back to downlink.

In each run of the test, the test equipment selects NPDCCH repetition level, and sends the RRC configuration to the UE. NPDCCH repetition level is determined by RRC parameter npdcch-NumRepetitions [3]. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.91.1-1: General test parameters for TDD in-sync test with DRX for UE category NB1 In-Band mode in enhanced coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IoT operational mode |  |  | In-band |  |
| Active cell |  |  | nCell 1 | NB-IOT cell nCell1 is within E-UTRA cell eCell1 |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | Format N1 | As defined in TS 36.212 [21] |
|  | Number of OFDM symbols for legacy control channels |  | 3 | In sync threshold Qin_NB-IoT and the corresponding hypothetical NPDCCH transmission parameters are as specified in clause 7.23.2 and Table 7.23.2-1 respectively. |
|  | NPDCCH aggregation level | eCCE | 2 |  |
|  | NPDCCH repetition level |  | 4 |  |
|  | Ratio of NPDSCH to NRS EPRE |  | 0 |  |
|  | Ratio of NPDCCH to NRS EPRE |  | 0 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | Format N1 | As defined in TS 36.212 [21] |
|  | Number of OFDM symbols for legacy control channels |  | 3 | Out of sync threshold Qout_NB-IoT and the corresponding hypothetical NPDCCH transmission parameters are as specified in clause 7.23.2 and Table 7.23.2-1 respectively. |
|  | NPDCCH aggregation level | eCCE | 2 |  |
|  | NPDCCH repetition level |  | 16 |  |
|  | Ratio of NPDSCH to NRS EPRE |  | 0 |  |
|  | Ratio of NPDCCH to RS EPRE | dB | 0 |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211 [16] |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-1 in TS 36.211 [16] |
| DRX cycle |  | ms | 256 | See Table A.7.3.62.1-4 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 4000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T1 |  | s | 4 |  |
| dT |  | s | 1.4 |  |
| T2 |  | s | 2.12 |  |
| dT |  | s | 1.4 |  |
| T3 |  | s | 4 |  |
| Note 1: NPDCCH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.91.1-2: nCell 1 specific test parameters for TDD in-sync radio link monitoring test with DRX for UE category NB1 In-Band mode in enhanced coverage

| Parameter | Unit | nCell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | dT | T2 | dT | T3 |
| BWchannel | kHz | 180 |  |  |  |  |
| PRB location within eCell | - | eCell 1 BWchannel 10MHz: 30 |  |  |  |  |
| NPDCCH parameters defined in A.3.1.5.7 |  | R.15 NB-TDD |  |  |  |  |
| Ratio of NPDSCH to NRS EPRE |  | -3 |  |  |  |  |
| NPDCCH_RA | dB | -3 |  |  |  |  |
| NPDCCH_RB | dB | -3 |  |  |  |  |
| NPBCH_RA | dB | -3 |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |
| NOCNG_RANote1 | dB |  |  |  |  |  |
| NOCNG_RBNote1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | Specified in Table A.7.3.91.1-3 |  |  |  |  |
| SNR Note 5, Note 6 | dB | -6.3 | Note 7 | -17.4 | Note 8 | -6.3 |
| Propagation condition |  | AWGN |  |  |  |  |
| Antenna Configuration |  | 2x1 |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in ncell1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains NPDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2, and SNR1 respectively in figure A.7.3.91.1-1. Note 7: The Test system shall reduce its transmit power in steps of (((SNR2-SNR1) / (10*dT)) dB every 100ms till SNR2 is achieved at the end of dT. Note 8: The Test system shall increase its transmit power in steps of (((SNR1-SNR2) / (10*dT)) dB every 100ms till SNR1 is achieved at the end of dT. |  |  |  |  |  |  |

Table A.7.3.91.1-3: eCell 1 specific test parameters for TDD in-sync radio link monitoring test with DRX for UE category NB1 In-Band mode in enhanced coverage

| Parameter | Unit | eCell 1 |
| --- | --- | --- |
|  |  | T1-T3 |
| BWchannel | MHz | 10 |
| NOCNG Pattern | - | BWchannel 10MHz: NOP.1 TDD |
| PBCH_RA | dB | -3 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| OCNG_RANote 1 | dB |  |
| OCNG_RBNote 1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
|  | dB | -12 |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 2x1 |
| Note 1: OCNG shall be used such that the eCell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |

Table A.7.3.91.1-4: DRX-Configuration for E-UTRAN TDD in-sync tests with DRX for UE category NB1 In-Band mode in enhanced coverage

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | pp1 | As specified in clause 6.3.2 in TS 36.331 [2] |
| drx-InactivityTimer | pp0 |  |
| drx-RetransmissionTimer | pp0 |  |
| longDRX-CycleStartOffset | Sf256 |  |
| shortDRX | disable |  |

Table A.7.3.91.1-5: TimeAlignmentTimer -Configuration for E-UTRAN TDD in-sync tests with DRX for UE category NB1 In-Band mode in enhanced coverage

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 [2] |

Figure A.7.3.91.1-1: SNR variation for in-sync testing with DRX

A.7.3.91.2	Test Requirements

The UE behaviour in each test shall be as follows:

During the period T3, the UE under test is expected to decode the uplink grant and switch to uplink and complete the uplink transmission. This is considered a correct event.

The rate of correct events observed during repeated tests shall be at least 90%.

A.7.3.92	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage

A.7.3.92.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD category NB1 UE configured in normal coverage properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the TDD radio link monitoring requirements in clause 7.23.

The test parameters are given in Tables A.7.3.92.1-1, A.7.3.92.1-2, A.7.3.92.1-3 and A.7.3.92.1-4. nCell 1 is the active cell in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively, excluding the transition time duration dT, where the SNR increases or decreases gradually in small steps. Figure A.7.3.92.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states.

Prior to the start of the time duration T1, the UE shall be fully be synchronized to nCell 1. The UE is scheduled in every uplink subframe to transmit NPUSCH, which is received by the test equipment. By measuring the reception of the NPUSCH, detection of out of sync and in sync requirements can be measured. In the test, DRX configuration is disabled, i.e. UE tries to decode NPDCCH and to send NPUSCH during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

The test setup in each test during time durations T1, T2, dT and T3 shall be as follows:

-	During the period from time point A to time point B, the SNR is decreasing linearly from SNR1 to SNR2.

-	During the period from time point C to time point D, the SNR is increasing linearly from SNR2 to SNR3.

-	During the period T3, the test system shall send the UE a grant to transmit in uplink. UE under test is expected to decode the uplink grant and switch to uplink and complete the uplink transmission.

-	Thereafter UE switches back to downlink.

In each run of the test, the test equipment selects NPDCCH repetition level, and sends the RRC configuration to the UE. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.92.1-1: General test parameters for TDD in-sync test without DRX for UE category NB1 In-Band mode in normal coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IoT operational mode |  |  | In-band |  |
| Active cell |  |  | nCell 1 | NB-IOT cell nCell1 is within E-UTRA cell eCell1 |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | Format N1 | As defined in TS 36.212 [21] |
|  | Number of OFDM symbols for legacy control channels |  | 3 | In sync threshold Qin_NB-IoT and the corresponding hypothetical NPDCCH transmission parameters are as specified in clause 7.23.2 and Table 7.23.2-1 respectively. |
|  | NPDCCH aggregation level | eCCE | 2 |  |
|  | NPDCCH repetition level |  | 2 |  |
|  | Ratio of NPDSCH to NRS EPRE |  | 0 |  |
|  | Ratio of NPDCCH to NRS EPRE |  | 0 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | Format N1 | As defined in TS 36.212 [21] |
|  | Number of OFDM symbols for legacy control channels |  | 3 | Out of sync threshold Qout_NB-IoT and the corresponding hypothetical NPDCCH transmission parameters are as specified in clause 7.23.2 and Table 7.23.2-1 respectively. |
|  | NPDCCH aggregation level | eCCE | 2 |  |
|  | NPDCCH repetition level |  | 8 |  |
|  | Ratio of NPDSCH to NRS EPRE |  | 0 |  |
|  | Ratio of NPDCCH to RS EPRE | dB | 0 |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211 [16] |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-1 in TS 36.211 [16] |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 4000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T1 |  | s | 4 |  |
| dT |  | s | 1.4 |  |
| T2 |  | s | 2.12 |  |
| dT |  | s | 1.4 |  |
| T3 |  | s | 4 |  |
| Note 1: NPDCCH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.92.1-2: nCell 1 specific test parameters for TDD in-sync radio link monitoring test without DRX for UE category NB1 In-Band mode in normal coverage

| Parameter | Unit | nCell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | dT | T2 | dT | T3 |
| BWchannel | kHz | 200 |  |  |  |  |
| PRB location within eCell | - | eCell 1 BWchannel 10MHz: 30 |  |  |  |  |
| NPDCCH parameters defined in A.3.1.5.7 |  | R.15 NB-TDD |  |  |  |  |
| NPDSCH_RA |  | -3 |  |  |  |  |
| NPDSCH_RB |  | -3 |  |  |  |  |
| NPDCCH_RA | dB | -3 |  |  |  |  |
| NPDCCH_RB | dB | -3 |  |  |  |  |
| NPBCH_RA | dB | -3 |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |
| NOCNG_RANote1 | dB |  |  |  |  |  |
| NOCNG_RBNote1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | Specified in Table A.7.3.92.1-3 |  |  |  |  |
| SNR Note 5, Note 6 | dB | -3.1 | Note 7 | -14.1 | Note 8 | -3.1 |
| Propagation condition |  | AWGN |  |  |  |  |
| Antenna Configuration |  | 2x1 |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in ncell1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains NPDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2, and SNR1 respectively in figure A.7.3.92.1-1. Note 7: The Test system shall reduce its transmit power in steps of (((SNR2-SNR1) / (10*dT)) dB every 100ms till SNR2 is achieved at the end of dT. Note 8: The Test system shall increase its transmit power in steps of (((SNR1-SNR2) / (10*dT)) dB every 100ms till SNR1 is achieved at the end of dT. |  |  |  |  |  |  |

Table A.7.3.92.1-3: eCell 1 specific test parameters for TDD in-sync radio link monitoring test without DRX for UE category NB1 In-Band mode in normal coverage

| Parameter | Unit | eCell 1 |
| --- | --- | --- |
|  |  | T1-T3 |
| BWchannel | MHz | 10 |
| NOCNG Pattern | - | BWchannel 10MHz: NOP.1 TDD |
| PBCH_RA | dB | -3 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| OCNG_RANote 1 | dB |  |
| OCNG_RBNote 1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
|  | dB | 6 |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 2x1 |
| Note 1: OCNG shall be used such that the eCell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |

Table A.7.3.92.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD in-sync tests without DRX for UE category NB1 In-Band mode in normal coverage

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 [2] |

Figure A.7.3.92.1-1: SNR variation for in-sync testing without DRX

A.7.3.92.2	Test Requirements

The UE behavior in each test shall be as follows:

During the period T3, the UE under test is expected to decode the uplink grant and switch to uplink and complete the uplink transmission. This is considered a correct event.

The rate of correct events observed during repeated tests shall be at least 90%.

A.7.3.93	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage

A.7.3.93.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD category NB1 UE configured in enhanced coverage properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the TDD radio link monitoring requirements in clause 7.23.

The test parameters are given in Tables A.7.3.93.1-1, A.7.3.93.1-2, A.7.3.93.1-3 and A.7.3.93.1-4. nCell 1 is the active cell in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively, excluding the transition time duration dT, where the SNR increases or decreases gradually in small steps. Figure A.7.3.93.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states.

Prior to the start of the time duration T1, the UE shall be fully be synchronized to nCell 1. The UE is scheduled in every uplink subframe to transmit NPUSCH, which is received by the test equipment. By measuring the reception of the NPUSCH, detection of out of sync and in sync requirements can be measured. In the test, DRX configuration is disabled, i.e. UE tries to decode NPDCCH and to send NPUSCH during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

The test setup in each test during time durations T1, T2, dT and T3 shall be as follows:

-	During the period from time point A to time point B, the SNR is decreasing linearly from SNR1 to SNR2.

-	During the period from time point C to time point D, the SNR is increasing linearly from SNR2 to SNR3.

-	During the period T3, the test system shall send the UE a grant to transmit in uplink. UE under test is expected to decode the uplink grant and switch to uplink and complete the uplink transmission.

-	Thereafter UE switches back to downlink.

In each run of the test, the test equipment selects NPDCCH repetition level, and sends the RRC configuration to the UE. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.93.1-1: General test parameters for TDD in-sync test without DRX for UE category NB1 In-Band mode in enhanced coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IoT operational mode |  |  | In-band |  |
| Active cell |  |  | nCell 1 | NB-IOT cell nCell1 is within E-UTRA cell eCell1 |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | Format N1 | As defined in TS 36.212 [21] |
|  | Number of OFDM symbols for legacy control channels |  | 3 | In sync threshold Qin_NB-IoT and the corresponding hypothetical NPDCCH transmission parameters are as specified in clause 7.23.2 and Table 7.23.2-1 respectively. |
|  | NPDCCH aggregation level | eCCE | 2 |  |
|  | NPDCCH repetition level |  | 4 |  |
|  | Ratio of NPDSCH to NRS EPRE |  | 0 |  |
|  | Ratio of NPDCCH to NRS EPRE |  | 0 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | Format N1 | As defined in TS 36.212 [21] |
|  | Number of OFDM symbols for legacy control channels |  | 3 | Out of sync threshold Qout_NB-IoT and the corresponding hypothetical NPDCCH transmission parameters are as specified in clause 7.23.2 and Table 7.23.2-1 respectively. |
|  | NPDCCH aggregation level | eCCE | 2 |  |
|  | NPDCCH repetition level |  | 16 |  |
|  | Ratio of NPDSCH to NRS EPRE |  | 0 |  |
|  | Ratio of NPDCCH to RS EPRE | dB | 0 |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211 [16] |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-1 in TS 36.211 [16] |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 4000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T1 |  | s | 4 |  |
| dT |  | s | 1.4 |  |
| T2 |  | s | 2.12 |  |
| dT |  | s | 1.4 |  |
| T3 |  | s | 4 |  |
| Note 1: NPDCCH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.93.1-2: nCell 1 specific test parameters for TDD in-sync radio link monitoring test without DRX for UE category NB1 In-Band mode in enhanced coverage

| Parameter | Unit | nCell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | dT | T2 | dT | T3 |
| BWchannel | kHz | 200 |  |  |  |  |
| PRB location within eCell | - | eCell 1 BWchannel 10MHz: 30 |  |  |  |  |
| NPDCCH parameters defined in A.3.1.5.7 |  | R.15 NB-TDD |  |  |  |  |
| NPDSCH_RA |  | -3 |  |  |  |  |
| NPDSCH_RB |  | -3 |  |  |  |  |
| NPDCCH_RA | dB | -3 |  |  |  |  |
| NPDCCH_RB | dB | -3 |  |  |  |  |
| NPBCH_RA | dB | -3 |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |
| NOCNG_RANote1 | dB |  |  |  |  |  |
| NOCNG_RBNote1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | Specified in Table A.7.3.93.1-3 |  |  |  |  |
| SNR Note 5, Note 6 | dB | -6.3 | Note 7 | -17.4 | Note 8 | -6.3 |
| Propagation condition |  | AWGN |  |  |  |  |
| Antenna Configuration |  | 2x1 |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in ncell1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains NPDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2, and SNR1 respectively in figure A.7.3.93.1-1. Note 7: The Test system shall reduce its transmit power in steps of (((SNR2-SNR1) / (10*dT)) dB every 100ms till SNR2 is achieved at the end of dT. Note 8: The Test system shall increase its transmit power in steps of (((SNR1-SNR2) / (10*dT)) dB every 100ms till SNR1 is achieved at the end of dT. |  |  |  |  |  |  |

Table A.7.3.93.1-3: eCell 1 specific test parameters for TDD in-sync radio link monitoring test without DRX for UE category NB1 In-Band mode in enhanced coverage

| Parameter | Unit | eCell 1 |
| --- | --- | --- |
|  |  | T1-T3 |
| BWchannel | MHz | 10 |
| NOCNG Pattern | - | BWchannel 10MHz: NOP.1 TDD |
| PBCH_RA | dB | -3 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| OCNG_RANote 1 | dB |  |
| OCNG_RBNote 1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
|  | dB | 0 |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 2x1 |
| Note 1: OCNG shall be used such that the eCell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |

Table A.7.3.93.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD in-sync tests without DRX for UE category NB1 In-Band mode in enhanced coverage

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 [2] |

Figure A.7.3.93.1-1: SNR variation for in-sync testing without DRX

A.7.3.93.2	Test Requirements

The UE behavior in each test shall be as follows:

During the period T3, the UE under test is expected to decode the uplink grant and switch to uplink and complete the uplink transmission. This is considered a correct event.

The rate of correct events observed during repeated tests shall be at least 90%.

A.7.3.94	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage

A.7.3.94.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD category NB1 UE properly detects the out of sync for the purpose of monitoring downlink radio link quality of the NB-IoT Cell. This test will partly verify the NB-IoT TDD radio link monitoring requirements in clause 7.23.

The test parameters are given in Tables A.7.3.94.1-1 and A.7.3.94.1-2. nCell1 is the active NB-IoT cell in the test. The test consists of four successive time periods with time duration of T1, T2, T3 and T4 respectively, excluding the transition time duration dT, where the SNR increases or decreases gradually in small steps. Figure A.7.3.94.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync state with the following testing procedure:

-	Prior to the start of the time duration T1, the UE shall be fully synchronized to nCell1

-	Starting at point A, the SNR is decreased in small steps from SNR1 to SNR2 within dT

-	At the start of the time duration T2, the UE is provided with a UL grant with NPDCCH

Note:	The UE is expected to decode the NPDCCH and complete the UL transmission during T2 according to the UL grant.

-	Starting at point B, the SNR is decreased in small steps from SNR2 to SNR3 within dT

-	During T3, the SNR is kept as SNR3

Note:	The UE is expected to detect OOS and declare RLF during T3.

-	Starting at point C, the SNR is increased in small steps from SNR3 to SNR1 with dT

-	At the start of the time period T4, the UE will be provided with another UL grant with NPDCCH

Note:	The UE is not expected to decode the UL grant and conduct any UL transmission during T4, since the UE is expected to declare RLF during T3.

Table A.7.3.94.1-1: General test parameters for TDD Radio Link Monitoring Test for out-of-sync tests without DRX for UE Category NB1 Standalone mode in normal coverage

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NB-IoT operational mode |  | Standalone |  |
| Active cell |  | nCell 1 |  |
| CP length |  | Normal |  |
| NPDCCH repetition level Rmax |  | 8 | Other NPDCCH parameters are defined in “ out-of-sync” column in Table 7.23.2-1 |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211 [16] |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-1 in TS 36.211 [16] |
| DRX |  | OFF |  |
| Layer 3 filtering Note 2,3 |  | Enabled | Counters: N310 = 1 N311 = 1 |
| T310 timer Note 2,3 | ms | 0 | T310 is disabled |
| T311 timer Note 2,3 | ms | 3000 | T311 is enabled |
| T1 | s | 2 |  |
| dT | s | 0.8 |  |
| T2 | s | 0.4 |  |
| dT | s | 0.7 |  |
| T3 | s | 0.5 |  |
| dT | s | 1.4 |  |
| T4 | s | 0.4 |  |
| Note 1: NPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: N310, N311, T310 and T311 are defined in TS 36.331 [2]. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. |  |  |  |

Table A.7.3.94.1-2: nCell1 specific test parameters for TDD Radio Link Monitoring Test for out-of-sync without DRX for UE Category NB1 Standalone mode in normal coverage

| Parameter | Unit | nCell 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | dT | T2 | dT | T3 | dT | T4 |
| NB-IoT Channel Bandwidth (BWchannel) | kHz | 180 |  |  |  |  |  |  |
| OCNG Pattern as defined in A.3.2.3.8 Note 1 |  | NOP.3 TDD |  |  |  |  |  |  |
| NPDCCH parameters as defined in A.3.1.5.8 |  | R.19 NB-TDD |  |  |  |  |  |  |
| NPBCH_RA | dB | -3 |  |  |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RA Note 1 | dB |  |  |  |  |  |  |  |
| OCNG_RB Note 1 | dB |  |  |  |  |  |  |  |
|  | dBm/15 KHz | -98 |  |  |  |  |  |  |
| SNR Note 4,5 | - | -3.1 | Note 6 | -9.1 | Note 7 | -14.1 | Note 6 | -3.1 |
| Propagation Condition | - | AWGN |  |  |  |  |  |  |
| Antenna Configuration | - | 2x1 |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: Void Note 3: Void Note 4: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 5: The SNRs in time periods T1, T2, T3 and T4 are denoted as SNR2, SNR3 and SNR1 respectively in figure A.7.3.94.1-1. Note 6: The Test system shall reduce its transmit power in steps of ((SNR2-SNR1) / (10*dT)) dB every 100ms until SNR2 is achieved at the end of dT. Note 7: The Test system shall reduce its transmit power in steps of ((SNR3-SNR2) / (10*dT)) dB every 100ms until SNR3 is achieved at the end of dT. Note 8: The Test system shall increase its transmit power in steps of ((SNR1-SNR3) / (10*dT)) dB every 100ms until SNR1 is achieved at the end of dT. |  |  |  |  |  |  |  |  |

Figure A.7.3.94.1-1: SNR variation for out-of-sync testing

A.7.3.94.2	Test Requirements

The UE behaviors in each test shall be as follows:

-	The UE shall complete the NPUSCH transmission during T2 according to the received UL grant;

-	The UE shall not conduct any NPUSCH transmission during T4

A correct event is defined as UE behaves correctly in all above steps. The correct events observed during repeated tests shall be at least 90%.

A.7.3.95	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage

A.7.3.95.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD category NB1 UE properly detects the out of sync for the purpose of monitoring downlink radio link quality of the NB-IoT Cell. This test will partly verify the NB-IoT TDD radio link monitoring requirements in clause 7.23.

The test parameters are given in Tables A.7.3.95.1-1 and A.7.3.95.1-2 below. nCell1 is the active NB-IoT cell, in the test. The test consists of four successive time periods with time duration of T1, T2, T3 and T4 respectively, excluding the transition time duration dT, where the SNR increases or decreases gradually in small steps. Figure A.7.3.95.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync state with the following testing procedure.

-	Before the start of the time duration T1, the UE shall be fully synchronized to nCell1

-	Starting at point A, the SNR is decreased in small steps from SNR1 to SNR2 with duration dT

-	At the start of the time duration T2, the UE is provided with a UL grant with NPDCCH.

Note:	The UE is expected to decode NPDCCH and complete the UL transmission during T2 according to the UL grant.

-	Starting at point B, the SNR is decreased in small steps from SNR2 to SNR3 with duration dT

-	During T3, the SNR is kept at SNR3.

Note:	The UE is expected to detect OOS and declare RLF during T3.

-	Starting at point C, the SNR is increased in small steps from SNR3 to SNR1 with duration dT

-	At the start of the time period T4, the UE will be provided with another UL grant with NPDCCH

Note:	The UE is not expected to decode the UL grant and conduct the UL transmission during T4 since the UE is expected to declare RLF during T3.

Table A.7.3.95.1-1: General test parameters for TDD Radio Link Monitoring Test for out-of-sync tests without DRX for UE Category NB1 Guard band mode in enhanced coverage

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NB-IoT operational mode |  | Guardband |  |
| Active cell |  | nCell 1 |  |
| CP length |  | Normal |  |
| NB-IoT RF Channel Number |  | 1 | One NB-IoT carrier frequency |
| NPDCCH repetition level Rmax |  | 16 | Other NPDCCH parameters are defined in “ out-of-sync” column in Table 7.23.2-1 |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211 [16] |
| Uplink-downlink configuration |  | 6 | As specified in table 4.2-1 in TS 36.211 [16] |
| DRX |  | OFF |  |
| Layer 3 filtering Note 2 |  | Enabled | Counters: N310 = 1 N311 = 1 |
| T310 timer Note 2 | ms | 0 | T310 is disabled |
| T311 timer Note 2 | ms | 3000 | T311 is enabled |
| T1 | s | 2 |  |
| dT | s | 0.7 |  |
| T2 | s | 0.4 |  |
| dT | s | 0.8 |  |
| T3 | s | 0.5 |  |
| dT | s | 1.4 |  |
| T4 | s | 0.4 |  |
| Note 1: NPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: N310, N311, T310 and T311 are defined in TS 36.331 [2]. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. |  |  |  |

Table A.7.3.95.1-2: nCell1 specific test parameters for TDD Radio Link Monitoring Test for out-of-sync without DRX for UE Category NB1 Guard band mode in enhanced coverage

| Parameter | Unit | nCell 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | dT | T2 | dT | T3 | dT | T4 |
| BWchannel | kHz | 180 |  |  |  |  |  |  |
| PRB location within eCell | - | eCell 1 BWchannel 10MHz: 50 |  |  |  |  |  |  |
| NPDCCH parameters as defined in A.3.1.5.9 |  | R.23 NB-TDD |  |  |  |  |  |  |
| NPBCH_RA | dB | -3 |  |  |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RA Note 1 | dB |  |  |  |  |  |  |  |
| OCNG_RB Note 1 | dB |  |  |  |  |  |  |  |
|  | dBm/15 KHz | Specified in Table A.7.3.95.1-3 |  |  |  |  |  |  |
| SNR Note 4,5 | - | -6.3 | Note 6 | -11.4 | Note 7 | -17.4 | Note 8 | -6.3 |
| Propagation Condition | - | AWGN |  |  |  |  |  |  |
| Antenna Configuration | - | 2x1 |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: Void Note 3: Void Note 4: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 5: The SNR in time periods T1, T2, T3 and T4 is denoted as SNR1, SNR2, SNR3 and SNR1 respectively in figure A.7.3.95.1-1. Note 6: The Test system shall reduce its transmit power in steps of ((SNR2-SNR1) / (10*dT)) dB every 100ms until SNR2 is achieved at the end of dT. Note 7: The Test system shall reduce its transmit power in steps of ((SNR3-SNR2) / (10*dT)) dB every 100ms until SNR3 is achieved at the end of dT. Note 8: The Test system shall increase its transmit power in steps of ((SNR1-SNR3) / (10*dT)) dB every 100ms until SNR1 is achieved at the end of dT. |  |  |  |  |  |  |  |  |

Table A.7.3.95.1-3: eCell 1 specific test parameters for TDD out-of-sync radio link monitoring test without DRX for UE category NB1 Guard band mode in enhanced coverage

| Parameter | Unit | eCell 1 |
| --- | --- | --- |
|  |  | T1-T4 |
| BWchannel | MHz | 10 |
| NOCNG Pattern | - | BWchannel 10MHz: NOP.2 TDD |
| PBCH_RA | dB | -3 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| OCNG_RANote 1 | dB |  |
| OCNG_RBNote 1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
|  | dB | -12 |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 2x1 |
| Note 1: OCNG shall be used such that the eCell 1 is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |

Figure A.7.3.95.1-1: SNR variation for out-of-sync testing

A.7.3.95.2	Test Requirements

The UE behaviors in each test shall be as follows:

-	The UE shall complete the NPUSCH transmission during T2 according to the received UL grant;

-	The UE shall not conduct any NPUSCH transmission during T4

A correct event is defined as UE behave correctly in all above steps. The correct events observed during repeated tests shall be at least 90%.

## << End of changes >>