3GPP TSG-RAN WG4 Meeting #100-e	R4- 2114160

Electronic Meeting, 16 – 27 August, 2020

Source:	Huawei, HiSilicon

Title:	TP to TR 37.717-31-11 for DC_3-7-7-28-n1

Agenda item:	8.16.2

Document for:	Approval

# Background

This contribution provides text proposal on the EN-DC band combination DC_3-7-7-28-n1 as defined in the basket WI [1].

# Text Proposal

##### ---Start of changes---

## 5.x.4	DC_3-7-7-28_n1

### 5.x.4.1	Configurations for EN-DC

Table 5.x.4.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-7A-7A-28A_n1A | DC_3A_n1A DC_7A_n1A DC_28A_n1A |

### 5.x.4.2	 ∆TIB and ∆RIB values

Table 5.x.4.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-7-28_n1 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 28 | 0.5 |
|  | n1 | 0.6 |

Table 5.x.4.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-7-28_n1 | 3 | 0 |
|  | 7 | 0 |
|  | 28 | 0.2 |
|  | n1 | 0 |

### 5.x.4.3		Reference sensitivity exceptions

REFSENS exceptions have already been specified for DC_3-7-28_n1.

##### ---End of changes---

# Reference

[1]	RP-211170, “Revised WID on Rel-17 Dual Connectivity (DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL)”, Ericsson