3GPP TSG-RAN4 Meeting #100-e	R4-2114297

# Electronic Meeting, 16 – 27 August, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to update gNB Rx-Tx time difference requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Perf |  |  |  |  |  | Date: |  |  | 2021-07-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are several issues with gNB Rx-Tx time difference requirements: - For some SRS BW ranges, the accuracy are derived not based on the lower bound of the range. - For some cases, the results from companies show large std deviation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update gNB Rx-Tx requirements to address the above issues: - Update accuracy based on the lower bound of the SRS BW range. - Add margin for cases where the results from companies show large std deviation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | gNB Rx-Tx time difference requirements are not fully aligned with simulation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.2.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

13.2.2	Measurement Accuracy Requirements

13.2.2.1	Introduction

This clause defines accuracy requirements for gNB Rx-Tx time difference measurement in FR1 and FR2. The requirements are applicable for gNB supporting gNB Rx-Tx time difference measurement. The gNB, which declares the support for gNB Rx-Tx time difference measurement also declares that it meets gNB Rx-Tx time difference accuracy requirements at least for one side condition Ês/Iot ≥ +3 dB or Ês/Iot ≥ -13 dB.

13.2.2.2	Requirements

The accuracy requirements for gNB Rx-Tx time difference measurement shall be within ±(X+Y) Tc under the following conditions:

-	AWGN propagation conditions.

-	The measured signals are in the directions covered by RoAoA of OTA reference sensitivity requirements for gNB type 1-O and 2-O BS

where

-	X is defined in Table 13.2.2.2-1 for gNB types 1-C, 1-H and 1-O and in Table 13.2.2.2-2 for gNB type 2-O.

-	Y is declared by manufacturer and can be different for different gNB types 1-C, 1-H, 1-O and 2-O.

Note: The measurement accuracy requirements in Table 13.2.2.2-1 and Table 13.2.2.2-2 are defined under an assumption that gNB is not mandated to perform receive beam sweeping.

Table 13.2.2.2-1: gNB Rx-Tx time difference absolute accuracy in FR1 for gNB type 1-C, 1-H and 1-O

| Accuracy | SRS Ês/Iot | SCS | SRS bandwidth range |
| --- | --- | --- | --- |
| Unit: Tc | Unit: dB | Unit: kHz | Unit: RB |
| [157] | ≥ -13 | 15 | 44 ≤ BW ≤ 84 |
| [56] |  |  | 88 ≤ BW ≤ 168 |
| [15] |  |  | 176 ≤ BW |
| [122] | ≥ +3 |  | 24 ≤ BW ≤ 40 |
| [60] |  |  | 44 ≤ BW ≤ 84 |
| [31] |  |  | 88 ≤ BW ≤ 168 |
| [15] |  |  | 176 ≤ BW |
| [42] | ≥ -13 | 30 | 48 ≤ BW ≤ 84 |
| [27] |  |  | 88 ≤ BW ≤ 168 |
| [8] |  |  | 176 ≤ BW |
| [31] | ≥ +3 |  | 48 ≤ BW ≤ 84 |
| [15] |  |  | 88 ≤ BW ≤ 168 |
| [8] |  |  | 176 ≤ BW |
| [19] | ≥ -13 | 60 | 48 ≤ BW ≤ 84 |
| [13] |  |  | 88 ≤ BW |
| [15] | ≥ +3 |  | 48 ≤ BW ≤ 84 |
| [8] |  |  | 88 ≤ BW |

Table 13.2.2.2-2: gNB Rx-Tx time difference absolute accuracy in FR2 for gNB type 2-O

| Accuracy | SRS Ês/Iot | SCS | SRS bandwidth range |
| --- | --- | --- | --- |
| Unit: Tc | Unit: dB | Unit: kHz | Unit: RB |
| [8] | ≥ -13 | 60 | 132 ≤ BW ≤ 168 |
| [6] |  |  | 176 ≤ BW |
| [8] | ≥ +3 |  | 132 ≤ BW ≤ 168 |
| [6] |  |  | 176 ≤ BW |
| [19] | ≥ -13 | 120 | 32 ≤ BW ≤ 40 |
| [18] |  |  | 44 ≤ BW ≤ 84 |
| [6] |  |  | 88 ≤ BW |
| [15] | ≥ +3 |  | 32 ≤ BW ≤ 40 |
| [8] |  |  | 44 ≤ BW ≤ 84 |
| [6] |  |  | 88 ≤ BW |

<End of Change 1>