3GPP TSG-RAN WG4 Meeting#100-e                	R4-2114418

Electronic Meeting, 19 – 27 August 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	On applicability of MRTD requirements for Rel-16 NR eMIMO

Agenda item:	6.1.5.1.1

Document for:	Discussion

Introduction

At the RAN4#99-e meeting, a way forward on NR eMIMO RRM requirement maintenance was approved [1], which lists the open issues. One open issue related to the core requirements is

Clarification on applicability of MRTD/MTTD requirements for Multi-TRxP Proposals: More technical discussion is needed in RAN4 to find a proper way to capture the meeting agreements in the spec. - Option 1: Capture that requirements are applicable to mTRP as discussed in GTW: Unless explicitly stated otherwise the Maximum Transmission Timing Difference (MTTD) and Maximum Receive Timing Difference (MRTD) requirements in clauses 7.5.3, 7.6.3 and 7.6.4 for co-located deployment are applicable when - When UE is configured to receive multiple PDCCH - When UE is configured by repetitionScheme set to one of ' fdmSchemeA', ' fdmSchemeB' and 'tdmSchemeA' - Option 2: Clarify what is the definition of MRTD for multi-TRP case. - The signals from different CCs are received within the specified MRTD, i.e., the MRTD is specified for signals from different CCs - The signals from different TRP are received within CP - Option 3: No need to add the applicability of MRTD requirements for multi-TRP transmission - The existing MRTD applies between any two serving cells regardless what is configured to transmit. - Adding applicability rule for multi-TRP transmission causes misunderstanding the MRTD requirements only applies to the specific configuration.

This contribution discusses the open issue and provides our views.

Discussion

Referring to [2], it was agreed that there is no RRM requirement impact identified regarding the existing MRTD/MTTD values as specified in Rel-15 for multi-TRxP transmission.

There is no RRM requirement impact identified regarding the existing MRTD/MTTD values as specified in Rel-15 for multi-TRxP transmission.

Consequently, the MRTD requirements in clauses 7.6.3 and 7.6.4, TS 38.133 apply to deployment scenarios without multi-TRxP as well as with multi-TRxP transmission.

MRTD requirements in clauses 7.6.3 and 7.6.4, TS 38.133 also apply to deployment scenarios without multi-TRxP as well as with multi-TRxP transmission.

There is no technical reason why RAN4 would need to explicitly state that the generally applicable requirements in sections 7.6.3 and 7.6.4 specifically apply to multi-TRxP deployment scenarios. The current requirements are general and the requirements generally applicable including multi-TRxP scenarios.

Hence, ‘clarifying’ that the generally applicable requirements in sections 7.6.3 and 7.6.4 apply for multi-TRxP does in our view not provide any clarification (as they do apply) but instead unclarified whether they apply in general. Hence, we have concerns with options 1 and 2 and do not see any technical reason for any change.

Adding applicability would have the opposite effect, that is, unclarified whether the requirements apply in general.

Option 3: No need to add the applicability of MRTD requirements for multi-TRP transmission

- The existing MRTD already applies generally and between any two serving cells regardless what is configured to transmit.

- Adding applicability rule for multi-TRP transmission causes misunderstanding whether the MRTD requirements applies generally or only applies to the specific configuration.

Conclusion

The document has discussed the applicability of MRTD requirements for multi-TRxP and our view is as follows:

- There is no RRM requirement impact identified regarding the existing MRTD/MTTD values as specified in Rel-15 for multi-TRxP transmission.

MRTD requirements in clauses 7.6.3 and 7.6.4, TS 38.133 also apply to deployment scenarios without multi-TRxP as well as with multi-TRxP transmission.

Adding applicability would have the opposite effect, that is, unclarified whether the requirements apply in general.

and

- Option 3: No need to add the applicability of MRTD requirements for multi-TRP transmission

- The existing MRTD already applies generally and between any two serving cells regardless what is configured to transmit.

- Adding applicability rule for multi-TRP transmission causes misunderstanding whether the MRTD requirements applies generally or only applies to the specific configuration.

References

[1]   R4-2108225, WF on NR eMIMO RRM requirements maintenance, Samsung

[2]    R4-2012146, WF on Completing Rel-16 eMIMO RRM Core Requirement, Samsung