# 3GPP TSG-RAN WG4 Meeting #100-e

# August 16–27, 2021	R4-2114469

# Title:	MSS S-Band range (1980-2010 and 2170-2200 MHz) for NTN-FR1 and its adjacent bands

# Source:	Hughes/EchoStar, Inmarsat, Sateliot, Thales

# Type:	Discussion

# Document for: 	Agreement

# Agenda Item:	9.13.1

# Introduction

RAN4#98-e agreed to have MSS S-band [1] i.e., 2GHz, considered for NTN FR1 exemplary band. It was also agreed to have NTN FR1 exemplary band in RAN4 be completed in Rel-17. This contribution clarifies the MSS S-Band exact frequency range and its NR adjacent bands in all regions.

# Discussion

## 2.1 NTN-FR1 Exemplary Band decision on 1980-2010 and 2170-2200 MHz

As was agreed in RAN4#98-e, MSS S-Band refers to 1980 -2010 (UL) and 2170 -2200 (DL) MHz will be the NTN FR1 exemplary band, to be completed in Rel-17. The chart below highlights its adjacent NR bands in all regions.

Figure 1: MSS S-Band 1980-2010 and 2170-2200 MHz [2] to be adapted for NTN-NR band

Observation 1: RAN4#98-e endorsed MSS S-Band [1980-2010 MHz (UL) and 2170-2200 MHz (DL)] as the NTN FR1 exemplary band, to be completed in Rel-17.

Proposal 1: RAN4 work to focus on the MSS specific range [1980-2010 MHz (UL) and 2170-2200 MHz (DL)] for the NTN FR1 exemplary band.

## 2.2 Adjacent NR Bands to MSS S-Band

As illustrated in Figure 1 the adjacent bands to MSS S-Band are as follows:

- Region 1: Adjacent with n1 (FDD).

- Region 3: Adjacent with n1 (FDD) and n34 (TDD).

- Region 2: In the US and Canada, the MSS band has been assigned for terrestrial use on a national basis; the MSS S-Band range for Mexico is not aligned with 1980-2010 and 2170-2200 MHz; and in Central/South America the MSS S-band is adjacent to n1 (FDD).

As part of the NTN-FR1 MSS S-Band analysis of co-existence with adjacent 3GPP NR bands, the following observations and proposals should be considered:

Observation 2: In the US and Canada, the MSS S-band (2000-2020 and 2180-2200 MHz) has been assigned for terrestrial use on a national basis. Therefore the MSS S-band definition for NTN-NR in this range will not apply for US and Canada [3].

Observation 3: The MSS S-Band range for Mexico is not aligned with 1980-2010 and 2170-2200 MHz.

Proposal 2: The MSS S-band definition for NTN-NR [1980-2010 MHz (UL) and 2170-2200 MHz (DL)] as part of the Rel-17 NR-NTN WI does not apply for North America (US, Canada and Mexico).

Proposal 3: RAN4 to consider analysis of co-existence with N1 and N34 as adjacent bands to MSS S-Band [1980-2010 MHz (UL) and 2170-2200 MHz (DL)].

## 2.3 NTN-NTN Adjacent Bands

Per agreement in RAN4#98-bis-e [4], RAN4 shall consider inputs from NTN operators for the NTN-NTN coexistence adjacent to MSS S-band.

Observation 4: As demonstrated in Figure 1, there are no NTN (satellite) bands adjacent to MSS S-band range of 1980-2010 and 2070–2200 MHz.

Proposal 4: RAN4 shall consider this as the input from operators that NTN-NTN (satellite) adjacent band co-existence for MSS S-band [1980-2010 MHz (UL) and 2170-2200 MHz (DL)] is not applicable and out of scope.

# Conclusions

Observation 1: RAN4#98-e endorsed MSS S-Band [1980-2010 MHz (UL) and 2170-2200 MHz (DL)] as the NTN FR1 exemplary band, to be completed in Rel-17.

Proposal 1: RAN4 work to focus on the MSS specific range [1980-2010 MHz (UL) and 2170-2200 MHz (DL)] for the NTN FR1 exemplary band.

Observation 2: In the US and Canada, the MSS S-band has been assigned for terrestrial use on a national basis. Therefore the MSS S-band definition for NTN-NR in this range will not apply for US and Canada [3].

Observation 3: The MSS S-Band range for Mexico is not aligned with 1980-2010 and 2170-2200 MHz.

Proposal 2: The MSS S-band definition for NTN-NR [1980-2010 MHz (UL) and 2170-2200 MHz (DL)] as part of the Rel-17 NR-NTN WI does not apply for North America (US, Canada and Mexico).

Proposal 3: RAN4 to consider analysis of co-existence with N1 and N34 as adjacent bands to MSS S-Band [1980-2010 MHz (UL) and 2170-2200 MHz (DL)].

Observation 4: As demonstrated in Figure 1, there are no NTN (satellite) bands adjacent to MSS S-band range of 1980-2010 and 2070–2200 MHz.

Proposal 4: RAN4 shall consider this as the input from operators that NTN-NTN (satellite) adjacent band co-existence for MSS S-band [1980-2010 MHz (UL) and 2170-2200 MHz (DL)] is not applicable and out of scope.

# References

[1] R4-2103877 - WF for NTN General Part

[2] ITU-R Radio Regulations, Article 5 – Frequency Allocations

[3] R4-2015915 - Possible FR1 exemplary band for NR based satellite networks, NTN system parameters and use cases – Thales

[4] R4-2106103 - WF on [307] NTN_Solutions_Part1