3GPP TSG-RAN WG4 Meeting # 100-e	R4-2114544

Electronic Meeting, August 16-27, 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	On 5MHz CBW in the IAB Rel-16 Specifications

Agenda item:	6.1.2.6.1 General

Document for:	Discussion

Introduction

At the previous RAN#99-e meeting the clauses on demodulation and CSI reporting testing were added to IAB Rel.16 TSs 38.174, 38.176-1 and 38.176-2.

Potentially, there are reasons to delete 5MHz channel bandwidth from the IAB technical specifications because they can never be utilized.

In the contribution, we discuss a need for 5MHz channel bandwidth in demodulation and CSI reporting sections.

Discussion of 5MHz CBW

It has previously been observed that the (RF) IAB specifications do not foresee operating bands that support 5MHz CBW. Only n41/n77/n78/n79 (see TS 38.174, section 5.2) are supported, and according to TS 38.104:

Table 5.3.5-1: BS channel bandwidths and SCS per operating band in FR1

| NR band / SCS / BS channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n41 | 15 |  | 10 | 15 | 20 |  | 30 | 40 | 50 |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 60 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| n77 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| n78 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| n79 | 15 |  |  |  |  |  |  | 40 | 50 |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |
|  | 60 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |

Following this agreement, the IAB Demod session has removed all IAB-MT requirements with 5MHz CBW.
The IAB-DU demodulation requirements with 5MHz CBW have been retained following the understanding that IAB-DUs also serve normal access UEs in addition to IAB-MTs.

Unfortunately, the RF session seems to have a different understanding and has also removed IAB-DU RF requirements with 5MHz CBW.
Upon closer inspection of the IAB specification it does not currently seem possible to serve access UEs with other bands, and in particular with bands that support 5MHz CBW. This behaviour is in line with the technical report for IAB (TR 38.809, Clause 5.2).

It seems impossible to serve either IAB-MTs or normal access UEs on bands that support 5MHz CBW.

In contrast to the RF session, we currently see no issue with including requirements for 5MHz CBW, as the CBW manufacturer declarations will make these requirements not applicable.

The current CBW manufacturer declaration structure allows to keep 5MHz CBW demodulation performance requirements.

Hence, we propose:

RAN4 to let 5MHz CBW IAB-DU demodulation performance requirements remain in the IAB specification, and to trust in the manufacturer declarations to have these requirements be non-applicable.

Conclusion

In the contribution, we discussed a need for 5MHz channel bandwidth in demodulation and CSI reporting sections.

The following observations and proposal are made:

- It seems impossible to serve either IAB-MTs or normal access UEs on bands that support 5MHz CBW.

The current CBW manufacturer declaration structure allows to keep 5MHz CBW demodulation performance requirements.

RAN4 to let 5MHz CBW IAB-DU demodulation performance requirements remain in the IAB specification, an

d to trust in the manufacturer declarations to have these requirements be non-applicable.