3GPP TSG-RAN WG4 Meeting # 100e	R4-2114580

# Electronic Meeting, Aug. 19-27, 2021

Source: 	Skyworks Solutions, Inc.

Title: 	n3 PC2 MSD

Agenda Item:	10.5.3 Analyses on receiver sensitivity degradation [FS_NR_PC2_UE_FDD]

Document for:	Approval

# Introduction

This document presents measurement data to evaluate the n3 PC2 MSD.

# Discussion

## Power Amplifier Calibration

- PA calibration point is 20 MHz, 15 kHz, QPSK, DFT-S-OFDM, 100 RB at lower channel edge with 1 dB MPR - PC2 ACLR -31dBc;

- Post PA losses: 4dB;

- Power Class 3 (PC3) operation;

- Local Oscillator (LO) leakage: -28dBc; and

- IQ Image rejection: -28dB.

## Measurement Results and MSD Proposal

Complete measurement dataset could not be entirely retrieved in due time. Partial dataset for 50MHz and 45MHz is presented and this data is subject to verification post-deadline, hence all values are presented in brackets until complete dataset is analyzed. The intent is to provide updates prior to meeting start or in the early days of round 1 discussions that will cover all of band n3 CBW.

UL RB allocations are those agreed for PC3, ie 50RB for SCS 15kHz located as close as possible to the downlink operating band.

The preliminary noise levels are (this data is subject to further checks) [-30.1]dBm/48.615MHz for 50MHz CBW and              [-35.1]dBm/43.755MHz for 45MHz CBW. MSD shown in Figure 1 is approximately [3.1]dB and [5.1]dB for 45MHz and 50MHz CBW respectively.

Figure 1 Measured Tx noise and corresponding MSD levels

These MSD levels, like for the case of our PC3 measurements [1], lead to SCS 15kHz REFSENS levels of -84.1dBm for 45MHz and -81.6dBm for 50MHz. These levels are lower than the legacy PC3 agreed levels of -81.3dBm and -79.7dBm for 45MHz/50MHz respectively. We therefore tentatively propose to use the same approach as in [1], i.e. we propose PC2 REFSENS levels by evaluating the MSD level difference between PC3 and PC2 and apply this MSD difference to PC3 REFSENS levels. For 50MHz, MSD increases by 1dB (5.1dB vs 4.1dB [1]), for 45MHz MSD increases by 1.1dB MSD (3.1dB vs 2dB [1]).

Proposal: Table 1 n3 PC2 REFSENS levels and Table 2 Uplink Configuration.

Table 1: n3 PC2 REFSENS

| CA configuration | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5B | 15 | [TBA] | [TBA] | [TBA] | [TBA] | [TBA] | [TBA] | [TBA] | [TBA] | [-80.2] | [-78.7] | FDD |
|  | 30 |  | [TBA] | [TBA] | [TBA] | [TBA] | [TBA] | [TBA] | [TBA] | [-80.3] | [-78.8] |  |
|  | 60 |  | [TBA] | [TBA] | [TBA] | [TBA] | [TBA] | [TBA] | [TBA] | [-80.4] | [-78.9] |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2: n3 PC2 REFSENS Uplink Configuration

| CA configuration | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5B | 15 | 25 | 501 | 501 | 501 | 501 | 501 | 501 | 501 | 501 | 501 | FDD |
|  | 30 |  | 241 | 241 | 241 | 241 | 241 | 241 | 241 | 241 | 241 |  |
|  | 60 |  | 101 | 101 | 101 | 101 | 101 | 101 | 101 | 101 | 101 |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |  |  |  |  |  |  |  |  |

# Conclusions

In this contribution, we present a partial measurement dataset that is subject to further verifications to evaluate n3 PC2 MSD. We intend to provide n3 PC2 REFSENS level for all CBW prior to the start of the meeting.

Proposal: Table 1 n3 PC2 REFSENS levels and Table 2 Uplink Configuration.

Table 1: n3 PC2 REFSENS

| CA configuration | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5B | 15 | [TBA] | [TBA] | [TBA] | [TBA] | [TBA] | [TBA] | [TBA] | [TBA] | [-80.2] | [-78.7] | FDD |
|  | 30 |  | [TBA] | [TBA] | [TBA] | [TBA] | [TBA] | [TBA] | [TBA] | [-80.3] | [-78.8] |  |
|  | 60 |  | [TBA] | [TBA] | [TBA] | [TBA] | [TBA] | [TBA] | [TBA] | [-80.4] | [-78.9] |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2: n3 PC2 REFSENS Uplink Configuration

| CA configuration | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5B | 15 | 25 | 501 | 501 | 501 | 501 | 501 | 501 | 501 | 501 | 501 | FDD |
|  | 30 |  | 241 | 241 | 241 | 241 | 241 | 241 | 241 | 241 | 241 |  |
|  | 60 |  | 101 | 101 | 101 | 101 | 101 | 101 | 101 | 101 | 101 |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |  |  |  |  |  |  |  |  |

# References

[1] 	R4-2111528, n3 50MHz REFSENS, A-MPR, 3GPP TSG-RAN WG4 Meeting#99-e, Skyworks Solutions, Inc.