3GPP TSG-RAN WG4 Meeting #100-e                                R4-2114806

Online, 16th-27th Aug, 2021

Source:	Samsung, Verizon

Title:	TP for TR 37.717-21-11: DC_2-46_n2

Agenda item:			8.15.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.717-21-11 to include DC_2-46_n2 according to the request in [1].

2. Reference

- R4-2110666 WID revision: Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL).

- R4-2110665 TR 37.717-21-11 v0.5.0_DC of 2 LTE band and 1 NR band

# 3. Text Proposal

<Start of Text Proposal>

5.x	DC_2-46_n2

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A-46A_n2A3 DC_2A-46C_n2A3 DC_2A-46D_n2A3 DC_2A-46E_n2A3 | DC_2A_n2A2 |
| NOTE 2: Only single switched UL is supported NOTE 3: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. |  |

### 5.x.2	Co-existence studies

For UL DC_2A_n2A, there is no need to study IMD for only single switch UL is supported.

3rd harmonic of n2 UL falls into b46 DL.

### 5.x.3	∆TIB and ∆RIB values

For DC_2-46_n2, the TIB,c and RIB,c values are reused from CA_2-46 shown below.

Table 5.x.3-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-46_n2 | 2 | 0 |
|  | 46 | 0 |
|  | n2 | 0 |

Table 5.x.3-2: ΔRIB due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-46_n2 | 2 | 0 |
|  | 46 | 0 |
|  | n2 | 0 |

### 5.x.4	REFSENS requirements

Based on 5.x.2 discussion above, REFSENS exceptions are specified below.(A new table below is introduced and could be number as Table 7.3B.2.3.1-6 in TS38.101-3 )

Table 5.x.4-1: Reference sensitivity QPSK PREFSENS (EN-DC with b46)

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n2 | 461 |  | N/A |  | N/A |  |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the downlink (victim) transmission bandwidth which falls into the reference sensitivity exclusion region overlapping with UL harmonic zones |  |  |  |  |  |  |  |  |  |  |  |  |

<End of Text Proposal>