3GPP TSG-WG RAN4 Meeting #100-e	R4-2114830

# Online, 16 – 27 August, 2021

Source: 				Nokia, US Cellular Corporation

Title: 					TP to TR 38.717-02-01: CA_n12-n71

Agenda Item: 		8.8.2 [NR_CADC_R17_2BDL_xBUL-Core]

Document for:		Approval

# Introduction

This TP introduces the following 2 band NR CA configurations (2DL/1UL).

| Band combination configuration | UpLink configuration | BCS |
| --- | --- | --- |
| CA_n12A-n71A | - | 0 |

## TP to TR 38.717-02-01

## <Start of Changes>

## 6.X		CA_n12-n71

### 6.X.1	Common for 1 band UL and 2 bands UL CA

#### 6.X.1.1	Operating bands for CA

Table 6.X.1.1-1: CA band combination of band n12+n71

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
| n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

#### 6.X.1.2	Channel bandwidths per operating band for CA

Table 6.X.1.2-1: Supported bandwidths per CA band combination of band n12+n71

| CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n12A-n71A | - | n12 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.X.1.3	UE co-existence studies

Table 6.X.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n12-n71.

Table 6.X.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n12 | 699 | 716 | 729 | 746 | 1398 | 1432 | 2097 | 2148 |  |  |
| n71 | 663 | 698 | 617 | 652 | 1326 | 1396 | 1989 | 2094 |  |  |

Table 6.X.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | mth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n12 | 699 | 716 | 729 | 746 | 1458 | 1492 | 2187 | 2238 |  |  |
| n71 | 663 | 698 | 617 | 652 | 1234 | 1304 | 1851 | 1956 |  |  |

#### 6.X.1.4	∆TIB and ∆RIB values

For CA_n12-n71, the TIB,c and RIB,c values are reused from the similar combination DC_12_n71 and are given in the tables below.

Table 6.X.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n12-n71 | n12 | 1 |
|  | n71 | 1 |

Table 6.X.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n12-n71 | n12 | 0.8 |
|  | n71 | 0.8 |

#### 6.X.1.5	REFSENS requirements

There is no REFSENS exception for 1UL/2DL of for CA_n12-n71.

#### 6.X.1.6	REFSENS requirements OOB blocking exception requirements

There is no OOB exception for the CA combination.

## <End of Changes>