3GPP TSG-RAN WG4 Meeting #100-e	R4-2114844

Electronic Meeting, 16 August – 27 August 2021

Source:	Ericsson, Telstra

Title:	TP for TR 38.717-02-01: CA_n1-n5

Agenda item:			8.8.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-02-01 to include CA_n1A -n5A  as defined in WID [1].

# ---Start of changes---

## 6.X	n1-n5

### 6.X.1	Common for 1 band UL and 2 bands UL CA

#### 6.X.1.1		Operating bands for CA

Table 6.X.1.1-1: CA band combination of band n1 and n5

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
| n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |

#### 6.X.1.2		Channel bandwidths per operating band for CA

Table 6.X.1.2-1: Supported bandwidths per CA band combination of band n5 and n1

| NR CA configuration | Uplink CA configuration | NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n5A | CA_n1A-n5A | n1 | 5 | 10 | 15 | 15 | 25 | 30 | 40 | 50 |  |  |  |  | 0 |
|  |  | n5 | 5 | 10 | 15 | 15 |  |  |  |  |  |  |  |  |  |

#### 6.X.1.3		UE co-existence studies

Table 6.X.1.3-1 lists up to 7th harmonics for n1-n5. As can be seen there are no harmonic issues.

Table 6.X.1.3-1: Impact of UL/DL Harmonic

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  | 6th Harmonic |  | 7th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n5 | 824 | 849 | 1648 | 1698 | 2472 | 2547 | 3296 | 3396 | 4120 | 4245 | 4944 | 5094 | 5768 | 5943 |
| n1 | 1920 | 1980 | 3840 | 3960 | 5760 | 5940 | 7680 | 7920 | 9600 | 9900 | 11520 | 11880 | 13440 | 13860 |

Table 6.X.1.3-2 list harmonic mixing issue for the 2DL bands CA with 1 UL. As can be seen there are no harmonic mixing issues.

Table 6.X.1.3-2 Harmonic mixing for 2DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1738 | 1788 | 2607 | 2682 | 3476 | 3576 |
| n1 | 1920 | 1980 | 2110 | 2170 | 4220 | 4340 | 6330 | 6510 | 8440 | 8680 |

#### 6.X.1.4		∆TIB and ∆RIB values

For CA_n1-n5, the TIB,c and RIB,c values are same as for DC_1_n5 and are given in the tables below.

Table 6.X.1.4-1: ΔTIB,c

| NR CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n5 | n1 | 0.3 |
|  | n5 | 0.3 |

Table 6.X.1.4-2: ΔRIB,c

| NR CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n5 | n1 | 0 |
|  | n5 | 0 |

#### 6.X.1.5 		REFSENS requirements

As can be seen in the co-existence studies in 6.X.1.3 there are no harmonics issues.

### 6.x.2		Specific for 2 bands UL CA

#### 6.x.2.1	Maximum output power for inter-band CA

Table 6.x.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n1A-n5A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

#### 6.x.2.2		UE co-existence studies

Table 6.x.2.2-1 lists Band n1 + Band n5 bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.x.2.2-1: Band n5 and Band n1 UL harmonics and IMD products

| UL frequency (MHz) | 824 | 849 | 1920 | 1980 |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1156 | 1071 | 2744 | 2829 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 332 | 222 | 2991 | 3136 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3568 | 3678 | 4664 | 4809 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2312 | 2142 | 5488 | 5658 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 492 | 627 | 4911 | 5116 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4392 | 4527 | 6584 | 6789 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7096 | 6831 | 1476 | 1316 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8504 | 8769 | 5216 | 5376 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4292 | 4062 | 1293 | 1488 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7408 | 7638 | 6312 | 6507 |

Based on the table above:

IMD4 generated by UL n5 and UL n1 might affect DL n1.

Table 6.x.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.x.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n1-n5 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 21, 22, 26, 28, 31, 38, 40, 42, 43, 50, 51, 65, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3,34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA band 41, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [4] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

#### 6.x.2.3		REFSENS requirements

Considering that the IMD4 does not fall at the duplex offset frequency of band n1, there is no need to define MSD value

### 6.6.1.6			OOB blocking exception requirements

Table 6.11.1.6-1: CA band combination with exceptions allowed

CA band combination

No exception

---End of changes---

# Reference

[1] 		RP-211058, Rel-17 NR Inter-band Carrier Aggregation/Dual Connectivity  for 2 bands DL with x bands UL (x=1,2), ZTE