3GPP TSG-RAN WG4 Meeting#100     	R4-2114858

E-meeting, 16th – 27th  Aug, 2021

Title:	TP for 37.717-11-21 to introduce DC_8A_n1A-n40A

Source:	Nokia

Agenda item:	8.19.2

Document for:	Approval

# Introduction

This is a TP for 37.717-11-21 to introduce DC_8A_n1A-n40A

********************** Start of TP *********************************

## 6.x	DC_8A_n1A-n40A

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n1-n40 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

### 6.x.2	Configuration for DC

Table 6.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_8A_n1A-n40A | DC_8A_n1A DC_8A_n40A | 8A | CA_n1A-n40A |

### 6.x.3	Co-existence studies

Co-existence analysis for DC_8_n1 UL shows no impact to NR Band n40 DL.

Co-existence analysis for DC_8_n40 UL shows 5th IMD might fall in NR Band n1 DL.

### 6.x.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n1-n40 | 8 | 0.3 |
|  | n1 | 0.3 |
|  | n40 | 0.5 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n1-n40 | 8 | 0.2 |
|  | n1 | 0 |
|  | n40 | 0.5 |

### 6.x.5	MSD

Based on co-existence studies MSD is needed.

Table 6. x.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n1A-n40A | 8 | 885 | 5 | 25 | 930 | N/A | N/A |
|  | n1 | 1935 | 5 | 25 | 2125 | 3.3 | IMD5 |
|  | n40 | 2390 | 5 | 25 | 2390 | N/A | N/A |

********************** End of TP *********************************