3GPP TSG-RAN WG4 Meeting #100-e 												R4-2114884

Electronic Meeting, 12th – 20th April, 2021

Agenda item:			8.2

Source:	Apple Inc., Skyworks Solutions Inc.

Title:	WF on introduction of the full unlicensed band

Document for:	Information

# Introduction

3GPP Rel-16 NR-U WI specified how the NR technology can be used on the unlicensed spectrum thus offering more resources in frequency bands, such as 5GHz and 6GHz.  5GHz is a well-known band for the unlicensed operation, but 6GHz is a relative new band usage of which was approved recently in different regulatory regions. While the 6GHz band for the US is already part of the Rel-16 core functionality, current 3GPP specifications do not support it for other countries, such as South Korea and Canada, which have finalised their regulatory requirements only recently. Thus, RAN#92 meeting approved a new WI aim of which is to enable support of the 6GHz unlicensed band for those countries and regions that have finalised recently the corresponding regulatory requirements.

This document presents a way forward based on the agreements made during the [100-e][109] NR_6GHz_unlic_full email discussion (RAN4#100 meeting).

# Way forward

## Regulatory parameters

Based on the comments from the 1st and the 2nd round, the following summary of the regulatory requirements is proposed.

| Country | Mode |  |  |
| --- | --- | --- | --- |
|  | SP | LPI | VLP |
| Canada | (new) | (new) | (new) |
| Brazil | N/A | same as US LPI | (new) |
| Peru | N/A | (new) Note1 | N/A |
| Chile | N/A | (new) Note1 | N/A |
| South Korea | N/A | (new) | (new)Note2 |

NOTE 1: Peru and Chile LPI regulatory parameters are the same as US/Brazil LPI, but there is no -27dBm/MHz out-of-band emission requirement.

NOTE 2: South Korea VLP parameters are the same as in EU/CEPT, but out-of-band emission requirement is set to -34dBm/MHz outside the VLP operational range of 5925-6425MHz.

Agreement:

-	Consider the presented regulatory summary for the next meeting technical discussions.

## Summary of the NS values

Based on the outcome of the 1st and 2nd round, the following summary is proposed that shows which NS values can be re-used and which new NS values are needed.

| Country | Mode |  |  |
| --- | --- | --- | --- |
|  | SP | LPI | VLP |
| Canada | NS_54 | NS_x1 (new) | TBD |
| Brazil | N/A | NS_53 | TBD |
| Peru | N/A | [NS_53] Note1 | N/A |
| Chile | N/A | [NS_53] Note1 | N/A |
| South Korea | N/A | NS_x2 (new) | TBD |

NOTE 1: Peru and Chile LPI regulatory parameters are the same as US/Brazil LPI, but there is no -27dBm/MHz out-of-band emission requirement. At the same time, since A-MPR values we are PSD limited, then NS_53 should also apply.

Agreement:

- NS_54 can be re-used for the SP mode in Canada.

- NS_53 can be re-used for the LPI mode in Brazil.

- New NS values are needed for LPI in Canada and South Korea.

- To be checked further whether NS_53 can be re-used for Peru and Chile.

## Band plan

Based on the comments from the 1st round, it is suggested to re-use band n96 for new countries (Canada, South Korea, Brazil, Peru, Chile).

Agreement:

-	Use existing band n96 for new countries (Canada, South Korea, Brazil, Peru, Chile).

## A-MPR values

Agreement:

-	Endorse the following A-MPR values for LPI in South Korea and Canada (can be further revised during next meetings).

Proposed A-MPR for LPI in Canada

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |
| --- | --- | --- | --- |
|  |  | 20 MHz |  |
|  |  | Full2 (dB) | Partial3 (dB) |
| DFT-s-ODFM | PI/2 BPSK | ≤ 3.0 | ≤ 5.5 |
|  | QPSK | ≤ 3.0 | ≤ 5.5 |
|  | 16 QAM | ≤ 3.0 | ≤ 5.5 |
|  | 64 QAM | ≤ 3.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 5.5 |
|  | 16 QAM | ≤ 4.0 | ≤ 5.5 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 |

Proposed A-MPR for LPI in South Korea

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | ≤ 6.0 | ≤ 8.5 | ≤ 4.0 | ≤ 5.5 | ≤ 3.5 | ≤ 4.5 | ≤ 3.0 | ≤ 4.5 |
|  | 16 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 5.0 | ≤ 3.5 | ≤ 5.0 |
|  | 64 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 5.0 | ≤ 3.5 | ≤ 5.0 |
|  | 256 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.0 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
|  | 16 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
|  | 64 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |