3GPP TSG-WG RAN4 Meeting #100-e	R4-2114928

Electronic meeting, August 16-27, 2021

WF on NS_50 and A-MPR for Band n39

Huawei, HiSilicon, Apple, Nokia, Nokia Shanghai Bell, Qualcomm

# Background

The PC2 A-MPR for n39 NS_50 has been discussed in RAN4#99-e and good progress were made [1]. The remaining issues are:

- A-MPR for BW=25/30/40 MHz

- A-MPR for BW ≤ 20 MHz

- Modification to NS_50 signalling

References:

[1] R4-2107738, “WF on A-MPR for PC2 n39”, Huawei, HiSilicon, Apple, Nokia, 3GPP TSG RAN4#99-e

[2] R4-2114696, PC2 A-MPR for NS_50 on n39, Qualcomm, RAN4#100-e

[3] R4-2111768, n39 PC2 A-MPR for NS_50, Nokia, Nokia Shanghai Bell, RAN4#100-e

[4] R4-2111769, Backward compatibility of an NS for n39 A-MPR for CBW < 25 MHz, Nokia, Nokia Shanghai Bell, RAN4#100-e

[5] R4-2112361, Considerations and A-MPR results for PC2 n39, Apple, RAN4#100-e

[6] R4-2114181, A-MPR for n39 NS_50 PC2, Huawei, HiSilicon, RAN4#100-e

# PC2 A-MPR for BW = 25/30/40 MHz (1/2)

The PC2 A-MPR for BW>20 MHz is defined as Table 1 and Table 2.

Table 1: A-MPR regions for NS_50 (Power Class 2, BW>20MHz)

| Channel Bandwidth (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 25 MHz | ≤ LCRB*12*SCS – 5 | > 5 | A2 |
|  | ≤ 6.3 | ≤ 1.44 | A5 |
|  | > 8.28 | > max (21.6 – RBstart*12*SCS, 0), <RBstart*12*SCS+5 | A4 |
|  | >1.8, ≤6.12 | > 1.44, ≤ 3.6 | A6 |
|  | > LCRB *12*SCS – 5, ≤ 5.04 | > 1.44 | A4 |
| 30 MHz | ≤ LCRB*12*SCS – 5 | >5 | A2 |
|  | ≤ 7.56 | ≤ 1.44 | A5 |
|  | >1.8, ≤7.56 | > 1.44, ≤ 3.6 | A6 |
|  | ≤ 1.8 | >1.44, <RBstart*12*SCS+5 | A4 |
|  | > 10.8 | > max (26.64 – RBstart*12*SCS, 0), <RBstart*12*SCS+5 | A4 |
| 40 MHz | ≤ 4.32 | > 0 | A1 |
|  | > 4.32 | > RBstart*12*SCS + 11.88 | A1 |
|  | > 4.32, ≤ 12.96 | ≤ 10.8 | A3 |
|  | > 4.32, ≤ 18 | > 10.8, <= RBstart*12*SCS + 11.88 | [A7] |
|  | > 18, ≤ 31.68 | > max (31.68 – RBstart*12*SCS, 0) | A4 |
|  | > 31.68 | > 0 | A1 |
| NOTE 1: The A-MPR values are specified in Table 2. |  |  |  |

# PC2 A-MPR for NS_50 (2/2)

Table 2: A-MPR for NS_50 (Power Class 2)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) | A7 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 11.5 | ≤4.5 | ≤ 4 | ≤ 2.5 | ≤ 4 | ≤ 1 | [≤ 3.5] |
|  | QPSK | ≤ 11.5 | ≤ 5.5 | ≤ 4 | ≤ 2.5 | ≤ 4 | ≤ 1 | [≤ 3.5] |
|  | 16 QAM | ≤ 11.5 | ≤ 5.5 | ≤ 4 | ≤ 2.5 | ≤ 4 | ≤ 1.5 | [≤ 3.5] |
|  | 64 QAM | ≤ 11.5 | ≤ 5.5 | ≤ 4 |  | ≤ 4 |  |  |
|  | 256 QAM | ≤ 11.5 | ≤ 5.5 |  |  |  |  |  |
| CP-OFDM | QPSK | ≤ 12.5 | ≤ 7 | ≤ 5.5 | ≤ 4 | ≤ 4 | ≤ 2 | [≤ 5] |
|  | 16 QAM | ≤ 12.5 | ≤ 7 | ≤ 5.5 | ≤ 4 | ≤ 4 |  | [≤ 5] |
|  | 64 QAM | ≤ 12.5 | ≤ 7 | ≤ 5.5 | ≤ 4 | ≤ 4 |  | [≤ 5] |
|  | 256 QAM | ≤ 12.5 | ≤ 7 |  |  |  |  |  |

# PC2 A-MPR for BW = 10/15/20 MHz

The PC2 A-MPR for BW<=20 MHz is defined as Table 3.

| Channel Bandwidth (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 10 MHz | ≤ 1.44 | < 1.44 | A5 |
|  | ≤ 1.8 | ≥ 2.7+2* RBstart*12*SCS | A4 |
|  | >1.8 | ≥ 8.1- RBstart*12*SCS | A4 |
| 15 MHz | ≤ 2.88 | < 2.7 | A5 |
|  | ≤ 3.24 | ≥ 2.7+2* RBstart*12*SCS | A3 |
|  | >3.24 | ≥ 12.42- RBstart*12*SCS | A4 |
| 20 MHz | ≤ 4.32 | < 3.6 | A5 |
|  | ≤ 4.5 | ≥ 3.6+2* RBstart*12*SCS | A3 |
|  | >4.5 | ≥ 17.1- RBstart*12*SCS | A4 |
| NOTE 1: The A-MPR values are specified in Table 2. |  |  |  |

Table 3: A-MPR regions for NS_50 (Power Class 2, BW≤20MHz)

# NS_50 Signalling

Agreements:

- Update NS_50 to include BW<=20MHz

- Define A-MPR for PC2 UEs covering all RB allocations

- FFS whether and how to remove RB restrictions for PC3