3GPP TSG-WG RAN4 Meeting #100-e	R4-2114943

Electronic meeting, August 16-27, 2021

WF on BS and UE RF requirements in support of NB-IoT 16QAM

Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Ericsson

# Background

In support of 16QAM for NB-IoT, relevant RF requirements need to be defined on both BS and UE sides.

References:

[1] R4-2112280 Proposals on BS RF requirements for support of 16QAM in NB-IoT, Nokia, Nokia Shanghai Bell

[2] R4-2113620, MPR for NB-IoT 16-QAM with modified IBE, Nokia, Nokia Shanghai Bell

[3] R4-2114002, CR on NB-IoT IBE mask to allow 16-QAM, Nokia, Nokia Shanghai Bell

[4] R4-2114216, Discussion on in-band emission requirements for 16QAM NB-IoT Uplink, Huawei, HiSilicon

[5] R4-2114217, Further discussion on BS RF requirements for 16QAM NB-IoT Downlink, Huawei, HiSilicon,

[6] R4-2114345, BS RF impact analysis on R17 NB_IoT, Ericsson

[7] R4-2114346, UE RF impact analysis on R17 NB_IoT, Ericsson

# Agreements on BS RF requirements

- Reuse the existing NB-IoT RB power dynamic range requirements for 16QAM for in-band or guard band operation or NB-IoT operation in NR in-band. And no changes to the specifications, nor new test cases for 16QAM RB power dynamic range are needed.

- The majority view is to set the EVM limit for 16QAM to 12.5%. FFS whether to allow different rated output power for 16QAM.

- FFS whether to define new 16QAM FRCs for BS Rx dynamic range tests

# Agreements on UE RF requirements

- The EVM limit for 16QAM to set to 12.5%.

- The IBE mask of NB-IoT is modified as in the CR of [3], i.e.,

| Parameter description | Unit | Limit (NOTE 1) | Applicable Frequencies |
| --- | --- | --- | --- |
| General | dB | max20⋅log10 EVM-10⋅log10(Ntone/LCtone), 20⋅log10 EVM-3-5⋅(Δtone-1)/LCtone, -57dBm/(3.75kHzor15kHz)-Ptone | Any non-allocated (NOTES 2, 10) |

- The MPR for 16QAM PC3 and PC5 is defined as the table below. FFS MPR for PC6.

Table 1: MPR for NB-IoT 16QAM PC3 and PC5

| Tone spacing | Number of tones | Allocated tones | MPR proposal [dB] |
| --- | --- | --- | --- |
| 15 kHz | 3 | 0-2 | [1.8] |
|  |  | 3-5 | [1.8] |
|  |  | 6-8 | [1.8] |
|  |  | 9-11 | [1.8] |
|  | 6 | 0-5 | [1.8] |
|  |  | 6-11 | [1.8] |
|  | 12 | 0-11 | [2.8] |