3GPP TSG-RAN WG4 Meeting # 100-e 												R4-2115030

Electronic Meeting, 16th – 27th August, 2021

Agenda item:			9.4.3, 9.4.6.3

Source:	Moderator (Apple)

Title:	Email discussion summary for [100-e][130] NR_RF_FR2_req_enh2_Part_2

Document for:	Information

# Introduction

FR2 UL gap is discussed in this email thread.

In RAN#92e, revised WID on NR RF enhancements for FR2 is approved [1]. The purpose of this WI is to specify related FR2 UE features and associated requirements, including

- UL gaps for self-calibration and monitoring: [RAN4 RF/RRM, RAN2] Study and, if feasible, introduce UE specific and NW configured gap for general self-calibration and monitoring purposes including

    - UE Tx power management

    - Other self-calibration and monitoring are not precluded

    - Coherent uplink MIMO

  - Phase 1: Study and clearly identify the performance gain over the current baseline (Rel.16 requirements) Study of RF performance evaluation/testability related to UE self-calibration and monitoring. Study network impact of UE emissions during UL gap, if any.

  - Phase 2: Specify the UL gap configuration(s), related UE capability and interruptions, if needed, based on the identified performance gain in Phase 1 and UE fall back behavior i.e. if gaps are not available for UE requesting gaps.

Agreements in RAN4#99-e on UL gap for Tx power management is summarized as below

Agreements:

- It is feasible to enable non-zero P-MPR in Tx power management and BPS related UL gap testing.

- zero P-MPR assumption for the existing test cases kept unchanged

On the test setups for UL gap based Tx power management.

- Option 1: Based on P-MPR report with/without blocking

- Option 2: Based on peak EIRP measurement with/without blocking

- Option 3: other method like jamming

On mandating P-MPR reporting for the UE who is configured with UL gap for BPS based Tx power management.

- Option 1: Yes

- Option 2: No.

- Option 3: depending on the conclusion of test case setup

On phantom or blocking be introduced in UL gap testing.

- Continue discussion the test setup and requirement with and without phantom or blocking

- Discuss phantom simplification, e.g., size, material and position

On the RF requirement for the UE who is configured with UL gap for BPS based Tx power management

- Exact RF requirement will be based on test set up.

# Topic #1: BPS related UL Gap

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2112088 | Apple | Proposal 1: No phantom/blocking in UL gap test set up. Proposal 2: Reported Delta P-MPR is used as metric. |
| R4-2112635 | NTT COCOMO | Observation 1: If we apply the test method to measure difference between the two cases in which UL gap is activated and deactivated, it may be difficult to evaluate performance of UE that does not require any UL gaps to perform the human detection. Observation 2: By checking the three P-MPR (X/Y/Z) in Table 2.2.1-2, we can confirm not only the performance of UE(s) that improve P-MPR values with UL gaps (type C and D UE), but also the performance of UE that can achieve lower P-MPR values without UL gaps (type B and D UE). Table 2.2.1-2: Clarification of the type of P-MPR (test case) Type of P-MPR; Blocking material; UL gap; Type of UE. A; B; C; D. X; No; No; High; Mid / Low; High; Mid / Low. Y; Yes; No; High; High; High; High. Z; No; Yes; High; Mid / Low; Low / Very Low; Low / Very Low. Proposal 1: To evaluate the improvement by UL gaps and the basic performance of all UEs, three P-MPR in Table 2.2.1-2 should be checked. Proposal 2: RF requirement for UL gaps is given the relaxation value (ω). - If [X (type X P-MPR) < β] or [Delta Y (type Y P-MPR) – X (type X P-MPR) > γ] ⇒ Delta X (type X P-MPR) – Z (type Z P-MPR) > α-ω (Further discussion is needed to determine α,β,γ,ω.) |
| R4-2112808 | Nokia | Observation 1: The test needs to ensure that UL gaps are used for body proximity sensing and P-MPR improvement with MPE events. Proposal 1: Support Option 1 with blocking as first priority if further discussion on the type of material that would trigger only radar is conclusive. Proposal 2: Do not support Option 2. EIRP is not a reliable metric and P-MPR report is sufficient. Proposal 3: Support Option 3with further discussion on jamming possibility as second priority, if consensus cannot be found regarding option 2. Proposal 4: Consider an extra test to validate UEs fulfill current requirements even without UL gaps scheduled. Observation 2: Another test may be defined for UEs embedding other means than radar for body proximity sensing. Proposal 5: Further discussion how to define UE core requirements in addition to the testing aspects. Observation 3: Gains should be shown between coherent MIMO without the gaps and coherent MIMO with the gaps. |
| R4-2113005 | vivo | Observation 1: Any UE implementation that can use gap to reduce P-MPR should be allowed and not limited to the use of BPS, so that the UEs can support this feature more flexibly. Proposal 1: What we should verify is the gain brought by the introduction of gap itself, and the influence of different calibration method or sensor capabilities need to be precluded. Proposal 2: The P-MPR reporting is more straightforward to reflect the performance gain of the UL gap for Tx power management and should be used as the baseline for test setups. At the same time, the P-MPR should be mandatory for the UE who support UL gap for Tx power management while the gap is configured. Observation 2: The delta P-MPR without blocking/phantom cannot identify whether the performance gain comes from the gap itself because there may be other calibration methods that do not require gap. Proposal 3: The blocking/phantom should be introduced in the verification of the calibration for Tx power management, which is helpful to distinguish whether the performance gain comes from the gap itself. Proposal 4: The performance gain of Tx power management should be defined as the difference between the PMPR changes before and after the gap is configured, as shown in Figure 1. |
| R4-2113211 | ZTE | Proposal 1: Once the configured UL gap activated, mandating P-MPR reporting. Proposal 2: The performance gain of self-calibration in UL gap should be reflected into requirement improvement, such as P-MPR or P-MPR reduction requirement. Proposal 3: UE should also meet SEM or transmit off power during the BPS UL gap to avoid causing interference. |
| R4-2113661 | Ericsson, Sony | Observation 1: RAN4 shall follow the directive from the meeting report “RAN4 will not define any requirements until the corresponding testing methodology for the performance enhancement is clear” Observation 2: 3-4 dB can be seen as a typical value of P-MPR (assuming a 20% uplink duty cycle). Proposal 1: Finalize the details regarding different gap configurations and their possible impact on activation, e.g. periodical/aperiodical/semi-persistant configuration. Proposal 2: Activation of the configured gaps shall be done by means of MAC CE. Proposal 3: UE vendors are encouraged to provide input of “Default/baseline” P-MPR that can be expected in regular operation in field. Proposal 4: Mandate test coverage for UEs configured with UL calibration gaps for Tx power management, i.e. test OFF power requirement in the gap time window (other tests might also be considered). Proposal 5: Investigate if the periodicity of PHR/P-MPR reports are sufficient to provide valid feedback for BPS calibration gaps. Proposal 6: Investigate further the need for 3GPP to standardize body phantom and the impact of a body phantom on OTA performance |
| R4-2113900 | OPPO | Observation 1: There is possibility in real testing, the phantom/blocking material cannot be detected by signals in the UL gap if positioned improperly, and no PMPR is triggered. Observation 2: Test mode based is more operable, but this can be left to RAN5 and is out of RAN4 scope. Proposal 1: It is proposed to leave the test method design to RAN5, e.g. whether to use test mode trigger PMPR etc. Observation 3: No gain can be observed by configuring UL gap, if the PMPR gain is less than 3dB or PMPR value is larger than 12dB. Observation 4: Peak EIRP based metric is more precise, and can rely on existing tests if no phantom is used. Proposal 2: It is proposed to adopt peak EIRP as the testing metric and no phantom used. Observation 5: How much dB gain in Peak EIRP deserves the UL gap is still unclear. Proposal 3: It is proposed to further discuss the required dB gain in UL gap feature, e.g. 1dB or 2dB. |
| R4-2114492 | Huawei, HiSilicon | Observation1: there is no RF requirements for coherent UL MIMO under multiple conditions, including DRX on, BWP switching, SRS switching, DL measurement gap, etc. Proposal 1: UE requirement improvement by using coherence calibration gap can go with one of following options: • Option 1: Specify 40 degree relative phase difference and 4dB power difference for side conditions including DRX on, BWP switching, SRS switching, DL measurement gap, etc. • Option 2: Improve the relative phase difference by using the calibration gap, e.g. 30 degree, but the conditions are limited as in current TS 38.101-2, e.g. no BWP switching, no measurement gap. Proposal 2: RAN4 confirms the work on performance gain introduced by coherence calibration gap (i.e. power and phase maintain between UE RF chains) can be concluded. Proposal 3: RAN4 confirms the work on performance loss caused by UL gaps for coherence is concluded, there is No performance loss caused by UL gaps configured for coherence calibration. |
| R4-2114555/R4-2114577 | Qualcomm | Observation 1: It is unclear how to communicate to the UE which supports UL gaps that it is under normal conformance testing and needs to set the P-MPR=0. Observation 2: How to test the functionality of this feature is unclear and more information is needed from the proponents and discussion in ran4 how this can be concluded And following proposals: Proposal 1: When UE declares support for UL gaps, it is mandatory for UE to also support mpe-Reporting-FR2 Proposal 2: 6 dB EIRP improvement requirement is defined as RF test for this feature when UL gaps are configured |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1: Performance and test metric

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 1-1-1: Metric used to evaluate the performance gain when UL gap is configured

- Proposals

  - Option 1: Delta P-MPR reported in existing PHR signaling (Apple, Nokia, vivo, ZTE)

  - Option 2: Peak EIRP measurement (OPPO)

  - Option 3: both P-MPR report and peak EIRP measurement (Qualcomm)

- Recommended WF

  - TBA

Issue 1-1-2: Performance gain

Moderator notes: the proponent of each options to provide the associated side condition including waveform, UL duty cycle,  max EIRP, etc.

- Peak EIRP based:

  - Option  1: 6dB

  - Option  2: depending on the “typical” value of in field, where “typical” value is TBD

  - Option 3: to be further discussed, e.g. 1dB or 2dB

- P-MPR report based

- Option 1: different reported value

### Sub-topic 1-2: Test methodology and phantom/blocking

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-2-1: Test methodology

In  R4-2112635, four types of UE are identified as

| Type of UE | P-MPR value when human targets are not close to Tx antenna |  | NOTE |
| --- | --- | --- | --- |
|  | Without UL gap | With UL gap |  |
| A | High | High | - UE cannot implement the human detection even if UL gaps are configured |
| B | Mid / Low | Mid / Low | - UE that shows good performance without UL gap and does not require any UL gaps |
| C (Targeted UE) | High | Low / Very Low | - UE that don't shows good performance without UL gap, so it requires any UL gaps for improvement |
| D | Mid / Low | Low / Very Low | - UE that shows good performance without UL gap, but it requires any UL gaps for further improvement. |

- Proposals

  - Option 1: Based on P-MPR reporting without phantom to distinguish UE type A, B and C.

| Test | Blocking/phantom | UL gap | Type of UE |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | A | B | C |
| Step 1 | No | Yes | High | Low | Low |
| Step 2 | No | No | High | Low | High |

  - Option 2: Based on P-MPR reporting with phantom to distinguish UE type A, B, C and D

| Test | Blocking/phantom | UL gap | Type of UE |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | A | B | C | D |
| Step 1 | No | No | High | Low | High | Low |
| Step 2 | No | Yes | High | Low | Low | Very Low |
| Step 3 | Yes | No | High | High | High | High |

  - Option 3: Based on peak EIRP reporting without phantom

  - Option 4: Based on hybrid of peak EIRP reporting and P-MPR reporting with phantom

| Check | Description | If not performed |
| --- | --- | --- |
| Regular conformance | All test cases from Rel-16 | No conformance acceptance |
| EIRP test 1 | UE EIRP test without gaps in live mode. In test lower EIRP performance should be observed | See below |
| EIRP test 2 | UE EIRP test with gaps in live mode, EIRP improvement of [6] dB should be observed and EIRP should meet conformance criteria. No phantom/blockage of antenna | Test 1 and 2 should show improvement. Otherwise UE is not taking advantage of gaps. |
| EIRP test 3 | UE EIRP test with phantom or other means to block antenna. EIRP reduction should be observed and P-MPR report accordingly | UE may not perform MPE scan and functionality is not according to gap justification. Possible improper use of P-MPR |

  - Option 5: Based on P-MPR reporting without phantom, but with RADAR jamming

  - Option 6: Leave the test method design to RAN5

- Recommended WF

  - TBA

Issue 1-2-2: Feasibility to introduce phantom in FR2 test in Rel-17

List of potential discussion points raised in the contributions

- Material including coating material

- Impacts on the existing FR2 test environment, e.g. antenna radiation pattern, quiet zone, etc.

- Blocking effects

- Measurement uncertainty

- Impact of the size and positioning of the phantom

### Sub-topic 1-3: Other related issues

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-3-1: “Default/baseline” P-MPR that can be expected in regular operation in field?

Issue 1-3-2: Is the periodicity of PHR/P-MPR reports are sufficient to provide valid feedback for BPS calibration gaps?

## Companies views’ collection for 1st round

### Open issues

Issue 1-1-1: Metric used to evaluate the performance gain when UL gap is configured

- Proposals

  - Option 1: Delta P-MPR reported in existing PHR signaling (Apple, Nokia, vivo, ZTE, Qualcomm)

  - Option 2: Peak EIRP measurement (OPPO, Qualcomm)

| Company | Comments |
| --- | --- |
| OPPO | Option 2, peak EIRP based measurement. For Option 1, we understand the benefit of using PMPR reporting is the simplicity in testing. However, in current PMPR reporting, it is 3dB step based and the value range is limited to 12dB as below figure shows. That means for the following two cases, no gain can be observed: - Case 1: The delta PMPR gain for with or w/o gap configured is within 3dB, then same PMPR will be reported, i.e. gain cannot be observed; - Case 2: If the PMPR absolute value is larger than 12dB, e.g. 20dB, then P-MPR_03 is reported and even the gain is 5dB, UE still report P-MPR_03, and no gain can be observed by configuring UL gap. Due to above two issues, our understanding is peak EIRP measurement is better in consistent gain observation. |
| Apple | Option 1. We see delta P-MPR is a straightforward method to test the UL gap performance gain. |
| ZTE | Option 1. Delta P-MPR is the most straightforward metric to verify the performance gain. |
| Qualcomm | Option 3 provides test that UE behaves the way it reports. We do not see why some proponent do not want the EIRP to be tested since it is obvious that EIRP should improve with the gaps. |
| Sony | Option 1 is more feasible to verify the proposed use case (BPS). Though the resolution of P-MPR reporting is not very high, it should be sufficient to verify the P-MPR improvement based on our estimated P-MPR value is at least 3-4 dB from a typical device. |
| Nokia | In our contribution we proposed P-MPR reporting with and without blocking as metric aligned with the earlier RAN4 agreement. Since some companies see that the support of phantom is too challenging within the Rel-17 time frame, we see that combined peak EIRP and P-MPR comparison could then be an alternative metric without phantom. The actual details of the signaling should be decided once the requirements and testing details are agreed. It is not clear what is meant with “Delta P-MPR” in the option 1 as it is not clearly defined. For example, “comparing different P-MPR values” may be more clear. The current MAC specification defines mpe-Reporting-FR2 reporting, which we should try to re-use. In the requirements and tests, it is important to ensure that clear performance gain in terms of reported P-MPR is achieved when the UE uses UL gaps for MPE proximity detection. As such, minimum gain of 3 dB is acceptable (i.e. P-MPR_00). However, the case 2 observed in OPPO’s comment is also a concern for us, particularly if no phantom can be used during the test. Therefore, to address this concern and to avoid the usage of phantom which may be challenging and delay the work, we support that the metric used to evaluate the performance gain is based on a combination of the Peak EIRP measurement and the comparison of P-MPR values. Thus, we support Option x: combined peak EIRP and P-MPR comparison (and then in the requirements improved requirements with UL gaps) |
| DOCOMO | Option 1. Delta P-MPR is a straightforward metric. In addition, if the absolute value of P-MPR can be measured, it may be possible to evaluate the performance of the UE without UE gap. |
| vivo | Option 1, delta P-MPR is more straightforward. |
| Ericsson | Option 1 Delta P-MPR, However the limits of reporting P-MPR, i.e. 3dB resolution and max 12 dB is a concern. Hence, we are open for including EIRP if possible from a testing P.O.V (in line with Qualcomm and Nokia) |
| Intel | Our preference is Option 1 as it is the most direct way, though we think greater granularity may need to be discussed |

Issue 1-1-2: Performance gain

- Peak EIRP based:

  - Option  1: 6dB

  - Option  2: depending on the “typical” value of in field, where “typical” value is TBD

  - Option 3: to be further discussed, e.g. 1dB or 2dB

  - Moderator notes: the proponent of each options to provide the associated side condition including waveform, UL duty cycle,  max EIRP, etc.

- P-MPR report based

  - Option 1: different reported value

| Company | Comments |
| --- | --- |
| OPPO | For peak EIRP based, Option 3 is ok. And the exact value actually in our view can be 2dB based, since any Tx power improvement is not easy and has benefit in extending UL coverage. |
| Apple | Support P-MPR report-based Option 1. P-MPR gain would depend on the side conditions including waveform, UL duty cycle and max EIRP, etc. Further, implementation margins also need to be considered. In the case of BPS, implementation margin is decided based on RF impairments and Tx power variations due to temperature variation, baseband estimation/detection inaccuracies, etc. The eventual P-MPR gain requirements need to be decided jointly based on side conditions and implementation margins. |
| ZTE | FFS |
| Qualcomm | In order to ensure there are gains for this feature, peak EIRP improvement should be specified. 6 dB was used to justify this feature by Apple so 6 dB should be used as criteria. |
| Sony | Option 2. The value should be estimated based on real devices performances. A typical 3-4 dB P-MPR value is estimated in our simulations with a 100% duty cycle with 26 dBm peak EIRP UE with CW. |
| Nokia | If phantom phased testing cannot be supported in Rel-17, in our view requirements for both peak EIRP and P-MPR report are needed. For both P-MPR and peak EIRP it is important that clear performance gains are achieved when UL gaps is used for proximity detection. It is necessary define requirement limits how much less P-MPR needs to be required when utilizing UL gaps. Similarly improved peak EIRP requirements are needed when UL gaps are utilized. If gap length and density are high, then even bigger requirement improvements for P-MPR and peak EIRP are necessary to justify UL gaps. The proposed option 1 above “Option 1: different reported value” is not clear. It is not sufficient that reported P-MPR value is just different. The difference i.e. gain in the requirements and tests to be sufficiently large, at least 6 dB. We support Peak EIRP option 1: with at least 6 dB improvement and P-MPR report option x: with at least 6 dB improvement (in the case that there is no phantom in the test setup) |
| DOCOMO | Support P-MPR report-based Option 1. (In addition, apart from this test, the UE performance without UL gap is evaluated by “absolute value of P-MPR (reported value)”. This is to evaluate UEs that show good performance without UL gap.) |
| vivo | - P-MPR report based In R16, we have discussed whether the PMPR=0 should be introduced to indicate that the UE is back to the normal state, but considering this state can also be derived from P-bit, the reporting of PMPR=0 is abandoned. For the performance gain of gap, the PMPR=0 may be needed to reflect the change of the reported value. |
| Ericsson | Option 1 &2. Indeed 6dB was used for justification but also typical field values should be counted for. No less then 3dB so Option 3 is not agreeable. |
| Intel | P-MPR report based For Option 1 (different reported value), considering the granularity and ranges, if the reported values are consecutive (e.g., P-MPR_00 and P-MPR_01), the gain may be as low as 1 dB and as high as 5 dB (assuming whole numbers). For this case, to ensure the gain is larger than 1 dB, greater granularity will be needed. This also helps with the two cases captured by OPPO in Issue 1-1-1. |

Issue 1-2-1: Test methodology

- Option 1: Based on P-MPR reporting without phantom to distinguish UE type A, B and C.

- Option 2: Based on P-MPR reporting with phantom to distinguish UE type A, B, C and/or D

- Option 3: Based on peak EIRP reporting without phantom

- Option 4: Based on hybrid of peak EIRP reporting and P-MPR reporting with phantom

- Option 5: Based on P-MPR reporting without phantom, but with RADAR jamming

- Option 6: Leave the test method design to RAN5

| Company | Comments |
| --- | --- |
| OPPO | Option 3 and Option 6. The test method design actually is RAN5 responsibility instead of RAN4. |
| Apple | Option 1. Challenges to introduce phantom are listed in issue 1-2-2 phantom discussion. The test cases should be developed with primary focus on how to show sufficient gain between with and without UL gap configured. We think both option 1 and 2, which are based on P-MPR reporting, can serve the purpose. However, due to the uncertainty to introduce phantom within R17 time frame, option 1 is preferred. Without phantom, an example shown below can be used to test device type A, B, C. Step; P-MPR of UE with BPS (Device C); P-MPR of UE with other non-UL gap dependent sensor (Device B); P-MPR of UE without BPS or other sensors (Device A). T1, gap activated, no blocking; Low; Low/medium; High. T2, gap de-activated, no blocking; High; Low/medium; High. |
| Huawei, HiSilicon | Generally support option 1. For UEs indicate not support PMPR reporting, use peak EIRP measurement. It can decouple the relation between PMPR reporting and BPS. For Ues indicate support PMPR reporting, directly test on PMPR. |
| ZTE | Support Option 1. Which can verify the performance gain given by BPS method required gap, not by other BPS method not required gap. |
| Qualcomm | There are challenges with phantom testing but the test procedure should be a combination of options. Three functionalities/performances should be tested: UE EIRP improvement and P-MPR reporting with and without gaps, UE MPE scan functionality with and without gaps, as this is the feature we are building. |
| Sony | Based on P-MPR reporting without phantom. |
| Nokia | We support P-MPR and peak EIRP based requirement and testing definitions but RAN4 still needs to have detailed discussion how to ensure that it can be verified that the UE actual provides the intended requirement gains without using phantom. Like discussed earlier we support a combination of option 1 and option 3 (when considering that phantom is seen too challenging to support in Rel-17) |
| DOCOMO | Option 1 and Option2 Option 2 is more preferred, but there are testing complexity and problem of phantom. For Option 1, we need not only delta P-MPR but also absolute values of P-MPR to distinguish between UE type A and UE type B. |
| vivo | Slightly prefer option 2 but option 1 is also acceptable. The phantom/blocking is helpful to distinguish whether the gain comes from gap itself, but considering that there is no such precedent and many details are unknown, if the different UE can be distinguished by not introducing phantom, then option 1 is also acceptable |
| Ericsson | Option 1 and possibly option 3, see our answer on Issue 1-1-1 |
| Intel | Based on P-MPR reporting without phantom |

Issue 1-2-2: Feasibility to introduce phantom in FR2 test in Rel-17

| Company | Comments |
| --- | --- |
| OPPO | Introduce is doable but not reliable, thus not recommended. |
| Apple | There are a lot of challenges to introduce phantom in FR2 test in R17. Given the current timeline, we think it is very challenging to introduce phantom test in R17 time frame. - Introducing phantoms into 3GPP FR2 OTA testing will require careful study and potentially a new testing methodology in the following respects: - Testing with Phantoms is a new paradigm in 3GPP. Even though other SDOs have some framework for sub-6GHz OTA testing with phantoms, which by itself continues to be under investigation, this is not investigated for FR2. As such, a dedicated study is required. - So far for sub-6GHz OTA testing, the main metrics have been TRP and TIS. While EIRP is measured in different directions to obtain TRP, the measurement of peak EIRP is not the focus and thus is not optimized. To be able to accurately measure peak EIRP, the properties of phantoms including its material, dimensions, and dielectric properties at the target FR2 operating frequencies need to be studied. For instance, the common understanding is mmWave signals do not penetrate deeply into human body, mostly likely the penetration is only a few millimeters, so the coating of phantoms will play a critical role in determining the RF property of phantoms (used to emulate human skin) such as reflection coefficients. It is worth noting that phantom packages targeting OTA testing of handheld devices inside compact antenna test ranges (CATR), at frequencies above 6 GHz, are currently under evaluation by CTIA [6]. - Furthermore, the introduction of phantom will affect the testing environment, including DUT antenna radiation pattern, the required size of quiet zone, etc., which may require reconfiguration/re-design of the layout of testing chamber. - The blocking effect of phantoms: When a phantom is placed right in between the DUT and link antenna, it may completely block the communication link in the test. During the test, link quality should be maintained so UE can correctly decode UL DCI for PUSCH transmission, and successfully transmit PHR back to gNB. - Reassessment of Measurement Uncertainty (MU): In current sub-6GHz OTA testing, even the mechanical support structure that is required to hold the DUT or head/hand phantom during a test causes interference while testing and needs to be calibrated for accounting in the tests. As a result, there will be an increase in the MU due to impact of phantoms on radiation pattern, calibration of pathloss, etc., that requires further assessment. The amount of increase in MU needs to be analyzed and in the best case can be assumed to be substantial. - In addition, there are no other FR2 RF test requirements that have been identified to be tested with phantoms. Therefore, introduction of phantoms for a few test points involving UL-Gaps has no value-addition to the conformance ecosystem. All it would entail is complex study item(s) with very little benefit in return. The focus can instead be on identifying alternatives. |
| Huawei, HiSilicon | Phantoms introducing makes the test highly complex. With comparing on PMPR or EIRP with and without gap, it already clearly show the performance gain by BPS, thus it is not necessary to introduce additional complexity. We prefer to simplify the feature test which has benefit to NR FR2. |
| Qualcomm | Phantom indeed has challenges and lot to discuss. If phatom testing is not introduced, how will proponent test the feature for themselves? What is the purpose of this feature that is meant to enable MPE scan in 3GPP environment? |
| Sony | Multiple challenges appear if body phantom needs to be introduced, where they have been briefly listed in R4-2113661. Therefore, we think it is not feasible to introduce phantom in FR2 in Rel-17. |
| vivo | Considering the issue listed by Apple above, the introduction of phantom does face many challenges and hard to complete in R17. |
| Intel | The complexities of introducing phantom testing merit a detailed study with RAN5 involvement. At this stage, completing the study within Rel-17 timeframe is not feasible. |

Issue 1-3-1: “Default/baseline” P-MPR that can be expected in regular operation in field?

| Company | Comments |
| --- | --- |
| OPPO | This is UE implementation specific issue, in our view, no default/baseline value. |
| Apple | Default/baseline is not easy to be quantified. In general P-MPR is jointly determined by peak EIRP, which varies depending on the distance between UE and BS, UL duty cycle and the waveform, etc. Meanwhile, UE’s implementation may also play a role here. In general, with a pre-determined side conditions as aforementioned, P-MPR can be roughly estimated. |
| Qualcomm | Default is zero. P-MPR is used if needed |
| Sony | A typical 3-4 dB P-MPR value is estimated in our simulation with a 100% duty cycle with 26 dBm peak EIRP UE with CW. |
| DOCOMO | If it is possible, we would like to quantify the baseline to evaluate the UE performance. |
| vivo | It depends on the UE implementation. |

Issue 1-3-2: Is the periodicity of PHR/P-MPR reports are sufficient to provide valid feedback for BPS calibration gaps?

| Company | Comments |
| --- | --- |
| XXX |  |
| Apple | We would like to know more what “sufficient” mean in this context. In current specification 38.331, PHR-Config, wide range of timers value can be configured. Network has sufficient flexibility to configure proper configuration. Phr-PeriodicTimer ENUMERATED {sf10, sf20, sf50, sf100, sf200,sf500, sf1000, infinity}, phr-ProhibitTimer ENUMERATED {sf0, sf10, sf20, sf50, sf100,sf200, sf500, sf1000}, phr-Tx-PowerFactorChange ENUMERATED {dB1, dB3, dB6, infinity}, MPE-Config-FR2-r16 ::= SEQUENCE { mpe-ProhibitTimer-r16 ENUMERATED {sf0, sf10, sf20, sf50, sf100, sf200, sf500, sf1000}, mpe-Threshold-r16 ENUMERATED {dB3, dB6, dB9, dB12} } |

### CRs/TPs comments collection

For close-to-finalize Wis and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing Wis, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 1-1-1: Metric used to evaluate the performance gain when UL gap is configured | - Option 1: Delta P-MPR reported in existing PHR signaling (Apple, Sony, vivo, ZTE, Ericsson, Intel, DCM) - Impacts of P-MPR report granularity should be discussed. - Option 2: Peak EIRP measurement (OPPO) - Option 3: P-MPR report+peak EIRP without phantom (Nokia, Ericsson) - Option 3a: P-MPR report+peak EIRP (Qualcomm) Recommendations for 2nd round: - Option 1: P-MPR report - Impacts of P-MPR report granularity should be discussed. - Option 3: P-MPR report+peak EIRP without phantom |
| Issue 1-1-2: Performance gain | - Option 1: 6dB for peak EIRP (Qualcomm, Ericsson) - Option 2: depending on the “typical” value of in field, where “typical” value is TBD for peak EIRP (Sony, Ericsson) - Option 3: to be further discussed, e.g. 1dB or 2dB for peak EIRP (OPPO) - Opiton 4: different P-MPR reported value for P-MPR reporting (apple, DCM, vivo) - Option 5: Peak EIRP option 1: with at least 6 dB improvement and P-MPR report option x: with at least 6 dB improvement (nokia) Recommendations for 2nd round: Depending on the agreements in issue 1-1-1, - Option 1: If “P-MPR report” is agreed, X dB P-MPR enhancement should be achieved. - X is defined as the relative gain of the reported P-MPR between with and without UL gap activated, which can be chosen as one of the options below - 6dB - A value between 6dB and 3dB, which is typical in the field - A value below 3dB - FFS on the impact of P-MPR report granularity - FFS on the implementation margin - Option 2: If “P-MPR report+peak EIRP without phantom” is agreed, X dB EIRP gain and P-MPR<YdB should be achieved. - X is defined as the relative gain of peak EIRP between with and without UL gap activated, which can be chosen as one of the options below - 6dB - A value between 6dB and 3dB, which is typical in the field - A value below 3dB - Y is defined as the maximum reported PMPR when UL gap is activated - A value below 3dB - 0dB - FFS on the implementation margin |
| Issue 1-2-1: Test methodology | - Option 1: Based on P-MPR reporting without phantom to distinguish UE type A, B and C.(apple, Huawei, ZTE, Sony, DCM, vivo, Ericsson, Intel) - Option 2: Based on P-MPR reporting with phantom to distinguish UE type A, B, C and/or D (DCM, vivo) - Option 3: Based on peak EIRP reporting without phantom (OPPO, Ericsson) - Option 4: Based on hybrid of peak EIRP reporting and P-MPR reporting with phantom (Qualcomm) - Option 5: Based on P-MPR reporting without phantom, but with RADAR jamming - Option 6: Leave the test method design to RAN5 (OPPO) - Option 7: combination of option 1 and 3 Recommendations for 2nd round: - Depending on the decision on issue 1-1-1, test method can be further decided. |
| Issue 1-2-2: Feasibility to introduce phantom in FR2 test in Rel-17 | - No phantom is introduced (OPPO, apple, Huawei, sony, vivo, intel, ) - Phantom should be introduced (qualcomm) Recommendations for 2nd round: - Recommended agreement: No phantom is introduced in R17 |
| Issue 1-3-1: “Default/baseline” P-MPR that can be expected in regular operation in field? | - UE implementation specific (OPPO, apple, vivo) - 3-4dB with 100% duty cycle and 26dBm peak EIRP (Sony) - Default is zero (Qualcomm) Recommendations for 2nd round: Merge this discussion with issue 1-1-2 |
| Issue 1-3-2: Is the periodicity of PHR/P-MPR reports are sufficient to provide valid feedback for BPS calibration gaps? | - No sufficient inputs |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

With the agreement on “P-MPR report+peak EIRP without phantom”, the following issues can be further discussed

Issue 1-4-1: the range of X, which is defined as the relative gain of the measured peak EIRP between with and without UL gap activated

- Option 1: at least 6dB

- Option 2: a value between 3dB and 6dB, which is typical in the field

| Company | Comments |
| --- | --- |
| Apple | Option 2. It is worth clarifying what has been evaluated in phase I study. The evaluation is based on 2-6dB power gain, and 0.25% to 5% gap overhead. Below is the SLS simulation results in R4-2104849. Throughput gain analysis based on different Tx power gain with 5% gap overhead Gap overhead = 5%; DDDSU; DDSUU. 3dB Tx power gain; 2.74%; 19.86%. 4dB Tx power gain; 10.29%; 28.68%. 5dB Tx power gain; 22.95%; 43.44%. 6dB Tx power gain; 44.23%; 68.27%. It has been shown that when gap overhead is 5%, positive throughput gain can be achieved based on 3-6dB power gain in both DDDSU and DDSUU. Obviously, the more tx power gain, the more throughput put gain. Since the requirement is minimum performance requirement, we think option 2 is reasonable considering additional implementation margin. If UE can achieve higher >6dB Tx power gain, it will have better performance comparing to the minimum requirement. |
| Qualcomm | We should remember that this is not 6 dB improvement over the rel-16 UE which was the baseline since rel-16 does not use any P-MPR for conformance. If UE gets to choose the amount of P-MPR it uses for MPE compliance, why the limit is so important? Couldn’t UE that needs the gaps set the baseline P-MPR very high without gaps and then be ok with any number. 6 dB makes the measurement much easier given all the tolerances in OTA measurements. Option 1 |
| ZTE | We are still ambiguous why we need both peak EIRP and P-MPR as the metric. P-MPR is the most straightforward metric, except for the issues of granularity and upper limit, is there any other reason? |
| vivo | We prefer option 2, in our understanding, the UL gap here is mainly used to more accurately assess the P-MPR actually needed to avoid unnecessary power back-off, and the P-MPR improvement larger than 6 dB is not typical case in the field. |
| Nokia | Option 1. In our view only significant gains like at least 6 dB justify the introduction of UL gaps. It is also important to note that for UL gaps rather frequent and long gaps are proposed and therefore it is even more important that gains are significant. |

Issue 1-4-2: Definition of Y

- Option 1: relative gain of the reported P-MPR

- Option 2: absolute P-MPR with UL gap activated

- Option 3: No Y specified.

| Company | Comments |
| --- | --- |
| Apple | Option 1 is OK for us. Based on our understanding of Monday’s GTW discussion, it seems that some companies want to consider both peak EIRP and P-MPR, in which the peak EIRP difference (relative gain) and P-MPR reporting difference (relative gain) can be verified against each other. Option 2 can provide an upper bound for P-MPR when UL gap is activated. We are also OK not to test it if there is a concern. Based on the comments in Monday’s GTW, the absolute value of Y might be hard to define or agree to, depending on UE implementations. In our understanding, it is the relative gain that really matters. |
| Qualcomm | What is an absolute level of P-MPR? P-MPR is in dB and dB is relative unit. Maybe Apple could share the specification language so we could understand this option better? In the 3GPP requirements, the P-MPR with gaps should be zero for this UE. With out gaps, the P-MPR can be set to anything, and based on issue 1-4-1, we propose it is >= 6 dB which is the same as observed EIPR improvement. Why would observed EIRP improvement be different than P-MPR chance? |
| ZTE | Support Option 1. Considering different UEs may have different P-MPR for the case of without UL gap, so we can not sure all UEs can obtain similar gain with the help of BPS during UL gap. Relative value can show the gain between without UL gap and with UL gap. |
| Nokia | Option 1: relative gain of the reported P-MPR |
| DOCOMO | Support Option 2. In deciding whether to introduce the UL gap, it is valuable for us that the improved final P-MPR values are guaranteed for all UEs configured UL gaps. We are interested in how small the P-MPR values are by configuring the UL gap. Also, we wonder if it is appropriate that UEs with high P-MPR despite the UL gap being active meet the requirements. How many such UEs are there? (Are they the majority?) In addition, we hope to distinguish between four UE types (A-D) below properly. Type of UE; P-MPR value. w/o UL gap; w/ UL gap. A; High. B; Mid / Low. C; High; Mid / Low / Very Low. D; Mid / Low; Low / Very Low. For example, suppose the P-MPR value is the following (i.e. underlined status in the table above): - ["A w/o gap" = "C w/o gap"] > ["B w/o gap" = "D w/o gap" = "C w/ gap"] > ["D w/ gap"] In this cases, if we select Option 1, we can only know the following: - "C w/o gap" > "C w/ gap" - "D w/o gap" > "D w/ gap" Therefore, we can see an appropriate relationship by measuring the following two absolute P-MPR value [dB]. - Additional test 1: P-MPR values of UE (all type) w/o gap - Additional test 2: P-MPR values of UE (B/D) w/ gap The former (test1) can be evaluated by another new test and the latter (test2) can be evaluated by Option2. If the latter (test2) is also evaluated by another new test, Option 2 may not be needed. |

# Topic #2: UL gap for coherence calibration

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2112808 | Nokia | Observation 1: The test needs to ensure that UL gaps are used for body proximity sensing and P-MPR improvement with MPE events. Proposal 1: Support Option 1 with blocking as first priority if further discussion on the type of material that would trigger only radar is conclusive. Proposal 2: Do not support Option 2. EIRP is not a reliable metric and P-MPR report is sufficient. Proposal 3: Support Option 3with further discussion on jamming possibility as second priority, if consensus cannot be found regarding option 2. Proposal 4: Consider an extra test to validate Ues fulfill current requirements even without UL gaps scheduled. Observation 2: Another test may be defined for Ues embedding other means than radar for body proximity sensing. Proposal 5: Further discussion how to define UE core requirements in addition to the testing aspects. Observation 3: Gains should be shown between coherent MIMO without the gaps and coherent MIMO with the gaps. |
| R4-2113004 | vivo | Observation 1: The UL gap may help UE to enable higher capability but current coherent MIMO capability structure cannot indicate the change clearly. Observation 2: The calibration of coherent MIMO is related to signal itself to ensure that the amplitude and phase error of the port meets the requirements. Proposal 1: The performance gain of coherent MIMO calibration can be shown by the side condition in current spec will be removed if the UL gap is configured. Proposal 2: New UE capability for coherent MIMO with UL gap should be introduced, e.g., pusch-TransCoherenceWithULgap {partialCoherent, fullCoherent}, to clarify the capability improvement of the calibration by UL gap. Proposal 3: It is suggested that the chip vendor confirm whether the coherent MIMO calibration can always be performed in the baseband at the same time as the PUSCH transmission, to confirm whether the UL gap is needed. Proposal 4: If the gap is needed, type 2 gap should be used for coherent MIMO calibration. |
| R4-2113212 | ZTE | Proposal 1: Overcoming the switching mentioned in Category 2 with the help of UL gap, which will be challenged by the Time Window requirement belonging to Category 1. Proposal 2: Compared with calibration using scheduled transmission signal, if the benefits of using type 2 UL gap can be verified, it is meaningful to calibrate coherent UL MIMO within type2 UL gap. Proposal 3: If no obvious gain can be justified between with UL gap case and without UL gap case for PA/Transceiver calibration, remove these use cases. |
| R4-2113662 | Ericsson, Sony | Observation 1: RAN4 shall follow the directive from the meeting report from RAN4#98-bis “RAN4 will not define any requirements until the corresponding testing methodology for the performance enhancement is clear” Observation 2: No “normal” gaps are needed to be pre-configured over RRC. Observation 3: The details of the UL calibration is not fully described, e.g. number of slots used by the UE. Observation 4: It’s indicated in [2] that there is no impact on performance loss caused by the calibration in UL. However, given observation 2 above, we believe that it’s too early to state no performance impact. Observation 5: There is no gain in performance using the calibration in comparison with legacy coherent UL MIMO. Proposal 1: Agree on the list of open issues. Based on the suggested solution in [2] we list the following list of open issues: - The usage of UL slots for UE calibration (for coherent UL MIMO) must be synchronized between gNB and UE. This will need RAN1 involvement if it will be done via e.g. codebook usage. Hence this has a possible impact on RAN1 specifications. - The details of e.g., the length (in UL slots) for a calibration must be settled/agreed between RAN4 and RAN1. Identifying any gain of the proposed calibration method compared to legacy coherent UL MIMO requirements |
| R4-2114492 | Huawei, HiSilicon | Observation1: there is no RF requirements for coherent UL MIMO under multiple conditions, including DRX on, BWP switching, SRS switching, DL measurement gap, etc. Proposal 1: UE requirement improvement by using coherence calibration gap can go with one of following options: • Option 1: Specify 40 degree relative phase difference and 4dB power difference for side conditions including DRX on, BWP switching, SRS switching, DL measurement gap, etc. • Option 2: Improve the relative phase difference by using the calibration gap, e.g. 30 degree, but the conditions are limited as in current TS 38.101-2, e.g. no BWP switching, no measurement gap. Proposal 2: RAN4 confirms the work on performance gain introduced by coherence calibration gap (i.e. power and phase maintain between UE RF chains) can be concluded. Proposal 3: RAN4 confirms the work on performance loss caused by UL gaps for coherence is concluded, there is No performance loss caused by UL gaps configured for coherence calibration. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1: performance gain

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-1-1: Performance gain and side conditions

- Nokia: The gain should be shown between coherent MIMO without the gaps and coherent MIMO with the gaps.

- vivo:

  - Option 1: If the calibration with gap is introduced, the requirement for relative difference of phase/power can be reduced.

  - Option 2: If the calibration with gap is introduced, the requirements remain the same, but the side condition can be removed.

- Ericsson: There is no gain in performance using the calibration in comparison with legacy coherent UL MIMO.

- Huawei: UE requirement improvement by using coherence calibration gap can go with one of following options:

- Option 1: Specify 40 degree relative phase difference and 4dB power difference for side conditions including DRX on, BWP switching, SRS switching, DL measurement gap, etc.

- Option 2: Improve the relative phase difference by using the calibration gap, e.g. 30 degree, but the conditions are limited as in current TS 38.101-2, e.g. no BWP switching, no measurement gap.

- Secondly, we consider whether 40 degree and 4dB requirement need to enhance further, which requires for simulation to prove there is corresponding performance improvement. We provide the simulation results here:

- Fig 2 performance gain by reaching 20 phase error and 40 degree phase error

- It is observed there is about 8% mean TP gain and max TP gain when compared between 20 degree and 40 degree phase error. With calibration gap, we can expect UE improves on the relative phase difference.

Moderator notes: companies are encouraged to provide your views on performance gain with detailed technical justifications.

Issue 2-1-2: Initial Gap examples to get the gain

- Example 1(Huawei R4-2114492): the calibration gap could be configured at the first X symbols(X can be 2OS) of the PUSCH transmission.

- Example 2(Huawei R4-2114492): gap period can be also outside the PUSCH transmission, it occupies additional symbols, which is similar mechanism as gap for BPS.

- Others

Companies are encouraged to provide your views on the examples of gap design to get the gain, and other scheme of gap design is not precluded.

## Companies views’ collection for 1st round

### Open issues

Sub topic 2-1: performance gain of coherent MIMO with UL gap

| Company | Comments |
| --- | --- |
| OPPO | Issue 2-1-1: Performance gain and side conditions Our understanding is there is gain in enabling coherent UL MIMO with UL gap configured. Currently there is no UE can support coherent UL MIMO in our knowledge. Issue 2-1-2: Initial Gap examples to get the gain Example 1 is preferred. |
| Apple | On performance gain, we acknowledge the gain for UL coherent MIMO due to calibration gap. We see both options proposed by vivo and Huawei can be considered for requirement improvement. |
| Huawei, HiSilicon | Issue 2-1-1: Performance gain and side conditions Firstly, in our understanding, removing the side conditions in the requirement, can clearly show the requirement improvement. we provide the comparing table before and after gap config: Gap config; Side conditions defined in Rel-15/16/17 spec for coherent UL MIMO; Relative phase error; Power imbalance. No; side conditions happen(e.g. switching happens); >40 degree; >4dB. Yes; side conditions happen(e.g. switching happens); <=40 degree; <=4dB. Furthermore, we found from simulation if we further decrease the relative phase error, e.g. 30 degree, we can see UL performance gain compared with 40degree, while power imbalance improvement has limited help on furtther UL performance gain. So there is an enhanced way to show the gain introduced by gap: Gap config; Side conditions defined in Rel-15/16/17 spec for coherent UL MIMO; Relative phase error; Power imbalance. No; side conditions happen(e.g. switching happens); >40 degree; >4dB. Yes; side conditions happen(e.g. switching happens); <=30 degree; <=4dB. We believe with either way provided above, it can prove requirement improvement. Issue 2-1-2: Initial Gap examples to get the gain We would like to further explain on the examples provided here: Example 1 actually requires for real time phase error detection, and compensation. The problem is, if we use the DMRS configured in following symbols, the gap symbols has risk on channel estimation. But we still can configure DMRS to gap symbols to ensure gap symbol performance. The benefit is, it will not occupy additional gap that configured to other users. However it requires for specific DMRS configuration. Example 2 requires for additional gap outside transmission. If side conditions happens between last transmitted SRS and PUSCH transmission, UE can use this gap to calculation and compensate the phase difference between SRS and PUSCH. We are open to discuss on other better solutions. |
| ZTE | For Example 1, In fact, the UL data transmitted in the first two symbols may not be correctly decoded due to the inter-layer interference, after all UE is still doing coherent calibration during the two symbols. So which may affect the BLER of the entire PUSCH. Therefore, evaluation verification is needed. |
| Sony | In Example 1 of calibration gap configuration, it seems there is no “gap” actually required. The main spec impact is the DMRS configuration which in our view is mainly a RAN1 issue. |
| Nokia | It is still unclear for the proposed alternatives (based on the contribution) how it is ensured that when UE uses UL gaps, the UE achieves better throughput in the requirements. For instance in Huawei’s option 1 (Specify 40 degree relative phase difference and 4dB power difference for side conditions including DRX on, BWP switching, SRS switching, DL measurement gap, etc) is the intention to define requirements such that for UE using UL gaps the throughput requirements are improved? Based on the Huawei comments above it would seem that no large throughput improvement is ensured or proposed with UL gaps. In such case UL gaps for this use case do not seem to be justified. |
| vivo | Issue 2-1-1: Performance gain and side conditions Both option 1 and option 2 that we proposed are acceptable. If the option 1 is chosen, we should further discuss how much improvement of the amplitude/phase difference is reasonable, and the original test process can be reused; if the option 2 is chosen, only the new test for side conditions should be introduced. From the perspective of expanding the applicability of coherent MIMO, we slightly prefer option 2. In addition, if the gap can help the UE achieve higher capability (e.g., non-coherent => partial coherent, partial coherent => full coherent), in our understanding, this can also be seen as a “gain”. Issue 2-1-2: Initial Gap examples to get the gain Example 1 is preferred. Considering the coherent MIMO need to calibrate the phase/amplitude, Using the “silent” gap (example 2) may means additional inner feedback is required, and it is not expected due to the higher complexity. |
| Ericsson | Issue 2-1-2: Initial Gap examples to get the gain For the two Examples provided it’s not clear how this would work. Example 1: Is there actually any GAP needed if UE shall send PUSCH in those red slots? Seems more like a DMRS config sync between UE and gNB (issue totally up to RAN1 of that’s the case). Will not the PUSCH be affected during those gaps as indicated by ZTE Example 2: Outside the PUSCH so this is a gap, Should the UE send some training sequence used for calibration on during the gap? The NW impact might be less for example1. |
| Qualcomm | To reflect Nokia comment, even the UE behavior is understood in principle, the benefit of this improvement is not clear. Say UE without this capability would lose the coherence under one of the side conditions, so how often the side condition occurs? And if side condition is waived, how much benefit there is. However, same could be said for the MPE based UL Gap, the benefit between the implementation that meets conformance with zero P-MPR and the one that needs gaps in the environment where human is blocking the antenna with some probability is not so clear, So if these gaps are defined, they are equally beneficial so maybe this gap to waive coherence side conditions is equally good. Question: Are all side conditions waived with the gap? |

### CRs/TPs comments collection

Major close to finalize Wis and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going Wis, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 2-1-1: Performance gain and side conditions Issue 2-1-2: Initial Gap examples to get the gain | - The gain of UL MIMO with UL gap configured has been demonstrated (OPPO, apple, Huawei, vivo,) - Further evaluation is needed (ZTE, Ericsson,) - No gain of UL MIMO with UL gap configured has been demonstrated (Sony, Nokia,) Recommendations for 2nd round: - It is recommended to focus on the following questions raised in round 1 - Is the metric of performance gain the throughput? If so, what’s the expected throughtput gain? - Is the metric of performance gain the coherent UL MIMO requirement by removing side conditions? If so, are all side conditions removed from coherent requirement with the gap? - To enable the metric of performance gain: - In example 1, is gap needed if UE is expected to send PUSCH in the red slots? Is there any solution on channel estimation on gap symbols? - In example 2, should UE send training sequence during the gap? If no, what is the UE behavior during the gap? |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

It is recommended to focus on the following questions raised in round 1

Issue 2-2-1: Is the metric of performance gain the throughput? If so, what’s the expected throughtput gain?

| Company | Comments |
| --- | --- |
| Apple | The metric can be throughput gain. Throughput gain has been shown in R4-2111383 and R4-2114492. We think the gain is sufficient to make the conclusion. |
| Qualcomm | We should be carefull not to compare coherent UL MIMO to non-coherent UL MIMO but compare coherent UL MIMO with gap to the case with coherent UL MIMO with side conditions happening and no gaps. What is the occurrence frequency and model for side conditions occurring when channel conditions are favorable for coherent UL MIMO? |
| ZTE | Yes, the metric can be throughput gain, and the evaluation should show the details of examples 1 and 2. |
| Huawei, HiSilicon | Yes, the benefit by using coherent codebook on PUSCH is to improve on the UL throughput. We provide simulation results shown in R4-2111383 and R4-2114492. From requirement perspective, there is no direct RF or demod requirement on UL throughput. So we have following choices on the metric: 1. RF requirement on phase and power by removing side conditions 2. other new requirements, e.g. EVM requirement under coherent MIMO To QC, with the current side conditions define in current spec, we can observe the occurrence frequency is very high, cause DRX and DL measurement on adjacent cell is normal UE behavior under NW. The problem is, with these side conditions existed in RF requirement, NW would have high limitation on configuring coherent codebook to the UE even under proper channel conditions. To ZTE, we can focus on example 2 according to your comments in 1st round. Since it is just example pattern, we may need discuss further in phase 2 after phase 1 completed. So we’d better to focus on the gain evaluation. |
| Nokia | We agree with Qualcomm’s comments. It is critical that significant gains are shown and ensured for coherent UL MIMO with UL gaps compared to without UL gaps. Although RAN4 has defined the coherent UL MIMO requirements with certain side conditions, it should not mean that UE cannot support coherent UL MIMO with any other conditions. Furthermore, if UEs require UL gaps in case any condition compared to the side conditions of the minimum requirements is changed, it would mean that coherent UL MIMO cannot be deployed without UL gaps. To our understanding this has not been the original intention in RAN1 and would make coherent UL MIMO feature much less attractive from the system perspective and limit the usage. |

Issue 2-2-2: Is the metric of performance gain the coherent UL MIMO requirement by removing side conditions? If so, are all side conditions removed from coherent requirement with the gap?

| Company | Comments |
| --- | --- |
| Apple | Yes. Removing side conditions is the metric of performance gain. With side condition, if any events listed in the side condition happen, such as measurement gap, DRX off, the gNB need to schedule ap-SRS transmission, before coherent UL transmission can start. This incurs scheduling complexity, addition ap-SRS overhead, and long delay before coherent UL MIMO can start. For non-codebook based coherent UL MIMO, additional ap-CSI-RS transmission, additional CPU processing time is needed before ap-SRS transmission starts. So even longer delay and larger overhead is expected. |
| Qualcomm | Same as 2-2-1, what is the occurrence of side conditions occurring when coherent UL MIMO is beneficial? How was this modeled? |
| ZTE | Based on analysis given by Huawei and vivo, one way to reflect the performance gain is to remove the side conditions, but still other way exists, before we decide which way to use, we suggest not to removing the side conditions. |
| Huawei, HiSilicon | Yes. Our intention is to remove all the side conditions in current spec by using the gap. Perf gain can be clearly shown by UE still can reach the coherent RF requirement or better coherent RF requirement with switching conditions. However, some other new requirement could also prove the perf gain by using the gap as provided in 2-2-1. To ZTE, even by other requirement we still need to remove the side conditions that highly limit on the gNB scheduling. Could you provide an example that we can go with? |
| vivo | Yes, the side condition in current spec will restrict the applicability obviously, and more details (gap pattern, test setup) can be discussed in phase II. |
| Nokia | No, it is important to ensure significant performance gains with UL gaps compared to no UL gaps. It is important to ensure that significant throughput gains can be achieved if UL gaps are defined and used for coherent UL MIMO. It is important to study how significant gains could be ensured in the UE requirements and test cases. Like for other cases this may mean new type of requirements and test cases. We also agree with Qualcomm’s comments. |

Issue 2-2-3: To enable the metric of performance gain:

- In example 1, is gap needed if UE is expected to send PUSCH in the red slots? Is there any solution on channel estimation on gap symbols?

- In example 2, should UE send training sequence during the gap? If no, what is the UE behavior during the gap?

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | In example 1, gap is needed because if DMRS cannot configured specifically to the calibration symbol, the perf on calibration symbol will be reduced. So definitely additional gap as in example 2 is better solution. In example 2, UE will send training sequence during the gap, also UE need to calculate and compensate during the gap. |
| Nokia | It is not possible to answer this type of questions until it is ensured that significant gains can be achieved for coherent UL MIMO if UL gaps are used (compared to coherent UL MIMO without UL gaps) |

# Topic #3: RRM requirements of UL gap for BPS

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2112089 | Apple | Proposal 1: Gap configuration should be flexible to support different BPS implementations. Proposal 2: UE report the supported UL gap configuration through UE capability report. Proposal 3: Consider maximum of 6.25% UL gap overhead. Proposal 4: UL gap location is determined by UL gap configuration and tdd-UL-DL-ConfiguraitonCommon and tdd-UL-DL-ConfigurationDedicated. Proposal 5: UE is not expected to transmit periodic/semi-persist/dynamic scheduled UL transmission in the UL gap slots when UL gap is activated. Proposal 6: Enable implicit activation based on P-MPR value in PHR. Proposal 7: Enable implicit de-activation based on UE de-activation request. |
| R4-2112635 | NTT COCOMO | Observation 3: There are no concerns about the configured UL gaps are activated/deactivated via MAC CE and/or DCI. Considering that UL gaps are dynamically activated/deactivated, DCI is appropriate in terms of immediacy. |
| R4-2112705 | Qualcomm | - UL gap configuration and Capability Proposal 1: UL gap configuration details shall be discussed in RF session and the signalling details, e.g. explicit vs. implicit configuration, dynamic (de)activation, should be left to RAN2 and RAN1. Proposal 2: UE capability and fallback ehaviour, if needed, shall be discussed/determined in RF session. - Interruption requirements Proposal 3: After the detailed UL gap configuration including UL gap type, e.g. type 1 and 2, is determined in RF session, RRM session starts discussion on UL interruption requirements, exception rules, etc. |
| R4-2114016 | Nokia, Nokia Shanghai Bell | Observation 1: A UE supporting UL gaps would need to meet the current requirements without UL gaps when not configured with UL gaps. Observation 2: The network decide if and when UL gaps are provided (agreement). Proposal 1: UE to report capability on the support of UL gaps. Proposal 2: RAN4 would need to define limited a number of UL gap patterns. Proposal 3: RAN4 will need to define one or more mandatory UL gap patterns. Proposal 4: RAN4 will not define any optional UL gap patterns. All defined UL gap patterns will be mandatory. Observation 3. Request for activation of UL gaps could either be explicit or implicit through P-MPR reporting Observation 4: The network may activate UL gap pattern to match the actual needs of a particular UE instead. Proposal 5: RAN4 to identify which UGLs are needed. Proposal 6: RAN4 to identify which uplink gap periodicities (UGRP) would be needed. Observation 5: If the UE provides an indication to the network of potential MPE issues (e.g. based on its EIRP level and/or P-MPR), the network could better determine and activate suitable UL gap periodicity or no UL gaps. These observations and proposals should be taken into considerations when further discussing UE requirements and UL gaps. |

## Open issues summary

In RAN4 #99-e, a WF [2] on introduction of UL gaps for proximity sensing was agreed, with the following conclusions and FFS for the RRM aspects:

- Agreements: The baseline assumption for UL gap related configurations: - UL gap is configured by NW via RRC signaling. Once UL gap is configured, it can be additionally activated or deactivated. - It is up to network decision on whether to configure UL gap or not. - It is FFS that the configured UL gap(s) can be activated/deactivated via MAC CE and/or DCI - Gap configuration candidates: - Candidates for gap periodicity: 5ms, 10ms, 20ms, 40ms, 80ms, 160ms, 320ms - Candidates for gap duration: 62.5us, 125us, 250us, 500us, 1000us, - Implicit activation to minimizing signaling, e.g., depending on PHR report.

### Issue 3-1: Gap configurations

- Gap configuration candidates identified in RAN4#99-e:

  - Candidates for gap periodicity: 5ms, 10ms, 20ms, 40ms, 80ms, 160ms, 320ms

  - Candidates for gap duration: 62.5us, 125us, 250us, 500us, 1000us,

Moderator note: to limit the overall number of gap configurations, companies are encouraged to provide up to two interested gap configurations, including the gap duration and periodicity.

### Sub-topic 3-2: Mechanism for UL gap configuration, activation and deactivation.

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 3-2-1: On UL gap activation

- Option 1: implicitly activated by P-MPR reporting from UE. The activation criteria is determined and signaled by the NW

- Option 2:  explicitly activated by NW

- Option 3: Support both. When network does not configure implicit activation, explicit activation is used by default.

Issue 3-2-2: On UL gap deactivation

- Option 1: implicitly deactivated by [TBD] reporting from UE. The deactivation criteria is determined and signaled by the NW

- Option 2:  explicitly deactivated by NW

- Option 3: Support both. When network does not configure implicit de-activation, explicit de-activation is used by default.

Issue 3-2-3: In case of explicit activation/deactivation, UL gap should be activated and deactivated by

- Option 1: MAC CE

- Option 2:  DCI

## Companies views’ collection for 1st round

### Open issues

Issue 3-1 Gap configurations

Moderator note: to limit the overall gap configurations, companies are encouraged to provide up to two interested gap configuration, , including the gap duration and periodicity.

| Company | Comments |
| --- | --- |
| Apple | 20ms periodicity with gap length of 1000us (8 slots in 120KHz SCS) - 20ms periodicity with gap length of 1250us (10 slots in 120KHz SCS) |
| Huawei, HiSilicon | Firstly, we think different gap duration can reach different resolution on BPS. Each one has its benefit. Our most preference is, 500us and 1000us. For gap periodicity, 20ms and 40ms would be more reasonable. We also support to define gap duration as UE capability. |
| Qualcomm | The moderator’s suggestion already implies “the number of UL gap patterns should be limited” and further says “up to two”. Although we don’t disagree that the number of UL gap patterns shouldn’t be more than necessary, we do not fully understand why it has to be limited to “two”. Besides, if this is motivated by the following argument provided in R4-2114016, please the author or anyone help us understand why any actual system gain is not expected in theory. - “Defining UL GPs which are optional may in fact only lead to increased overall complexity without providing any actual system gain in real field deployments.” One thing that may need to be clarified regarding UL gap length is, - Is the gap length dependent on the number of valid UL slots? For example, the number of UL slots that are actually available to the UE within the gap length can be different depending on UL-DL pattern, other dynamic parameters, etc. With the question, what we ultimately want to know is whether UE and NW can expect the same performance gain out of the same gap length at a given periodicity. Last but not least, we are not sure if RRM session/section is the right place to determine which gap patterns are better than the others in terms of performance gain, system impact, etc. In our understanding, the motivation and use case for UL gap are different from DL gap in a sense that the benefit and expected gain of each pattern can’t be justified by RRM aspect. It would be the right procedure that RF group first identifies patters that should be prioritized and then RRM group can investigate whether the identified patterns will have too much impact to RRM requirements to be considered. If we had to pick a few couple of gap configurations, we would select the following: - gap periodicity: 5ms, 10ms, 20ms gap duration: 125us (1 slot in 120KHz SCS) |
| Sony | In general, we think the user body detection and P-MPR adjustment is not a fast-changing procedure in real life. Therefore, we think a fairly large periodicity, e.g., 160 ms or 320 ms is sufficient for the BPS use case. However, if smaller periodicity would be needed, clear motivation should be provided. On the other hand, the duration of the gap may depend on the sensor capability, which proponent companies need to show a feasible duration time for the device to perform the sensing accurately. |
| Nokia | We agree with the moderator note. RAN4 should try to limit the number of different UL gap configuration options aiming at perhaps a couple of MGPs as also proposed by Apple. As discussed in our paper, in order to define the MGPs such that they also reflect the MGPs which would likely be able to provide best gain performance overall, RAN4 will need to have more stable input on needed measurement gap length and the repetition period. UE RF requirements like gains obtained with UL gaps need to justify needed gap periodicity and length. Thus, UE RF requirements need to progress first. Hence, we support only defining a limited number of MGPs. But RAN4 still need to evaluate and down select among the listed options in order to decide which gap lengths and repetition periods options. |
| Vivo | For the gap periodicity, share views similar to Sony and we prefer 160ms or 320 ms. The gap duration is related to the sensor capability and it seems that the choice between different companies is very different (1 slot / 10 slot), can companies provide more technical justifications? |
| Ericsson | One general comment on RRM (Similar to Qualcomm statement above). We believe that these gaps do not need to be handled as RRM “defined” since no RRM requirements are defined. The gaps definitions should be part of RF. This is also reflected during the meeting since the RRM “related” papers are handled in the RF thread during meeting time. We share the input from Sony, Vivo and Nokia. I.e. prefer fairly large periodicity 160ms and/or 320ms and also to limit number of UL GAP and justification of needed GAP period/length. One question is e.g. Rx switching included within the defined gap length? |
| Intel | We agree with the comment from Sony. Further justification on small periodicities is required for BPS use case. Large periodicities (160ms and 320ms) are ok for us. As mentioned by other companies, the Gap duration may depend on the sensor capability. More inputs are needed from the proponents of some certain value. We are wondering if this gap will also be applicable to FR2-2. Should we consider 480kHz and 960kHz SCS? If no more than 120kHz is considered, then we prefer not to have Gap duration in the fraction of slot and propose to exclude 62.5us from the list. |

Issue 3-2-1: On UL gap activation

- Option 1: implicitly activated by P-MPR reporting from UE. The activation criteria is determined and signaled by the NW

- Option 2:  explicitly activated by NW

| Company | Comments |
| --- | --- |
| Apple | Both option 1 and option 2 can be supported to balance control overhead and network control. Implicit activation can be supported only when configured by NW. Otherwise, explicit activation is used. |
| Huawei, HiSilicon | Option 2. We would like the UE not supporting PMPR can also use this BPS feature. |
| ZTE | Support Option 2. Which is more straightforward and can be better controlled by NW. |
| Qualcomm | In principle, by-default option should be always “explicit” manner. And the right question to the group should be “whether implicit manner is necessary on top of explicit one”. For Option 1, we are not yet convinced if the ‘overhead’ is so large that it can be justified. And it should be also noted that ‘implicit activation’ doesn’t necessarily mean ‘no’ signaling overhead. |
| Sony | Option 2. |
| Nokia | We should not rule the option that the UL MGP is configured and activated using RRC signaling without additional lower layer activation/deactivation signaling. We do not necessarily see the two options being exclusive at this moment but at least Option 2 needs to be supported. |
| DOCOMO | We prefer Option 2. |
| Vivo | Both implicit and explicit activation can be supported. The implicit method can reduce the overhead, but the UE reporting P-MPR does not always mean requesting gap, so explicit method is also required. |
| Ericsson | Option 2 but (as stated by Nokia) we should not prevent direct “activation” via RRC (i.e. configuration and activation) without lower layer signaling. |
| Intel | Support Option 2 |

Issue 3-2-2: On UL gap deactivation

- Option 1: implicitly deactivated by [TBD] reporting from UE. The deactivation criteria is determined and signaled by the NW

- Option 2:  explicitly deactivated by NW

| Company | Comments |
| --- | --- |
| Apple | Both option 1 and option 2 can be supported to balance control overhead and network control. Implicit de-activation can be supported only when configured by NW. Otherwise, explicit de-activation is used. |
| Huawei, HiSilicon | Option 2. |
| ZTE | Option 2. We think no matter activate or deactivate an UL gap, which will not be too frequent, so using the explicitly indication by NW is suitable and reliable. |
| Qualcomm | The same comment as Issue 3-2-1. |
| Sony | Option 2. |
| Nokia | Our view here is similar to Issue 3.2.1 |
| DOCOMO | We prefer Option 2. |
| vivo | Both deactivation method can be supported. |
| Ericsson | Similar to Issue 3-2-1 |
| Intel | Support Option 2 |

Issue 3-2-3: In case of explicit activation/deactivation, UL gap should be activated and deactivated by

- Option 1: MAC CE

- Option 2:  DCI

| Company | Comments |
| --- | --- |
| Apple | MAC CE can be used. The 3ms MAC CE processing is acceptable for activation/de-activation delay. MAC CE are explicitly acknowledged, reduce the potential misalignment between UE and network comparing to DCI based approach. Also it saved potential additional DCI bits. |
| Huawei, HiSilicon | Option 1. |
| ZTE | Option 1 is enough. |
| Qualcomm | When MAC and DCI are considered for explicit activation/deactivation signaling, traditionally the first option should always be MAC, and whether DCI based solution is needed or not is something debatable even when there is a technical justification. In this case, however, we do not even see any technical justification in terms of, e.g. tolerance towards (de)activation latency, yet. Therefore, it is premature to consider Option 2 at this point in time. |
| Sony | Option 1. MAC CE seems sufficient for this case. P-MPR reporting is also on the MAC layer, where we have considered it is fast enough to indicate the EMF exposure event. |
| Nokia | Our view here is that we should at least also consider the simplest approach of using RRC signaling to configure and de-configure the UL gaps. We are aware of the WI on MG enhancements but currently we think UL gaps are a bit orthogonal to that WI. We can discuss further the need for additional lower layer activation and deactivation mechanisms. As said RRC and MAC/DCI options – we do not see this as being exclusive but we should consider also RRC. |
| DOCOMO | We thought DCI is preferred in terms of immediacy, but either is OK if 3ms is acceptable. |
| Vivo | MAC CE is enough. |
| Ericsson | MAC CE if handled by lower layes but also possible via RRC at configuration. |
| Intel | Support Option 1 |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 3-1: Gap configurations | Candidate gap configurations - UL gap length (UGL): 0.5ms, UL gap repetition periodicity (UGRP): 20ms (Huawei) - UGL: 1ms, UGRP:20ms (Huawei, apple) - UGL: 1.25ms, UGRP: 20ms (apple) - UGL: 0.5ms, UGRP:40ms (Huawei) - UGL: 1ms, UGRP:40ms (Huawei) - UGL: 0.125ms, UGRP:5ms (Qualcomm) - UGL: 0.125ms, UGRP:10ms (Qualcomm) - UGL: 0.125ms, UGRP:20ms (Qualcomm) - UGRP: 160ms (Sony, vivo, Ericsson, intel) - UGRP: 320ms (Sony, vivo, Ericsson, intel) Recommendations for 2nd round: - Further discuss in the 2nd round |
| Issue 3-2-1: On UL gap activation | - Option 1: implicitly activated by P-MPR reporting from UE. The activation criteria is determined and signaled by the NW (apple) - Option 2: explicitly activated by NW (apple, Huawei, ZTE, qualcomm, sony, nokia, DCM, vivo, Ericsson, intel) Recommendations for 2nd round: - Recommended agreements: UL gap should be explicitly activated by NW via signaling - How can UE indicate the NW UL gap activation is needed? - Option 1: UE explicitly indicates to NW by signaling - Option 2: Network decision based on P-MPR reporting. This requires P-MPR reporting mandatory for UE supporting UL gap. |
| Issue 3-2-2: On UL gap deactivation | - Option 1: implicitly deactivated by [TBD] reporting from UE. The deactivation criteria is determined and signaled by the NW (apple) - Option 2: explicitly deactivated by NW (apple, Huawei, ZTE, qualcomm, sony, nokia, DCM, vivo, Ericsson, intel) Recommendations for 2nd round: - Recommended agreements: UL gap should be explicitly deactivated by NW via signaling - How can UE indicate the NW UL gap deactivation is needed? - Option 1: UE explicitly indicates to NW by signaling - Option 2: Network decision based on existing report such as L3-RSRP, BSR etc. |
| Issue 3-2-3: In case of explicit activation/deactivation, UL gap should be activated and deactivated by | - Option 1: MAC CE (apple, Huawei, ZTE, Qualcomm, sony, DCM, vivo, Ericsson, Intel ) - Option 2: DCI (DCM) - Option 3: RRC (nokia, Ericsson) Recommendations for 2nd round: Recommended agreements: UL gap should be explicitly activated and deactivated by MAC CE - Moderator note: depending on the progress of issue 3-2-1, 3-2-2 and 3-2-3, we can inform RAN2 by sending LS. In LS, the RRC configuration and UL gap activation and deactivation related agreement in RAN4#99e should be included too. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

- Issue 3-3-1: How can UE indicate the NW UL gap activation is needed?

  - Option 1: UE explicitly indicates to NW by signaling

  - Option 2: UE implicitly indicate to NW by P-MPR reporting. The exact P-MPR value is FFS.

| Company | Comments |
| --- | --- |
| Apple | Prefer option 1 Option 1 provides a flexibility for UE to indicate based on the implementation, and simplicity for the network to decide whether UL gap should be activated or not. For option 2, we can leave it as NW's implementation issue. Regardless, NW will decide if to activate the gap it provides UE flexibility to indicate whether UE requires UL gap based on implementation. |
| Qualcomm | We don’t think Option 1 is controversial whereas Option 2 might be. As mentioned during GTW session, until we see a clear framework/example of Option 2, we don’t think the group can tell if that would work in general. Besides, we are not yet convinced if the ‘signaling overhead’ of Option 1 is so large that Option 2 can be justified. And it should also be noted that ‘implicit activation’ doesn’t necessarily mean ‘no’ signaling overhead. Again, if there is any ambiguity that can be caused by Option 2 approach, it will require additional signaling, fallback operation, exception handling, etc. Based on this understanding, we support Option 1. |
| ZTE | We are OK for Option 1 and Option 2. |
| vivo | At least the explicitly indication is needed which is more flexible, the P-MPR reporting does not always mean the UE require the UL gap. The detail of option2 need further discussion. |
| Nokia | At least option 1 needs to be supported. |

  - Issue 3-3-2: How can UE indicate the NW UL gap deactivation is needed?

    - Option 1: UE explicitly indicates to NW by signaling

    - Option 2: UE implicitly indicate to NW by [TBD] reporting.

| Company | Comments |
| --- | --- |
| Apple | Prefer option 1. UE explicitly indicates to the NW by signaling. Unlike activation, the P-MPR in PHR cannot be used as the deactivation metric, since P-MPR=0 can be a result of BPS sensing and no target is nearby. For option 2, network might be able to make decision when to de-activate UL gap based on L3-RSRP report, which indicates whether UE is in cell edge or not, and/or BSR which indicates UL traffic, and/or power head room value where a large value indicate UL gap might be able to turn off. However none of the reports are direct indication that UL gap can be de-activated, we think allow UE to send explicitly de-activation request to the network is preferrable and much simpler. |
| Qualcomm | The same comment as Issue 3-2-1. And we share a similar view as Apple’s |
| ZTE | We are OK for Option 1 and Option 2. |
| Nokia | At least option 1 needs to be supported |

Issue 3-3-3: Should RAN4  inform RAN2 the following UL gap configuration/deconfiguration/activation/deactivation related agreements by sending a LS?

- Agreements in RAN4#99e: The baseline assumption for UL gap related configurations:

  - UL gap is configured by NW via RRC signaling. Once UL gap is configured, it can be additionally activated or deactivated.

    - It is up to network decision on whether to configure UL gap or not.

Agreements in RAN4#100e:

  - UL gap can be directly activated/deactivated by RRC where the UL gap is configured/deconfigured

  - UL gap can also be activated and deactivated by MAC-CE after it is configured by RRC

- Option 1: Yes

- Option 2: No

| Company | Comments |
| --- | --- |
| Apple | Option 1. We agree to send LS to RAN2 to inform RAN4 decision. We think sufficient progress, which is RAN2 relevant, in RAN4 has been made and RAN2 should be informed. Firstly, RAN4 has reached the consensus on the overall frame of UL gap configuration and activation in the recent two meetings. Due to RAN2 spec related impacts, we should follow a general practice to inform other WG on the related progress. Secondly, one company refer to one of the decisions in RAN4#98e to block sending such LS. In RAN4#98e, it was captured in the chairman notes on UL gap that “RAN4 will not define any requirements until the corresponding testing methodology for the performance enhancement is clear.” Clearly, the referred agreement should not prevent any technical discussion in both RAN4 and RAN2 and it applies to the RAN4-defined requirements only. Additionally, considering the timelines of this WI, i.e. RAN2/4 need to complete the core part by March 2022, timely information sharing between WGs is necessary. |
| ZTE | We believe the two candidates “UL gap can be directly activated/deactivated by RRC where the UL gap is configured/deconfigured” , “UL gap can also be activated and deactivated by MAC-CE after it is configured by RRC” can not be both supported, down selection between the two is needed. Since if both of them are supported, when a UL gap is configured by RRC signaling, UE is ambiguous whether to wait for a MAC-CE to activate such UL gap, or automatically activate such UL gap. After we decide which candidate should be supported, we can send LS to RAN2. |
| vivo | We prefer Yes. it’s more efficient to inform RAN2 about the progress of RAN4, and RAN2 can start related work in parallel. |
| Nokia | Our understanding of the discussion is that at least the RRC configuration which configures and simultaneously means ‘activated’ (like legacy DL MGs configured with RRC) was one basic option to be supported. In addition the MAC CE activation/deactivation could also be supported, but can be seen as an additional feature on top of the basic RRC signaling solution. If RAN4 agree to send an LS to RAN2 the options need to be clear: - Use only RRC configuration to configure and deconfigure gaps. And once a gap is configured the gap is also in active use by the UE. Use RRC configuration to configure gap which can then be activated and deactivated later by MAC CE. |

- Issue 3-3-4: continue discussing the candidate gap configurations

- UL gap length (UGL): 0.5ms, UL gap repetition periodicity (UGRP): 20ms (Huawei)

- UGL: 1ms, UGRP:20ms (Huawei, apple)

- UGL: 1.25ms, UGRP: 20ms (apple)

- UGL: 0.5ms, UGRP:40ms (Huawei)

- UGL: 1ms, UGRP:40ms (Huawei)

- UGL: 0.125ms, UGRP:5ms (Qualcomm)

- UGL: 0.125ms, UGRP:10ms (Qualcomm)

- UGL: 0.125ms, UGRP:20ms (Qualcomm)

- UGRP: 160ms (Sony, vivo, Ericsson, intel)

- UGRP: 320ms (Sony, vivo, Ericsson, intel)

| Company | Comments |
| --- | --- |
| Apple | Typically, the body proximity sensor combines sensing measurements from various gaps to improve target detection performance. Different sensing gap configurations help support the diverse requirements of different UEs. For example, sensing measurement combining over distributed pattern can be enabled by 5ms periodicity with 125us gap length, while combining over clustered sensing occasions can be enabled by 20ms periodicity with 500us gap length. We propose 20ms with 1000us gap length which is an optimization between Tx power gain versus gap overhead. When low gap overhead is used, due to detection performance, lower Tx power gain is expected as well. |
| Qualcomm | We believe the UL gap pattern is RF centric issue. We would like to ask moderator to consider handling the issue as a part of RF discussion unless RRM specific aspects need to be taken into account in selecting UL gap patterns. We echo our comment provided in the first round: In our understanding, the motivation and use case for UL gap are different from DL gap in a sense that the benefit and expected gain of each pattern can’t be justified by RRM aspect. It would be the right procedure that RF group first identifies patters that should be prioritized, if needed, and then RRM group can subsequently investigate whether the identified patterns will have too much impact on RRM requirements to be considered. In the meantime, we’d like to ask companies for clarification about the following. - Is the UGL irrespective of UL-DL pattern and/or SCS of active UL BWP? - What is the switching time assumed to be included in gap period? - Is the switching gap supposed to be included in UGL or UGRP? For RRM work, companies may need to establish a common understanding of the above three aspects. |
| ZTE | For UGRP, we support 160ms and 320 ms. |
| Nokia | As discussed in 1st round the gap patterns needs to be defined such that they also provide best gain in performance overall. RAN4 will need to have more stable input on needed measurement gap length and the repetition period. We see that UE RF requirements like gains obtained with UL gaps need to part of the discussion and the GPs (gap periodicity and length) needs to be justified by the gain they can provide. Thus, UE RF requirements need to progress first. Additionally, we support defining a limited number of MGPs. We prefer that RAN4 does evaluation and down selection among the listed options in order to decide which gap lengths and repetition periods options to support. options now seem to be: ULG: 0.125, 0.5, 1 and 1.25ms UGRP: 5, 10, 20, 40,160 and 320ms Supporting all these combinations seems unnecessary. We suggest one GP with short UGRP and long UGRP. And down selection of UGL option would also be preferred – e.g. no more than 2. |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on UL gap for BPS | Apple |  |
| LS on UL gap in FR2 RF enhancement | Apple | To: RAN_2; |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Sony | Kun Zhao | kun.1.zhao@sony.com |
| DOCOMO | Ryu Kitagawa | ryuu.kitagawa.pn@nttdocomo.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)