3GPP TSG-RAN WG4 Meeting #100-e                                         R4-2115055

# Electronic Meeting, Aug. 16-27, 2021

Source:	ZTE Corporation

Title:	TP to TR38.862: Guidelines on the band edge relaxation for MOP for CA/DC band combination

Agenda Item:	10.3.5

Document for:	Approval

- 1	Introduction

In current TS38.101-1 spec, the band edge relaxations for MOP are applied for almost all of NR uplink inter-band CA/DC configurations. The reasons are mainly because currently there were no agreements on how to apply the band edge relaxations for MOP for uplink inter-band NR CA/DC configurations.

In last meeting, this issue was discussed in [1]. Although the contribution [1] was noted, the companion CRs [2][3] which are based on the principle proposed in [1] were agreed. Also according to the email discussion in [4], it seems there are no objections on the principle proposed in [1].

Meanwhile, RAN4 have already agreed a SI for band combination handling and the corresponding TR38.862[5], where the purpose is to include the information such as guidelines/principle/approach etc for band combination.

Therefore, we think it is beneficial for future work by capturing some guidelines on the band edge relaxation for MOP for band combination in the TR38.862.

- 2	Discussion

Thanks contribution [1], some backgrounds and principles are provided, including intra-band contiguous/non-contiguous CA, UL inter-band CA. For UL inter-band CA, the proposal is:

Proposal 3: For uplink inter band CA or MR-DC, apply band edge relaxation to the uplink configurations whose at least one of the bands has band edge relaxation for MOP as single band usage. Note that corrections for CA are needed from Rel-16 while those for MR-DC are needed from Rel-17.

Actually, in the past meetings, there were no agreements how to apply the MOP relaxation on the band edge for uplink intra-band CA and uplink inter-band CA configurations, so companies usually apply the band edge relaxation note to their combinations.

Meanwhile, separated clauses/tables are used for intra-band and inter-band MOP requirements in current TS38.101-1 specifications.

The guidelines are summarized as follow:

For the band edge relaxation for MOP for CA/DC band combination:

Guideline 1: The superscript of the NOTE on band edge relaxation is not needed in the Tables for band configurations consisting of different bands in TS38.101-1 specification.

For

uplink intra-band contiguous CA or non-contiguous CA

Also, no matter PC1.5, PC2 or PC3, we think it is no doubt that the band edge relaxation for MOP is applied for a certain NR band operating in single band and uplink intra-band contiguous CA.

Guideline 2: For uplink intra-band contiguous CA or intra-band non-contiguous CA, apply band edge relaxation to the uplink configurations if this band has band edge relaxation for MOP as single band usage.

Next, we take some examples to explain the above guidelines further.

First, assuming NR FDD band nX and NR TDD bands nYand nZ, where PC3 is only applied to band nX, PC1.5/PC2/PC3 are applied to band nY, and PC2/PC3 are applied to band nZ. See table 1.

Table 1. Power class only for NR single band PC1.5/2/3

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| nX |  |  |  |  |  |  | 23 | ±2 |
| nY |  |  | 29 | +2/-33 | 26 | +2/-33 | 23 | ±23 |
| nZ |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. |  |  |  |  |  |  |  |  |

Where NOTE 3 is defined in Table 6.2.1-1 in TS38.101-1.

In table 1, band edge relaxation (i.e. NOTE 3) for MOP is only applied for band nY, and no band edge relaxation for MOP is applied for band nX and nZ.

- When HPUE NR inter-band CA constitute of bands nX, nY and nZ, i.e. HPUE NR inter-band CA CA_nXA-nYA, CA_nXA-nZA

According to guideline 1, due to the band nY as single band usage has band edge relaxation for MOP and band nX and band nZ as single band usage have no such band edge relaxation for MOP, so for the uplink HPUE band(s) in CA_nXA-nYA and CA_nXA-nZA, the band edge relaxation  is applied for HPUE TDD band nY, while no band edge relaxation is applied for HPUE TDD band nZ

2. When uplink inter-band CA or DC constitute of band nX, nY and nZ, i.e. uplink inter-band CA CA_nXA-nYA, CA_nXA-nZA, and uplink inter-band DC_nXA-nYA, DC_nXA-nZA

According to guideline 1, due to the band nY as single band usage has band edge relaxation for MOP and band nX and band nZ as single band usage have no such band edge relaxation for MOP, therefore band edge relaxation for PC1.5, PC2 and PC3 to the uplink configurations of CA_nXA-nYA are applied, while no band edge relaxation for  PC2 and PC3 to the uplink configurations of CA_nXA-nZA are applied.

Similarly, band edge relaxation to the uplink configurations of DC_nXA-nYA are applied, while no band edge relaxation to the uplink configurations of DC_nXA-nZA are applied.

- When NR band nX, nY, nZ support intra-band contiguous/non-contiguous CA

According to guideline 2, due to the band nY as single band usage has band edge relaxation for MOP and band nX and band nZ as single band usage have no such band edge relaxation for MOP, therefore band edge relaxation for PC1.5, PC2 and PC3 to the uplink configurations of band nYC or band nY(2A) are applied. While no band edge relaxation for PC2 and PC3 to the uplink configurations of band nZC or band nZ(2A) are applied, and also no band edge relaxation for PC3 to the uplink configurations of band nXC or band nX(2A) are applied.

- 3 Conclusion

In this contribution, we give some guidelines on the band edge relaxation for MOP for band combination with some examples for further explanation. The conclusions are summarized as:

Proposal: Approve the following guidelines:

For the band edge relaxation for MOP for CA/DC band combination:

Guideline 1: The superscript of the NOTE on band edge relaxation is not needed in the Tables for band configurations consisting of different bands in TS38.101-1 specification.

For uplink NR intra-band contiguous CA or non-contiguous CA:

Guideline 2: For uplink intra-band contiguous CA or intra-band non-contiguous CA, apply band edge relaxation to the uplink configurations if this band has band edge relaxation for MOP as single band usage.

Meanwhile, TP to TR38.862 is provided.

- 4	Reference

[1] R4-2109127, Correction of an improper usage of band edge relaxation for MOP, Nokia, Nokia Shanghai Bell

[2] R4-2108016, Correction of an improper usage of band edge relaxation for MOP, Nokia, Nokia Shanghai Bell

[3] R4-2108017, Correction of an improper usage of band edge relaxation for MOP, Nokia, Nokia Shanghai Bell

[4] R4-2107913, Email discussion summary for [99-e][103] NR_Maintenance_R16, Moderator (Apple)

[5] TR38.862, Study on band combination handling in RAN4

- 5 TP to TR38.862

7.X 	Guidelines on the band edge relaxation for MOP for CA/DC band combination

The following are the guidelines for the band edge relaxation for MOP for CA/DC band combination

Guideline 1: The superscript of the NOTE on band edge relaxation is not needed in the Tables for band configurations consisting of different bands in TS38.101-1 specification.

Guideline 2:

For intra-band contiguous CA or intra-band non-contiguous CA, apply band edge relaxation to the uplink configurations if this band has band edge relaxation for MOP as single band usage.

For guideline 2, the NOTE for single band, i.e. “Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB.” should be applied.

The following are some examples to explain the above guidelines further.

First, assuming NR FDD band nX and NR TDD bands nYand nZ, where PC3 is only applied to band nX, PC1.5/PC2/PC3 are applied to band nY, and PC2/PC3 are applied to band nZ. See table 7.x.1.

Table 7.x.1. Power class only for NR single band PC1.5/2/3

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| nX |  |  |  |  |  |  | 23 | ±2 |
| nY |  |  | 29 | +2/-33 | 26 | +2/-33 | 23 | ±23 |
| nZ |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. |  |  |  |  |  |  |  |  |

In table 7.x.1, band edge relaxation (i.e. NOTE 3) for MOP is only applied for band nY, and no band edge relaxation for MOP is applied for band nX and nZ.

- When HPUE NR inter-band CA constitute of bands nX, nY and nZ, i.e. HPUE NR inter-band CA CA_nXA-nYA, CA_nXA-nZA

According to guideline 1, due to the band nY as single band usage has band edge relaxation for MOP and band nX and band nZ as single band usage have no such band edge relaxation for MOP, so for the uplink HPUE band(s) in CA_nXA-nYA and CA_nXA-nZA, the band edge relaxation is applied for HPUE TDD band nY, while no band edge relaxation is applied for HPUE TDD band nZ.

- 2. When uplink inter-band CA or DC constitute of band nX, nY and nZ, i.e. uplink inter-band CA CA_nXA-nYA, CA_nXA-nZA, and uplink inter-band DC_nXA-nYA, DC_nXA-nZA

According to guideline 1, due to the band nY as single band usage has band edge relaxation for MOP and band nX and band nZ as single band usage have no such band edge relaxation for MOP, therefore band edge relaxation for PC1.5, PC2 and PC3 to the uplink configurations of CA_nXA-nYA are applied, while no band edge relaxation for  PC2 and PC3 to the uplink configurations of CA_nXA-nZA are applied.

Similarly, band edge relaxation to the uplink configurations of DC_nXA-nYA are applied, while no band edge relaxation to the uplink configurations of DC_nXA-nZA are applied.

- When NR band nX, nY, nZ support intra-band contiguous/non-contiguous CA

According to guideline 2, due to the band nY as single band usage has band edge relaxation for MOP and band nX and band nZ as single band usage have no such band edge relaxation for MOP, therefore band edge relaxation for PC1.5, PC2 and PC3 to the uplink configurations of band nYC or band nY(2A) are applied. While no band edge relaxation for PC2 and PC3 to the uplink configurations of band nZC or band nZ(2A) are applied, and also no band edge relaxation for PC3 to the uplink configurations of band nXC or band nX(2A) are applied.