3GPP TSG RAN WG4 Meeting #100-e	R4-2115240

Electronic Meeting, 16-27 August 2021

Agenda Item:	5.1.8

Source: 	Ericsson

Title:	WF on Rel-15 NR RRM test case related issues

Document for:	Approval

- Introduction

The WF is related to correction of RRM test configuration and test cases discussed under the following email thread:

- [100-e][202] NR_RRM_maintenance_R15_Perf

The following sections contain agreements at RAN4#100-e.

# Channel BW configuration for RRM CA TCs

- For channel BW other than 10 MH for SSB SCS 15 kHz or 40 MHz for SSB SCS 30 kHz, define test parameters as follows.

  - Define channel bandwidth with “≥” mark in the test configuration tables to add flexibility to the CA bandwidth combination

    - Clarify the channel bandwidth in the configuration table is BWchannel

  - Introduce a new parameter “BWoccupied” which is the actual BW allocation in each test parameter table (confined with 10 MHz or 40 MHz)

  - Configure PDSCH RMC with the same number of RBs as 10 MHz for 15 kHz SSB SCS or 40 MHz for 30 kHz SSB SCS

  - Configure CORESET for RMSI and RMC scheduling with same configurations as CBW of 10 MHz for 15 kHz SSB SCS or 40 MHz for 30 kHz SSB SCS

  - Fill in 10 MHz or 40 MHz channel BW with OCNG.

  - Confine also the bandwidth of Noc within BWoccupied

# FR2 inter-frequency relative RSRP accuracy

- The margin in the FR2 inter-frequency relative RSRP accuracy tests depend on how close the inter-frequency carriers are in frequency domain e.g. whether the carriers are in the same band or not:

  - FFS: margins for both cases.

# FR1/LTE + FR2 testability

## Criteria for selecting FR1/LTE+FR2 test with OTA testability problem

- FR1/LTE+FR2 test has OTA testability problem if at least one of the following criteria is met:

  - Tests where any requirement is tested for FR1/LTE,

  - Tests where UE receives any DL message (e.g. RRC/DCI/MAC-CE configuration message/command etc) on FR1/LTE between the starting point and ending point of the test, and

  - Tests where UE transmits any UL signal (e.g. measurement report, ACK/NACK, CSI etc) b on FR1/LTE between the starting point and ending point of the test.

## Rel-15 FR1/LTE+FR2 test case list with OTA testability problem

Table 1: Rel-15 FR1/LTE+FR2 test cases with OTA testability problem

| No. | Test case |
| --- | --- |
| 1 | A.5.5.2.3 E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC |
| 2 | A.5.5.2.4 E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC |
| 3 | A.5.5.2.5 E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC |
| 4 | A.5.5.2.6 E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC |
| 5 | A.5.5.3.1 SCell Activation and deactivation of SCell in FR2 intra-band |
| 6 | A.5.5.3.2 SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle |
| 7 | A.5.5.3.5 SCell Activation and deactivation of SCell in FR2 |
| 8 | A.5.5.6.1.1 E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC |
| 9 | A.5.5.6.1.2 E-UTRAN – NR PSCell FR2 DL active BWP switch with FR2 SCell in non-DRX in synchronous EN-DC |
| 10 | A.5.5.6.2.1 E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC |
| 11 | A.7.3.1.1 Inter-frequency handover from FR1 to FR2; unknown target cell |
| 12 | A.7.5.3.2 SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2 |
| 13 | A.7.5.6.1.2 NR FR1-NR FR2 DL active BWP switch of PCell with non-DRX in SA |
| 14 | A.7.5.7.1 Addition and Release Delay of known NR PSCell |
| 15 | A.7.5.7.2 Addition and Release Delay of unknown NR PSCell |
| 16 | A.7.6.2.5 SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1) |
| 17 | A.7.6.2.6 SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1) |
| 18 | A.7.6.2.7 SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1) |
| 19 | A.7.6.2.8 SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1) |
| 20 | A.7.7.1.3 SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell |
| 21 | A.8.4.2.5 NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used |
| 22 | A.8.4.2.6 NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used |
| 23 | A.8.4.2.7 NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used |
| 24 | A.8.4.2.8 NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used |

## Rel-16 FR1/LTE+FR2 test case list with OTA testability problem

- All Rel-15 tests in table 1,

- Rel-16 specific tests in table 2,

- More Rel-16 tests are FFS.

Table 2: Rel-16 specific FR1/LTE+FR2 test cases with OTA testability problem (other than Rel-15 tests)

| No. | Test case |
| --- | --- |
| 25 | A.7.3.1.4 Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2 |
| 26 | A.7.3.1.5 Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2 |
| More Rel-16 tests with OTA testability issues are FFS |  |

## Solutions for R1/LTE + FR2 tests with testability issue

- Some of the LTE/FR1+FR2 tests can be performed with some modification:

  - Modify each individual test e.g. remove testing of LTE/FR1 cell in the test

- Exclude testing of remaining tests by defining general rule/applicability rule in Annex of TS 38.133

## List of FR1/LTE+FR2 tests which can be tested with modification

Table 3: Rel-15 LTE/FR1+FR2 test cases which can be tested with modification

| No. | Test case |
| --- | --- |
| 1 | A.5.5.2.3 E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC |
| 2 | A.5.5.2.4 E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC |
| 3 | A.5.5.2.5 E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC |
| 4 | A.5.5.2.6 E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC |
| 5 | A.5.5.3.1 SCell Activation and deactivation of SCell in FR2 intra-band |
| 8 | A.5.5.6.1.1 E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC |
| 9 | A.5.5.6.1.2 E-UTRAN – NR PSCell FR2 DL active BWP switch with FR2 SCell in non-DRX in synchronous EN-DC |
| 10 | A.5.5.6.2.1 E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC |
| 20 | A.7.7.1.3 SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell |

Table 4: Rel-16 LTE/FR1+FR2 test cases which can be tested with modification

| No. | Test case |
| --- | --- |
| 25 | A.7.3.1.4 Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2 |
| 26 | A.7.3.1.5 Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2 |
| Note 1: More LTE/FR1+FR2 test cases are FFS |  |

## General modification related to FR1/LTE+FR2 testability

General modification related to FR1/LTE+FR2 testability is needed in clauses A.3.7A, A.3.7B and A.3.7.2.2 of TS 38.133.