3GPP TSG RAN WG4 Meeting #100-e	R4-2115316

Electronic Meeting, 16-27 August 2021

Agenda Item:	6.1.6.2.3

Source: 	Ericsson

Title:	WF on gNB positioning measurement requirements

Document for:	Approval

- Introduction

The WF is related to gNB Rx-Tx and SRS-RSRP positioning measurement requirements discussed under the following email thread:

- [100-e][211] NR_pos_3

The following sections contain agreements at RAN4#100-e.

# SRS-RSRP measurement requirements

- The latest companies’ link simulation results show that the existing SRS-RSRP measurement accuracies in TS 38.133 v16.8.0 do not need any modification.

- Existing SRS-RSRP accuracy requirements in TS 38.133 v16.8.0 are updated by removing the brackets.

# gNB Rx-Tx Time Difference measurement requirements

- The latest companies’ link simulation results show that for some SRS BW ranges the existing gNB Rx-Tx time difference measurement accuracies in TS 38.133 v16.8.0 need to be updated.

- gNB Rx-Tx accuracy requirements in TS 38.133 v16.8.0 are updated for certain SRS BW ranges based on the following principles:

  - Take into account new data points from simulation results for lowest RB number of bandwidth group to define gNB Rx-Tx measurement accuracy requirements.

  - Add half of standard deviation rounded to the next integer as additional margin.