3GPP TSG-RAN WG4 Meeting # 100-e 												R4-2115334

Electronic Meeting, 16th – 27th Aug, 2021

Agenda item:			9.9.4.1, 9.9.4.2, 9.9.4.3

Source:	Moderator (Nokia, Nokia Shanghai Bell)

Title:	WF on FR2 HST RRM (part 1)

Document for:	Approval

# Background

This contribution is to capture the agreements for the email discussion for the topic of Rel-17 NR HST FR2 enhancements RRM core requirements in RAN4 #100, with the email thread identifier “[100-e][218] NR_HST_FR2_RRM_1”.

This e-mail thread will capture the e-mail discussions for the following sub-agenda items for FR2 HST RRM

- 9.9.4.1 General

- 9.9.4.2 Number of RX beams

- 9.9.4.3 Idle/Inactive and connected state mobility requirements

The detailed discussions on these agreements can be found in the e-mail discussion summary.

# WF on Topic#1: General

## Network signaling

Identification of different/enhanced RRM requirements

Agreement: Add a flag in a form of cell-specific signalling to indicate UE to use different/enhanced RRM requirements in HST FR2 deployments. Way forward: Following the GtW agreement, FFS on feasibility and methods to differentiate scenarios from UE perspective

Signaling of uni-/bi-directional operation

Way forward: Discuss further if there is a need to signal uni-bi-directional mode of operation: - Option 1: Network signals type of deployment (uni- or bi-direction) to UE. - Option 2: Signalling of uni-/bi-directional operation is not needed.

Signalling of network assistance information

Way forward: FFS signalling of network assistance information: - Option 1: In uni-directional deployment, network signals DL beam w.r.t. UE moving direction to UE. - Option 2: Enable network signaling of SSB index per RRH - Option 3: Network can indicate different SSBs on adjacent RRHs having the same QCL property: signal the mapping between the repeated sets of beams from the adjacent RRHs - Option 4: Scaling factor based on explicit signalling from network to UE or implicit signalling based on UE’s identification of SSB index/TCI-state, position of UE relative to RRHs and so on - Option 5: The following additional beam coverage related information can be signaled to UE - Distance between the projections of adjacent beam peaks on the track - Beam peak direction angle relative to track - The 6 dB beam-width projection on track - In addition, UE can report speed to the network. - The options are not mutually exclusive and other options are not precluded. Companies are encouraged to provide further analysis/details of network assisted signalling addressing potential benefits and drawbacks including but not limited to reducing UE RX beams and L1/L3 measurement delay, differentiating uni- and bi-directional modes, signalling overhead, etc. Companies are recommended to capture their analysis in TR 38.854.

## UE capabilities

CPE support for HST FR2 deployment

Agreement: HST FR2 CPE has a capacity to support both uni- and bi-directional operation. Way forward: Following the GtW agreement, FFS if different UE capabilities shall be used for Scenario A and B support Continue the discussion of CPE support for HST FR2 deployment: - Option 1: It is not necessary to introduce UE capability to indicate the support of FR2 HST. - Option 2: Define UE capability for FR2 HST enhancement support. FFS the ways to differentiate HST FR2 capable CPEs from any other UEs in the specification: - Option 1: Apply enhanced RRM requirements for FR2 HST based on Power Class corresponding to FR2 UE in TS 38.101-2. - Other options are not precluded. FFS a need for CPE capability to change characteristics, e.g. RX beam sweep number in uni-/bi-directional operation.

# WF on Topic#2: Number of RX beams from RRM perspective

Number of RX beams

GtW agreements: - RX beam number for RRM requirements definition - Define two set of requirements for Scenario A and Scenario B in terms of number of RX beams per UE - Scenario A: [2] RX beams for all scenarios - Scenario B: [6] RX beams for all scenarios - FFS on feasibility and methods to differentiate scenarios from UE perspective - FFS if different UE capabilities shall be used for Scenario A and B support - Note: if there is insignificant difference between Scenario A and B requirements, then further discussion on unified requirements can take place Way forward: Discuss the FFS issues from GTW agreement and the possibility to unify the requirements further.

Impact of RRH position at one/both sides of rail track

Agreement: There is no impact of RRH position at one/both sides of rail track in Scenario-A under the assumption that UE boresight direction (or the beam direction if there is only one beam) is parallel to the track Way forward: Companies are encouraged to evaluate candidate options for Scenario-B considering the GtW agreements - Option 1: RRM requirements are defined with scaling factor which is double of number of RX sweep number in scenario study. - Option 2: RRM requirements are defined with adapted scaling factor based on explicit or implicit signalling from network to UE. - Other options are not precluded.

Impact of obstruction between RRH and UE

Agreement: Impact of obstruction between RRH and UE on RRM e.g., beam management/RX beam number, doesn’t need to be discussed in this session.

Unified uni- and bi-directional RRM requirements

Way forward: Companies, which have concerns with regard to defining a unified set of enhanced RRM requirements for uni- and bi-directional modes, are encouraged to bring technical analysis outlining the cause/reason for the difference in uni and bi-directional modes taking into account different (or range of) Dmin, etc. Companies are recommended to capture their analysis in TR 38.854.

# WF on Topic#3: RRC Idle/Inactive and connected state mobility requirements

## CONNECTED state mobility

DRX upper bound

Way forward: FFS the DRX upper bound for enhanced RRM HST FR2 requirements: - Option 1: Up to [256] ms - Option 2: [80] ms - Option 3: [60] ms - Other options are not precluded

Requirements for long DRX

Agreement: Apply existing R16 requirements for long DRX cycles, i.e. above the upper bound of DRX cycle.

SMTC periodicity

Agreement: HST FR2 enhanced requirement is applied to SMTC <=40ms. SMTC periodicity is not restricted.

Enhancements for short DRX

Way forward: Further consider the possibility of requirement enhancement for SMTC periodicity <= [40] ms - Option 1: M2 = 1.5 if SMTC periodicity > [40] ms, otherwise M2=1. - Option 2: Keep the 1.5 relaxation factor.

Connection Re-establishment to known cell

Way forward: FFS time to identify target NR cell for RRC connection re-establishment to known NR intra-frequency cell: - Option 1: MAX (TBD ms, 5 x N1 x TSMTC). - Option 3: Do not introduce any enhancements. - Other options are not precluded

Connection Re-establishment to unknown cell

Way forward: FFS time to identify target NR cell for RRC connection re-establishment to unknown NR intra-frequency cell: - Option 1: Keep current requirements with N1 scaling factor corresponding the number of RX beams: MAX (1000 ms, 10 x N1 x TSMTC) - Option 2: Do not introduce any enhancements. - Other options are not precluded

## Handover

Agreement: Criteria of known cell for FR2: The target FR2 cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds; otherwise, it is unknown. Way forward: FFS the following issues: - The scaling factor in HO requirement corresponding the number of RX beam sweep. - A feasibility of HO to unknow cell in HST FR2 deployments.

## IDEL/INACTIVE state mobility

Agreements: - Use cell selection criterion S and number of DRX cycles as a baseline for cell selection requirement. - Use table below as a background for further discussion of HST FR2 IDLE mode requirement: Tdtect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra for UE configured with RRM enhancements for high speed (Frequency range FR2) DRX cycle length [s]; Tdetect,NR_Intra [s] (number of DRX cycles); Tmeasure,NR_Intra [s] (number of DRX cycles); Tevaluate,NR_Intra [s] (number of DRX cycles). 0.32; 2.56 x N1 x M2 (8 x N1 x M2); 0.32 x N1 x M3 (1 x N1 x M3); 0.96 x N1 x M4 (3 x M4). 0.64; 5.12 x N1 (8 x N1); 0.64 x N1 (1 x N1); 1.92 x N1 (3 x N1). 1.28; 8.96 x N1 (7 x N1); 1.28 x N1 (1 x N1); 3.84 x N1 (3 x N1). 2.56; 58.88 x N1 (23 x N1); 2.56 x N1 (1 x N1); 7.68 x N1 (3 x N1). Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2. Way forward: FFS the following issues: - The upper bound of DRX cycle with enhanced requirements: - Option 1: Use 320 ms DRX cycle as baseline for following analysis and requirement definition - Other options are not precluded - The values of scaling factors: N1, M2, and SMTC: - Option 1: N1 is not necessarily the agreed Rx numbers for each scenario, N1 = 3 - Option 2: N1 refers to agreed Rx numbers - Other options are not precluded

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Moderator (Nokia, Nokia Shanghai Bell) | Dmitry Petrov | dmitry.a.petrov@nokia-bell-labs.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)