3GPP TSG-RAN WG4 Meeting # 100-e 												R4-2115338

Electronic Meeting, August 16-27, 2021

Agenda item:			9.10.2.3

Source:	CATT

Title:	WF on further RRM enhancement for NR and MR-DC - PUCCH SCell activation/deactivation requirements

Document for:	Approval

- Sub-topic 1-1 Ending point of PUCCH SCell activation

Issue 1-1-1: The ending point of PUCCH SCell activation for invalid TA case?

- Agreements:

  - The point when UE transmits valid CSI report on the target PUCCH SCell

    - Note: Network is not precluded to schedule UE with DL/UL transmission before the ending point but UE performance is not guaranteed

- Sub-topic 1-2 Beam information for PUCCH SCell activation

Issue 1-2-1: How to indicate the beam information for PUCCH Scell activation for unknown cell (The procedure for beam indication for PUCCH Scell activation)?

Agreement:

RAN4 send LS to RAN1/2 asking for the feasibility and potential solutions for the support of PUCCH SCell activation procedure for unknown cell.

Issue 1-2-2: Whether the CSI reporting type (periodic, aperiodic, semi-persistent) about PUCCH SCell activation is needed to be specified?

FFS:

- Option 1: (NTT DOCOMO, Nokia)

  - No. Any kind of reporting type can be used for the PUCCH SCell activation procedure.

- Option 2: (Apple, OPPO)

  - Periodic and semi-persistent CSI reporting shall be considered for PUCCH SCell activation, like the legacy SCell activation requirement. FFS for aperiodic CSI reporting.

- Option 3: (MTK, Qualcomm, Huawei, CATT)

  - FFS.

Issue 1-2-3: Whether the beam information (L1-RSRP measurement result) of PUCCH SCell for TCI determination is needed or not for unknown cell?

FFS:

- Option 1: (Apple, CATT, Qualcomm, vivo, OPPO, Ericsson, Intel)

  - Same as the beam information indication for determining the associated SSB in PDCCH order for RA.

  - If the target PUCCH Scell is unknown cell in FR2:

    - If there is at least one active serving cell on that FR2 band (following the same conditions in TS38.133 section 8.3.2 for intra-band FR2 Scell activation), no need to indicate the beam information of PUCCH Scell to network for TCI determination.

    - Otherwise, need to indicate the beam information of PUCCH Scell to network for TCI determination.

  - If the target PUCCH Scell is unknown cell in FR1:

    - If it is contiguous to an active serving cell in the same band (following the same conditions in TS38.133 section 8.3.2 for intra-band contiguous FR1 Scell activation), no need to indicate the beam information of PUCCH Scell to network for TCI determination.

    - Otherwise, need to indicate the beam information of PUCCH Scell to network for TCI determination.

- Option 2: (Huawei, Xiaomi, NTT DOCOMO)

  - Beam information is need for unknown PUCCH Scell activation for TCI determination for both valid TA and invalid TA and both FR1 and FR2.

- Option 3: (Nokia, Qualcomm, Huawei)

  - Beam information is need for unknown PUCCH Scell activation for TCI determination for both valid TA and invalid TA.

Issue 1-2-4: Whether the UL spatial relation is needed for PUCCH SCell activation in FR2 for invalid TA case?

Agreement:

The UL spatial relation is needed for PUCCH SCell activation in FR2 for invalid TA case.

Issue 1-2-5: If the answer of issue 1-2-4 is yes, whether the extra delay time due to UL spatial relation activation is needed for PUCCH SCell activation requirements?

Agreement:

- The time uncertainty of the MAC CE for UL spatial relation activation of PUCCH in target being-activated SCell shall be considered.

- FFS: No extra delay time is needed if spatial relation activation command and TCI activation command are received in the same MAC CE.

- Sub-topic 1-3 PUCCH SCell activation requirements applicability regarding to UE capability

Issue 1-3-1: PUCCH Scell activation requirements applicability regarding to UE capability?

Agreements:

For Ues do not support beamCorrespondenceWithoutUL-BeamSweeping, FR2 PUCCH Scell (de)activation requirements are not applied.

- Sub-topic 1-4 PUCCH Scell activation delay requirement for valid TA case

Issue 1-4-1: Whether the Tx power of target PUCCH should be considered in PUCCH Scell activation requirements?

Agreements:

The Tx power of target PUCCH should be considered in PUCCH Scell activation requirements.

Issue 1-4-2: PUCCH Scell activation delay requirement for valid TA case?

Agreements:

  - In FR1, reuse the Rel-15 Scell activation delay requirement which is (( THARQ + Tactivation_time +TCSI_Reporting)/ NR slot length).

  - In FR2, use normal Scell activation delay (i.e., (THARQ + Tactivation_time +TCSI_Reporting)/ NR slot length ) in TS38.133 section 8.3.2 as baseline, but the time uncertainty of the single MAC CE for both UL spatial relation and PL-RS activation of PUCCH in target being-activated Scell shall be considered in the baseline Tactivation_time.

  - FFS: whether additional delay will be introduced due to the time uncertainty.

- Sub-topic 1-5 PUCCH Scell activation delay requirement for invalid TA case

Issue 1-5-1: The PUCCH SCell activation requirements for invalid TA case

FFS:

- Option 1: (CATT)

  - Delay = (( THARQ + Tactivation_time + TCSI_Reporting + TPDCCH + T1 + T2 + T3)/ NR slot length) for invalid TA case.

    - TPDCCH is time from the end of basic Scell activation to the start of PDCCH signal receiving for PRACH transmission.

    - If the PDCCH signal is sent during TCSI_Reporting, TPDCCH  = 0.

- Option 2: (NTT DOCOMO, Apple, Xiaomi, MTK, vivo, OPPO, Ericsson, Qualcomm, Huawei, Intel)

  - If UE does not have the valid TA on the PUCCH Scell being activated, an additional UL synchronization procedure to obtain the valid TA comparing to ( THARQ + Tactivation_time +TCSI_Reporting) shall be considered which including the following factors:

    - the delay uncertainty in acquiring the first available PRACH occasion in the PUCCH Scell(T1);

    - the delay for obtaining a valid TA command for the sTAG to which the Scell configured with PUCCH belongs(T2);

    - the delay for applying the received TA for uplink transmission(T3)

- Option 3: (CMCC)

  - For DL, the Scell activation delay is: (( THARQ + Tactivation_time +TCSI_Reporting)/ NR slot length)

  - For UL, the Scell activation delay is: except THARQ + Tactivation_time +TCSI_Reporting, additional delay including following parts need to be considered for the Scell activation delay requirements specification:

    - the delay uncertainty in acquiring the first available PRACH occasion in the PUCCH Scell

    - the delay for obtaining a valid TA command for the sTAG

    - the delay for applying the received TA for uplink transmission

- Option 4: (Nokia)

  - If the UE does not have a valid TA for transmitting on an Scell, the UE shall be capable to perform downlink actions related to the Scell activation command for the Scell being activated on the PUCCH Scell no later than in slot n+THARQ+Tactivation_timeNR slot length.

  - If the UE does not have a valid TA for transmitting on an Scell, the UE shall be capable to perform uplink actions related to the Scell activation command for the Scell being activated on the PUCCH Scell no later than in slot n+THARQ+Tactivation_time+TRACHNR slot length, where TRACH is the delay to perform RACH procedure and apply the TA.

  - The activation delay requirement for PUCCH Scell shall be defined assuming no dedicated time period for CSI measurements and reporting.

Issue 1-5-2: Whether to define separated requirements for downlink actions and uplink actions?

Agreements:

The timeline for downlink actions and uplink actions could be clarified in the spec similar as LTE, but the delay requirement shall be specified covering DL/UL actions for PUCCH Scell activation with invalid TA.

Issue 1-5-3: the delay for obtaining a valid TA command for the sTAG to which the Scell configured with PUCCH belongs (i.e. T2)

FFS:

- Option 1: (MTK, Apple, Qualcomm, vivo, Xiaomi, NTT DOCOMO, OPPO, Ericsson, Huawei, CATT, Intel)

  - T2 is the delay from slot n + (Tactivate_basic +T1)/NR slot length until UE has obtained a valid TA command for the target PUCCH Scell being activated. Tactivate_basic is the normal Scell activation delay in TS38.133 section 8.3.2. slot n is the slot when UE received PUCCH Scell activation MAC CE.

- Option 2: (Nokia)

  - T2 is the delay from slot n + (THARQ + Tactivatation_time +T1)/NR slot length until UE has obtained a valid TA command for the target PUCCH Scell being activated. Tactivatation_time is defined in TS38.133 section 8.3.2. slot n is the slot when UE received PUCCH Scell activation MAC CE.

Issue 1-5-4: the delay for applying the received TA for uplink transmission on target PUCCH Scell being activated (i.e. T3)

Agreements:

T3 is the delay for applying the received TA for uplink transmission on target PUCCH Scell being activated, and greater than or equal to k+1 slot, where k is defined in clause 4.2 in TS 38.213.

- Sub-topic 1-6 Interruption requirements for PUCCH SCell activation in invalide TA case

Issue 1-6-1: Interruption requirements for PUCCH Scell activation in invalide TA case

FFS:

RAN4 further study the possibility of interruption due to PRACH based on the following options:

- Option 1: (CATT, Nokia)

  - Reuse the interruption requirement of normal Scell activation (i.e. not to define the interruption requirement due to PRACH)

- Option 2: (Apple, OPPO, Qualcomm)

  - The interruption requirement shall include the existing requirement for Scell activation in Rel-15.

  - Introduce additional interruption by PRACH transmission when target PUCCH Scell RACH has different SCS from spCell data/control channel and UE does not support diffNumerologyAcrossPUCCH-Group.

  - FFS: Need to revisit R15 RACH requirement

  - For unknown PUCCH Scell activation, whether or not there can be additional interruptions is up to the final solution, if supported

- Option 3: (Ericsson, Huawei, Nokia)

  - Ask RAN1 whether this is a valid case and how to prioritize between the channels

- Sub-topic 1-7 Applicability of PUCCH SCell activation requirements

Issue 1-7-1 Applicability on interruption:

FFS:

- Option 1: (Apple, CATT, Qualcomm, Huawei, Intel, OPPO)

  - No interruption occurs in same FR as the target PUCCH Scell during the Scell activation procedure if UE supports per-FR MG, otherwise the PUCCH Scell activation delay can be extended, and

  - No interruption occurs during the Scell activation procedure if UE does not support per-FR MG, otherwise the PUCCH Scell activation delay can be extended.

  - The above interruption is caused by factor defined in TS38.133 section 8.2.1.1 for EN-DC, in TS38.133 section 8.2.2.1 for NR SA, in TS38.133 section 8.2.3.1 for NE-DC and in TS38.133 section 8.2.4.1 for NR-DC.

- Option 2: (MTK)

  - No interruption occurs in same FR as the target PUCCH Scell during other Scell activation procedure if UE supports per-FR MG, otherwise the PUCCH Scell activation delay can be extended, and

  - No interruption occurs during other Scell activation procedure if UE does not support per-FR MG, otherwise the PUCCH Scell activation delay can be extended.

  - The above interruption is caused by factor defined in TS38.133 section 8.2.1.1 for EN-DC, in TS38.133 section 8.2.2.1 for NR SA, in TS38.133 section 8.2.3.1 for NE-DC and in TS38.133 section 8.2.4.1 for NR-DC.

Issue 1-7-2: Applicability on PDCCH order receiving:

FFS:

- Option 1: (CATT, Qualcomm)

  - The UE has received a PDCCH order to initiate RA procedure on the PUCCH Scell within TCSI_Reporting (can’t earlier than THARQ + Tactivation_time) otherwise additional delay to activate the Scell is expected as TPDCCH defined in issue 1-5-1.

- Option 2: (Apple, Nokia, Intel, OPPO)

  - The UE has received a PDCCH order to initiate RA procedure on the PUCCH SCell within Tactivate_basic otherwise additional delay to activate the SCell is expected.

- Option 3: (Ericsson)

  - Delay requirements for PUCCH Scell activation shall account for additional time when PDCCH order is received outside Tactivate_basic. The additional time shall be accounted for by an expression and/or a delay component, e.g. max(Tactivate_basic, TPDCCH_order).

Issue 1-7-3: Applicability on SSB configuration:

Agreements:

No requirement defined when the target PUCCH SCell is a SSB-less SCell (i.e. SSB is not configured).

Issue 1-7-4: Applicability on use cases:

FFS:

- Option 1: (vivo, OPPO)

  - RAN4 to discuss and clarify whether the following cases are valid and/or to be considered for PUCCH Scell (de)activation requirements.

    - Single-TAG vs. Multi-TAG for Dual PUCCH

    - Intra- vs. Inter-band between PUCCH cells

- Option 2: (MTK, Huawei, CATT)

  - There is no needed to bundle the PUCCH Scell with single/multiple TAGs or intra-/inter band cases.

Issue 1-7-5 Applicability on multiple SCells:

FFS:

- Option 1: (Ericsson)

  - In activation of multiple Scells with one PUCCH Scell, activation delay requirement shall apply at least for the PUCCH Scell in the event that one or more Scells have configurations that render parallel activation impossible for the UE. FFS on whether activation delay requirement also is to apply for Scells that are compatible with parallel activation with PUCCH Scell.