[]3GPP TSG-RAN WG4 Meeting # 100-e 	R4-2115349

# Electronic Meeting, Aug. 16-27, 2021

Title:	LS on criteria for RLM/BFD relaxation

Response to:

Release:	Rel-17

Work Item:	NR_UE_pow_sav_enh-Core

Source:	RAN WG4

To:	RAN WG2

Cc:	RAN WG1

Contact Person:

Name:	Yanliang SUN, Althea Huang.

E-mail Address:	yanliang.sun@vivo.com

Althea.Huang@mediatek.com

Attachments:

1. Overall Description:

In RAN4 #98e-bis, RAN4 #99e and RAN4 #100e meeting, RAN4 has discussed RLM/BFD relaxation for power saving enhancements.

Conclusions on feasible scenarios have been draw as follows.

- RAN4 concluded the feasible scenario and will define the RLM/BFD requirements for R17 UE measurements relaxation for RLM and/or BFD in work phase for the following cases (in RAN4 #98e-bis)

- Case 1: SSB based RLM/BFD measurement relaxation in FR1

- Case 2: CSI-RS based RLM/BFD measurement relaxation in FR1

- Case 3: CSI-RS based RLM/BFD measurement relaxation in FR2

- Case 4: SSB based RLM/BFD measurement relaxation in FR2

- Relaxed BFD/RLM requirements shall be supported for all deployment scenarios supported by current specification which includes: NR SA, EN-DC, NE-DC, NR intra-band CA, NR inter-band CA and NR-DC. (in RAN4 #99e)

For the relaxation criteria, RAN4 have also achieved agreements as follows.

- Network to enable and disable this feature. (in RAN4 #98e)

- Whether relaxed RLM/BFD requirements can be applied depends on both the serving cell quality and UE mobility state. (in RAN4 #98e-bis)

- If the UE fulfills any of serving cell quality exit condition or low mobility exit condition, or DRX cycle length is NOT allowed for relaxation, UE will exit relaxation mode. (in RAN4 #99e)

- If the UE applies a DRX cycle longer than 80ms, the UE is assumed not to perform relaxed RLM/BFD measurements and the existing RLM/BFD requirements would apply. (in RAN4 #100e)

- When neither serving cell quality criteria nor low mobility criteria is configured, the existing RLM/BFD requirements shall apply. (in RAN4 #100e)

- Note: It can be revisited if

  - dedicated or broadcast signalling to indicate the UE when it is allowed to relax the RLM/BFD measurements is agreed, or

  - good serving cell criteria is agreed to be predefined.

- FFS: Whether low mobility criteria are necessary to be configured

For the low mobility criterion, RAN4 have achieved agreements so far as follows

- Low mobility criteria (in RAN4 #100e)

  - Reuse Rel-16 low mobility criterion based on L3 RSRP measurement variation.

    - FFS the RSs for L3 RSRP measurement

For the cell quality criterion, RAN4 have achieved agreements so far as follows

- Good serving cell quality criteria of RLM/BFD relaxation is defined as the radio link quality is better than a threshold. The radio link quality in good serving cell quality criteria for R17 RLM/BFD relaxation is based on SINR. (in RAN4 #98e-bis)

- UE reuse the SINR for RLM/BFD evaluation when determine whether the serving cell quality criteria is fulfilled or not (in RAN4 #99e)

- FFS: (in RAN4 #100e)

  - SINR definition for good serving cell quality criteria

  - predefined or configured threshold

According to the WID RP-193239, RAN2 work will be triggered by RAN4 LS.

  - Study the feasibility and performance impact of relaxing UE measurements for RLM and/or BFD, particularly for low mobility UE with short DRX periodicity/cycle, and specify, if agreed, relaxation in the corresponding requirements [RAN4]

- NOTE: Supplementary RAN2 work, if needed, can be triggered by RAN4 LS

RAN4 respectfully ask RAN2 take above conclusions into considerations and start work in RAN2. RAN2 may work on at least following aspects:

- Specify corresponding signalling for the relaxation criteria defined by RAN4

If there is further RAN4 conclusions that need RAN2 work in the future meetings, it will be informed to RAN2.

2. To RAN WG2 group.

ACTION: 	RAN4 respectfully asks RAN2 to take RAN4 conclusions in consideration and start RAN2 work on this topic.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 Meeting #101e	1 - 12, Nov, 2021	Online

TSG-RAN4 Meeting #101e-bis	TBD	Online