3GPP TSG-RAN WG4 Meeting # 100-e 												R4-2115384

Electronic Meeting, 16th – 27th August, 2021

Agenda item:			6.1.6.2.2

Source:	Moderator (Intel Corporation)

Title:	Email discussion summary for [100-e][210] NR_pos_RRM_Part_2

Document for:	Information

# Introduction

The scope of this email discussion is UE RRM requirements for NR positioning from the following agenda items:

- AI 6.1.6.2.1 RRM Perf requirements: General

- AI 6.1.6.2.2. UE requirements and test cases

In providing comments, companies are encouraged to:

- Be concise

- Provide comments on all topics/sub-topics of interest

- Ensure that comments are inserted in the latest version of the document by checking the folder before uploading

- Use “Track changes” to help identify added comments/changes

# Topic #1: General performance requirements for NR Positioning (AI 6.1.6.2.2.1)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2114451 Positioning RRM performance requirements in Rel-17 | Ericsson | CR for Rel17 |

## Open issues summary and companies views’ collection for 1st round

N.A.

### Open issues

N.A.

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2114451 Ericsson | Moderator: Shall be discussed in 4.2. |

## Summary for 1st round

### Open issues

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |

## Discussion on 2nd round

Please only comment on topics that are selected for discussion in 2nd round.

Sub-topic#1-1

## Summary on 2nd round

# Topic #2: Measurement Accuracy Requirements for PRS RSTD (AI6.1.6.2.2.2.1)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2112544 | vivo | Proposal 1: RF margin, including group delay margin and frequency drift margin, for the RSTD accuracy requirements is 1Ts (64Tc) in both FR1 and FR2 |
| R4-2113154 | Intel Corporation | Proposal 1a: The requirements for AWGN in FR1 based on the different PRS measurement bandwidth can be: Accuracy, Tc+; PRS BW, PRB; PRS SCS, kHz; Repetition factor (TrepPRS*LPRS/KcombPRS). [132 + margin]; ≥ [24]; 15; ≥4. [98 + margin]; ≥ [52]; All. [42 + margin]; ≥ [104]; All. [53 + margin]; ≥ [24]; 30; ≥4. [48 + margin]; ≥ [48]; All. [24 + margin]; ≥ [132]; All. [50 + margin]; ≥ [24]; 60; ≥4. [24+ margin]; ≥ [64]; All. [10+ margin]; ≥ [132]; All. Note 1: Margin value is FFS. Proposal 1b: The requirements for fading channel based on the different PRS measurement bandwidth can be: Accuracy, Tc+; PRS BW, PRB; PRS SCS, kHz; Repetition factor (TrepPRS*LPRS/KcombPRS). [247 + margin]; ≥ [24]; 15; ≥4. [140 + margin]; ≥ [52]; All. [86 + margin]; ≥ [104]; All. [169 + margin]; ≥ [24]; 30; ≥4. [109 + margin]; ≥ [48]; All. [28 + margin]; ≥ [132]; All. [147 + margin]; ≥ [24]; 60; ≥4. [27+ margin]; ≥ [64]; All. [21+ margin]; ≥ [132]; All. Note 1: Margin value is FFS. Observation 1: The group delay calibration margin should be specified depending on PRS bandwidth (e.g. inversely scaled by the bandwidth). Observation 2 : The maximum frequency drift between the measured TRPs can be dependent with the maximum time offsets among the measured TRPs/cells (e.g. . 2* expectedRSTD) |
| R4-2113264 | OPPO | Proposal 1: Group delay margin should scale inversely with PRS subcarrier spacing. Proposal 2: The values of group delay margin in table 1 could be used as the starting point. Table 1: Group delay margin for RSTD accuracy Subcarrier Spacing(kHz); 15; 30; 60; 120. UE Timing Advance adjustment accuracy; ±256 Tc; ±256 Tc; ±128 Tc; ±32 Tc. Group delay margin for RSTD accuracy (1/4); ±64 Tc; ±64Tc; ±32 Tc; ±8 Tc. Proposal 3: The value of additional margin with different PFLs should be inversely correlated with PRS subcarrier spacing and bandwidth. Proposal 4: The frequency drift margin for FR1 could be +/-32Tc. Proposal 5: The frequency drift margin for FR2 could be +/-16Tc. |
| R4-2114196 | Qualcomm Incorporated | Proposal 1: RAN4 should add different group delay margins for RSTD when neighbor and reference PRS resources are in the same PFL and in different PFLs. Observation 1: In general, the group delay calibration margin should scale inversely with PRS bandwidth. Whether a fixed margin value can be used for the case of neighbor and reference PRS resources in the same PFL is FFS. Proposal 2: For PRS-RSTD measurements in FR1 with neighbor and reference PRS resources in the same PFL, the value of the group delay calibration margin is FFS. Proposal 3: For PRS-RSTD measurements in FR1 with neighbor and reference PRS resources in different PFLs, the value of the group delay calibration margin is FFS. Proposal 4: For PRS-RSTD measurements in FR2 with neighbor and reference PRS resources in the same PFL, the value of the group delay calibration margin is FFS. Proposal 5: For PRS-RSTD measurements in FR1 with neighbor and reference PRS resources in different PFLs, the value of the group delay calibration margin is FFS. Observation 2: Degradation in RSTD accuracy due to frequency error will depend on TRS configuration. Observation 3: Degradation in RSTD accuracy due to frequency error is expected to increase with the time separation between neighbor and reference PRS resources. Proposal 6: RAN4 will discuss TRS configuration assumptions for specifying the frequency error margin for RSTD. e.g. TRS BW ≥ [52] RB and TRS periodicity [20] msec. Proposal 7: RAN4 will discuss whether to limit the applicability of RSTD measurement accuracy requirements based on time proximity between neighbor and reference PRS resources or to add a frequency error margin that scales with the time separation between neighbor and reference PRS resources. |
| R4-2114282 | Huawei, HiSilicon | Proposal 1: Add the following group delay calibration margin for RSTD accuracy. - For FR1, when the reference cell and neighbour cell are on same PFL: ±8ns for PRS BW ≥ 50MHz and ±16ns for PRS BW < 50MHz - For FR1, when the reference cell and neighbour cell are on different PFLs: ±16ns for PRS BW ≥ 50MHz and ±32ns for PRS BW < 50MHz - For FR2: ±16ns for PRS BW >= 50MHz and ±32ns for PRS BW < 50MHz Proposal 2: For the case where reference resource and neighbor resource are on same PLF, add a margin of +/-32Tc for RSTD accuracy requirements, provided that the separation between the reference resource and the neighbor resource is within 160ms. Proposal 3: For the case where reference resource and neighbor resource are on different PLFs, no RSTD accuracy requirements are defined in Rel-16. Proposal 4: Ask RAN2 to update the RSTD reporting signaling in Rel-17 to allow UE reporting an RSTD reference resource for each PFL. Proposal 5: Confirm the condition of ‘no-repetition’ for smallest RB number in case of AWGN FR2 |
| R4-2114283 | Huawei, HiSilicon | Draft CR : - Add the the applicability that requirements apply only when reference and neighbour resource are on the same PFL due to frequency drift issue |
| R4-2114284 | Huawei, HiSilicon | Draft CR |

## Open issues summary and companies’ views collection for 1st round

### Sub-topic 2-1 Group delay calibration margin

- Option 1(vivo): fixed value for all parameters combinations.

- Option 2 : different values depending on PRS  parameters combination

  - PRS BW (Intel, Qualcomm, Huawei)

  - SCS(OPPO)

  - PFLs (Qualcomm, Huawei)

- Option 2a (Huawei) :

  - For FR1, when the reference cell and neighbour cell are on same PFL: ±8ns for PRS BW ≥ 50MHz and ±16ns for PRS BW < 50MHz

  - For FR1, when the reference cell and neighbour cell are on different PFLs: ±16ns for PRS BW ≥ 50MHz and ±32ns for PRS BW < 50MHz

  - For FR2: ±16ns for PRS BW >= 50MHz and ±32ns for PRS BW < 50MHz

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Intel | Support Option 2. Regarding to the PRS measurement accuracy depended on several factors e.g. PRS BW, PFL, we believed that the margin shall be dynamic for the requirement sets. |
| Qualcomm | We support option 2. Overall, for FR1 there are five PRS BW (MHz) values in the accuracy requirements: 5, 10, 20, 50, 100 MHz. For FR2 there are four values: 20, 50, 100, 200 MHz. We think it is reasonable to differentiate the delay margin across these different BW values. Also, as option 2 indicates, we support different margin values for single PFL and dual PFL. |
| Huawei | Option 2, but we are not sure if PRs SCS needs to be considered. Option 2a is our proposal is on the exact values and we look forward to other opinions. |
| OPPO | Support option 2, and we also support PRS BW. We noticed that most timing requirements in TS 38.133 is related to the SCS of reference signal. That is the motivation to introduce SCS here. |
| vivo | We do not fully understand what option 2 means. In our view, RF group delay calibration error for RSTD measurement would be mainly to account for group delay error (difference) introduced by different receiver chains (including antenna). How is that dependent on PRS BW or SCS? |

### Sub-topic 2-2 Frequency drift margin

- Option 1 (vivo, OPPO): fixed value

- Option 2(Intel, Huawei, Qualcomm): The maximum frequency drift between the measured TRPs can be dependent with the maximum time offsets among the measured TRPs/cells.

- Option 2a(Huawei) :

  - For the case where reference resource and neighbor resource are on same PLF, add a margin of +/-32Tc for RSTD accuracy requirements, provided that the separation between the reference resource and the neighbor resource is within 160ms

  - For the case where reference resource and neighbor resource are on different PLFs, no RSTD accuracy requirements are defined in Rel-16. Ask RAN2 to update the RSTD reporting signaling in Rel-17 to allow UE reporting an RSTD reference resource for each PFL.

- Option 3 (Qualcomm) Depending on TRS BW

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Intel | Support Option 2. For option 2a, we need check how much the maximum offset between the measured TRPs |
| Qualcomm | All options are not mutually exclusive. We agree with the statement in option 2 and, therefore, we support limiting applicability of RSTD accuracy requirements to a maximum separation between neighbor and reference PRS resources. In our view, RAN4 should seek agreement on this principle first. Regarding option 2a, there wouldn’t be a need to differentiate between one PFL and two different PFLs provided that the proximity condition (to be agreed) is satisfied. |
| Huawei | Option 2a. On option 1, are fine to use a fixed value, but it should be conditioned on same PFL and a max separation between reference and neighbor resources. On option 2 and response to QC, we understand when reference and neighbor resources are on different PFLs, even they are in the same symbol, the time when they are measured can be far apart because UE is assumed to measure each PFL in sequential manner, and the measurement period for each PFL can be rather long. On option 3, we are not sure if we need to define conditions on TRS. Current Te requirements is based on SSB, so does option 3 mean to do positioning, UE needs to do better or more frequent T/F tracking based on serving cell TRS, more than what is needed for data communication? |
| OPPO | Option 1. Generally, we are not opposed to option 2. The frequency drift should be dependent on the max time offset between measured TRPs/cells. But the PRS configuration is quite flexible and we do not expect to define too many different frequency drift values like PRS bandwidth. So, it is proposed to use single fixed frequency drift margin derived by the max time offset. |
| vivo | Option 1. In our view the UE frequency should be updated periodically so the drifted is limited. |

### Sub-topic 2-3 RSTD accuracy requirements for FR1/FR2

[Moderator notes:

Background:  In the last meeting the RSTD accuracy requirement are TBD for the case below.

FR1, PRS BW≥ [24],  SCS=30kHz.

Based on the collected simulation results in this meeting [R4-2109238], the performance requirements for the difference SCS with the similar PRS BW(one PRS BW  group)  and repetition factor can be summarized below.

- For AWGN:

| FR | PRS BW (PRBs) | SCS(kHz) | rep | Requirement(N.A.) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Intel | Ericsson | vivo | Huawei | QC | CATT | OPPO | avg |  |
| FR1 | 24 | 30 | 4 | 53 | 75 | 70 | 89 |  |  |  | 80 |  |

- For fading channel :

| FR | PRS BW (PRBs) | SCS(kHz) | rep | Requirement(N.A.) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Intel | Ericsson | vivo | Huawei | QC | CATT | OPPO | avg |
| FR1 | 24 | 30 | 4 | 169 | 93 | 144 | 113 |  |  |  | 129 |

Therefore, could we agree the proposal below.

Proposal 1: RSTD accuracy requirements under the AWGN can be:

| Accuracy, Tc+ | PRS BW, PRB | PRS SCS, kHz | Repetition factor (TrepPRS*LPRS/KcombPRS) |
| --- | --- | --- | --- |
| [132 + margin] | ≥ [24] | 15 | ≥4 |
| [98 + margin] | ≥ [52] |  | All |
| [42 + margin] | ≥ [104] |  | All |
| [80 + margin] | ≥ [24] | 30 | ≥4 |
| [48 + margin] | ≥ [48] |  | All |
| [24 + margin] | ≥ [132] |  | All |
| [50 + margin] | ≥ [24] | 60 | ≥4 |
| [24+ margin] | ≥ [64] |  | All |
| [10+ margin] | ≥ [132] |  | All |
| Note 1: Margin value is FFS |  |  |  |

Proposal 2: RSTD accuracy requirements under the fading channels can be:

| Accuracy, Tc+ | PRS BW, PRB | PRS SCS, kHz | Repetition factor (TrepPRS*LPRS/KcombPRS) |
| --- | --- | --- | --- |
| [247 + margin] | ≥ [24] | 15 | ≥4 |
| [140 + margin] | ≥ [52] |  | All |
| [86 + margin] | ≥ [104] |  | All |
| [129 + margin] | ≥ [24] | 30 | ≥4 |
| [109 + margin] | ≥ [48] |  | All |
| [28 + margin] | ≥ [132] |  | All |
| [147 + margin] | ≥ [24] | 60 | ≥4 |
| [27+ margin] | ≥ [64] |  | All |
| [21+ margin] | ≥ [132] |  | All |
| Note 1: Margin value is FFS |  |  |  |

]

Recommended WF: Check the proposals on the RSTD accuracy requirements above can be agreeable for companies.

| Company | Comments |
| --- | --- |
| Intel | Up to the latest simulation results collected in [R4-2113156], the requirements of the TBD cases can be agreed as in the recommended WF |
| Qualcomm | AWGN FR; PRS BW (PRBs); SCS(kHz); rep; Requirement(N.A.). Intel; Ericsson; vivo; Huawei; QC; CATT; OPPO; avg. FR1; 24; 30; 4; 53; 75; 70; 89; 66; 70. TDLA FR; PRS BW (PRBs); SCS(kHz); rep; Requirement(N.A.). Intel; Ericsson; vivo; Huawei; QC; CATT; OPPO; avg. FR1; 24; 30; 4; 169; 93; 144; 113; 97; 118. |
| Huawei | Fine to derive the requirement based on average of simulation results. |
| vivo | Support the accuracy requirements are derived from simulation results. |

### CRs/TPs

[Moderator notes: suggest take one of these CR drafts as the baseline which can be revised in 2nd round discussion.]

| CR/TP number | Comments collection |
| --- | --- |
| R4-2114283 (Huawei, Hi Silicon) | Intel: Revision needed to include the agreements of issue 2-5 |
|  | Qualcomm2: Pending issue 2-2 |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 | Group delay calibration margin Tentative agreements: None. Candidate options: - Option 1(vivo): fixed value for all parameters combinations. - Option 2 : different values depending on PRS parameters combination - PRS BW (Intel, Qualcomm, Huawei) - SCS(OPPO) - PFLs (Qualcomm, Huawei) - Option 2a (Huawei) : - For FR1, when the reference cell and neighbour cell are on same PFL: ±8ns for PRS BW ≥ 50MHz and ±16ns for PRS BW < 50MHz - For FR1, when the reference cell and neighbour cell are on different PFLs: ±16ns for PRS BW ≥ 50MHz and ±32ns for PRS BW < 50MHz - For FR2: ±16ns for PRS BW >= 50MHz and ±32ns for PRS BW < 50MHz Recommendations for 2nd round: Can be FFS. |
| Sub-topic#2-2 | Frequency drift margin Tentative agreements: - Option 1 (vivo, OPPO): fixed value - Option 2(Intel, Huawei, Qualcomm): The maximum frequency drift between the measured TRPs can be dependent with the maximum time offsets among the measured TRPs/cells. - Option 2a(Huawei) : - For the case where reference resource and neighbor resource are on same PLF, add a margin of +/-32Tc for RSTD accuracy requirements, provided that the separation between the reference resource and the neighbor resource is within 160ms - For the case where reference resource and neighbor resource are on different PLFs, no RSTD accuracy requirements are defined in Rel-16. Ask RAN2 to update the RSTD reporting signaling in Rel-17 to allow UE reporting an RSTD reference resource for each PFL. - Option 3 (Qualcomm) Depending on TRS BW Candidate options: Recommendations for 2nd round: Can be FFS. |
| Sub-topic#2-3 | RSTD accuracy requirements for FR1/FR2 Tentative agreements: RSTD measurement accuracy requirements can be defined as below according to [R4-2113156] RSTD accuracy requirements under the AWGN can be: Accuracy, Tc+; PRS BW, PRB; PRS SCS, kHz; Repetition factor (TrepPRS*LPRS/KcombPRS). [132 + margin]; ≥ [24]; 15; ≥4. [98 + margin]; ≥ [52]; All. [42 + margin]; ≥ [104]; All. [75 + margin]; ≥ [24]; 30; ≥4. [48 + margin]; ≥ [48]; All. [24 + margin]; ≥ [132]; All. [50 + margin]; ≥ [24]; 60; ≥4. [24+ margin]; ≥ [64]; All. [10+ margin]; ≥ [132]; All. Note 1: Margin value is FFS. RSTD accuracy requirements under the fading channels can be: Accuracy, Tc+; PRS BW, PRB; PRS SCS, kHz; Repetition factor (TrepPRS*LPRS/KcombPRS). [247 + margin]; ≥ [24]; 15; ≥4. [140 + margin]; ≥ [52]; All. [86 + margin]; ≥ [104]; All. [118 + margin]; ≥ [24]; 30; ≥4. [109 + margin]; ≥ [48]; All. [28 + margin]; ≥ [132]; All. [147 + margin]; ≥ [24]; 60; ≥4. [27+ margin]; ≥ [64]; All. [21+ margin]; ≥ [132]; All. Note 1: Margin value is FFS. Candidate options: Recommendations for 2nd round: No need further discussion. |

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2114283 | Revised |

## Discussion on 2nd round

Please only comment on topics that are selected for discussion in 2nd round.

#### Sub-topic 2-1 Group delay calibration margin

Recommended WF: In order to converge the discussion step by step , please companies provide further views on the proposals below firstly.

- Option 1(vivo): fixed value for all parameters combinations.

- Option 2 : different values depending on PRS  parameters combination

  - PRS BW (Intel, Qualcomm, Huawei)

  - SCS(OPPO)

  - PFLs (Qualcomm, Huawei)

| Company | Comments |
| --- | --- |
| Intel | As a compromise, the option 2 can be revised as: - Option 2 : different values depending on PRS parameters combination, which can be FFS: - PRS BW (Intel, Qualcomm, Huawei) - SCS(OPPO) - PFLs (Qualcomm, Huawei) |
| Qualcomm | We support option 2. Overall, for FR1 there are five PRS BW (MHz) values in the accuracy requirements: 5, 10, 20, 50, 100 MHz. For FR2 there are four values: 20, 50, 100, 200 MHz. We think it is reasonable to differentiate the delay margin across these different BW values. Also, as option 2 indicates, we support different margin values for single PFL and dual PFL. |
| E/// | Option 2 |
| Huawei | Option 2 |
| Moderator | Please companies check the further tentative agreements below: - the group delay calibration margin for the RSTD accuracy requirements can be defined as the different values depending on PRS parameters combination, - which PRS parameter combination can be FFS - PRS BW (Intel, Qualcomm, Huawei) - SCS(OPPO) - PFLs (Qualcomm, Huawei) |
| GTW agreements | - Group delay calibration margin for the RSTD accuracy requirements is FFS - Different values for the group delay calibration margin for the RSTD accuracy requirements can be defined depending on PRS parameters combination - FR(frequency range) - PRS BW (absolute BW) - Number of PFLs and whether reference/neighbor cells belong to same PFL |

#### Sub-topic 2-2 Frequency drift margin

- Option 1 (vivo, OPPO): fixed value

- Option 2(Intel, Huawei, Qualcomm): The maximum frequency drift between the measured TRPs can be dependent with the maximum time offsets among the measured TRPs/cells.

- Option 2a(Huawei) :

  - For the case where reference resource and neighbor resource are on same PLF, add a margin of +/-32Tc for RSTD accuracy requirements, provided that the separation between the reference resource and the neighbor resource is within 160ms

  - For the case where reference resource and neighbor resource are on different PLFs, no RSTD accuracy requirements are defined in Rel-16. Ask RAN2 to update the RSTD reporting signaling in Rel-17 to allow UE reporting an RSTD reference resource for each PFL.

- Option 2b (Huawei): The frequency drift margin in RSTD accuracy can be dependent with the time offsets between the time points when reference resource and neighbor resource are measured.

- Option 3 (Qualcomm) Depending on TRS BW

| Company | Comments |
| --- | --- |
| Moderator | In our understanding, Option 2 is the general principle to define such frequency drfit margin, which is not exclusive with Option 1. For an example, if the maximum time offset (separation) between the measurements on the different TRPs is static, the value of such margin shall be fixed. Thus, could we accept the Option 2, and the exact value can be FFS. |
| Qualcomm | Option 2. Let’s agree on the general principle first. The exact value is FFS. |
| E/// | Option 2 |
| Huawei | We can support option 2 but the wording may to be fine-tuned. The exact TP suggestion can be found in option 2b above with change marks based on option 2. |
| Moderator | Please companies check the further tentative agreements below: The frequency drift margin in RSTD accuracy can be dependent with the time offsets between the two measurements on the PRS resources from the reference cell and neighbor cells which will be used to a single RSTD estimation. |
| Moderator | Revision of above tentative agreements: - “The frequency drift margin in RSTD accuracy can be dependent with the maximum time offsets between a pair of measurements on the PRS resources from the reference cell and neighbor cells which will be used to a single RSTD estimation. - The maximum time offset between a pair of measurements on the PRS resources from the reference cell and neighbor cells can be FFS - The exact value of the frequency drift margin can be FFS |
| GTW agreements | - The frequency drift margin in RSTD accuracy is FFS - Frequency drift margin will be derived on the maximum time offsets between the two PRS resources instance from the reference cell and neighbor cells which will be used to a single RSTD measurement. - The maximum time offset for margin definition can be FFS - A single fixed margin will be used for requirements definition |

## Summary on 2nd round

| CR/TP/LS/WF number | T-doc status update recommendation |
| --- | --- |
| R4-2115308 | Agreeable |
| R4-2115307 | Agreeable |

# Topic #3: Measurement Accuracy Requirements for PRS RSRP (AI6.1.6.2.2.2.2)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2111989 | CATT | Proposal 1: The RF calibration margin for PRS RSRP measurement is 2dB for FR1 and FR2. Proposal 2: The PRS RSRP accuracy requirements in extreme condition are X dB larger than that in normal condition, and X is: - 3dB for absolute accuracy for FR1. - 3dB for absolute accuracy for FR2. - 1dB for relative accuracy for FR1. - 3dB for relative accuracy for FR2. Proposal 3: For the PRS RSRP accuracy requirements in extreme condition, define a single value for each SINR side condition. |
| R4-2111991 | CATT | draftCR |
| R4-2112545 | vivo | Proposal 1: For PRS-RSRP measurement from resources in the same resource set in FR1, no RF calibration margin is added in the relative accuracy requirements. Proposal 2: The margin for PRS-RSRP accuracy requirements under extreme conditions are - 3dB for absolute accuracy for FR1. - 3dB for absolute accuracy for FR2. - 1dB for relative accuracy for FR1. - 3dB for relative accuracy for FR2. |
| R4-2113265 | OPPO | Proposal 1: The PRS-RSRP measurement accuracy requirements in extreme condition are X dB larger than that in normal condition, and X is: - [3]dB for absolute accuracy for FR1. - 3dB for absolute accuracy for FR2. - [1]dB for relative accuracy for FR1. - 3dB for relative accuracy for FR2. Proposal 2: Use 2dB as the RF calibration margin for relative PRS-RSRP accuracy requirements for FR1. |
| R4-2113869 | ZTE | - Proposal 1: PRS RSRP measurement requirements in extreme condition are X dB larger than that in normal condition, and X is: - 3dB for absolute accuracy for FR1. - 3dB for absolute accuracy for FR2. - 1dB for relative accuracy for FR1. - 3dB for relative accuracy for FR2. |
| R4-2114285 | Huawei, HiSilicon | Proposal 1: RAN4 does not define PRS-RSRP accuracy requirements for extreme condition in Rel-16. Proposal 2: confirm that relative PRS-RSRP accuracy requirements only apply for PRS-RSRP measured from resources in the same resource set, and with same Rx beam in case of FR2. Proposal 3: Add a margin of 2dB for FR1 and 4dB for FR2 for relative PRS-RSRP accuracy requirements. |
| R4-2114203 Draft CR: Corrections to NR positioning performance requirements | Qualcomm Incorporated | Draft CR: Delete PRS-RSRP relative accuracy requirements in FR1 since RAN4 did not agree those requirements are applicable to FR1. |

## Open issues summary and companies’ views collection for 1st round

### Sub-topic 3-1 RSRP requirements under the extreme conditions

- Option 1 (CATT, vivo, OPPO, ZTE, Ericsson). The PRS RSRP measurement requirements in extreme condition are X dB larger than that in normal condition, and X is:

  - 3dB for absolute accuracy for FR1.

  - 3dB for absolute accuracy for FR2.

  - 1dB for relative accuracy for FR1.

  - 3dB for relative accuracy for FR2

- Option 2 (Huawei, Intel, Qualcomm): No need to define PRS RSRP accuracy requirements with the extreme conditions

Recommended WF:  Further discussion needed. Collect companies’ views

| Company | Comments |
| --- | --- |
| ZTE | Support Option 1. |
| CATT | Support option 1. Different from the timing measurement, for power measurement, the requirements for extreme condition are defined in the existing specification. We think PRS-RSRP should also follow the principle. |
| Intel | Slightly prefer Option 2 |
| Qualcomm | We tend to align with option 2. We are not sure if there is a strong need to add such requirements. |
| Huawei | Prefer option 2. |
| OPPO | Option 1. |
| Ericsson | Option 1. |
| vivo | Option 1. |

### Sub-topic 3-2 RF calibration margin for the relative accuracy requirements

[Moderator notes:

Background:  The agreements in the last meeting were:

  - FFS on the exact value of RF calibration margin

    - For FR1:

      - Option 1: [2dB] for FR1

      - Option 2: [0dB] for FR1

]

Candidate options:

- Option 1 (CATT, OPPO, Huawei, vivo): [2dB] for FR1

- Option 2 (vivo): [0dB] for FR1

Recommended WF:   Further discussion needed. Collect companies’ views

| Company | Comments |
| --- | --- |
| CATT | We are fine with either option. |
| Intel | Option 1 |
| Qualcomm | Neither option. In RAN4#99-e, RAN4 agreed that PRS-RSRP relative accuracy requirements apply only to FR2. |
| Huawei | Option 1. To QC, we have different understanding about the agreement last meeting. We think DL-AoD can be also used in FR1, so relative RSRP accuracy requirements should also apply in FR1. For FR2 we have an additional applicability condition that the two PRS resources are measured with same Rx beam, but it does not mean the requirements apply only for FR2. |
| OPPO | Option 1. |
| Ericsson | We can support Option 1 under the assumption that 2 dB will be added to the compiled simulation results. Otherwise we support Option 2. |
| vivo | Option 2. Like intra-frequency relative accuracy requirements for SSB based measurement, no RF calibration is needed. If majority companies prefer option 1, we are also fine. |

### CRs/TPs

[Moderator notes: suggest take one of these CR drafts as the baseline which can be revised in 2nd round discussion.]

| CR/TP number | Comments collection |
| --- | --- |
| R4-2111991 (CATT) | Intel: Can be revised after issues above agreed |
|  | Qualcomm2: RSRP relative accuracy requirements for FR1 should be removed. |
| R4-2114203 Qualcomm (Correction #1) | CATT: we think the first deletion is not necessary since it is the definition of PRS-RSRP measurement and not contradictory with the applicability. Also we think the relative accuracy requirements have been defined in previous meeting for FR1 and FR2 respectively. We don’t understand where we have the agreement that the requirements are not applicable for FR1 and we think the requirements should not be removed. |
|  | Intel: The part of RSRP can be merged with R4-2111991 |

‘

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic#3-1 | RSRP requirements under the extreme conditions Tentative agreements: None Candidate options: - Option 1 (CATT, vivo, OPPO, ZTE, Ericsson). The PRS RSRP measurement requirements in extreme condition are X dB larger than that in normal condition, and X is: - 3dB for absolute accuracy for FR1. - 3dB for absolute accuracy for FR2. - 1dB for relative accuracy for FR1. - 3dB for relative accuracy for FR2 - Option 2 (Huawei, Intel, Qualcomm): No need to define PRS RSRP accuracy requirements with the extreme conditions Recommendations for 2nd round: Can be FFS |
| Sub-topic#3-2 | RF calibration margin for the relative accuracy requirements Tentative agreements: - RF calibration margin for FR1 PRS RSRP relative accuracy requirements can be [2dB] Candidate options: Recommendations for 2nd round: Need not further discussion |

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2111991 | Revised |

## Discussion on 2nd round

Please only comment on topics that are selected for discussion in 2nd round.

#### Sub-topic 2-1 RSRP requirements under the extreme conditions

Recommended WF: Please companies provide further views on the proposals below.

- Option 1 (CATT, vivo, OPPO, ZTE, Ericsson). The PRS RSRP measurement requirements in extreme condition are X dB larger than that in normal condition, and X is:

  - 3dB for absolute accuracy for FR1.

  - 3dB for absolute accuracy for FR2.

  - 1dB for relative accuracy for FR1.

  - 3dB for relative accuracy for FR2

- Option 2 (Huawei, Intel, Qualcomm): No need to define PRS RSRP accuracy requirements with the extreme conditions

| Company | Comments |
| --- | --- |
| ZTE | Prefer Option 1. This is a common practice and in 38.133 and a lot of accuracy requirements are defined with extreme conditions. Thus, we suggest to also define extreme conditions for PRS-RSRP. |
| Intel | We are fine either of them. |
| Qualcomm | Prefer option 2. |
| E/// | Option 1 |
| Huawei | Slightly prefer option 2. |
| CATT | Prefer option 1. Similar view as ZTE, we don’t see the difference between PRS-RSRP measurement and other RSRP measurement. |

## Summary on 2nd round

| CR/TP/LS/WF number | T-doc status update recommendation |
| --- | --- |
| R4-2115307 | Agreeable |
| R4-2115309 | Agreeable |

# Topic #4: Measurement Accuracy Requirements for UE Rx-Tx Time Difference (AI6.1.6.2.2..3.)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2111990 | CATT | N.A |
| R4-2112546 | vivo | Proposal 1: UE Rx-Tx measurement accuracy requirements shall apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment. Proposal 2: Group delay calibration margin for the UE Rx-Tx time difference accuracy requirements is 0.5Ts (32Tc) in both FR1 and FR2. |
| R4-2113155 | Intel Corporation | Proposal 2a: The requirements for AWGN in FR1 based on the different PRS measurement bandwidth can be: Table 1a. UE Rx-Tx time difference accuracy requirements in FR1 Accuracy, Tc; Es/Iot, dB; PRS BW, PRB; PRS SCS, kHz; Repetition factor (TrepPRS*LPRS/KcombPRS). [78+margin]; -3; ≥[24]; 15; ≥4. [59 +margin]; ≥[52]; All. [30 +margin]; >[104]; All. [12+margin]; ≥[24]; 30; ≥4. [30 +margin]; ≥[48]; All. [15 +margin]; ≥[132]; All. [29 +margin]; ≥[24]; 60; ≥4. [15 +margin]; ≥[64]; All. [7+margin]; ≥[132]; All. [101 +margin]; -13; ≥[24]; 15; ≥4. [75 +margin]; ≥[52]; All. [39+margin]; >[104]; All. [43 +margin]; ≥[24]; 30; ≥4. [37+margin]; ≥[48]; All. [16 +margin]; ≥[132]; All. [36 +margin]; ≥[24]; 60; ≥4. [16+margin]; ≥[64]; All. [8 +margin]; ≥[132]; All. Note 1: Margin value is FFS. Proposal 2b: The requirements for fading channel in FR1 based on the different PRS measurement bandwidth can be: Table 1b. UE Rx-Tx time difference accuracy requirements in FR1 Accuracy, Tc; Es/Iot, dB; PRS BW, PRB; PRS SCS, kHz; Repetition factor (TrepPRS*LPRS/KcombPRS). [137+margin]; -3; ≥[24]; 15; ≥4. [96+margin]; ≥[52]; All. [62 +margin]; >[104]; All. [56+margin]; ≥[24]; 30; ≥4. [68 +margin]; ≥[48]; All. [44 +margin]; ≥[132]; All. [59 +margin]; ≥[24]; 60; ≥4. [42 +margin]; ≥[64]; All. [36 +margin]; ≥[132]; All. [180+margin]; -13; ≥[24]; 15; ≥4. [98+margin]; ≥[52]; All. [68 +margin]; >[104]; All. [71+margin]; ≥[24]; 30; ≥4. [85 +margin]; ≥[48]; All. [44 +margin]; ≥[132]; All. [139 +margin]; ≥[24]; 60; ≥4. [46 +margin]; ≥[64]; All. [30 +margin]; ≥[132]; All. Note 1: Margin value is FFS. |
| R4-2113870 | ZTE Corporation | Proposal 1: UE Rx-Tx measurement accuracy requirements shall apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment. |
| R4-2114197 | Qualcomm Incorporated | Proposal 1: UE Rx-Tx measurement accuracy requirements shall apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment. Observation 1: The group delay calibration margin should scale inversely with PRS bandwidth. Proposal 2: For UE Rx-Tx measurements in FR1, add a group delay calibration margin of [4] ns * (100/BW), where BW is the PRS bandwidth in MHz, to the accuracy requirements. Proposal 3: For UE Rx-Tx measurements in FR2, the value of the group delay calibration margin is FFS. |
| R4-2114286 | Huawei, HiSilicon | Proposal 1: UE Rx-Tx accuracy requirements do not apply in case there is UE autonomous timing adjustment during the measurement period. Proposal 2: Confirm the condition of ‘no-repetition’ for smallest RB number in case of AWGN FR2. Proposal 3: Add the following group delay calibration margin for UE Rx-Tx accuracy. - ±24ns for PRS BW ≥ 50MHz - ±48ns for PRS BW < 50MHz |
| R4-2114459 | Ericsson | - Observation 1: The UL autonomous timing adjustment, which is based on the DL timing, follows the DL timing change, resulting in the UE transmit timing changes in the same direction in which the DL timing - Proposal 1: The UE Rx-Tx measurement accuracy requirements shall apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to UE autonomous timing adjustment defined in clause 7.1.2, TS 38.133. |
| R4-2114460 | Ericsson | draft CR for Rel16 |
| R4-2114203 Draft CR: Corrections to NR positioning performance requirements | Qualcomm Incorporated | Correct errors UE Rx-Tx time difference accuracy requirements based on agreements from RAN4#99-e. |
| R4-2114204 Draft CR: Corrections to NR positioning performance requirements | Qualcomm Incorporated | CR for Rel17 |

## Open issues summary and companies’ views collection for 1st round

### Sub-topic 4-1 Applicability of accuracy requirements under TA adjustment

- Option 1. (Huawei )

  - UE Rx-Tx measurement accuracy requirements shall not apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment

- Option 2. (vivo,ZTE,  Qualcomm, OPPO, Ericsson)

  - UE Rx-Tx measurement accuracy requirements shall apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment

Recommended WF:  Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| ZTE | Support Option 2, the UE can maintain its timing under this case. |
| CATT | Fine with option 2. |
| Intel | Option 2 |
| Qualcomm | Option 2 |
| Huawei | Option 1 The problem to apply the accuracy requirements is how to define the ideal value. With UE autonomous timing adjustment, UE could have more than one UL timing during the measurement period, and it’s unclear which one should be used to determine the ideal Rx-Tx. It is noted that the UL timing can change up to 5.5 Ts with a single adjustment, which is much larger than the Ue Rx-Tx errors we are discussing for positioning. Some companies commented that for UE autonomous timing adjustment, the UL is following the DL, and this is the difference from the NW commanded TA. We agree that this is the case for the serving cell, where the ideal UE Rx-Tx for the serving cell does not change much with UE autonomous timing adjustment (but still the accuracy is Te as UE is only required to meet the UL timing accuracy), but this is clearly not the case for eighbor cells, for which the ideal UE Rx-Tx changes. |
| Ericsson | In principle we support Option 2. There is at least no issue on serving cell because UE TX and RX timings change in the same direction. But we agree with HW that UE Rx-Tx measurement performed the on neighbor cell under autonomous timing adjustment will cause additional error. This is because the UE TX timing is changed due to DL reference cell but there is no similar change in the UE RX in the measured neighbor cell. But in this context use of serving vs neighbor cells may be confusing and bit misleading. It is more correct to state the impact on accuracy depending on whether UE Rx-Tx is measured on the cell which also contains SRS or not? One compromise solution can be as follows. The wording can be improved: - If SRS is transmitted on a cell where UE Rx-Tx measurement is performed then the UE Rx-Tx measurement accuracy requirements shall apply even if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment. - If SRS is transmitted on a cell different than the cell where UE Rx-Tx measurement is performed then the UE Rx-Tx measurement accuracy requirements shall not apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment. |
| Vivo | Option 2. |

### Sub-topic 4-2 Group delay calibration margin

- Option 1(vivo): fixed value for both FR1 and FR2, e.g. 0.5Ts (32Tc) in both FR1 and FR2

- Option 2a(Qualcomm):

  - FR1: depending on the PRS parameters. E.g. scaling inversely with PRS BW

  - FR2: FFS

- Option 2b(Huawei): FR1 &2 : depending on the PRS parameters. E.g.

  - ±24ns for PRS BW ≥ 50MHz

  - ±48ns for PRS BW < 50MHz

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| ZTE | Support Option 2a. |
| Intel | The same principle to define RSTD group delay can be reused here. |
| Qualcomm | We would like to clarify our proposal. In option 2a, the inverse scaling with PRS BW should apply to both FR1 and FR2. Also our margin proposal for FR1 was not included above. Proposal 2: For UE Rx-Tx measurements in FR1, add a group delay calibration margin of [4] ns * (100/BW), where BW is the PRS bandwidth in MHz, to the accuracy requirements. |
| Huawei | Option 2b. |
| vivo | Option 1. The value can be further discussed. The group delay calibration error for UE Rx-Tx measurement is to account for calibration error for single RF chain (including antenna). It is not clear how it is dependent on PRS BW. |

### Sub-topic 4-3 UE Rx-Tx time difference measurement accuracy requirements

[Moderator notes:

Background:  In the last meeting the RSTD accuracy requirement are TBD for the case below.

FR1, PRS BW≥ [24],  SCS=30kHz.

Based on the collected simulation results in this meeting [R4-2113156], the performance requirements for the difference SCS with the similar PRS BW(one PRS BW  group)  and repetition factor can be defined.

Proposal a: The requirements for AWGN in FR1 based on the different PRS measurement bandwidth can be:

Table 1a. UE Rx-Tx time difference accuracy requirements in FR1

| Accuracy, Tc | Es/Iot, dB | PRS BW, PRB | PRS SCS, kHz | Repetition factor (TrepPRS*LPRS/KcombPRS) |
| --- | --- | --- | --- | --- |
| [78+margin] | -3 | ≥[24] | 15 | ≥4 |
| [59 +margin] |  | ≥[52] |  | All |
| [30 +margin] |  | >[104] |  | All |
| [57+margin] |  | ≥[24] | 30 | ≥4 |
| [30 +margin] |  | ≥[48] |  | All |
| [15 +margin] |  | ≥[132] |  | All |
| [29 +margin] |  | ≥[24] | 60 | ≥4 |
| [15 +margin] |  | ≥[64] |  | All |
| [7+margin] |  | ≥[132] |  | All |
| [101 +margin] | -13 | ≥[24] | 15 | ≥4 |
| [75 +margin] |  | ≥[52] |  | All |
| [39+margin] |  | >[104] |  | All |
| [58 +margin] |  | ≥[24] | 30 | ≥4 |
| [37+margin] |  | ≥[48] |  | All |
| [16 +margin] |  | ≥[132] |  | All |
| [36 +margin] |  | ≥[24] | 60 | ≥4 |
| [16+margin] |  | ≥[64] |  | All |
| [8 +margin] |  | ≥[132] |  | All |
| Note 1: Margin value is FFS |  |  |  |  |

Proposal b: The requirements for fading channel in FR1 based on the different PRS measurement bandwidth can be:

Table 1b. UE Rx-Tx time difference accuracy requirements in FR1

| Accuracy, Tc | Es/Iot, dB | PRS BW, PRB | PRS SCS, kHz | Repetition factor (TrepPRS*LPRS/KcombPRS) |
| --- | --- | --- | --- | --- |
| [137+margin] | -3 | ≥[24] | 15 | ≥4 |
| [96+margin] |  | ≥[52] |  | All |
| [62 +margin] |  | >[104] |  | All |
| [80+margin] |  | ≥[24] | 30 | ≥4 |
| [68 +margin] |  | ≥[48] |  | All |
| [44 +margin] |  | ≥[132] |  | All |
| [59 +margin] |  | ≥[24] | 60 | ≥4 |
| [42 +margin] |  | ≥[64] |  | All |
| [36 +margin] |  | ≥[132] |  | All |
| [180+margin] | -13 | ≥[24] | 15 | ≥4 |
| [98+margin] |  | ≥[52] |  | All |
| [68 +margin] |  | >[104] |  | All |
| [79+margin] |  | ≥[24] | 30 | ≥4 |
| [85 +margin] |  | ≥[48] |  | All |
| [44 +margin] |  | ≥[132] |  | All |
| [139 +margin] |  | ≥[24] | 60 | ≥4 |
| [46 +margin] |  | ≥[64] |  | All |
| [30 +margin] |  | ≥[132] |  | All |
| Note 1: Margin value is FFS |  |  |  |  |

]

Recommended WF: Check the proposals on the accuracy requirements above can be agreeable for companies.

| Company | Comments |
| --- | --- |
| Intel | The recommended WF and requirements can be agreed. |
| Qualcomm | AWGN Es/IoT; Huawei; QC; vivo; Intel; Ericsson; Average. -3dB; 57; 57; 62; 12; 56.46; 57. -13 dB; 58; 58; 68; 43; 57.03; 58. TDLA Es/IoT; Huawei; QC; vivo; Intel; Ericsson; Average. -3dB; 58; 103; 104; 71; 88.28; 87. -13 dB; 59; 104; 109; 56; 98.3; 87. |
| Huawei | Fine to derive the requirement based on average of simulation results. |
| vivo | Support the accuracy requirements are derived from simulation results. |

### CRs/TPs

| CR/TP number | Comments collection |
| --- | --- |
| R4-2114460 (Ericsson) | Intel:Need to be revised after issues above agreed. |
|  | R&S: Is the test expected to be changed given the extension of the core requirements? |
| R4-2114203 Qualcomm Incorporated (correction #2) | Intel: The part of UE Rx-Tx time difference measurement can be merged with R4-2114460 |
| R4-2114451 Ericsson | Intel: For Rel17 |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic#4-1 | Applicability of accuracy requirements in the case of in the case of TA adjustment Tentative agreements: Candidate options: - Option 1. (Huawei ) - UE Rx-Tx measurement accuracy requirements shall not apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment - Option 2. (vivo,ZTE, Qualcomm, OPPO, Ericsson, Intel) - UE Rx-Tx measurement accuracy requirements shall apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment - Option 3 (Ericsson) - If SRS is transmitted on a cell where UE Rx-Tx measurement is performed then the UE Rx-Tx measurement accuracy requirements shall apply even if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment. - If SRS is transmitted on a cell different than the cell where UE Rx-Tx measurement is performed then the UE Rx-Tx measurement accuracy requirements shall not apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment Recommendations for 2nd round: Can be FFS. |
| Sub-topic#4-2 | Group delay calibration margin Tentative agreements: None. Candidate options: - Option 1(vivo): fixed value for both FR1 and FR2, e.g. 0.5Ts (32Tc) in both FR1 and FR2 - Option 2a(Qualcomm): - FR1: depending on the PRS parameters. E.g. [4] ns * (100/BW) - FR2: depending on the PRS parameters - Option 2b(Huawei): FR1 &2 : depending on the PRS parameters. E.g. - ±24ns for PRS BW ≥ 50MHz - ±48ns for PRS BW < 50MHz Recommendations for 2nd round: Can be FFS |
| Sub-topic#4-3 | UE Rx-Tx time difference measurement accuracy requirements Tentative agreements: UE Rx-Tx time difference measurement accuracy requirements can be defined as below according to [R4-2113156] Table 1a. UE Rx-Tx time difference accuracy requirements in FR1 under AWGN Accuracy, Tc; Es/Iot, dB; PRS BW, PRB; PRS SCS, kHz; Repetition factor (TrepPRS*LPRS/KcombPRS). [78+margin]; -3; ≥[24]; 15; ≥4. [59 +margin]; ≥[52]; All. [30 +margin]; >[104]; All. [57+margin]; ≥[24]; 30; ≥4. [30 +margin]; ≥[48]; All. [15 +margin]; ≥[132]; All. [29 +margin]; ≥[24]; 60; ≥4. [15 +margin]; ≥[64]; All. [7+margin]; ≥[132]; All. [101 +margin]; -13; ≥[24]; 15; ≥4. [75 +margin]; ≥[52]; All. [39+margin]; >[104]; All. [58 +margin]; ≥[24]; 30; ≥4. [37+margin]; ≥[48]; All. [16 +margin]; ≥[132]; All. [36 +margin]; ≥[24]; 60; ≥4. [16+margin]; ≥[64]; All. [8 +margin]; ≥[132]; All. Note 1: Margin value is FFS. Table 1b. UE Rx-Tx time difference accuracy requirements in FR1 under fading channel Accuracy, Tc; Es/Iot, dB; PRS BW, PRB; PRS SCS, kHz; Repetition factor (TrepPRS*LPRS/KcombPRS). [137+margin]; -3; ≥[24]; 15; ≥4. [96+margin]; ≥[52]; All. [62 +margin]; >[104]; All. [87+margin]; ≥[24]; 30; ≥4. [68 +margin]; ≥[48]; All. [44 +margin]; ≥[132]; All. [59 +margin]; ≥[24]; 60; ≥4. [42 +margin]; ≥[64]; All. [36 +margin]; ≥[132]; All. [180+margin]; -13; ≥[24]; 15; ≥4. [98+margin]; ≥[52]; All. [68 +margin]; >[104]; All. [87+margin]; ≥[24]; 30; ≥4. [85 +margin]; ≥[48]; All. [44 +margin]; ≥[132]; All. [139 +margin]; ≥[24]; 60; ≥4. [46 +margin]; ≥[64]; All. [30 +margin]; ≥[132]; All. Note 1: Margin value is FFS. Candidate options: Recommendations for 2nd round: No further discussion. These requirements will be included in the corresponding CRs. |

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2114203 | Merged |
| R4-2114460 | Revised |
| R4-2114451 | Postpone |

## Discussion on 2nd round

Please only comment on topics that are selected for discussion in 2nd round.

#### Sub-topic 4-1 Applicability of accuracy requirements in the case of in the case of TA adjustment

Recommended WF: Please companies provide further views on the proposals below, especially check the Option 3 as a compromise can be acceptable.

- Option 1. (Huawei )

  - UE Rx-Tx measurement accuracy requirements shall not apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment

- Option 2. (vivo,ZTE,  Qualcomm, OPPO, Ericsson, Intel)

  - UE Rx-Tx measurement accuracy requirements shall apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment

- Option 3 (Ericsson)

  - If SRS is transmitted on a cell where UE Rx-Tx measurement is performed then the UE Rx-Tx measurement accuracy requirements shall apply even if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment.

  - If SRS is transmitted on a cell different than the cell where UE Rx-Tx measurement is performed then the UE Rx-Tx measurement accuracy requirements shall not apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment

| Company | Comments |
| --- | --- |
| ZTE | Support Option 2 and can also support Option 3. |
| Intel | Both option 2 and 3 are fine for us. |
| Qualcomm | We support option 2. We need more time to evaluate option 3. |
| Ericsson | While we have been supportive of option 2. But technically it is not correct for measurement on cells where SRS is not transmitted. Option 1 is too conservative: Autonomous adjustment may be applied any time. So practically the UE may often not meet accuracy for all multi-RTT cells. So we support Option 3. |
| Huawei | We can go with option 3 as a compromise, but we suggest to use the wording of ‘serving cell’ and ‘neighbor cell’ because SRS can be only transmitted on a serving cell. So the current wording in option 3 is bit confusing. We cannot agree to option 2 because the ideal Rx-Tx may change by several Ts due to UE autonomous adjustment. |
| CATT | Option 2 and can option 3. |
| Ericsson2 | To HW: It is valid scenario that UE Rx-Tx is configured on serving cell different than SpCell where typically SRS is transmitted i.e. SRS can be on any serving cell (section 9.9.4.2, 38.133). That’s UE measures UE Rx-Tx based on PRS from SCell and SRS sent on SpCell (e.g. PCell). In such case (even though is serving cell i.e. SCell) the UE Rx-Tx accuracy should not apply. But UE Rx-Tx accuracy on SpCell can apply. So, in our view condition for applying UE Rx-Tx accuracy linked to serving vs neighbor cell can be misleading. But condition for applying UE Rx-Tx accuracy for a cell based on whether SRS is transmitted on that cell is sufficient and unambiguous. If companies want then we can also include PRS i.e. whether SRS and PRS are transmitted on the same cell where measurement is done or not e.g. - If SRS and PRS are transmitted on the same cell where UE Rx-Tx measurement is performed then the UE Rx-Tx measurement accuracy requirements shall apply even if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment. - If SRS transmitted on a cell different than the cell on which the PRS is transmitted and UE Rx-Tx measurement is performed then the UE Rx-Tx measurement accuracy requirements shall not apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment |

#### Sub-topic 4-2 Group delay calibration margin

Recommended WF: Please companies provide further views on the proposals below, and the same principle as RSTD RF margin can be considered.

- Option 1(vivo): fixed value for both FR1 and FR2, e.g. 0.5Ts (32Tc) in both FR1 and FR2

- Option 2a(Qualcomm):

  - FR1: depending on the PRS parameters. E.g. [4] ns * (100/BW)

  - FR2: depending on the PRS parameters

- Option 2b(Huawei): FR1 &2 : depending on the PRS parameters. E.g.

  - ±24ns for PRS BW ≥ 50MHz

  - ±48ns for PRS BW < 50MHz

| Company | Comments |
| --- | --- |
| ZTE | Support Option 2a. Fixed values might not be accurate enough, thus prefer not to go with Option 1 or 2b. |
| Intel | We can follow the same principle for RSTD margin. |
| Qualcomm | Option 2a. We would like to clarify that this margin would only account for group delay calibration error. |
| E/// | Agree with Intel that same principle as for RSTD is used for deriving the margin. We should avoid fixed margin like in option 1 since in NR there are several variables: SCS, BW, PFL, FR1 vs FR2 |
| Huawei | Option 2b. On option 2a, as the requirements are derived based on only 1) the baseband error we got from simulations, and 2) the calibration margin discussed here, so it should include all imperfections other than the baseband estimation error. |

## Summary on 2nd round

| CR/TP/LS/WF number | T-doc status update recommendation |
| --- | --- |
| R4-2115307 | Agreeable |
| R4-2115310 | Agreeable |

# Topic #5: Test cases (AI6.1.6.2.2.3)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2111993 | CATT | DraftCR on test case for PRS-RSRP measurement requirements for FR2 in SA |
| R4-2111994 | CATT | DraftCR on test case for PRS-RSRP measurement requirements for FR2 in SA |
| R4-2113091 | Rohde & Schwarz | Draft-CR to TS 38.133: Correction to PRS configuration for positioning test cases (Rel 16) |
| R4-2113092 | Rohde & Schwarz | Draft-CR to TS 38.133: Correction to PRS configuration for positioning test cases (Rel 17) |
| R4-2113445 | CATT | Draft CR on test case for RSTD measurement requirements in SA |
| R4-2113446 | CATT | Draft CR on test case for RSTD measurement requirements in SA |
| R4-2113447 | CATT | Draft CR on test case for RSTD accuracy requirements |
| R4-2113448 | CATT | Draft CR on test case for RSTD accuracy requirements |
| R4-2114287 | Huawei, HiSilicon | Proposal 1: Starting point of T2 (measurement period) in the tests should be the first MG occasion with PRS after n+T, where n is the last PDSCH for the LPP messages including the assistance data and measurement request, and T=50ms. Proposal 2: Update the PRS RMC to introduce PRS configurations with two PRS resources per resource set. Proposal 3: Simplify the test procedure by using 2 time periods in the tests: - T1: UE receives assistance data and measurement request - T2: UE performs the measurement |
| R4-2114288 | Huawei, HiSilicon | CR to update PRS RMC for positioning tests |
| R4-2114289 | Huawei, HiSilicon | CR to update PRS RMC for positioning tests R17 |
| R4-2114290 | Huawei, HiSilicon | CR to update TC for PRS-RSRP measurement requirements for FR1 in SA |
| R4-2114291 | Huawei, HiSilicon | CR to update TC for PRS-RSRP measurement requirements for FR1 in SA R17 |
| R4-2114292 | Huawei, HiSilicon | CR to update TC for RSTD measurement accuracy for FR1 and FR2 in SA |
| R4-2114293 | Huawei, HiSilicon | CR to update TC for RSTD measurement accuracy for FR1 and FR2 in SA R17 |

## Open issues summary and companies’ views collection for 1st round

### Sub-topic 5-1 Processing time for the assistance data and measurement request

- Option 1 (Huawei)

  - Starting point of T2 (measurement period) in the tests should be the first MG occasion with PRS after n+T, where n is the last PDSCH for the LPP messages including the assistance data and measurement request, and T=50ms.

Recommended WF: Further discussion needed. Collect companies’ views

| Company | Comments |
| --- | --- |
| Intel | Can be FFS. |
| Qualcomm | The current value assumed in the tests is T=150ms, which was copied from LTE. If there is consensus that the current value is sufficient for NR positioning then we might leave it as is. In our view, it is OK to be conservative and this test setting should not impact the test outcome. |
| Huawei | No strong view. If there is concern from companies to update the value, we are also fine to use the current value 150ms which is from LTE. |
| Ericsson | We are fine with changing to new value i.e. T=50ms |

### Sub-topic 5-2	 Updated PRS RMC

- Option 1 (Huawei):

  - Update the PRS RMC to introduce PRS configurations with two PRS resources per resource set.

Recommended WF: Further discussion needed. Collect companies’ views

| Company | Comments |
| --- | --- |
| Intel | Option 1 can be fine for us. |
| Qualcomm | Option 1. It follows a prior agreement. |
| Huawei | Option 1 |
| Ericsson | Option 1 |

### Sub-topic 5-3 Test procedure simplification

- Option 1 (Huawei) : Simplify the test procedure by using 2 time periods in the tests:

  - T1: UE receives assistance data and measurement request

  - T2: UE performs the measurement

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| CATT | Support option 1. |
| Intel | Can be FFS to check the potential standardization efforts since many existing TCs shall be updated. |
| Qualcomm | Support option 1, unless any issues have been identified with the proposed simplification. |
| Huawei | Option 1 |

### CRs/TPs

| CR/TP number | Comments collection |
| --- | --- |
| R4-2111993 draftCR on test case for PRS-RSRP measurement requirements for FR2 in SA (CATT) | Intel: Agreeable |
|  | Huawei: we noticed that the title of this CR is for FR2 TC but it also contains changes to FR1 TC. We have a CR R4-2114290 to update PRS-RSRP delay TCs for FR1 based on the work split, so would it be possible to revise this CR to focus on FR2 TC and keep all changes to FR1 TC in R4-2114290? Technically, Expected RSTD and Expected RSTD uncertainty should be kept, as they are used for all positioning methods for UE to determine the location of neighbor cell PRS. |
|  | Qualcomm2: Update PRS configurations. Use new PRS.1.3 FR2 and PRS.1.4 FR2 from Huawei. |
| R4-2113091 Draft-CR to TS 38.133: Correction to PRS configuration for positioning test cases (Rel 16) (R&S) | Intel: Can be merged with R4-2114288 |
|  | Huawei: the PRS BW is not limited by the serving cell CBW. We think we should rather remove the Channel BW row and keep the PRS RB number. We have done this change in our CR R4-2114288 |
|  | Qualcomm2: Not agreeable. Remove channel BW from section titles and table captions instead. |
| R4-2113445 draftCR for the test case of RSTD measurement requirements in SA (CATT) | Intel: need not to define the SSB SINR |
|  | Qualcomm2: Fix repeated corrections. e.g. Reference cell is the cell in the DL-TDOADL-TDOA, TS 37.355[34]37.355[34]. |
| R4-2113447 Draft CR on test case for RSTD accuracy requirement (CATT) | Qualcomm2: Change #2 is not needed. Expected RSTD and Expected RSTD uncertainty are relative to the reference cell. |
| R4-2114288 CR to update PRS RMC for positioning tests (Huawei) | Qualcomm2: Agree with adding the new RMC but the number of RBs shouldn’t be changed. |
| R4-2114290 draftCR to introduce TC for PRS-RSRP measurement requirements for FR1 in SA (Huawei) | Qualcomm2: PRS configs should be updated to PRS.1.3 FR1 and PRS.2.3 FR1 from R4-2114288. T may need to be updated based on conclusion of issue 5-1. |
| R4-2114292 CR to update TC for RSTD measurement accuracy for FR1 and FR2 in SA (Huawei) | Qualcomm2: Pending issue 2-2. |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic#5-1 | Processing time for the assistance data and measurement request Tentative agreements: Starting point of T2 (measurement period) in the tests should be the first MG occasion with PRS after n+T, where n is the last PDSCH for the LPP messages including the assistance data and measurement request, and T=50ms Candidate options: Recommendations for 2nd round: No need to further discussion. |
| Sub-topic #5-2 | Updated PRS RMC Tentative agreements: Update the PRS RMC to introduce PRS configurations with two PRS resources per resource set Candidate options: Recommendations for 2nd round: No need further discussion. Check the corresponding CRs directly. |
| Sub-topic#5-3 | Test procedure simplification Tentative agreements: - Simplify the test procedure by using 2 time periods in the tests: - T1: UE receives assistance data and measurement request - T2: UE performs the measurement Candidate options: Recommendations for 2nd round : No need further discussion. The CR to update TCs can be provided in the future. |

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2111993 | Revised |
| R4-2113091 | Merged |
| R4-2113445 | Revised |
| R4-2113447 | Merged |
| R4-2114288 | Revised |
| R4-2114290 | Revised |
| R4-2114292 | Revised |

## Discussion on 2nd round

Please only comment on topics that are selected for discussion in 2nd round.

## Summary on 2nd round

| CR/TP/LS/WF number | T-doc status update recommendation |
| --- | --- |
| R4-2115307 | Return |
| R4-2115311 (from R4-2111993). | Agreeable |
| R4-2115312 (from R4-2113445). | Agreeable |
| Revised to R4-2115313 (from R4-2114288). | Agreeable |
| R4-2115314 (from R4-2114290). | Agreeable |
| R4-2115315 (from R4-2114292). | Agreeable |

# Simulation results

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2113156 | Intel Corporation | Summary of link level simulation results for RSTD, PRS RSRP and UE Rx-Tx time difference |
| R4-2112547 | vivo | Additional link level simulation results for NR positioning |
| R4-2114281 | Huawei, HiSilicon | Additional simulation results for PRS measurement performance |
| R4-2114458 | Ericsson | Link level simulation results for RSTD, PRS RSRP and UE Rx-Tx time difference |

## Open issues summary

[Moderator notes: all simulation results from the different companies were be included in R4-2113156

for information only.]

### Sub-topic 6-1 Simulation results collection (For information only)

## Summary for 1st round

# Recommendation for Tdocs

After first round:

| Tdoc No. | Source company |  | Recommendation | Remarks |  |
| --- | --- | --- | --- | --- | --- |
| Big CR |  |  |  |  |  |
| CR for accuracy requirements |  |  |  |  |  |
| R4-2114283 | Huawei, Hi Silicon |  | Revised |  |  |
| R4-2111991 | CATT |  | Revised |  |  |
| R4-2114203 | Qualcomm |  | Merged | Correction #1 can be merged with R4-2111991 Correction #2 can be merged with R4-2114460 |  |
| R4-2114460 | Ericsson |  | Revised |  |  |
| R4-2114451 | Ericsson |  | Postpone | Rel17 mirror CRs can be postpone |  |
| CR for TC |  |  |  |  |  |
| R4-2111993 | CATT |  | Revised | test case for PRS-RSRP measurement requirements for FR2 in SA |  |
| R4-2113091 | R&S |  | Merged | CR for PRS configuration merged with R4-2114288 |  |
| R4-2113445 | CATT |  | Revised | test case for RSTD measurement requirements in SA |  |
| R4-2113447 | CATT |  | Merged | test case for RSTD accuracy requirements Merged with R4-2114292 |  |
| R4-2114288 | Huawei, Hi Silicon |  | Revised | PRS RMC |  |
| R4-2114290 | Huawei, Hi Silicon |  | Revised | TC for PRS-RSRP measurement requirements for FR1 in SA Merged with |  |
| R4-2114292 | Huawei, Hi Silicon |  | Revised | TC for RSTD measurement accuracy for FR1 and FR2 in SA |  |
| WF |  |  |  |  |  |
| New | Intel |  | Return |  |  |
| Simulation results collection |  |  |  |  |  |
| R4-2113156 | Intel | Noted |  |  |  |

After 2nd round:

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2115308 (from R4-2114283). | CR on accuracy requirements for RSTD measurement | Huawei, Hi Silicon | Agreeable |  |
| R4-2115309 (from R4-2111991). | DraftCR on PRS-RSRP accuracy requirements | CATT | Agreeable |  |
| R4-2115310 (from R4-2114460). | CR on UE Rx-Tx measurement accuracy requirements | Ericsson | Agreeable |  |
| R4-2115311 (from R4-2111993). | DraftCR on test case for PRS-RSRP measurement requirements for FR2 in SA | CATT | Agreeable |  |
| R4-2115312 (from R4-2113445). | Draft CR on test case for RSTD measurement requirements in SA | CATT | Agreeable |  |
| Revised to R4-2115313 (from R4-2114288). | CR to update PRS RMC for positioning tests | Huawei, Hi Silicon | Agreeable |  |
| R4-2115314 (from R4-2114290). | CR to update TC for PRS-RSRP measurement requirements for FR1 in SA | Huawei, Hi Silicon | Agreeable |  |
| R4-2115315 (from R4-2114292). | CR to update TC for RSTD measurement accuracy for FR1 and FR2 in SA | Huawei, Hi Silicon | Agreeable |  |
| R4-2115307 | WF on NR Positioning Performance Requirements | Intel | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents