3GPP TSG- RAN WG4 Meeting #100-e	R4-2115698

# Electronic Meeting, August 16-27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | draft | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS38.101-4 on PDSCH requirements for standalone NR-U (Rel-16) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Perf |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The requiremnt values for both 2x2 and 2x4 cases are not captured correctly - Reference channel number should follow the last two digit of table number |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The requiremnt values for both 2x2 and 2x4 cases are corrected according to R4-2109351 - Modify the reference channel number in Table A.3.2.2.2-18 and the corresponding reference channel number in Table 5.2.2.2.15-3 and Table 5.2.3.2.15-3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The values for requirements are wrong |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 5.2.2.1, 5.2.2.2, 5.2.2.2.15, 5.2.3.2.15 and A.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE 1

##### 5.2.2.2.15	Minimum requirements for PDSCH of PCell on band with shared spectrum access

<SKIP UNCHANGED PART>

Table 5.2.2.2.15-3: Minimum performance for Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-18.1 TDD | 20 / 30 | 16QAM, 0.48 | FR1.30-7 | TDLA30-10 | 2x2, ULA Low | 70 | 13.8 |
| 1-2 | R.PDSCH.2-18.2 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-7 | TDLA30-10 | 2x2, ULA Low | 70 | 14.1 |
| 1-3 | R.PDSCH.2-18.3 TDD | 60 / 30 | 16QAM, 0.48 | FR1.30-7 | TDLA30-10 | 2x2, ULA Low | 70 | 14.2 |
| 1-4 | R.PDSCH.2-18.4 TDD | 80 / 30 | 16QAM, 0.48 | FR1.30-7 | TDLA30-10 | 2x2, ULA Low | 70 | 14.5 |

END OF CHANGE 1

START OF CHANGE 2

##### 5.2.3.2.15	Minimum requirements for PDSCH of PCell on band with shared spectrum access

<SKIP UNCHANGED PART>

Table 5.2.3.2.15-3: Minimum performance for Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-18.1 TDD | 20 / 30 | 16QAM, 0.48 | FR1.30-7 | TDLA30-10 | 2x4, ULA Low | 70 | 8.7 |
| 1-2 | R.PDSCH.2-18.2 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-7 | TDLA30-10 | 2x4, ULA Low | 70 | 8.7 |
| 1-3 | R.PDSCH.2-18.3 TDD | 60 / 30 | 16QAM, 0.48 | FR1.30-7 | TDLA30-10 | 2x4, ULA Low | 70 | 8.9 |
| 1-4 | R.PDSCH.2-18.4 TDD | 80 / 30 | 16QAM, 0.48 | FR1.30-7 | TDLA30-10 | 2x4, ULA Low | 70 | 9.1 |

END OF CHANGE 2

START OF CHANGE 3

#### A.3.2.2.2	Reference measurement channels for SCS 30 kHz FR1

<SKIP UNCHANGED PART>

Table A.3.2.2.2-18: PDSCH Reference Channel for PDSCH on band with shared spectrum access with TDD UL-DL pattern FR1.30-7

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-18.1 TDD | R.PDSCH.2-18.2 TDD | R.PDSCH.2-18.3 TDD | R.PDSCH.2-18.4 TDD |
| Channel bandwidth | MHz | 20 | 40 | 60 | 80 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 |
| Allocated resource blocks | PRBs | 51 | 106 | 162 | 217 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |
| For Slot 0 and slot i, if mod(i, 10) =7 for i from {0,…,39} |  | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {3, 5, 6} for i from {1,…,39} | symbol | {4,7,10,12} | {4,7,10,12} | {4,7,10,12} | {4,7,10,12} |
| For Slot i, if mod(i, 10) = {0, 1, 2, 4} for i from {1,…,39} | symbol | 12 | 12 | 12 | 12 |
| 3 Allocated slots per 2 frames | slot | 31 | 31 | 31 | 31 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 13 | 13 | 13 | 13 |
| Modulation |  | 16QAM | 16QAM | 16QAM | 16QAM |
| Target Coding Rate |  | 0.48 | 0.48 | 0.48 | 0.48 |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 |
| Number of DMRS REs |  |  |  |  |  |
| For Slot 0 and slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {3, 5, 6} for i from {1,…,39} | symbol | {6,12} | {6,12} | {6,12} | {6,12} |
| For Slot i, if mod(i, 10) = {0, 1, 2, 4} for i from {1,…,39} | symbol | 12 | 12 | 12 | 12 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slot 0 and slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {3, 5, 6} for i from {1,…,39} | Bits | {8192,14088， 16392,25608} | {16896,29192,44040,53288} | {26120,45096, 67584,81976,} | {34816,60456, 90176,110632} |
| For Slot i, if mod(i, 10) = {0, 1, 2, 4} for i from {1,…,39} | Bits | 25608 | 53288 | 81976 | 110632 |
| Transport block CRC per Slot |  |  |  |  |  |
| For Slot 0 and slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slot 0 and slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {3, 5, 6} for i from {1,…,39} | CBs | {1,2,4,4} | {3,4,6,7} | {4,6,9,10} | {5,8,11,14} |
| For Slot i, if mod(i, 10) = {0, 1, 2, 4} for i from {1,…,39} | CBs | 4 | 7 | 10 | 14 |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slot 0 and slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {3, 5, 6} for i from {1,…,39} | Bits | {17136,29376,44064,53865} | {35616,61056,91854,11193} | {54432,93312, 139968,171072} | {72912,124992,187488,229152} |
| For Slot i, if mod(i, 10) = {0, 1, 2, 4} for i from {1,…,39} | Bits | 53865 | 111936 | 171073 | 229152 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: For {a1,a2,a3,a4 }, a1, a2, a3 and a4 stand for the setup when the number of OFDM sybmols is 6,9,12,14 respectively. Note 5: The slot i, mod (i,10)=9 is idle slot with no UL transmission. |  |  |  |  |  |

END OF CHANGE 3