3GPP TSG-RAN WG4 Meeting # 100-e 												R4-2115724

Electronic Meeting, 16 – 27 August, 2021

Agenda item:			9.8.3

Source:	CMCC

Title:	WF on FR1 HST demodulation

Document for:	Approval

Introduction

This is the WF to capture all agreements and open issues in [100-e][324] NR_HST_FR1_Demod for RAN4#100e meeting

Topic #1: PDSCH requirements for CA scenarios

Issue 1-1: SCS configurations and applicability rules for SCS configuration

GTW agreement: Option 1 is agreed

|  | Number of CA Duplex modes | Number of required tests if UE supports all duplex modes |
| --- | --- | --- |
| Option 1 (Apple, ZTE, Ericsson, Huawei, Qualcomm, Intel) | 3 (FDD 15+FDD 15, FDD 15+TDD 30, TDD 30+TDD 30) | 2 (FDD 15 + TDD 30; TDD 30 + TDD 30) If UE supports both FDD 15 kHz + TDD 30 kHz and FDD 15 kHz + FDD 15 kHz CA duplex modes, apply requirements only to the first one |

Issue 1-2: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme

- Option 1:

- When UE declares the capability demodulationEnhancement-r16, UE need to pass HST-SFN JT for CA. UE can skip HST-DPS for CA

- When UE does not declare the capability demodulationEnhancement-r16, UE need to pass HST-DPS for CA. UE can skip HST-SFN for CA.

- Option 2: Distribute the HST CA test cases into different SCS combinations. it can be ensured that performance under both HST-SFN CA and HST-DPS CA are verified by this compromise way

| UE capability |  |  | Cases to be tested |  |  |
| --- | --- | --- | --- | --- | --- |
| demodulationEnhancement-r16 | more than 1 SCS configurations | more than 1 CC can track 2 active TCI states | FDD 15 + TDD 30 | and/or | TDD 30 + TDD 30 |
| x | x | x | DPS 1a | or | DPS 1a |
| x | x | √ | DPS 1b | or | DPS 1b |
| x | √ | x | DPS 1a | and | DPS 1a |
| x | √ | √ | DPS 1b | and | DPS 1b |
| √ | x | x | SFN and DPS 1a | or | SFN and DPS 1a |
| √ | x | √ | SFN and DPS 1b | or | SFN and DPS 1b |
| √ | √ | x | SFN | and | DPS 1a |
| √ | √ | √ | SFN | and | DPS 1b |

Issue 1-2a: How to handle a Rel-15/16 UE supporting normal HST-SFN and CA separately but not supporting normal HST-SFN CA defined in Rel-17?

- Option 1:

- Define UE capability for HST-SFN CA, such as demodulationEnhancementCA-r17 (This capability can be release independent from Rel-15), that is different from the capability of demodulationEnhancement-r16 for HST-SFN single carrier

  - The granularity is per band to allow UE to report the supporting HST-SFN CA in some bands

- Other option is not precluded

Issue 1-3: Applicability rule between single carrier and CA

- Option 1：

- UE skip single carrier test case if it explicitly passes corresponding CA test case

  - If UE pass the HST-SFN JT requirements for CA, UE can skip HST-SFN JT requirements for single carrier defined in Rel-16.

    - FFS: UE need to pass Rel-16 HST-DPS single carrier test according to the capability of active TCI state handling

    - UE need to pass Rel-16 FDD HST single tap test.

    - UE can skip Rel-16 TDD and Rel-15 HST single tap test.

  - If UE pass the HST-DPS requirements for CA, UE can skip HST-DPS requirements for single carrier defined in Rel-16.

    - FFS: UE don’t need to pass Rel-16 HST-SFN JT single carrier test if UE does not have the capability demodulationEnhancement-r16

    - FFS: UE need to pass Rel-16 HST-SFN JT single carrier test if UE have the capability demodulationEnhancement-r16

    - UE need to pass Rel-16 FDD HST single tap test.

    - UE can skip Rel-16 TDD and Rel-15 HST single tap tests.

- Other options are not precluded

Option 1 is baseline assumption.

Issue 1-4: Release independent

- Option 1: Rel-17 FR1 HST PDSCH CA requirements are release independent from Rel-15.

- Option 2: Do not define release independent from Rel-15 for HST PDSCH CA requirements

- Option 3:

  - HST-DPS CA requirements are release independent from Rel-15

  - HST-SFN CA requirements are applicable from Rel-17

- Option 4: Rel-17 FR1 HST PDSCH CA requirements can be release independent from Rel-15, if Rel-17 RRM requirements for HST are defined release independent from Rel-15.

Issue 1-5: Simulation

- Since huge performance gap for HST-SFN is still observed, companies are encouraged to check the summary on simulation results (R4-2113455) and come back in the next meeting if necessary

- RAN4 discuss how to derive the final PDSCH demodulation requirements for the test cases where the result span of the ideal results among companies’ is larger than [2.5] dB:

  - Option 1: RAN4 does not consider the farthest result(s) from the ideal AVERAGE value, until the span becomes [2.5] dB or less. The final requirements are derived from AVERAGE impairment results with the corresponding ideal results whose span is within [2.5] dB

  - Option 2: RAN4 does not consider the smallest result(s) from the ideal AVERAGE value, until the span becomes [2.5] dB or less. The final requirements are derived from AVERAGE impairment results with the corresponding ideal results whose span is within [2.5] dB

  - Option 3: The results farthest from the AVERAGE value is taken out for the AVERAGE and SPAN re-calculation until the ideal span is <= [2.5] dB but still with at least 3 companies’ results available

  - Other options are not precluded.

- Note: The threshold, [2.5] dB, can be discussed in RAN4#101-e.

- Reference

R4-2103876 WF on UE demodulation for FR1 HST, RAN4#98-e

R4-2106098 WF on UE demodulation for FR1 HST, RAN4#98-bis-e

R4-2108635 WF on UE demodulation for FR1 HST, RAN4#99-e