3GPP TSG-RAN WG4 #100-e	R4-2115729

Electronic Meeting, August 16-27, 2021

Agenda item:	9.12.2.2

Source:	MediaTek Inc.

Title:	TP to TR 38.833: Scenario for inter-user interference suppression for MU-MIMO

Document for:	Approval

- Introduction

In this contribution, we propose TP for the scenario section of MU-MIMO in TR 38.833.

- Reference

- R4-2108666, “Way Forward on MMSE-IRC for intra-cell inter-user interference”, Huawei, HiSilicon

- TP to TR 38.833

The following TP to TR 38.860 is proposed for approval.

<< Start of text proposal >>

## 4.1	Scenario and interference modelling

MU-MIMO allows gNB to transmit data to multiple UEs in the same time-frequency resources through spatial multiplexing. The process of selecting paired UEs is called ‘pairing’. As illustrated in Figure 1, UE1 and UE2 are paired and gNB transmits data to both UEs with suitable precoders through the same time-frequency resources. However, the gNB cannot guarantee the perfect pairing of multiple users in the real network. The paired UEs may not perfectly spatially orthogonal to each other and this will induce the intra-cell interference between paired UEs.

Figure 1. gNB tranmsmit data to paired UE1 and UE2 with the same time-frequency resources

To evaluate the performance of UE with intra-cell interference induced by spatial multiplexing, the following scenarios in Figure 2 are considered for the case of number of paired UEs is 2.

- Target UE with single DMRS antenna port:

  - Scenario 1-1: Number of CDM group without data is 1

    - AP1000 for target UE, AP1001 for interference UE

  - Scenario 1-2: Number of CDM group without data is 2

    - AP1000 for target UE, AP1002 for interference UE

- Target UE with two DMRS antenna ports:

  - Scenario 2-1: Number of CDM group without data is 2

    - AP1000 and 1001 for target UE, AP1002 and 1003 for interference UE

  - Scenario 2-2: Number of CDM group without data is 2

    - AP1000 and 1001 for target UE, AP1002 for interference UE

| Scenario 1-1 | Scenario 1-2 |
| --- | --- |
| Scenario 2-1 | Scenario 2-2 |
| Figure 2. Scenarios for the case of number of paired UEs is 2 |  |

<< End of text proposal >>