3GPP TSG-RAN WG4 Meeting # 100-e 	R4-2115803

Electronic Meeting, 16th – 27th Aug, 2021

Agenda item:			10.7

Source:	Moderator (Qualcomm Incorporated)

Title:	Email discussion summary for [100-e][330] NR_ATP

Document for:	Information

# Introduction

The discussions in this thread includes study on 5G NR UE Application Layer Data Throughput performance requirements.

# Topic #1: General

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2113124 | Intel | Proposal 1: The requirements of Absolute Physical Layer Throughput with Link Adaptation can be declared feasible if RAN4 can find several SNR points in different regions where companies’ simulation results are aligned. |
| R4-2113642 | Ericsson | Proposal 3: RAN4 shall try to align the simulation results before discussing the feasibility criteria. |
| R4-2113787 | Huawei, HiSilicon | Proposal 2: Choose Option 2, i.e. RAN4 can find several SNR points where companies’ simulation results align depending on Rank 1 and Rank 2 regime. |
| R4-2114477 | Qualcomm | Draft CR on General and Summary Sections in RAN4 study on Application Layer Throughput Requirements |

## Open issues summary

### Sub-topic 1-1: Feasibility

Issue 1-1-1: Feasibility Criteria

- The requirements of Absolute Physical Layer Throughput with Link Adaptation can be declared feasible if

  - Option 1: RAN4 can find at least one SNR point where companies’ simulation results align. [QC]

  - Option 2: RAN4 can find several SNR points where companies’ simulation results align depending on Rank 1 and Rank 2 regime. [Intel?, Huawei]

    - Note from Moderator: Based on simulation results, Rank2 may be limited to very few SNR points for 2Rx cases in simulation campaign.

- Recommended WF

  - TBA

Issue 1-1-2: Feasibility

- Proposals

  - Option1: Yes [Ericsson, Intel, QC, MediaTek]

  - Option 2: No

- Recommended WF

  - TBA

    - Based on simulation results submitted by companies, below are the preliminary observations:

      - Option 1 Alignment Criteria:

        - FDD 2x2: Thpt span is >10% for all SNRs

        - FDD 2x4: Thpt span is within 10% of Avg Thpt for 12, 16 and 18dB.

        - FR1 TDD 2x2: Thpt span is within 10% of Avg Thpt for 10dB.

        - FR1 TDD 2x4: Thpt span is within 10% of Avg Thpt for 12, 14, 16 and 18dB.

        - FR2 TDD 2x2: Thpt span is >10% for all SNRs.

      - Option 2 Alignment Criteria:

        - FDD 2x2: SNR span is within 2.5dB for 10% of peak thpt.

        - FDD 2x4: SNR span is within 2.5dB for 10%,20%,30%,40%,50% and 60% of peak thpt.

        - FR1 TDD 2x2: SNR span is within 2.5dB for 10%,20%, and 30% of peak thpt.

        - FR1 TDD 2x4: SNR span is within 2.5dB for 10%,20%,30%,40%,50% and 60% of peak thpt.

        - FR2 TDD 2x2: SNR span is within 2.5dB for 10%, 15%, 35%, 40% of peak thpt.

    - Request companies to provide simulation results as soon as possible. Chair guideline is to only include simulation results provided by Wed of 1st week of the meeting.

    - Based on above preliminary comparison, we can find several SNR points where results are aligned for Option 2. Therefore, we suggest to first confirm above observations and if similar observations are confirmed, declare the application layer throughput tests to be feasible with Option 2 alignment criteria.

### Sub-topic 1-2: LS to RAN5

Issue 1-2-1: LS Content

Discuss LS content in the 2nd round to inform RAN5 about endorsed RAN4 draft CRs for implementation in TR 37.901-5.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Issue 1-1-1: Feasibility Criteria Issue 1-1-2: Feasibility |
| Apple | Issue 1-1-1: Feasibility Criteria Perhaps doesn’t matter given that we have alignment for few SNR points based on simulation results based on option 2 alignment criteria. Issue 1-1-2: Feasibility FR1-FDD 2x2 and FR2 results are aligned (within SNR span) at only 1 or 2 points. Perhaps some further check is required to understand the delta based on other metrics. |
| Ericsson | Issue 1-1-1: Feasibility Criteria We have same views as Apple. Based on GTW’s discussion, we can just check SNR span, but it may further be checked based on update simulation results. Issue 1-1-2: Feasibility Option 1. |
| Intel | Issue 1-1-1: Feasibility Criteria Based on current results for 3 scenarios we have rather good SNR alignment almost for all considered throughput levels. For other 2 scenarios, we have alignment for one or two throughput levels. Therefore, we can consider Option 2 to draw the conclusion on feasibility. Issue 1-1-2: Feasibility Support Option 1 based on comment from Issue 1-1-1. We can further work on alignment of simulation results during WI stage in case we observe some misalignment for simulation assumptions agreed for requirements definition. |
| Qualcomm | Issue 1-1-1: Feasibility Criteria Given that simulation results are aligned for multiple SNR points for 3 scenarios and 1-2 SNR points for other two scenarios, it should not matter which option is chosen. Issue 1-1-2: Feasibility Support Option 1. |
| MediaTek | Issue 1-1-1: Feasibility Criteria Basically, we think the results are quite aligned for most scenarios. We can further check the updated simulation results if there are any. Issue 1-1-2: Feasibility OK with Option 1 |
| Huawei | Issue 1-1-1: Feasibility Criteria Based on the alignment criteria agreed in GTW, the simulation results seems not aligned for FDD 2T2R. Companies should double check the simulation results and updated results are needed. Issue 1-1-2: Feasibility In FR1 FDD 2x2, we have alignment only for level with 10% of max TP which seems be not enough to declare the feasibility of this test. We should further check the simulation results. |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2114477 (Qualcomm) | Moderator: Need to be revised based on outcome of Issue 1-1-2. |
|  | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 1-1-1: Feasibility Criteria | Tentative agreements: RAN4 can find several SNR points where companies’ simulation results align depending on Rank 1 and Rank 2 regime. |
| Issue 1-1-2: Feasibility | Candidate options: Option 1: Yes [Ericsson, Intel, QC, MediaTek] Option 2: No Simulation results are aligned for all scenarios except FDD 2x2 Rank2. In case of FDD 2x2 Rank2, there is span of 2.7dB for 35% Thpt. 4 companies think that it should be declared feasible. Recommendations for 2nd round: Continue the effort to align the simulation results for FDD 2x2 based on simulation assumptions in Huawei’s draft CR and conclude on feasibility. |
| Issue 1-2-1: LS Content | Discuss the contents in the 2nd round. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2114477 (Qualcomm) | To be revised. Update based on the outcome of Issue 1-1-2. |

## Discussion on 2nd round

Issue 1-1: Feasibility

- Proposals

  - Option1: Yes [Ericsson, Intel, QC, MediaTek]

  - Option 2: No

- Recommended WF

Option 1

R4-2115745 (Qualcomm): draft CR on General and Summary sections

Qualcomm: Based on the current status of simulation results, we can find aligned results for all scenarios except FDD 2x2 Rank2 regime. We request Huawei and Intel to double check their simulation results since they are on the either extremes.

However, we would like to point out that the span for 35% of peak throughput for FDD 2x2 is 2.7dB, which is very close to 2.5dB. So, we expect that companies should be able to align the results in the future meeting, if it doesn’t happen in this meeting. Otherwise, we can resolve the misalignment similar to other PDSCH tests by either dropping the outlier results from the average computation or increasing the margin. Therefore, we recommend to declare the feasibility in this meeting and remove the [] in the draft CR.

Huawei: Thanks for your summary, we are checking our results and trying to get feedback to you as early as possible.

Qualcomm: As no comments received so far, we have removed the []

Issue 1-2: LS Content

It will capture the list of endorsed RAN4 draft CRs and indicate to RAN5 that RAN4 work should be captured in Rel-17 version of TR since the RAN4 secondary responsibility was approved for Rel17. Rel-17 version of the TR needs to be requested by RAN5.

Further discuss the LS content in the 2nd round after draft version of R4-2114569 is shared by Qualcomm.

Qualcomm: I have uploaded the final version to drafts. The only change is that I fixed the date for RAN4#100-e meeting.

# Topic #2: Simulation Results Alignment

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2112110 | Apple | Simulation results and below proposals. Proposal #1: To align results, criteria used is: SNR G±Gspan can be reached for the T% of maximum throughput. Proposal #2: Define physical layer TP requirements based on methodology for PDSCH demodulation requirements. |
| R4-2113124 | Intel | Simulation results. |
| R4-2113130 | Intel | Draft CR to TR 37.901-5: Simulation results section |
| R4-2113642 | Ericsson | Proposal 1: Target BLER for simulations can be 2%-20% for FR1; 1%-20% for FR2 due to different CQI reporting strategy. Proposal 2: RAN4 shall further discuss which simulation results alignment criteria is more suitable based on companies’ simulation results. Proposal 4: The requirement definition for link adaptation (LA) physical layer throughput shall follow the same criteria in simulation results alignment. |
| R4-2113643 | Ericsson | Simulation results. |
| R4-2113644 | Ericsson | Draft CR on ATP performance simulation alignment criteria |
| R4-2113787 | Huawei, HiSilicon | Proposal 1: For simulation results alignment, use Option 2, i.e. 2%~30% for target BLER. Meanwhile, not introduce the BLER requirements if agreed to define performance requirements for link adaptation physical layer throughput. Proposal 3: Introduce one SNR point in rank 2 operation SNR range for test. |
| R4-2113788 | Huawei, HiSilicon | Simulation results. |
| R4-2113789 | Huawei, HiSilicon | Draft CR: Introduction the simulation assumptions for ATP to TR.37.901-5 |
| R4-2114042 | MediaTek | Simulation results. |
| R4-2114474 | Qualcomm | Simulation results and below observations/proposals: Observation 1: Throughput span in % is very large for lower SNR regime since absolute throughput itself is very low. Observation 2: If RAN4 decides to define requirements for both Rank1 and Rank2 regime, SNR at T% of maximum throughput (Option 2) may be better for alignment. Observation 3: If RAN4 decides to define requirements for both only Rank2 regime, as asked by RAN5, both options can be used for alignment. Proposal 1: Use Option 2 for alignment results criteria. |

## Open issues summary

### Sub-topic 2-1: Ways to Align Simulation Results

In this subtopic, we discuss possible ways to improve the alignment.

Issue 2-1-1: Target BLER

- Proposals

- Option 1: 2%-20% for FR1; 1%-20% for FR2 [Ericsson, MediaTek]

- Option 2: 2%-30% for FR1; 1%-30% for FR2 [Huawei]

- Other options not precluded

- Recommended WF

  - Focus on Issue 2-1-2 and aligning the results among companies. We can discuss this issue in the 2nd round, if needed.

Issue 2-1-2: Simulation results alignment criteria

- Proposals

  - Option 1: Absolute throughput span within X% of average throughput across companies at a given SNR.

    - Decide X based on simulation results. Possible values of X = [5]% or [10]%.

  - Option 2: SNR span (Gspan) can be reached for the T% of maximum throughput [Apple, QC]

    - Maximum throughput is derived with TBS corresponding to CQI index 15 with rank 2 for 2Rx/4Rx UE.

    - Gspan = Max (G) – Min (G), where G is the set of SNRs submitted by different companies to achieve T% of maximum throughput

    - Decide Gspan based on simulation results. Candidate option is Gspan = [2.5] dB.

- Recommended WF

  - Option 2 based on GTW agreement.

### Sub-topic 2-2: Requirements Definition

If it is found to be feasible to define absolute throughput requirements, following issues will be considered for defining the requirements.

Issue 2-2-1: How to set the requirements (if found feasible to define such requirements)

- Proposals

  - Set the physical layer throughput requirements by

    - Option 1: Multiplying the averaged throughput by Y (%), e.g., Y=95% or 90%.

    - Option 2: Using methodology from PDSCH demodulation requirements with fixed RMC (i.e. average of impairments results + X dB margin).

- Recommended WF

  - Option 2.

Issue 2-2-2: Number of SNR points for defining requirements (if found feasible to define such requirements)

- Proposals

  - Option 1: one in rank 1 and one in rank 2 operation SNR range.

  - Option 2: one in rank 2 operation SNR range. [Huawei]

- Recommended WF

  - Discuss this in WI phase as per agreement in the last meeting.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub-topic 2-1: Ways to Align Simulation Results Issue 2-1-1: Target BLER Issue 2-1-2: Simulation results alignment criteria Sub-topic 2-2: Requirements Definition Issue 2-2-1: How to set the requirements Issue 2-2-2: Number of SNR points for defining requirements |
| Apple | Sub-topic 2-1: Ways to Align Simulation Results Issue 2-1-1: Target BLER We are fine with recommended WF. Issue 2-1-2: Simulation results alignment criteria We support option 2 and fine with recommended WF. Sub-topic 2-2: Requirements Definition Issue 2-2-1: How to set the requirements We support the recommended WF. Issue 2-2-2: Number of SNR points for defining requirements We support the recommended WF. |
| Ericsson | Sub-topic 2-1: Ways to Align Simulation Results Issue 2-1-1: Target BLER Option 1. This issue can be discussed in WI. Issue 2-1-2: Simulation results alignment criteria We’re fine with both options, a little prefer option 1. It depends on simulation alignment. Sub-topic 2-2: Requirements Definition Issue 2-2-1: How to set the requirements We support the recommended WF. Issue 2-2-2: Number of SNR points for defining requirements We support the recommended WF. |
| Intel | Sub-topic 2-1: Ways to Align Simulation Results Issue 2-1-1: Target BLER We think that we can focus on alignment of throughput results. Whether we need to target the certain BLER level, we can discuss during WI stage. Issue 2-1-2: Simulation results alignment criteria Option 2 based on GTW discussion. We can consider the following modification of Option 2 - SNR span (Gspan) can be reached for the T% of maximum throughput - Gspan = Gmax(T%) – Gmin(T%) - Gspan = [2.5] dB Sub-topic 2-2: Requirements Definition Support recommended WFs for all issues. |
| Qualcomm | Sub-topic 2-1: Ways to Align Simulation Results Issue 2-1-1: Target BLER Ok to discuss this further at WI stage. Issue 2-1-2: Simulation results alignment criteria Option 2 based on GTW agreement. Sub-topic 2-2: Requirements Definition Issue 2-2-1: How to set the requirements Ok with recommended WF. Issue 2-2-2: Number of SNR points for defining requirements Ok with recommended WF. |
| MediaTek | Sub-topic 2-1: Ways to Align Simulation Results Issue 2-1-1: Target BLER We prefer Option 1 but we are OK to discuss during the WI stage. Issue 2-1-2: Simulation results alignment criteria OK with the recommended WF. Sub-topic 2-2: Requirements Definition Issue 2-2-1: How to set the requirements OK with the recommended WF. Issue 2-2-2: Number of SNR points for defining requirements OK with the recommended WF. |
| Huawei | Sub-topic 2-1: Ways to Align Simulation Results Issue 2-1-1: Target BLER OK with recommended WF Issue 2-1-2: Simulation results alignment criteria OK with recommended WF. Issue 2-2-1: How to set the requirements (if found feasible to define such requirements) OK with recommended WF. Issue 2-2-2: Number of SNR points for defining requirements (if found feasible to define such requirements) OK with recommended WF. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2113130 (Intel) | Moderator: Need to be revised based on latest simulation results. |
|  | Company A |
|  | Company B |
| R4-2113644 (Ericsson) | Moderator: Need to be revised based on outcome of Issue 2-1-2. |
|  | Apple: Section number: 5.10.2.1 |
|  | Company B |
| R4-2113789 (Huawei, HiSilicon) | Apple: Rename title: as Table 5.10.3-1: Simulation assumptions for Absolute Physical Layer Throughput alignment with link adaptation It should be 5 tests or with separate table for each config and 1 test |
|  | Intel: Please check our suggestions on TP improvement - It is better to have three columns for simulation assumptions table for different scenarios: FR1 FDD, FR1 TDD and FR2 - Add information about PDSCH scheduling assumptions - We can add reference to CSI RMC from 38.101-4 just for information - List of test metrics can be extended |
|  | Qualcomm: BW/SCS, Periodicity/offset, CSI-IM config, subband size etc missing for FR2. Antenna config for FR2 needs to be clarified. It will be better to have 3 columns as suggested by Intel. |
|  | Huawei: Thanks for comments, the updated TP has been uploaded for further review. Company is welcome to double check the simulation assumptions captured in the TP and used in your simulation to facilitate the results alignment, especially for FR1 FDD 2x2 case. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 2-1-1: Target BLER | Tentative agreements: Discuss this issue in WI phase, if needed. Candidate options: - Option 1: 2%-20% for FR1; 1%-20% for FR2 [Ericsson, MediaTek] - Option 2: 2%-30% for FR1; 1%-30% for FR2 [Huawei] - Other options not precluded Recommendations for 2nd round: Focus on aligning the simulation results. |
| Issue 2-1-2: Simulation results alignment criteria | Tentative agreements: - SNR span (Gspan) can be reached for the T% of maximum throughput - Maximum throughput is derived with TBS corresponding to CQI index 15 with rank 2 for 2Rx/4Rx UE. - Gspan = Max (G) – Min (G), where G is the set of SNRs submitted by different companies to achieve T% of maximum throughput - Decide Gspan based on simulation results. Candidate option is Gspan = [2.5] dB. Recommendations for 2nd round: Continue working on refining the language as part of Ericsson’s draft CR on simulation alignment criteria |
| Issue 2-2-1: How to set the requirements | Tentative agreements: Using methodology from PDSCH demodulation requirements with fixed RMC. Recommendations for 2nd round: Capture this agreement in Ericsson’s draft CR in new Section 5.10.2.2 |
| Issue 2-2-2: Number of SNR points for defining requirements | Tentative agreements: Discuss this issue in WI phase. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round

Further discuss the language of simulation alignment criteria and requirement methodology as part of Ericsson’s draft CR.

Companies are requested to share the updated draft CRs and further discuss its contents in the 2nd round.

R4-2115743 (Intel) draft CR on simulation results

Apple: We are wondering if we should include BLER, CQI and RI results in the draft CR/ TP. Companies were requested to submit BLER, CQI, RI statistics to help with understanding any performance deltas in TP performance. The final requirements should be only for TP, but not for CQI, RI. Hence we think only TP results and possibly BLER results should be captured in the TP. Would like to understand other companies’ views.

Intel: Thank you for comments and update.

We don’t have strong preference on simulation results which will be captured in TP, taking into account that excel file with detailed simulation results will be attached to TP. We can consider one of the following options:

- Option 1: Keep all results as it is and add note in the begging of section: “Results from Table 5.10.4.1-2 are used for feasibility analysis. Results from other tables in this section are just for information.”

- Option 2: Keep only Table 5.10.4.1-2 which is used for feasibility analysis.

In case you and other companies have strong concern to have tables with results for all metrics in TP, we can update TP using Option 2.

Ericsson: Sorry for later feedback. We slightly update our median RI values in FDD 2*2.

Huawei: With the updated simulation results for FDD 2T2R, we think that we can conclude that we are aligned in all cases. Also the simulation results captured in the corresponding TP needs to be updated to include the latest results.

Intel2: We would like to let you know that TP is updated based on updated results from Huawei and our Option 1

Apple2: Thanks for your reply in response to adding the CQI, RI, BLER results. Since we already include the spreadsheet with company results in the TP, our preference is to only have the table with TP results used for feasibility analysis.

Intel3: We’ve prepared the updated TP based on comment from Apple.

Ericsson2: We’re fine this version since all the related information have already captured in the excel. BTW, We have updated our simulation results for RI values in 2*2 case. Could you help to update the excel also?

R4-2115744 (Ericsson) draft CR on test methodology

Ericsson: As no further comments so far, we update the CR based on QC’s changes.

R4-2115746 (Huawei, HiSilicon) draft CR on simulation assumptions

Intel: Thank you for draft CR update. We’ve add several notes in the table. One note is related to PDSCH scheduling assumptions. Taking into account that CQI/RI PDSCH assumptions are under discussion in another e-mail thread (320) and they can be different in comparison to agreed assumptions for analysis for this SI, it is better to add clarification in TR.

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on … | YYY |  |
| LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2114477 | Draft CR on General and Summary Sections in RAN4 study on Application Layer Throughput Requirements | Qualcomm | Revised |  |
| R4-2113130 | Draft CR to TR 37.901-5: Simulation results section | Intel | Revised |  |
| R4-2113644 | Draft CR on ATP performance simulation alignment criteria | Ericsson | Revised |  |
| R4-2113789 | Draft CR: Introduction the simulation assumptions for ATP to TR.37.901-5 | Huawei, HiSilicon | Revised |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2114569 | LS on RAN4 updates to TR 37.901-5 | Qualcomm | Agreeable |  |
| R4-2115745 | Draft CR on General and Summary Sections in RAN4 study on Application Layer Throughput Requirements | Qualcomm | Agreeable |  |
| R4-2115743 | Draft CR to TR 37.901-5: Simulation results section | Intel | Agreeable |  |
| R4-2115744 | Draft CR on ATP performance simulation alignment criteria | Ericsson | Agreeable |  |
| R4-2115746 | Draft CR: Introduction the simulation assumptions for ATP to TR.37.901-5 | Huawei, HiSilicon | Agreeable |  |
| R4-2113123 | Summary of simulation results for NR UE Application Layer Data Throughput Performance | Intel | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Apple | Manasa Raghavan | Manasa.raghavan@apple.com |
| Intel | Dmitry Belov | dmitry.belov@intel.com |
| Qualcomm | Gaurav Nigam | gnigam@qti.qualcomm.com |
| MediaTek | Licheng Lin | Licheng.lin@mediatek.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)