3GPP TSG-RAN WG4 Meeting # 100-e		                       R4-2111947

Electronic Meeting, August. 16-27, 2021

Title: 	On notation for V2X band combinations

Source: 	CATT

Agenda item:	8.25.1

Document for:	Approval

- Introduction

A number of V2X band combinations have been introduced, including con-current operation of NR Uu/NR PC5, NR Uu/LTE PC5 and LTE Uu/NR PC5. Nevertheless, the notations for different band combinations probably vary among different inputs from companies such as V2X_39-n47 and V2X_39_n47 (hyphens or underscore). This contribution will discuss the basic principle of natation for V2X band combinations similar to CA, DC and LTE V2X con-current operation.

- Discussion

Regarding notation for V2X band combinations, there is no detailed discussion or common understanding on how to represent V2X band combinations. To make a clear picture of V2X band combinations, it is expected to discuss the basic principle of nation for different forms of V2X con-current operations based on CA, DC and LTE V2X con-current operation.

As per TR 38.817-01, the examples for notation of NR CA, DC and EN-DC are given in Table 1, 2 and 3.

Table 1: Example notation for DC and CA

| Representation | Corresponding functionality |
| --- | --- |
| CA_n77-n78 | NR CA of band n77 and band n78 |
| DC_1-2_n77 | NR-LTE DC with LTE CA of band 1 and band 2 and NR band n77. |
| DC_1-2_n77-n78 | NR-LTE DC with LTE CA of band 1 and band 2 and NR CA of band n77 and band n78. |

Table 2: Example notation for EN-DC bandwidth class

| Type |  | Notation | Note |
| --- | --- | --- | --- |
| EN-DC | Intraband contiguous | DC_(n)XAA | 1 CC LTE Band X + 1 CC NR Band nX with max BW ≤ max LTE BW + max NR BW |
|  |  | DC_(n)XB | 1 CC LTE Band X + 1 CC NR Band nX with max BW ≤ 20 MHz |
|  |  | DC_(n)XJK | LTE Band X with class J + NR Band nX with class K |
|  | Intraband non-contiguous | DC_XA_nXA | Intraband non-contiguous EN-DC with 2 sub-blocks |
|  |  | DC_XC_nXC | Intraband non-contiguous EN-DC with 2 contiguous LTE sub blocks and 2 contiguous NR sub-blocks |
|  |  | DC_XA-XA_nX(3A) | Intraband non-contiguous EN-DC with 2 non-contiguous LTE sub blocks and 3 non-contiguous NR sub-blocks |
|  | Interband | DC_XA_nYA | Interband EN-DC with 1CC LTE Band X + 1CC NR Band nY |
| NOTE: LTE CA and NR CA bandwidth classes are used in EN-DC respectively |  |  |  |

Table 3: Example notation for NR DC bandwidth class

| Type |  | Notation | Note |
| --- | --- | --- | --- |
| NR DC | Interband | DC_nXA-nYA | NR DC with 1CC NR Band nX + 1CC NR Band nY |

Based on above examples for CA and DC, some observations can be derived to further determine the notation of V2X band combinations. For CA, NR CA inter-band band combinations are represented as CA_nX-nY such as CA_n77-n78, in which only NR bands are involved and “-” between two bands is used.

As for DC, EN-DC inter-band band combinations are represented as DC_X_nY, in which both LTE band and NR band are involved and “_” between two bands is used. NR DC inter-band band combinations are represented as DC_nX-nY, in which only NR bands are involved and “-” between two bands is used. Also based on TS 38.101-3 clause 5.5B.4a, NE-DC inter-band band combinations are represented as DC_nX_Y, in which both NR band and LTE band are involved and “_” between two bands is used.

LTE V2X inter-band band combinations for con-current operation are represented as V2X_X-Y, in which only LTE bands are involved and “-” between two bands is used.

It can be observed that for CA and DC, “nX-nY” is used for notation of inter-band band combinations that involve only NR bands, while “X_nY” or “nX_Y” is used for notation of inter-band band combinations that involve both LTE band and NR band.

Observation 1: NR CA inter-band band combinations are represented as CA_nX-nY, in which only NR bands are involved and “-” between two bands is used.

Observation 2: EN-DC inter-band band combinations are represented as DC_X_nY, in which both LTE band and NR band are involved. NE-DC inter-band band combinations are represented as DC_nX_Y, in which both NR band and LTE band are involved and “_” between two bands is used. NR DC inter-band band combinations are represented as DC_nX-nY, in which only NR bands are involved and “-” between two bands is used.

Observation 3: For CA and DC, “nX-nY” is used for notation of inter-band band combinations that involve only NR bands, while “X_nY” or “nX_Y” is used for notation of inter-band band combinations that involve both LTE band and NR band.

Observation 4: LTE V2X inter-band band combinations for con-current operation are represented as V2X_X-Y, in which only LTE bands are involved and “-” between two bands is used.

In Rel-16 NR V2X, RAN4 already have a common understanding on notation for V2X inter-band band combinations that Uu band is placed in front of V2X band regardless of which type of V2X con-current operation, such as V2X_n71-n47, V2X_n71_47, and V2X_20_n38. But no consensus on detailed notation is achieved. When it comes to NR V2X con-current operation, two options can be considered.

Option 1: Reuse the principle of CA and DC, i.e. V2X_nX-nY, V2X_X_nY, V2X_nX_Y

Option 2: Use “_” for all cases, i.e. V2X_nX_nY, V2X_X_nY, V2X_nX_Y

Option 1 is to follow the principle of CA and DC. The benefit is that different forms of con-current operation can be distinguished with different notations. Option 2 is to use “_” for all cases mainly considering NR CA is possible to be introduced with V2X con-current operation. For V2X con-current operation plus NR CA case, Option 2 would be helpful to present the clear relationship between each band in a more concise manner, such as V2X_nX-nZ_nY (CA between band nX and nZ, V2X band nY con-current with CA).

To avoid potential confusion and misalignment, it is proposed to adopt either Option 1 or Option 2 for NR V2X inter-band band combinations.

Proposal 1: To adopt either Option 1 or Option 2 for NR V2X inter-band band combinations.

Option 1: Reuse the principle of CA and DC, i.e. V2X_nX-nY, V2X_X_nY, V2X_nX_Y

Option 2: Use “_” for all cases, i.e. V2X_nX_nY, V2X_X_nY, V2X_nX_Y

- Conclusion

This contribution discusses the notion of V2X inter-band band combinations based on CA and DC. The following observations and proposal are derived:

Observation 1: NR CA inter-band band combinations are represented as CA_nX-nY, in which only NR bands are involved and “-” between two bands is used.

Observation 2: EN-DC inter-band band combinations are represented as DC_X_nY, in which both LTE band and NR band are involved. NE-DC inter-band band combinations are represented as DC_nX_Y, in which both NR band and LTE band are involved and “_” between two bands is used. NR DC inter-band band combinations are represented as DC_nX-nY, in which only NR bands are involved and “-” between two bands is used.

Observation 3: For CA and DC, “nX-nY” is used for notation of inter-band band combinations that involve only NR bands, while “X_nY” or “nX_Y” is used for notation of inter-band band combinations that involve both LTE band and NR band.

Observation 4: LTE V2X inter-band band combinations for con-current operation are represented as V2X_X-Y, in which only LTE bands are involved and “-” between two bands is used.

Proposal 1: To adopt either Option 1 or Option 2 for NR V2X inter-band band combinations.

Option 1: Reuse the principle of CA and DC, i.e. V2X_nX-nY, V2X_X_nY, V2X_nX_Y

Option 2: Use “_” for all cases, i.e. V2X_nX_nY, V2X_X_nY, V2X_nX_Y

- Reference

- TR 38.817-01, General aspects for User Equipment (UE) Radio Frequency (RF) for NR V16.2.0

- TS 38.101-3, User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios V17.2.0