3GPP TSG-RAN WG4 Meeting # 100-e 	R4-2112681

# Electronic Meeting, 16th – 27th August 2021

Source:		Verizon, Samsung

Title:			TP for TR 37.827 for DC_2-48_n77

Agenda item:		8.38.2

Document for:		Approval

# Introduction

This contribution is a text proposal for TR 37.827 to include DC_2-48_n77 according to the request in [1].

Reference

[1] RP-211172, Revised WID: Power Class 2 for EN-DC with x LTE band + y NR TDD band

# Text Proposal

<Start of Text Proposal>

## 5.x	DC_2-48_n77

### 5.x.1	Transmitter Characteristics

#### 5.x.1.1	Maximum Output Power

Table 5.x.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.x.1.2	Configurations for EN-DC

Table 5.x.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-48A_n77A DC_2A-48C_n77A DC_2A-48D_n77A | DC_2A_n77A |

#### 5.x.1.3	Co-existence study

According to the co-existence studies performed, no IMD issue is identified for this configuration.

### 5.x.2	Receiver Characteristics

#### 5.x.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

There is no additional MSD requirement for this PC2 band combination.

<End of Text Proposal>