3GPP TSG-RAN WG4 Meeting #100-e	                               R4-2112737

Electronic Meeting, 16 Aug – 27 Aug, 2021

Source: 	Murata Manufacturing Co Ltd.

Title: 	REFSENS of n71 for 25MHz and 30MHz channel bandwidth

Agenda Item:	8.26.2.1 Addition of bandwidth and Tx/Rx requirements [NR_bands_R17_BWs -Core]

Document for:	Approval

Introduction

In the RAN#92 plenary meeting, Band n71 25MHz and 30MHz channel bandwidth were newly added in the revised WID [1]. In this contribution, we provide the measurement of the Tx noise and MSD estimation.

Discussion

Analysis of Tx noise

With Low-band (Operating frequency is lower than 1GHz) channel bandwidth expansion, IMD noise from the RIFC enters Rx band after through PA because the duplex offset of Tx-Rx is narrow. These IMDs will cause PA Tx noise to increase, thereby greatly reducing receiver sensitivity. Using UL bandwidth restriction method can avoid larger MSD.

Another WID [2] discussed the REFSENSE specification for band n71 35MHz channel bandwidth. Asymmetric operation at UL/DL bandwidth = 20MHz/35MHz was approved. Also, the existing duplex offset was maintained [3],[4]. The same rule should be applied to the 25MHz and30 MHz channel bandwidths of band n71.

Figure. 1 and 2 indicate the location of the IMD when the channel bandwidths are 25MHz and 30MHz respectively. When the UL bandwidth is limited to 20 MHz, the IM3 of Tx signal and Tx image does not fall into the Rx band. In other words, reduction of PA Tx noise can be expected.

Figure 1: PA output spectrum for UL/DL = 20/25MHz and 25/25MHz

Figure 2: PA output spectrum for UL/DL = 20/30MHz and 30/30MHz

Tx noise measurement and MSD Estimation

Power Amplifier Calibration and Test Waveforms

PA calibration conditions:

The power amplifier (PA) is calibrated to meet 1 dB MPR for Power Class 3 using a 100 RB QPSK DFT-s-OFDM SCS 15kHz with RBs allocated at the channel edge.

Test waveforms:

- QPSK DFT-s-OFDM SCS 15kHz with the UL RBs be located as close as possible to the DL operating band but confined within the transmission bandwidth configuration for the CBW;

- Image and Local Oscillator (LO) rejection is set to -28 dBc;

- Image and Local Oscillator (LO) rejection is set to -28 dBc;

- Counter-IM3 (C-IM3) set to -60 dBc;

- Counter-IM5 (C-IM5) set to -70 dBc;

Measurement Results and MSD Estimation

We measured PA Tx Noise in Rx band (RxBN) at center frequency to maintain the duplex offset. The measurement results are shown in Table 1.

Table 1: RxBN measurement results

| UL/DL BW | Tx freq. | Rx freq. | Tx RBs Allocation | RxBN |
| --- | --- | --- | --- | --- |
| (MHz) | (MHz) | (MHz) | (-) | (dBm/BW) |
| 20/25 | 680.5 | 634.5 | 20@0 | -37.4 |
| 25/25 | ↓ | ↓ | ↓ | -22.1 |
| 20/30 | ↓ | ↓ | ↓ | -35.0 |
| 30/30 | ↓ | ↓ | ↓ | -13.2 |

Comparing to UL/DL = 25/25MHz and 30/30MHz, the RxBN is reduced by 15.3dB and 21.8dB respectively when UL bandwidth is limited to 20MHz.

Observation 1:  With limiting the UL BW to 20MHz, PA Tx noise can be significantly reduced.

Proposal 1:

For Band n71 25MHz and 30MHz channel bandwidth operation, restrict the UL channel bandwidth to 20MHz, and keep existing duplex offset between center of RX and TX channel BWs.

MSD Estimation

The link-budget calculation assumptions and MSD estimation are shown in Table 2.

Table 2: MSD Estimation for n71

| Parameter | Unit | n71 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| UL/DL BW | MHz | 20/25 |  | 20/30 |  |
| Antenna isolation | dB | 10.0 |  | 10.0 |  |
| Front-end loss | dB | 4.0 |  | 4.0 |  |
| PA output power | dBm | 27.0 |  | 27.0 |  |
| PA RxBN | dBm | -37.4 |  | -35.0 |  |
| DPX_ISO_from Tx to Rx | dB | 50.0 |  | 50.0 |  |
| DPX ATT RX band | dB | 40.0 |  | 40.0 |  |
| Div_filter_Tx rejection | dB | 40.0 |  | 40.0 |  |
| Transceiver effective phase noise | dBc/Hz | -141.0 |  | -141.0 |  |
| Thermal noise at PRx/DRX ANT port | dBm/Hz | -164.0 |  | -164.0 |  |
| SNR requirement for QPSK | dB | -1.0 |  | -1.0 |  |
| TX noise power at PRX/DRx ANT port | dBm | -80.4 | -84.4 | -78.7 | -82.7 |
| TX IM2 due to wider CBW at PRx/DRx ANT port | dBm | -86.0 | -94.0 | -86.0 | -94.0 |
| Thermal noise at PRx/DRx ANT Port | dBm | -87.2 | -87.2 | -86.4 | -86.4 |
| Total noise level at PRx/DRx ANT Port | dBm | -78.7 | -82.3 | -77.4 | -80.9 |
| MSD | dB | 6.4 |  | 6.9 |  |

Proposal 2:

REFSENS and UL configurations for band n71 25MHz and 30MHz Channel bandwidth operation are proposed highlighted as Table 3 and Table 4.

Table 3: Two antenna port reference sensitivity QPSK PREFSENS

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| n71 | 15 | -97.2 | -94 | -91.6 | -86 | -83.6 | -82.3 | [-80.7] |  |  |  | FDD |
|  | 30 |  | -94.3 | -91.9 | -87.4 | -83.7 | -82.4 | [-80.8] |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |

Table 4: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| n71 | 15 | 25 | 25 | 20 | 20 | 20 | 20 | [20] |  |  |  | FDD |
|  | 30 |  | 12 | 10 | 10 | 10 | 10 | [10] |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |  |  |  |  |  |  |  |  |

Conclusion

In this contribution, we present PA Tx noise in Rx band of n71 25MHz and 30MHz Channel bandwidth and propose the following reference sensitivity levels and UL configurations.

Observation 1:  With limiting the UL BW to 20MHz, PA Tx noise can be significantly reduced.

Proposal 1:

For Band n71 25MHz and 30MHz channel bandwidth operation, restrict the UL channel bandwidth to 20MHz, and keep existing duplex offset between center of RX and TX channel BWs.

Proposal 2:

REFSENS and UL configurations for band n71 25MHz and 30MHz Channel bandwidth operation are proposed highlighted as Table 3 and Table 4.

Table 3: Two antenna port reference sensitivity QPSK PREFSENS

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| n71 | 15 | -97.2 | -94 | -91.6 | -86 | -83.6 | -82.3 | [-80.7] |  |  |  | FDD |
|  | 30 |  | -94.3 | -91.9 | -87.4 | -83.7 | -82.4 | [-80.8] |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |

Table 4: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| n71 | 15 | 25 | 25 | 20 | 20 | 20 | 20 | [20] |  |  |  | FDD |
|  | 30 |  | 12 | 10 | 10 | 10 | 10 | [10] |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |  |  |  |  |  |  |  |  |

References

[1]		RP-211599, Revised Basket WID on adding channel bandwidth support to existing NR bands, Ericsson

[2]		RP-201321, New WID proposal: introduction of channel bandwidths 35MHz and 45MHz for NR, Huawei, HiSilicon, Etisalat

[3]		R4-2105450, Email discussion summary for [98-bis-e][113] NR_FR1_35MHz_45MHz_BW, Huawei

[4]		R4-2105364, WF on UE REFSENS for 35 MHz and 45 MHz, Huawei, HiSilicon