3GPP TSG-RAN WG4 Meeting #100-e	R4-2113516

# Electronic Meeting, May. 16-27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DRAFT | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TS38.133 for dual active protocol stack handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-Core |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current specification it is indicated that the UE is not expected to transmit /not expected to receive if less than 26500 Tc is available for DL to UL / UL to DL switching on the same cell. It was discussed but not agreed that in RAN4#97 that this should be modifed to the time differeence between both source and targget cell. To enable this approach, it needs to be specifed what is expected from the UE. In addition, the value 26500 Tc is a typo and should have been 25600 Tc |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add conditions for not expected to transmit / not expected to receive covering both source and target cell. Add autonomous interruption allowance if these conditions are unspecified. Correct Ntx-rx and Nrx-tx to 25600 Tc |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE behaviour in case it does not have sufficient switching time between both source and target cell is not specied. This implies that network should try to avoid operating in such condition, but this is not feasible becase NW is not fully aware of receive time difference between source and target cell, TAE between transmission of source and target cell, and exact TA used by UE for each link. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------------------- Beginning of Change ------------------------------------------------------------

### 6.1.3	NR DAPS Handover

#### 6.1.3.1	Introduction

The requirements in this clause are applicable to DAPS handover to change the NR PCell to another NR cell.

Note:	requirements only apply if

-	the UE indicates ‘no-gap’ via intraFreq-needForGap for intra-frequency measurement of source cell and intra-frequency measurement of target cell, or

-	the SSB of source cell is completely contained in the active DL BWP of the source cell, and the SSB of target cell is completely contained in the active DL BWP of the target cell, or

-	the initial DL and UL BWP of source cell is confined within the active DL and UL BWP of the source cell respectively, and the initial DL and UL BWP of target cell is confined within the active DL and UL BWP of the target cell respectively.

#### 6.1.3.2	NR FR1 - NR FR1 DAPS Handover

The requirements in this clause are applicable to both intra-frequency and inter-frequency handovers from NR FR1 cell to NR FR1 cell. A DAPS handover is intra-frequency if the centre frequency of the SSB of the source cell and the centre frequency of the SSB of the target cell are the same, and	the subcarrier spacing of the two SSBs are also the same.

Note:	For intra-frequency DAPS handover, no requirement applies if active DL and UL BWP of target cell is not confined within the active DL and UL BWP of the source cell respectively.

Note:	For inter-frequency DAPS handover, no requirement applies if the BWP of target cell is overlaped with the BWP of source cell in frequency domain.

An FR1 DAPS handover is synchronous if it meets the conditions in table  6.1.3.2-1, otherwise it is asynchronous

Table  6.1.3.2-1: Sync conditions for FR1 DAPS handover

| Type of handover | Maximum receive timing difference between source and taget cell (µs) for sync DAPS handover | Maximum transmit timing difference between source and taget cell (µs) for sync DAPS handover |
| --- | --- | --- |
| Intra-frequencyNote 1,2,3 | 6µs | 7.6 µs |
| Intra-band inter-frequency Note 1,2,3 | 6µs | 7.6 µs |
| Inter-band inter-frequency | 33 µs | 34.6 µs |
| Note 1: For synchonous DAPS handover, if the receive time difference exceeds the cyclic prefix length of that SCS, demodulation performance degradation is expected for the first symbol of the slot. For asynchronous DAPS handover, if the receive time difference exceeds the cyclic prefix length of that SCS, interruptions may occur depending on UE implementation. The duration and frequency of occurrence of such interruptions is not specified. Note 2: For DAPS handover on a TDD band, a UE shall be capable to transmit in the uplink to source or target cell after NRX-TX from the end of the last received downlink symbol from source or target cell in the same TDD-band where NRX-TX=25600Tc. When this condition is not met, the UE may perfrom autonomous interruption as shown in Table 6.1.3.2-2. Note 3: For DAPS handover on a TDD band, a UE is shall be capable to receive in the downlink from source or target cell after NTX-RX after the end of the last transmitted uplink symbol towards sorce or target cell in the same TDD-band where NTX-RX=25600Tc. When this condition is not met, the UE mau perfrom autonomous interruption as shown in table 6.1.3.2-3. |  |  |

Table  6.1.3.2-2: Autonomous interruptions related to DL to UL switching for syncrounous TDD DAPS handover in the same band

| Scenario | Allowed interruption |
| --- | --- |
| Target cell earlier than source cellNote 1, prior to start of random access | Not applicable |
| Target cell later than source cell Note 1, prior to start of random access | The UE may stop receiving the target DL up to 25600 Tc prior to the start of the source UL |
| Target cell earlier than source cell Note 1, after start of random access | The UE may stop receiving the source DL up to 25600 Tc prior to the start of the target UL |
| Target cell later than source cell Note 1, after start of random access | The UE may start transmitting the source UL up to 25600 Tc after the start of the source DL |
| Note 1: As observed by UE at antenna connector |  |

Table  6.1.3.2-3: Autonomous interruptions related to UL to DL switching for syncrounous TDD DAPS handover in the same band

| Scenario | Allowed interruption |
| --- | --- |
| Target cell earlier than source cell Note 1, prior to start of random access | The UE may start receiving the target DL up to 25600 Tc after the end of the source UL |
| Target cell later than source cell Note 1 prior to start of random access | Not applicable |
| Target cell earlier than source cell Note 1, after start of random access | The UE may stop transmissions of the source UL up to 25600 Tc prior to the start of target DL reception. |
| Target cell later than source cell Note 1, after start of random access | The UE may start receiving the source DL up to 25600 Tc after the end of the target UL |
| Note 1: As observed by UE at antenna connector |  |

------------------------------------------------------------ End of Change ----------------------------------------------------------------