| 3GPP TR 37.827 V0.2.0 (2021-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Networks; Power Class 2 for EN-DC with xLTE band + yNR DL with 1LTE+1(TDD) NR UL band (x= 2, 3, 4, y=1; x=1, 2, y=2). (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	8

1	Scope	10

2	References	10

3	Definitions of terms, symbols and abbreviations	10

3.1	Terms	10

3.2	Symbols	10

3.3	Abbreviations	10

4	Background	10

4.1	TR maintenance	11

5	EN-DC Power Class 2: Specific Band Combination Part	11

5.1	DC_2A-5A_n77A	11

5.1.1	Transmitter Characteristics	11

5.1.1.1	Maximum Output Power	11

5.1.1.2	Co-existence study	11

5.1.2	Receiver Characteristics	11

5.1.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	11

5.1.2.1.1	Power class 2 Case A	11

5.1.2.1.2	Power class 2 Case B	12

5.2	DC_2A-13A_n77A	12

5.2.1	Transmitter Characteristics	12

5.2.1.1	Maximum Output Power	12

5.2.1.2	Co-existence study	12

5.2.2	Receiver Characteristics	12

5.2.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	12

5.2.2.1.1	Power class 2 Case A	12

5.2.2.1.2	Power class 2 Case B	13

5.3	DC_2A-66A_n77A	13

5.3.1	Transmitter Characteristics	13

5.3.1.1	Maximum Output Power	13

5.3.1.2	Co-existence study	13

5.3.2	Receiver Characteristics	13

5.3.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	13

5.3.2.1.1	Power class 2 Case A	13

5.3.2.1.2	Power class 2 Case B	14

5.4	DC_5A-66A_n77A	14

5.4.1	Transmitter Characteristics	14

5.4.1.1	Maximum Output Power	14

5.4.1.2	Co-existence study	14

5.4.2	Receiver Characteristics	14

5.4.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	14

5.4.2.1.1	Power class 2 Case A	14

5.4.2.1.2	Power class 2 Case B	15

5.5	DC_13A-66A_n77A	15

5.5.1	Transmitter Characteristics	15

5.5.1.1	Maximum Output Power	15

5.5.1.2	Co-existence study	15

5.5.2	Receiver Characteristics	15

5.5.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	15

5.5.2.1.1	Power class 2 Case A	15

5.5.2.1.2	Power class 2 Case B	16

5.6	DC_2A_n5A-n77A	16

5.6.1	Transmitter Characteristics	16

5.6.1.1	Maximum Output Power	16

5.6.1.2	Co-existence study	16

5.6.2	Receiver Characteristics	16

5.6.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	16

5.6.2.1.1	Power class 2 Case A	16

5.6.2.1.2	Power class 2 Case B	17

5.7	DC_66A_n2A-n77A	17

5.7.1	Transmitter Characteristics	17

5.7.1.1	Maximum Output Power	17

5.7.1.2	Co-existence study	17

5.7.2	Receiver Characteristics	17

5.7.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	17

5.7.2.1.1	Power class 2 Case A	17

5.7.2.1.2	Power class 2 Case B	18

5.8	DC_66A_n5A-n77A	18

5.8.1	Transmitter Characteristics	18

5.8.1.1	Maximum Output Power	18

5.8.1.2	Co-existence study	18

5.8.2	Receiver Characteristics	18

5.8.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	18

5.8.2.1.1	Power class 2 Case A	18

5.8.2.1.2	Power class 2 Case B	19

5.9	DC_2-13_n66-n77	19

5.9.1	Maximum Output Power	19

5.9.3	Co-existence study	19

5.10	DC_2-13-66_n77	20

5.10.1	Maximum Output Power	20

5.10.2	Configuration for EN-DC	20

5.10.3	Co-existence study	20

5.11	DC_13-66_n2-n77	20

5.11.1	Maximum Output Power	20

5.11.2	Co-existence study	20

5.12	DC_2-66_n5-n77	21

5.12.1	Maximum Output Power	21

5.12.3	Co-existence study	21

5.13	DC_2-5-66_n77	21

5.13.1	Maximum Output Power	21

5.13.3	Co-existence study	21

5.14	DC_13_n66-n77	22

5.14.1	Transmitter Characteristics	22

5.14.1.1	Maximum Output Power	22

5.14.1.3	Co-existence study	22

5.14.2	Receiver Characteristics	22

5.14.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	22

5.14.2.1.1	Power class 2 Case A	22

5.14.2.1.2	Power class 2 Case B	23

5.15	DC_13_n2-n77	23

5.15.1	Transmitter Characteristics	23

5.15.1.1	Maximum Output Power	23

5.15.1.3	Co-existence study	23

5.15.2	Receiver Characteristics	23

5.15.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	23

5.15.2.1.1	Power class 2 Case A	23

5.15.2.1.2	Power class 2 Case B	24

5.16	DC_2A-66A_n41A	24

5.16.1	Transmitter Characteristics	24

5.16.1.1	Maximum Output Power	24

5.16.1.2	Co-existence study	24

5.16.2	Receiver Characteristics	24

5.16.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	24

5.16.2.1.1	Power class 2 Case A	24

5.16.2.1.2	Power class 2 Case B	25

5.17	DC_66_n66-n77	25

5.17.1	Transmitter Characteristics	25

5.17.1.1	Maximum Output Power	25

5.17.1.2	Co-existence study	25

5.17.2	Receiver Characteristics	25

5.17.2.1	 MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	25

5.17.2.1.1	Power class 2 Case A	25

5.17.2.1.2	Power class 2 Case B	26

5.18	DC_48-66_n77	26

5.18.1	Transmitter Characteristics	26

5.18.1.1	Maximum Output Power	26

5.18.1.3	Co-existence study	26

5.18.2	Receiver Characteristics	26

5.18.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	26

5.19	DC_13_n5-n77	27

5.19.1	Transmitter Characteristics	27

5.19.1.1	Maximum Output Power	27

5.19.1.3	Co-existence study	27

5.19.2	Receiver Characteristics	27

5.19.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	27

5.19.2.1.1	Power class 2 Case A	27

5.19.2.1.2	Power class 2 Case B	27

5.20	DC_5_n66-n77	28

5.20.1	Transmitter Characteristics	28

5.20.1.1	Maximum Output Power	28

5.20.1.3	Co-existence study	28

5.20.2	Receiver Characteristics	28

5.20.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	28

5.20.2.1.1	Power class 2 Case A	28

5.20.2.1.2	Power class 2 Case B	29

5.21	DC_5A_n5A-n77A	29

5.21.1	Transmitter Characteristics	29

5.21.1.1	Maximum Output Power	29

5.21.1.3	Co-existence study	29

5.21.2	Receiver Characteristics	29

5.21.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	29

5.21.2.1.1	Power class 2 Case A	29

5.21.2.1.2	Power class 2 Case B	30

5.22	DC_5_n2-n77	30

5.22.1	Transmitter Characteristics	30

5.22.1.1	Maximum Output Power	30

5.22.1.3	Co-existence study	30

5.22.2	Receiver Characteristics	30

5.22.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	30

5.22.2.1.1	Power class 2 Case A	30

5.22.2.1.2	Power class 2 Case B	31

5.23	DC_2_n66-n77	31

5.23.1	Transmitter Characteristics	31

5.23.1.1	Maximum Output Power	31

5.23.1.3	Co-existence study	31

5.23.2	Receiver Characteristics	31

5.23.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	31

5.23.2.1.1	Power class 2 Case A	31

5.23.2.1.2	Power class 2 Case B	32

5.24	DC_2-48_n77	32

5.24.1	Transmitter Characteristics	32

5.24.1.1	Maximum Output Power	32

5.24.1.3	Co-existence study	32

5.24.2	Receiver Characteristics	32

5.24.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	32

5.25	DC_2A_n2A-n77A	33

5.25.1	Transmitter Characteristics	33

5.25.1.1	Maximum Output Power	33

5.25.1.3	Co-existence study	33

5.25.2	Receiver Characteristics	33

5.25.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	33

5.25.2.1.1	Power class 2 Case A	33

5.25.2.1.2	Power class 2 Case B	34

5.26	DC_1A-5A_n78A	34

5.26.1	Transmitter Characteristics	34

5.26.1.1	Maximum Output Power	34

5.26.1.2	Co-existence study	34

5.26.2	Receiver Characteristics	34

5.26.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	34

5.26.2.1.1	Power class 2 Case A	34

5.26.2.1.2	Power class 2 Case B	35

5.27	DC_1A-n7A_n78A	35

5.27.1	Transmitter Characteristics	35

5.27.1.1	Maximum Output Power	35

5.27.1.2	Co-existence study	35

5.27.2	Receiver Characteristics	36

5.27.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	36

5.27.2.1.1	Power class 2 Case A	36

5.27.2.1.2	Power class 2 Case B	36

Annex A - Change history	37

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is a technical report Power Class 2 for EN-DC with xLTE band + yNR DL with 1LTE+1(TDD) NR UL band (x= 2, 3, 4, y=1; x=1, 2, y=2) under Rel-17 time frame. The purpose is to gather the relevant background information and studies in order to address relevant requirements for the Rel-17 EN-DC band combinations with Power Class 2 requested by proponents and captured in the WID...

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-210816, “New WID on PC2 EN-DC with x LTE band + y NR band (x= 2, 3, 4, y=1; x=1, 2, y=2)”, RAN#91-e

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Background

The present document is a technical report for Power Class 2 band combinations under Rel-17 timeframe. The document covers each band combination specific issues (i.e. one sub-clause defined per band combination)

## 4.1	TR maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

# 5	EN-DC Power Class 2: Specific Band Combination Part

## 5.1	DC_2A-5A_n77A

### 5.1.1	Transmitter Characteristics

#### 5.1.1.1	Maximum Output Power

Table 5.1.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 266 | +2/-3 |
| DC_5A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.1.1.2	Co-existence study

According to the PC3 DC_2A-5A_n77A study in 37.717-21-11, the Rx impacts are identified as below,

- 3rd order IMD products generated by DC_2_5 uplink may fall into own Rx of band n77.

- 5th order IMD products generated by DC_2_n77 uplink may fall into own Rx of band 5.

Thus, additional MSD for IMD 3 and 5 should be considered to mitigate the impact of the interference for PC2 DC_2A-5A_n77A combination.

### 5.1.2	Receiver Characteristics

#### 5.1.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.1.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_2A-5A_n77A are defined in table 5.1.2.2.1-1.

Table 5.1.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-5A_n77A | 2 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |
|  | 5 | 842.5 | 5 | 25 | 887.5 | 16.6 | IMD5 |
|  | n77 | 3305 | 5 | 25 | 3305 | N/A | N/A |
|  | 2 | 1907 | 5 | 25 | 1987 | 24.8 | IMD3 |
|  | 5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |
|  | n77 | 3680 | 5 | 25 | 3680 | N/A | N/A |

#### 5.1.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_2A-5A_n77A are the same as the Case A defined in table 5.1.2.1.1-1.

## 5.2	DC_2A-13A_n77A

### 5.2.1	Transmitter Characteristics

#### 5.2.1.1	Maximum Output Power

Table 5.2.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 266 | +2/-3 |
| DC_13A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.2.1.2	Co-existence study

According to the PC3 DC_2A-13A_n77A study in 37.717-21-11, the Rx impacts are identified as below,

- No IMD issue for Rx of band 13 with UL DC_2_n77.

- 3rd order IMD products generated by DC_13_n77 uplink may fall into own Rx of band 2

Thus, additional MSD for IMD 3 should be considered to mitigate the impact of the interference for PC2 DC_2A-13A_n77A combination.

### 5.2.2	Receiver Characteristics

#### 5.2.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.2.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_2A-13A_n77A are defined in table 5.2.2.2.1-1.

Table 5.2.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-13A_n77A | 2 | 1864 | 5 | 25 | 1944 | 24.2 | IMD3 |
|  | 13 | 783 | 5 | 25 | 752 | N/A | N/A |
|  | n77 | 3510 | 5 | 25 | 3510 | N/A | N/A |

#### 5.2.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_2A-13A_n77A are the same as the Case A defined in table 5.2.2.1.1-1.

## 5.3	DC_2A-66A_n77A

### 5.3.1	Transmitter Characteristics

#### 5.3.1.1	Maximum Output Power

Table 5.3.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 266 | +2/-3 |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.3.1.2	Co-existence study

According to the PC3 DC_2A-66A_n77A study in 37.717-21-11, the Rx impacts are identified as below,

- 2nd, 4th and 5th order IMD products generated by DC_2_n77 uplink may fall into own Rx of band 66

- 2nd, 4th and 5th order IMD products generated by DC_66_n77 uplink may fall into own Rx of band 2

Thus, additional MSD for IMD 3 and 5 should be considered to mitigate the impact of the interference for PC2 DC_2A-66A_n77A combination.

### 5.3.2	Receiver Characteristics

#### 5.3.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.3.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_2A-66A_n77A are defined in table 5.3.2.2.1-1.

Table 5.3.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-66A_n77A | 2 | 1855 | 5 | 25 | 1935 | N/A | N/A |
|  | 66 | 1765 | 5 | 25 | 2185 | 34.7 | IMD2 |
|  | n77 | 4040 | 5 | 25 | 4040 | N/A | N/A |
|  | 2 | 1905 | 5 | 25 | 1985 | M/A | N/A |
|  | 66 | 1720 | 5 | 25 | 2120 | 21.1 | IMD44 |
|  | n77 | 3595 | 5 | 25 | 3595 | N/A | N/A |
|  | 2 | 1880 | 5 | 25 | 1960 | 37.6 | IMD2 |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n77 | 3700 | 5 | 25 | 3700 | N/A | N/A |
|  | 2 | 1860 | 5 | 25 | 1940 | 19.8 | IMD44 |
|  | 66 | 1775 | 5 | 25 | 2195 | N/A | N/A |
|  | n77 | 3385 | 5 | 25 | 3385 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

#### 5.3.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_2A-66A_n77A are the same as the Case A defined in table 5.3.2.1.1-1.

## 5.4	DC_5A-66A_n77A

### 5.4.1	Transmitter Characteristics

#### 5.4.1.1	Maximum Output Power

Table 5.4.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_5A_n77A | 266 | +2/-3 |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.4.1.2	Co-existence study

According to the PC3 DC_5A-66A_n77A study in 37.717-21-11, the Rx impacts are identified as below,

- The IMD3 of UL DC_5A_n77A may impact to band 66 Rx.

Thus, additional MSD for IMD 3 should be considered to mitigate the impact of the interference for PC2 DC_5A-66A_n77A combination.

### 5.4.2	Receiver Characteristics

#### 5.4.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.4.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_5A-66A_n77A are defined in table 5.4.2.2.1-1.

Table 5.4.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A-66A_n77A | 5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |
|  | 66 | 1742 | 5 | 25 | 2142 | 22.2 | IMD3 |
|  | n77 | 3795 | 10 | 50 | 3795 | N/A | N/A |

#### 5.4.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_5A-66A_n77A are the same as the Case A defined in table 5.4.2.1.1-1.

## 5.5	DC_13A-66A_n77A

### 5.5.1	Transmitter Characteristics

#### 5.5.1.1	Maximum Output Power

Table 5.5.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_13A_n77A | 266 | +2/-3 |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.5.1.2	Co-existence study

According to the PC3 DC_13A-66A_n77A study in 37.717-21-11, the Rx impacts are identified as below,

- 3rd order IMD products generated by DC_13_n77 uplink may fall into own Rx of band 66.

- 3rd order IMD products generated by DC_66_n77 uplink may fall into own Rx of band 13

Thus, additional MSD for IMD 3 should be considered to mitigate the impact of the interference for PC2 DC_13A-66A_n77A combination.

### 5.5.2	Receiver Characteristics

#### 5.5.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.5.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_13A-66A_n77A are defined in table 5.5.2.2.1-1.

Table 5.5.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_13A-66A_n77A | 13 | 777 | 5 | 25 | 746 | N/A | N/A |
|  | 66 | 1746 | 5 | 25 | 2146 | 25.3 | IMD3 |
|  | n77 | 3700 | 10 | 50 | 3700 | N/A | N/A |
|  | 13 | 781 | 5 | 25 | 750 | 23.4 | IMD3 |
|  | 66 | 1710 | 5 | 25 | 2110 | N/A | N/A |
|  | n77 | 4170 | 10 | 50 | 4170 | N/A | N/A |

#### 5.5.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_13A-66A_n77A are the same as the Case A defined in table 5.5.2.1.1-1.

## 5.6	DC_2A_n5A-n77A

### 5.6.1	Transmitter Characteristics

#### 5.6.1.1	Maximum Output Power

Table 5.6.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.6.1.2	Co-existence study

According to the PC3 DC_2A_n5A-n77A study in 37.717-11-21, the Rx impacts are identified as below,

- 3rd and 5th order IMD generated by dual uplink of 2_n5 may fall into own Rx of band n77.

- 5th order IMD generated by dual uplink of 2_n77 may fall into own Rx of band n5.

Thus, additional MSD for IMD 3 and 5 should be considered to mitigate the impact of the interference for PC2 DC_2A_n5A-n77A combination.

### 5.6.2	Receiver Characteristics

#### 5.6.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.6.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_2A_n5A-n77A are defined in table 5.6.2.2.1-1.

Table 5.6.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A_n5A-n77A | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n77 | 3540 | 10 | 50 | 3540 | 24.5 | IMD3 |
|  | 2 | 1907 | 5 | 25 | 1987 | N/A | N/A |
|  | n5 | 844 | 5 | 25 | 889 | 16.6 | IMD5 |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | N/A |

#### 5.6.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_2A_n5A-n77A are the same as the Case A defined in table 5.6.2.1.1-1.

## 5.7	DC_66A_n2A-n77A

### 5.7.1	Transmitter Characteristics

#### 5.7.1.1	Maximum Output Power

Table 5.7.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.7.1.2	Co-existence study

According to the PC3 DC_66A_n2A-n77A study in 37.717-11-21, the Rx impacts are identified as below,

- 2nd, 4th and 5th order IMD generated by dual uplink of 66_n77 may fall into own Rx of band n2.

- 2nd and 4th order IMD generated by dual uplink of 66_n2 may fall into own Rx of band n77.

Thus, additional MSD for IMD 2, 4 and 5 should be considered to mitigate the impact of the interference for PC2 DC_66A_n2A-n77A combination.

### 5.7.2	Receiver Characteristics

#### 5.7.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.7.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_66A_n2A-n77A are defined in table 5.7.2.2.1-1.

Table 5.7.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A_n2A-n77A | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | 34.9 | IMD2 |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n77 | 3340 | 10 | 50 | 3340 | 20.9 | IMD44 |
|  | n2 | 1880 | 5 | 25 | 1960 | 37.6 | IMD2 |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n77 | 3700 | 10 | 50 | 3700 | N/A | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | 21.1 | IMD44 |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
|  | n77 | 3350 | 10 | 50 | 3350 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

#### 5.7.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_66A_n2A-n77A are the same as the Case A defined in table 5.7.2.1.1-1.

## 5.8	DC_66A_n5A-n77A

### 5.8.1	Transmitter Characteristics

#### 5.8.1.1	Maximum Output Power

Table 5.8.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.8.1.2	Co-existence study

According to the PC3 DC_66A_n5A-n77A study in 37.717-11-21, the Rx impacts are identified as below,

- 3rd 4th and 5th order IMD generated by dual uplink of 66_n5 may fall into own Rx of band n77

Thus, additional MSD for IMD 3, 4 and 5 should be considered to mitigate the impact of the interference for PC2 DC_66A_n5A-n77A combination.

### 5.8.2	Receiver Characteristics

#### 5.8.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.8.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_66A_n5A-n77A are defined in table 5.8.2.2.1-1.

Table 5.8.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A_n5A-n77A DC_66A-66A_n5A-n77A | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n77 | 3420 | 10 | 50 | 3420 | 24.9 | IMD3 |
|  | 66 | 1714 | 5 | 25 | 2114 | N/A | N/A |
|  | n5 | 827 | 5 | 25 | 872 | N/A | N/A |
|  | n77 | 4195 | 10 | 50 | 4195 | 24.1 | IMD44 |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

#### 5.8.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_66A_n5A-n77A are the same as the Case A defined in table 5.8.2.1.1-1.

## 5.9	DC_2-13_n66-n77

### 5.9.1	Maximum Output Power

Table 5.9.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A DC_13A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.9.2		Configurations for EN-DC

Table 5.9.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-13A_n66A-n77A | DC_2A_n77A DC_13A_n77A |

#### 5.9.3	Co-existence study

MSD have been defined for lower order combinations [2, 3]. No further MSD is needed for both Case A and B.

## 5.10	DC_2-13-66_n77

#### 5.10.1	Maximum Output Power

Table 5.10.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A, DC_13A_n77A, DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.10.2	Configuration for EN-DC

Table 5.10.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-13A-66A_n77A | DC_2A_n77A, DC_13A_n77A, DC_66A_n77A |

#### 5.10.3	Co-existence study

MSD have been defined for lower order combinations [2]. No further MSD is needed for both Case A and B.

## 5.11	DC_13-66_n2-n77

### 5.11.1	Maximum Output Power

Table 5.11.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_13A_n77A, DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.11.2	Configurations for EN-DC

Table 5.11.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_13A-66A_n2A-n77A | DC_13A_n77A, DC_66A_n77A |

#### 5.11.2	Co-existence study

MSD have been defined for lower order combinations [2, 3]. No further MSD is needed for both Case A and B

## 5.12	DC_2-66_n5-n77

### 5.12.1	Maximum Output Power

Table 5.12.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_66A_n77A, DC_2A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.12.2	Configurations for EN-DC

Table 5.12.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-66A_n5A-n77A | DC_66A_n77A, DC_2A_n77A |

#### 5.12.3	Co-existence study

MSD have been defined for lower order combinations [2]. No further MSD is needed for both Case A and B

## 5.13	DC_2-5-66_n77

### 5.13.1	Maximum Output Power

Table 5.13.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A, DC_5A_n77A, DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.13.2	Configurations for EN-DC

Table 5.13.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-5A-66A_n77A | DC_2A_n77A, DC_5A_n77A, DC_66A_n77A |

#### 5.13.3	Co-existence study

MSD have been defined for lower order combinations [2]. No further MSD is needed for both Case A and B.

## 5.14	DC_13_n66-n77

### 5.14.1	Transmitter Characteristics

#### 5.14.1.1	Maximum Output Power

Table 5.14.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_13A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.14.1.2	Configurations for EN-DC

Table 5.14.1.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_13A_n66A-n77A | DC_13A_n77A |

#### 5.14.1.3	Co-existence study

According to the both PC3 DC_13A_n66A-n77A study in 37.717-11-21 and only DC_13A_n77A uplink configuration applied in this band combination, additional MSD due to IMD 3 to the band n66 from the configured uplink EN-DC should be considered to mitigate the impact of the interference to the PC2 DC_13A_n66A-n77A combination.

IMD generated by DC_13A_n77A that falls into the third band, n66

### 5.14.2	Receiver Characteristics

#### 5.14.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.14.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A of DC_13A_n66A-n77A are defined in table 5.14.2.1.1-1.

Table 5.14.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_13A_n66A-n77A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n66 | 1756 | 5 | 25 | 2156 | 26.1 | IMD3 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |

#### 5.14.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B of DC_13A_n66A-n77A are the same as the Case A defined in table 5.14.2.1.1-1.

## 5.15	DC_13_n2-n77

### 5.15.1	Transmitter Characteristics

#### 5.15.1.1	Maximum Output Power

Table 5.15.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_13A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.15.1.2	Configurations for EN-DC

Table 5.15.1.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_13A_n2A-n77A | DC_13A_n77A |

#### 5.15.1.3	Co-existence study

According to the both PC3 DC_13A_n2A-n77A study in 37.717-11-21 and only DC_13A_n77A uplink configuration applied in this band combination, additional MSD due to IMD 3 to the band n2 from the configured uplink EN-DC should be considered to mitigate the impact of the interference to the PC2 DC_13A_n66A-n77A combination.

### 5.15.2	Receiver Characteristics

#### 5.15.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.15.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A of DC_13A_n2A-n77A are defined in table 5.15.2.2.1-1.

Table 5.15.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_13A_n2A-n77A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | 25.0 | IMD3 |
|  | n77 | 3524 | 10 | 50 | 3524 | N/A | N/A |

#### 5.15.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B of DC_13A_n2A-n77A are the same as the Case A defined in table 5.15.2.1.1-1.

## 5.16	DC_2A-66A_n41A

### 5.16.1	Transmitter Characteristics

#### 5.16.1.1	Maximum Output Power

Table 5.16.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_66A_n41A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.16.1.2	Co-existence study

According to the PC3 DC_2A_66A-n41A study in 37.716-21-11, the Rx impacts are identified as below,

- Co-existence analysis for DC_2_n41 shows that there is no impact from DC_2_n41 UL to Band 66 DL.

- Co-existence analysis for DC_66_n41 shows that there is IMD4 impact from DC_66_n41 UL to Band 2 DL.

Thus, additional MSD for IMD 4 should be considered to mitigate the impact of the interference for PC2 DC_2A_66A-n41A combination.

### 5.16.2	Receiver Characteristics

#### 5.16.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.16.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_2A_66A-n41A are defined in table 5.16.2.2.1-1.

Table 5.16.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A-66A_n41A | 2 | 1860 | 5 | 25 | 1940 | 22.6 | IMD4 |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n41 | 2685 | 5 | 25 | 2685 | N/A | N/A |

#### 5.16.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_2A_66A-n41A are the same as the Case A defined in table 5.16.2.1.1-1.

## 5.17	DC_66_n66-n77

### 5.17.1	Transmitter Characteristics

#### 5.17.1.1	Maximum Output Power

Table 5.17.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.17.1.2	Configurations for EN-DC

Table 5.17.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_66A_n66A-n77A | DC_66A_n77A |

#### 5.17.1.2	Co-existence study

According to the PC2 coexistence studies performed in the lower order combinations, the Rx impacts are identified as below,

- For UL DC_66A_n77A configuration, IMD2 and IMD5 products fall into the band n66 Rx

Thus additional MSD exception should be considered to mitigate the impact of the interference.

### 5.17.2	Receiver Characteristics

#### 5.17.2.1		MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.17.2.1.1	Power class 2 Case A

Based on co-existence study, additional MSD are specified Table 5.17.2.1.1-1 for this dual connectivity configuration.

Table 5.17.2.1.1-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A_n66A-n77A | 66 | 1730 | 5 | 25 | 2130 | N/A | N/A |
|  | n66 | 1770 | 5 | 25 | 2170 | 37 | IMD2 |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | N/A |
|  | 66 | 1730 | 5 | 25 | 2130 | N/A | N/A |
|  | n66 | 1770 | 5 | 25 | 2170 | 20 | IMD5 |
|  | n77 | 3680 | 10 | 50 | 3680 | N/A | N/A |

#### 5.17.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B configuration are same as the Case A defined in table 5.17.2.1.1-1.

## 5.18	DC_48-66_n77

### 5.18.1	Transmitter Characteristics

#### 5.18.1.1	Maximum Output Power

Table 5.18.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.18.1.2	Configurations for EN-DC

Table 5.18.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_48A-66A_n77A DC_48C-66A_n77A | DC_66A_n77A |

#### 5.18.1.3	Co-existence study

According to coxeistence studies for the PC3 DC_48A-66A_n77A in TR 37.717-21-11, no harmonics and intermodulation products is present as the duplex mode for both band 48 and n77 is TDD and opertion in fully synchronizing in US.

### 5.18.2	Receiver Characteristics

#### 5.18.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

There is no additional MSD requirement for this PC2 band combination.

## 5.19	DC_13_n5-n77

### 5.19.1	Transmitter Characteristics

#### 5.19.1.1	Maximum Output Power

Table 5.19.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_13A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.19.1.2	Configurations for EN-DC

Table 5.19.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_13A_n5A-n77A | DC_13A_n77A |

#### 5.19.1.3	Co-existence study

According to the PC3 DC_13A_n77A study in TR 37.717-21-11, the Rx impacts are identified as below,

- For UL DC_13A_n77A, IMD5 of the 2UL falls into band n5 Rx

Thus additional MSD should be considered to mitigate the impact of the interference.

### 5.19.2	Receiver Characteristics

#### 5.19.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.19.2.1.1	Power class 2 Case A

Based on co-existence study, additional MSD are specified Table 5.19.2.1.1-1 for this dual connectivity configuration.

Table 5.19.2.1.1-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_13A_n5A-n77A11 | n5 | 840 | 5 | 25 | 885 | 19.5 | IMD5 |
|  | 13 | 782 | 5 | 20 | 751 | N/A | N/A |
|  | n77 | 4013 | 10 | 50 | 4013 | N/A | N/A |
| NOTE 11: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |

#### 5.19.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B configuratoin are same as the Case A defined in table 5.19.2.1.1-1.

## 5.20	DC_5_n66-n77

### 5.20.1	Transmitter Characteristics

#### 5.20.1.1	Maximum Output Power

Table 5.20.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_5A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.20.1.2	Configurations for EN-DC

Table 5.20.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A_n66A-n77A | DC_5A_n77A |

#### 5.20.1.3	Co-existence study

For UL DC_5A_n77A, IMD3 generated by dual uplink of 5_n77 may fall into own Rx of band n66.

Thus additional MSD exception should be considered to mitigate the impact of the interference.

### 5.20.2	Receiver Characteristics

#### 5.20.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.20.2.1.1	Power class 2 Case A

Based on co-existence study, additional MSD are specified Table 5.20.2.1.1-1 for this dual connectivity configuration.

Table 5.20.2.1.1-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A_n66A-n77A | 5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |
|  | n66 | 1742 | 5 | 25 | 2142 | 22.2 | IMD3 |
|  | n77 | 3795 | 10 | 50 | 3795 | N/A | N/A |

#### 5.20.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B configuration are same as the Case A defined in table 5.20.2.1.1-1.

## 5.21	DC_5A_n5A-n77A

### 5.21.1	Transmitter Characteristics

#### 5.21.1.1	Maximum Output Power

Table 5.21.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_5A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.21.1.2	Configurations for EN-DC

Table 5.21.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A_n5A-n77A | DC_5A_n77A |

#### 5.21.1.3	Co-existence study

According to the co-existence studies performed in the lower order combinations, the Rx impacts are identified as below,

- For UL DC_5A_n77A configuration, IMD4 and IMD5 products fall into the band n5 Rx

Thus additional MSD should be considered to mitigate the impact of the interference

### 5.21.2	Receiver Characteristics

#### 5.21.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.21.2.1.1	Power class 2 Case A

Based on co-existence study, additional MSD are specified Table 5.21.2.1.1-1 for this dual connectivity configuration.

Table 5.21.2.1.1-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR EN-DC Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD for PC2 (dB) | Duplex mode | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A_n5A-n77A11 | 5 | 844 | 5 | 25 | 889 | N/A | FDD | N/A |
|  | n5 | 844 | 5 | 25 | 889 | 20.3 | FDD | IMD44 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 11: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |  |

#### 5.21.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B configuration are same as the Case A defined in table 5.21.2.1.1-1.

## 5.22	DC_5_n2-n77

### 5.22.1	Transmitter Characteristics

#### 5.22.1.1	Maximum Output Power

Table 5.22.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_5A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.22.1.2	Configurations for EN-DC

Table 5.22.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A_n2A-n77A | DC_5A_n77A |

#### 5.22.1.3	Co-existence study

According to coexistence studies performed in the PC3 DC_5A_n2A-n77A configuration, the Rx impacts are identified as below

- For UL DC_5A_n77A, IMD3 of 2UL may fall into own Rx of band n2.

Thus additional MSD should be considered to mitigate the impact of the interference for DC_5A_n2A-n77A combination.

### 5.22.2	Receiver Characteristics

#### 5.22.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.22.2.1.1	Power class 2 Case A

Based on co-existence study, additional MSD are specified Table 5.22.2.1.1-1 for this dual connectivity configuration.

Table 5.22.2.1.1-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A_n2A-n77A11 | n2 | 1907 | 5 | 25 | 1987 | 25.5 | IMD3 |
|  | 5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |
|  | n77 | 3680 | 5 | 25 | 3680 | N/A | N/A |
| NOTE 11: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |

#### 5.22.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B configuratoin are same as the Case A defined in table 5.22.2.1.1-1.

## 5.23	DC_2_n66-n77

### 5.23.1	Transmitter Characteristics

#### 5.23.1.1	Maximum Output Power

Table 5.23.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.23.1.2	Configurations for EN-DC

Table 5.23.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A_n66A-n77A DC_2A-2A_n66A-n77A | DC_2A_n77A |

#### 5.23.1.3	Co-existence study

Based on co-existence specified in 38.101-3 for the configuration,

- For UL DC_2A_n77A configuration, IMD2, IMD4 and IMD5 fall into own band n66 Rx.

Thus additional MSD should be considered to mitigate the impact of the interference.

### 5.23.2	Receiver Characteristics

#### 5.23.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.23.2.1.1	Power class 2 Case A

Based on co-existence study, additional MSD are specified Table 5.23.2.1.1-1 for this dual connectivity configuration.

Table 5.23.2.1.1-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A_n66A-n77A DC_2A-2A_n66A-n77A | 2 | 1855 | 5 | 25 | 1935 | N/A | N/A |
|  | n66 | 1715 | 5 | 25 | 2115 | 35.2 | IMD2 |
|  | n77 | 3970 | 10 | 50 | 3970 | N/A | N/A |
|  | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 22.3 | IMD44 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | N/A |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |

#### 5.23.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B configuratoin are same as the Case A defined in table 5.23.2.1.1-1.

## 5.24	DC_2-48_n77

### 5.24.1	Transmitter Characteristics

#### 5.24.1.1	Maximum Output Power

Table 5.24.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.24.1.2	Configurations for EN-DC

Table 5.24.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-48A_n77A DC_2A-48C_n77A DC_2A-48D_n77A | DC_2A_n77A |

#### 5.24.1.3	Co-existence study

According to the co-existence studies performed, no IMD issue is identified for this configuration.

### 5.24.2	Receiver Characteristics

#### 5.24.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

There is no additional MSD requirement for this PC2 band combination.

## 5.25	DC_2A_n2A-n77A

### 5.25.1	Transmitter Characteristics

#### 5.25.1.1	Maximum Output Power

Table 5.25.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.25.1.2	Configurations for EN-DC

Table 5.25.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A_n2A-n77A | DC_2A_n77A |

#### 5.25.1.3	Co-existence study

According to the PC2 coexistence studies performed in the lower order combinations, the Rx impacts are identified as below,

- For UL DC_2A_n77A, IMD2, IMD4 and IMD5 products fall into the band n2 Rx

Thus additional MSD should be considered to mitigate the impact of the interference.

### 5.25.2	Receiver Characteristics

#### 5.25.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.25.2.1.1	Power class 2 Case A

Based on co-existence study, additional MSD are specified Table 5.25.2.1.1-1 for this dual connectivity configuration.

Table 5.25.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD for PC2 | Duplex mode | Source of IMD |
| Configuration |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| DC_2A_n2A-n77A | 2 | 1855 | 5 | 25 | 1935 | N/A | FDD | N/A |
|  | n2 | 1855 | 5 | 25 | 1935 | 32.0 | FDD | IMD2 |
|  |  |  |  |  |  | 34.75 |  |  |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | 2 | 1885 | 5 | 25 | 1965 | N/A | FDD | N/A |
|  | n2 | 1865 | 5 | 25 | 1945 | 20.0 | FDD | IMD44 |
|  |  |  |  |  |  | 22.75 |  |  |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

#### 5.25.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B configuration are same as the Case A defined in table 5.25.2.1.1-1.

## 5.26	DC_1A-5A_n78A

### 5.26.1	Transmitter Characteristics

#### 5.26.1.1	Maximum Output Power

Table 5.26.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_aA_n78A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.26.1.2	Co-existence study

According to the PC3 DC_1A_5A-n78A study in 37.863-02-01, the Rx impacts are identified as below,

- Co-existence analysis for DC_1_n78 shows that there is IMD4 impact from DC_1_n78 UL to Band 1 and IMD5 impact from DC_1_n78 UL to Band 5 DL.

- Co-existence analysis for DC_5_n78 shows that there is IMD4 impact from DC_5_n78 UL to Band 5 DL and IMD3 impact from DC_5_n78 UL to Band 1 DL.

Thus, additional MSD for IMD 4 should be considered to mitigate the impact of the interference for PC2 DC_1A_5A-n78A combination.

### 5.26.2	Receiver Characteristics

#### 5.26.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.26.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_1A_5A-n78A are defined in table 5.26.2.2.1-1.

Table 5.26.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LLRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-5A_n78A | 1 | 1930 | 5 | 25 | 2120 | 19.2 | IMD4 |
|  | 5 | 844 | 5 | 25 | 889 | N/A | N/A |
|  | n78 | 3670 | 10 | 52 | 3670 | N/A | N/A |
| DC_1A-5A_n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 5 | 844 | 5 | 25 | 889 | 19.2 | IMD4 |
|  | n78 | 3421 | 10 | 52 | 3421 | N/A | N/A |
| DC_1A-5A_n78A | 1 | 1932 | 5 | 25 | 2122 | 27.0 | IMD3 |
|  | 5 | 829 | 5 | 25 | 874 | N/A | N/A |
|  | n78 | 3780 | 10 | 52 | 3780 | N/A | N/A |
| DC_1A-5A_n78A | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | 5 | 840 | 5 | 25 | 885 | 13.2 | IMD5 |
|  | n78 | 3405 | 10 | 52 | 3405 | N/A | N/A |

#### 5.26.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_1A_5A-n78A are the same as the Case A defined in table 5.26.2.1.1-1.

## 5.27	DC_1A-n7A_n78A

### 5.27.1	Transmitter Characteristics

#### 5.27.1.1	Maximum Output Power

Table 5.27.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_aA_n78A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.27.1.2	Co-existence study

According to the PC3 DC_1A_7A-n78A study in 37.863-02-01, the Rx impacts are identified as below,

- Co-existence analysis for DC_1_n78 shows that there is IMD4 impact from DC_1_n78 UL to Band 1 and Band 7 DL.

- Co-existence analysis for DC_7_n78 shows that there is IMD4 impact from DC_7_n78 UL to Band 1 DL.

Thus, additional MSD for IMD 4 should be considered to mitigate the impact of the interference for PC2 DC_1A_7A-n78A combination.

### 5.27.2	Receiver Characteristics

#### 5.27.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.27.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_1A_7A-n78A are defined in table 5.27.2.2.1-1.

Table 5.27.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LLRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-7A_n78A | 1 | 1930 | 5 | 25 | 2120 | 19.2 | IMD4 |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | n78 | 3670 | 10 | 52 | 3670 | N/A | N/A |
| DC_1A-7A_n78A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | 7 | 2507.5 | 5 | 25 | 2627.5 | 20.2 | IMD4 |
|  | n78 | 3305 | 10 | 52 | 3305 | N/A | N/A |
| DC_1A-7A_n78A | 1 | 1950 | 5 | 25 | 2140 | 19.7 | IMD4 |
|  | 7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
|  | n78 | 3580 | 10 | 52 | 3580 | N/A | N/A |

#### 5.27.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_1A_7A-n78A are the same as the Case A defined in table 5.27.2.1.1-1.

# Annex A - Change history

| Date | Meeting | TDoc | Subject/Comment | New version |
| --- | --- | --- | --- | --- |
| 2021-04 | 3GPP RAN4#98-bis-e | R4-2105489 | Initial version Implemented TP’s from RAN4 #98-bis-e: R4-2105351, “TP for TR 37.xxx for DC_2-5_n77”, Verizon Denmark R4-2105352, “TP for TR 37.xxx for DC_2-13_n77”, Verizon Denmark R4-2105353, “TP for TR 37.xxx for DC_2-66_n77”, Verizon Denmark R4-2105354, “TP for TR 37.xxx for DC_5-66_n77”, Verizon Denmark R4-2105355, “TP for TR 37.xxx for DC_13-66_n77”, Verizon Denmark R4-2105356, “TP for TR 37.xxx for DC_2_n5-n77”, Verizon Denmark R4-2105357, “TP for TR 37.xxx for DC_66_n2-n77”, Verizon Denmark R4-2105358, “TP for TR 37.xxx for DC_66_n5-n77”, Verizon Denmark | 0.0.1 |
| 2021-08 | 3GPP RAN4#100-e | R4-2113565 | Implemented TP’s from RAN4 #99-e: R4-2108901, TP for TR 37.827 for DC_2-13_n66-n77, Verizon Denmark R4-2108902, TP for TR 37.827 for DC_2-13-66_n77, Verizon Denmark R4-2108903, TP for TR 37.827 for DC_13-66_n2-n77, Verizon Denmark R4-2108904, TP for TR 37.827 for DC_2-66_n5-n77, Verizon Denmark R4-2108905, TP for TR 37.827 for DC_2-5-66_n77, Verizon Denmark R4-2107837, TP for TR 37.827 for DC_13_n66-n77, Verizon Denmark R4-2107838, TP for TR 37.827 for DC_13_n2-n77, Verizon Denmark R4-2110957, TP for PC2 DC_2_66-n41, Huawei Tech.(UK) Co.. Ltd | 0.1.0 |
| 2021-08 | 3GPP RAN4#100-e | R4-2113566 | Implemented TP’s from RAN4 #100-e: R4-2112669, TP for TR 37.827 for DC_66_n66-n77, Verizon, Samsung R4-2112670, TP to TR 37.717-21-11 for DC_48-66_n77, Verizon, Samsung R4-2112672 TP for TR 37.827 for DC_13_n5-n77, Verizon, Samsung R4-2112673, TP for TR 37.827 for DC_5_n66-n77, Verizon, Samsung R4-2114940 TP for TR 37.827 for DC_5_n5-n77, Verizon, Samsung R4-2114941 TP for TR 37.827 for DC_5_n2-n77, Verizon, Samsung R4-2112679, TP for TR 37.827 for DC_2_n66-n77, Verizon, Samsung R4-2112681, TP for TR 37.827 for DC_2-48_n77, Verizon, Samsung R4-2114942, TP for TR 37.827 for DC_2_n2-n77, Verizon, Samsung R4-2114043, Text proposal for TR 37.827 to include DC_1A-5A_n78A, Huawei Tech.(UK) Co.. Ltd R4-2114054, Text proposal for TR 37.827 to include DC_1A-n7A_n78A, Huawei Tech.(UK) Co.. Ltd | 0.2.0 |