3GPP TSG-RAN WG4 Meeting # 100-e	R4-2113782

# Electronic Meeting, August 16th – 27th, 2021

Title: 	TP: Simulation assumptions for intra-cell MMSE-IRC receiver

Source: 	Huawei, HiSilicon

Agenda item:	9.12.2.2

Document for:	Discussion

- Background

In RAN4 #99-e meeting, WF R4-2108666 is approved and corresponding initial simulation assumptions are determined. In this TP, we provide the simulation assumptions for intra-cell MMSE-IRC receiver.

Inter-user interference suppression for MU-MIMO

  - Link performance characterization

- 4.3.1 Parameters for link level evaluation

Simulation assumptions for target UE and interference UE are captured in Table 2-1:

Table 2-1: Common assumptions for target UE and interference UE

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Channel Bandwidth |  | MHz | 10MHz for FDD and 40MHz for TDD |
| SCS |  | kHz | 15kHz for FDD and 30kHz for TDD |
| Duplex mode |  |  | FDD,TDD |
| Interference model |  |  | 1 target UE + 1interference UE |
| MCS for target UE |  |  | 4 and 13 if rank of target UE is 1, 13 and 19 if rank of target UE is 2 |
| Allocation for interference UE and target UE | Rank allocation |  | Rank 1(Target UE)+1(Interference UE) Rank 2(Target UE)+1(Interference UE) Rank 2(Target UE)+2(Interference UE) |
|  | DMRS port allocation |  | For Rank 1+1 - Port 1000 for target UE and Port 1001 for interference UE - Port 1000 for target UE and Port 1002 for interference UE For Rank 2+1 - Port 1000 and 1001 for target UE and Port 1002 for interference UE For Rank 2+2 - Port 1000 and 1001 for target UE and Port 1002 and 1003 for interference UE |
|  | Scrambling ID |  | Same scrambling ID for both UEs |
| Antenna configuration |  |  | 2T2R for case with rank1+1 and 4T4R for case with rank 2+1 and rank 2+2 |
| HARQ process number for target UE |  |  | 4 |
| Precoding model | Target UE |  | Random precoding with Single panel Type 1 per PRB bundling size per slot |
|  | Interference UE |  | - Select the precoding matrix to ensure orthogonality with target UE - Select the precoding matrix randomly ensuring the selected precoding matrix shall not be identical to the precoding matrix of target UE |
| PDSCH configuration for target UE and interference UE | Mapping type |  | Type A |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling size |  | 2 |
|  | PRB bundling type |  | Static |
| PDSCH DMRS configuration for target UE and interference UE | DMRS Type |  | DMRS Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| SSB position |  |  | First SSB in slot #0 in every 20 ms periodicity that is not scheduled for PDSCH transmission |
| Propagation conditions and MIMO configuration |  |  | TDLA30-10 ULA Low, TDLA30-10 XPL Low, TDLC300-100 ULA Low, TDLC300-100 XPL Low. |
| Receiver type |  |  | MMSE-IRC and MMSE-MRC |
| Test metric |  |  | SNR @ %70 of maximum Throughput of target UE |

- Reference

- R4-2108666, Way Forward on MMSE-IRC for intra-cell inter-user interference, Huawei, HiSilicon.