3GPP TSG-RAN WG4 #100-e	R4-2114040

Electronic Meeting, August 16-27, 2021

Agenda item:	9.12.2.2

Source:	MediaTek Inc.

Title:	Discussion on the MMSE-IRC receiver requirements for intra-cell inter-user interference

Document for:	Discussion

- Introduction

In RAN4#99-e, a way forward for MMSE-IRC receiver for intra-cell inter-user interference was agreed [1]. In this contribution, we provide some simulation results based on the agreed simulation assumption and provide our views.

- Simulation Results

The simulation results are based on the table 1 in Appendix A. Figure 1 shows the results for rank1, with different setting for CDM group and scrambling ID. For both MCS4 and MCS13, we have the following observations. For the case of same CDM group, the difference between same scrambling ID and different scrambling ID is quite small. For the case of different scrambling ID, there is no difference between same scrambling ID and different scrambling ID. Besides, as the number of available REs are different for the case of same CDM group and different CDM group, the maximum achievable throughout is different. With fixed MCS, the TBS for the case of different CDM group is lower than that of same CDM group, and then achieve lower maximum throughput. Although the maximum achievable throughput is different, we think this is not the criterion to choose between them to define the requirement.

| Figure 1. Simulation results for rank1 for both MCS4 and MCS13 for 4x2, ULA-low and TDLA30-10 |  |
| --- | --- |

Figure 2 shows the results for rank2, with different setting for scrambling ID. For both MCS13 and MCS19, there is no difference between same scrambling ID and different scrambling ID for the case of different CDM group.

| Figure 2. Simulation results for rank2 for both MCS13 and MCS19 for 4x4, ULA-low and TDLA30-10 |  |
| --- | --- |

According the simulation results, we have the following observations.

Observation 1: For the case of same CDM group, the difference between same scrambling ID and different scrambling ID is quite small.

Observation 2: For the case of different CDM group, no difference between same scrambling ID and different scrambling ID.

Observation 3: The maximum achievable throughout for same CDM group and different CDM group is different.

Also, from our understanding, whether the co-scheduled UEs are in the same CDM group or with same/different scrambling ID are up to the configuration of gNB. If it is difficult to determine the configuration of CDM group and scrambling ID for test cases, we can have merged configurations in test cases. For example, we can use 50% of slots with same CDM group and 50% of slots with different CDM group.

Proposal: RAN4 can consider to merger different configurations to define the test case.

- Conclusion

The observations and proposal are summarized as below:

Observation 1: For the case of same CDM group, the difference between same scrambling ID and different scrambling ID is quite small.

Observation 2: For the case of different CDM group, no difference between same scrambling ID and different scrambling ID.

Observation 3: The maximum achievable throughout for same CDM group and different CDM group is different.

Proposal: RAN4 can consider to merger different configurations to define the test case.

- Reference

- R4-2108666, “Way Forward on MMSE-IRC for intra-cell inter-user interference”, Huawei, HiSilicon

Appendix A

Table 1: Simulation parameters for MU-MIMO evaluation

| Parameter |  | Value |  |
| --- | --- | --- | --- |
| CBW/SCS |  | 10 MHz/15 kHz for FDD |  |
| Antenna configuration |  | 4x2 and 4x4 ULA Low |  |
| Channel model |  | TDLA30-10 |  |
| Codebook |  | Type I SP codebook |  |
| Precoder selection for target UE |  | Random |  |
| Precoder selection for interference co-scheduled UE |  | Select the precoder to ensure orthogonality with percoder for target UE |  |
| Precoding granularity |  | 2 PRBs |  |
| PRB bundling size |  | 2 PRBs |  |
| Scrambling ID for target UE and co-scheduled UE |  | Option1: same Option2: different |  |
| Rank and DMRS ports |  | Option 1A [1+1] | DMRS port 0 for target UE, DMRS port 1 for the interference UE, i.e., same CDM group |
|  |  | Option 1B [1+1] | DMRS port 0 for target UE, DMRS port 2 for the interference UE, i.e., different CDM groups |
|  |  | Option 2A [2+2] | DMRS port 0 (and 1) for target UE, port 2 (and 3) for the interference UE, i.e., use different CDM groups for the target and interference UEs |
| Target NR PDSCH parameters | Mapping type | Type A |  |
|  | Number of front-loaded DMRS | 1 |  |
|  | Number of additional DMRS | 1 |  |
|  | Rank and MCS | Rank 1: QPSK (MCS4), 16QAM (MCS 13) Rank 2: 16QAM (MCS 13), 64QAM (MCS 19) |  |
|  | HARQ process number for target NR PDSCH | 4 for FDD |  |