# 3GPP TSG-RAN WG4 Meeting # 100-e	R4-2114094

# Electronic Meeting, August 16-27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | Draft | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on clarification on SMTC determination in DC 36133 R17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-08-01 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In RAN4#99e meeting, it was identified that when STMC is not provided within the corresponding command, UE is required to follow the SMTC in the MO with same SSB frequency and SCS, but MN and SN could configure different SMTC in different MOs. Then which SMTC configuration that UE should follow is not clear. It is agreed that it is up to UE implementation which SMTC to use under such case. Then necessary clarification shall be made. - Agreements: - When SMTC configuration is not provided within the corresponding command (e.g. Handover, RRC release with redirection, SCell activation and PSCell addition/change), and MN and SN configure measObjectNR having same SSB frequency and subcarrier spacing but with different SMTC configurations, - It is up to UE implementation which SMTC configuration to use - UE requirements will be based on the SMTC configuration used by the UE |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that If such measObjectNRs configured by MN and SN have different SMTC, Trs is the periodicity of one of the SMTC which is up to UE implementation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements are ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1,6.2.3, 8.3.2 and 8.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <End of Change 1>

#### 6.3.2.4	RRC connection release with redirection to NR

The UE shall be capable of performing the RRC connection release with redirection to the target NR cell within Tconnection_release_redirect_NR.

The time delay (Tconnection_release_redirect_NR) is the time between the end of the last TTI containing the RRC command, “RRCConnectionRelease” (TS 36.331 [2]) on the E-UTRAN PDSCH and the time the UE starts to send random access to the target NR cell. The time delay (Tconnection_release_redirect_NR) shall be less than:

Tconnection_release_redirect_NR = TRRC_procedure_delay + Tidentify-NR + TSI-NR + TRACH

The target NR cell shall be considered detectable when for each relevant SSB:

-	SSB_RP and SSB Ês/Iot according to Annex B.2.5 of TS 38.133 [50] for a corresponding NR Band.

TRRC_procedure_delay: It is the RRC procedure delay for processing the received message “RRCConnectionRelease” as defined in clause 6.2.2 of TS 36.331 [2]. It shall be less than 110 ms.

Tidentify-NR: It is the time to identify the target NR cell and depends on the frequency range (FR) of the target NR cell. It is defined in table 6.3.2.4-1. Tidentify-NR = TPSS/SSS-sync + Tmeas, whereTPSS/SSS-sync is the cell search time and Tmeas is the measurement time due to cell selection criteria evaluation.

TSI-NR: It is the time required for acquiring all the relevant system information of the target NR cell. This time depends upon whether the UE is provided with the relevant system information of the target NR cell or not by the old NR cell before the RRC connection is released.

TRACH: It is the delay caused due to the random access procedure when sending random access to the target NR cell. TRACH can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [39].

Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cell in the redirection command, otherwise Trs is the SMTC periodicity configured in the measObjectNR having the same SSB frequency and subcarrier spacing configured for the RRC connection release with redirection. If the measObjectNRs having the same SSB frequency and subcarrier spacing configured by MN and SN have different SMTC, Trs is the periodicity of one of the SMTC which is up to UE implementation. If the UE is not provided with SMTC configuration or measurement object for the frequency which is also configured for the RRC connection release with redirection then:

-	the requirement in this section is applied with Trs = 20 ms assuming the SSB transmission periodicity is not larger than 20 ms,

-	there is no requirement if the SSB transmission periodicity is larger than 20 ms.

Table 6.3.2.4-1: Time to identify target NR cell for RRC connection release with redirection to NR

| Frequency range (FR) of target NR cell | Tidentify-NR |
| --- | --- |
| FR1 | MAX (680 ms, 11 x Trs) |
| FR2 | MAX (880 ms, 88 x Trs) |

### <End of Change 1>