3GPP TSG-RAN WG4 Meeting#100-e	R4-2114566

Online, 16th Aug – 27th Aug, 2021

Source: 	Qualcomm Incorporated

Title: 	Views on Inter-cell Interference PDSCH Tests

Agenda item:	9.12.2.1.1

Document for:	Discussion

- Introduction

In RAN4#99-e, WF [1] was agreed and it had a lot of open issues. In this paper, we provide our views on those open issues for inter-cell interference scenarios.

- PDSCH Test Parameters

Channel Bandwidth

There were couple of options listed in [1] for channel bandwidths. However, bandwidth is not going to alter UE receiver processing. So, we don’t see any benefit of defining the requirements with multiple bandwidths and it unnecessarily increases the test burden. Therefore, we propose the following.

Proposal 1: Only consider 10MHz CBW for 15kHz SCS and 40MHz CBW for 30kHz SCS.

SSB Configuration

In the last meeting, it was agreed that TRS of serving cell will collide with TRS of interfering cell. This will degrade the quality of TRS for tracking timing/frequency drift. So, only other reference signal which can help with the tracking is SSB and we prefer to not have colliding SSB. Otherwise, UE will not have any clean reference signal and it will impact UE’s performance in real world. Therefore, we propose the following.

Proposal 2: Serving cell SSB and interference cell(s) SSB(s) are specified in the different time/frequency resources.

HetNet Deployment

As we are considering MMSE-IRC receiver, UE will be agnostic to homogeneous or heterogeneous requirements. Therefore, we propose the following.

Proposal 3: Do not define test cases for HetNet deployment.

Number of Interfering Cells

As we go from one interfering cell to two interfering cells, it does not change interference structure. Having two cells only increase the interference level. If we maintain the same SINR, 1 cell vs 2 cell will not matter much. Therefore, we propose the following.

Proposal 4: Only consider 1 interfering cell for defining the requirements.

Methodology

We prefer to use INR and SNR methodology because it is easier to read in the spec. Typically, spec reader is interested in knowing serving signal level and interference signal levels to get an idea of SIR. With DIP methodology, reader will have to make extra computations to get to SNR and INR levels. Even in later LTE releases, INR methodology was used for the same reason. Therefore, we propose the following.

Proposal 5: Use INR methodology for interference profile configuration.

- Conclusions

This paper provides our views on open issues for inter-cell interference scenarios in RAN4. Following has been observed and proposed.

Proposal 1: Only consider 10MHz CBW for 15kHz SCS and 40MHz CBW for 30kHz SCS.

Proposal 2: Serving cell SSB and interference cell(s) SSB(s) are specified in the different time/frequency resources.

Proposal 3: Do not define test cases for HetNet deployment.

Proposal 4: Only consider 1 interfering cell for defining the requirements.

Proposal 5: Use INR methodology for interference profile configuration.

# References

[1] R4-2108664, “Way Forward on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC”, 3GPP TSG RAN WG4 Meeting #99-e, Online, May 19 - May 27, 2021.