3GPP TSG-RAN WG4 Meeting # 100e	R4-2114695

# Electronic Meeting, Aug. 19-27, 2021

Source: 	Skyworks Solutions, Inc.

Title: 	n3 PC2 MSD

Agenda Item:	10.5.3 Analyses on receiver sensitivity degradation [FS_NR_PC2_UE_FDD]

Document for:	Approval

# Introduction

This document is a revision of R4-2114580. It presents a complete measurement dataset to evaluate the n3 PC2 MSD for all channel bandwidth (CBW).

# Discussion

## Power Amplifier Calibration

- PA calibration point is 20 MHz, 15 kHz, QPSK, DFT-S-OFDM, 100 RB at lower channel edge with 1 dB MPR - PC2 ACLR -31dBc;

- Post PA losses: 4dB;

- Power Class 3 (PC3) operation;

- Local Oscillator (LO) leakage: -28dBc; and

- IQ Image rejection: -28dB.

## Measurement Results and MSD Proposal

Complete measurement dataset could not be entirely retrieved in due time for R4-2114580. This revision provides the complete dataset for all CBW. It should be noted that PA calibration has been corrected. The measured noise level for 50MHz CBW is approximately 3.7dB higher than initially reported.

We use the same UL RB allocations than those agreed for PC3.

The PA noise levels in the receiver band and corresponding MSD levels can be found in Figure 1 below

.

Figure 1 Measured Tx noise and corresponding MSD levels

The estimated MSD levels for in 35MHz, 40MHz, 45MHz and 50MHz CBW lead to REFSENS levels that are lower than those agreed for PC3 operation. For these CBW, we tentatively propose to use the same approach as in [1], i.e. we propose PC2 REFSENS levels by evaluating the MSD level difference between PC3 and PC2 and apply this MSD difference to PC3 REFSENS levels. We observe the following MSD difference vs CBW:

- 3.1dB for 50MHz CBW: PC2 MSD of 7.2dB vs PC3 MSD of 4.1dB [1],

- 2.5dB for 45MHz CBW: PC2 MSD of 4.5dB vs PC3 MSD 2dB [1],

- 0.6dB for 40MHz CBW: PC2 MSD of 2.0 dB vs PC3 MSD of 1.5dB [1],

- 0.2dB for 35MHz CBW: PC2 MSD of 0.9 dB vs PC3 MSD of 0.7 dB [1].

For CBW less than or equal to 30MHz, the PC2 MSD can be neglected.

n3 PC2 REFSENS levels and UL configurations are proposed in Table 1 and Table 2.

Proposal: Table 1 n3 PC2 REFSENS levels and Table 2 Uplink Configuration.

Table 1: n3 PC2 REFSENS

| CA configuration | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n3 | 15 | -97.0 | -93.8 | -92.0 | -90.8 | -89.7 | -88.9 | -86.0 | -81.7 | -78.8 | -76.6 | FDD |
|  | 30 |  | -94.1 | -92.1 | -91.0 | -89.8 | -89.0 | -86.1 | -81.8 | -78.9 | -76.7 |  |
|  | 60 |  | -94.5 | -92.4 | -91.2 | -90.0 | -89.1 | -86.2 | -82.0 | -79.0 | -76.8 |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2: n3 PC2 REFSENS Uplink Configuration

| CA configuration | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n3 | 15 | 25 | 501 | 501 | 501 | 501 | 501 | 501 | 501 | 501 | 501 | FDD |
|  | 30 |  | 241 | 241 | 241 | 241 | 241 | 241 | 241 | 241 | 241 |  |
|  | 60 |  | 101 | 101 | 101 | 101 | 101 | 101 | 101 | 101 | 101 |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |  |  |  |  |  |  |  |  |

# Conclusions

This revised document presents a complete PC2 measurement dataset and evaluates the corresponding n3 PC2 MSD levels. REFSENS are proposed in Table 1 and uplink configurations in Table 2 below.

Proposal: Table 1 n3 PC2 REFSENS levels and Table 2 Uplink Configuration.

Table 1: n3 PC2 REFSENS

| CA configuration | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n3 | 15 | -97.0 | -93.8 | -92.0 | -90.8 | -89.7 | -88.9 | -86.0 | -81.7 | -78.8 | -76.6 | FDD |
|  | 30 |  | -94.1 | -92.1 | -91.0 | -89.8 | -89.0 | -86.1 | -81.8 | -78.9 | -76.7 |  |
|  | 60 |  | -94.5 | -92.4 | -91.2 | -90.0 | -89.1 | -86.2 | -82.0 | -79.0 | -76.8 |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2: n3 PC2 REFSENS Uplink Configuration

| CA configuration | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n3 | 15 | 25 | 501 | 501 | 501 | 501 | 501 | 501 | 501 | 501 | 501 | FDD |
|  | 30 |  | 241 | 241 | 241 | 241 | 241 | 241 | 241 | 241 | 241 |  |
|  | 60 |  | 101 | 101 | 101 | 101 | 101 | 101 | 101 | 101 | 101 |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |  |  |  |  |  |  |  |  |

# References

[1] 	R4-2111528, n3 50MHz REFSENS, A-MPR, 3GPP TSG-RAN WG4 Meeting#99-e, Skyworks Solutions, Inc.