3GPP TSG-RAN4#100-e	R4-2114696 revivion of R4-2111741

16th– 27th August 2021

Source:                 Qualcomm Incorporated

Title: 		    PC2 A-MPR for NS_50 on n39

Agenda Item: 	    8.32.2.1

Document for:     Approval

1 Introduction

##### 6.5.3.3.16	Requirement for network signalling value "NS_50"

When "NS_50" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.16-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.16-1: Additional requirements for "NS_50"

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 1805 | - | 1855 | -40 | 1 | 1 |
| Frequency range | 1855 | - | 1880 | -15.5 | 5 | 1, 2, 3 |
| NOTE 1: This requirement is applicable for carriers with aggregated channel bandwidths confined in 1885-1920 MHz for 25MHz and 30MHz channel BWs and confined in 1880-1920 MHz for 40MHz channel BW. NOTE 2: The requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 3: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |

According to the WF [1] some preliminary simulation results are provided for the discussion on n39 PC2 A-MPR. New A-MPR may be needed for PC2 UEs to meet the additional emission requirements as signalled by NS_50 [2]. In addition, the n39 -> EUTRA band 3 coexistence requirements are removed, and it is necessary to compute the back off for 15MHz and 20MHz channel BWs.

2 Discussion

The following assumptions for the simulations were used:

- Power Class 2

- Calibration: 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB, -31dBc ACLR

- Carrier Leakage: 28dBc

- Image: 28dBc

- CIM3: 60dBc

- CIM5: 70dBc

The PC2 A-MPR measurement and simulation results for NS_50 is presented in Table 1.

| Waveform | Mode | CBW (MHz) | Tx Freq. (MHz) | RBStart | LCRB | Measurement PC2 | Simulation | Region | WF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s | ET | 15 | 1892.5 | 0 | 75 | 2.6 | 3.5 | new | TBD |
| DFT-s | ET | 20 | 1895 | 0 | 54 | 2.1 | 3 | new | TBD |
| DFT-s | ET | 20 | 1895 | 0 | 100 | 2.63 | 3.5 | new | TBD |
| DFT-s | ET | 25 | 1897.5 | 0 | 128 | 3 | 4.5 | A7 | 5.5 |
| DFT-s | ET | 25 | 1897.5 | 52 | 81 | 1.1 | 4 | A4 | 2.5 |
| DFT-s | ET | 30 | 1900 | 0 | 160 | 5.3 | 4.5 | A7 | 5.5 |
| DFT-s | ET | 30 | 1900 | 66 | 90 | 1.7 | 4 | A4 | 2.5 |
| DFT-s | APT | 40 | 1900 | 0 | 216 | 11.6/10.4 | 11.5 | A1 | 11.5 |
| DFT-s | APT | 40 | 1900 | 24 | 192 | 9.8/110.2 | 9.4 | A2 | 8 |
| DFT-s | ET | 40 | 1900 | 24 | 60 | 3 | 3 | A3 | 4 |

| Waveform | Mode | CBW (MHz) | Tx Freq. (MHz) | RBStart | LCRB | Measurement (PC2) | Simulation (PC2) | Region | WF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | ET | 15 | 1892.5 | 0 | 79 | 4.51 | 4.5 | new | TBD |
| CP-OFDM | ET | 20 | 1895 | 0 | 54 | 4 | 4 | new | TBD |
| CP-OFDM | ET | 20 | 1895 | 0 | 106 | 4.51 | 4.5 | new | TBD |
| CP-OFDM | ET | 25 | 1897.5 | 0 | 133 | 6.2 | 6.5 | A7 | 7 |
| CP-OFDM | ET | 25 | 1897.5 | 52 | 81 | 3 | 4 | A4 | 4 |
| CP-OFDM | ET | 30 | 1900 | 0 | 160 | 6.6 | 6.5 | A7 | 7 |
| CP-OFDM | ET | 30 | 1900 | 66 | 94 | 3 | 4 | A4 | 4 |
| CP-OFDM | APT | 40 | 1900 | 0 | 216 | 13.3/12.3 | 12 | A1 | 12.5 |
| CP-OFDM | APT | 40 | 1900 | 24 | 192 | 10.6/10.1 | 9.6 | A2 | 9 |
| CP-OFDM | ET | 40 | 1900 | 24 | 60 | 4.4 | 4.5 | A3 | 5.5 |

3 Proposal

There are few changes (highlighted in yellow) than in WF. Region A9 is removed and region A6 is corrected for 25MHz. It now matches with the A6 region in 30MHz.

Table 1. A-MPR regions for NS_50, together with A-MPR values

| Channel Bandwidth (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 15 MHz | ≤ LCRB*12*SCS – 6.48 | >6.48 | A5 |
| 20 MHz | ≤ LCRB*12*SCS – 6.48 | >6.48 | A5 |
|  | ≤ 1.44 | <1.44 | A8 |
| 25 MHz | ≤ LCRB*12*SCS - 5 | > 5 | A7 |
|  | ≤ 6.3 | ≤ 1.44 | A8 |
|  | ≤ 1.8 | >1.44,< RBstart*12*SCS+5 | A6 |
|  | >8.28 | > max (21.6 – RBstart*12*SCS, 0), < RBstart*12*SCS+5 | A4 |
| 30 MHz | ≤ LCRB*12*SCS - 5 | > 5 | A7 |
|  | ≤ 7.56 | ≤ 1.44 | A8 |
|  | ≤ 1.8 | >1.44,< RBstart*12*SCS+5 | A6 |
|  | >10.8 | > max (26.64 – RBstart*12*SCS, 0), < RBstart*12*SCS+5 | A4 |
| 40 MHz | ≤ 4.32 | > 0 | A1 |
|  | > 4.32, ≤ 12.96 | ≤ 10.8 | A3 |
|  | > 4.32, ≤ 18 | > 10.8 | A2 |
|  | > 18, ≤ 31.68 | > max (31.68 – RBstart*12*SCS, 0) | A6 |
|  | > 31.68 | > 0 | A1 |
| NOTE 1: The A-MPR values are specified in Table 2. |  |  |  |

Table 2. A-MPR values for NS_50

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) | A7 (dB) | A8 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 11.5 | ≤ 10 | ≤ 4 | ≤ 2.5 | ≤ 3 | ≤ 2.5 | ≤ 4.5 | ≤ 4 |
|  | QPSK | ≤ 11.5 | ≤ 10 | ≤ 4 | ≤ 2.5 | ≤ 3 | ≤ 2.5 | ≤ 5.5 | ≤ 4 |
|  | 16 QAM | ≤ 11.5 | ≤ 10 | ≤ 4 | ≤ 2.5 | ≤ 3 | ≤ 2.5 | ≤ 5.5 | ≤ 4 |
|  | 64 QAM | ≤ 11.5 | ≤ 10 | ≤ 4 |  | ≤ 3 |  | ≤ 5.5 | ≤ 4 |
|  | 256 QAM | ≤ 11.5 | ≤ 10 |  |  |  |  | ≤ 5.5 |  |
| CP-OFDM | QPSK | ≤ 12.5 | ≤ 10 | ≤ 5.5 | ≤ 4 | ≤ 4.5 | ≤ 4 | ≤ 7 | ≤ 4 |
|  | 16 QAM | ≤ 12.5 | ≤ 10 | ≤ 5.5 | ≤ 4 | ≤ 4.5 | ≤ 4 | ≤ 7 | ≤ 4 |
|  | 64 QAM | ≤ 12.5 | ≤ 10 | ≤ 5.5 |  | ≤ 4.5 | ≤ 4 | ≤ 7 | ≤ 4 |
|  | 256 QAM | ≤ 12.5 | ≤ 10 |  |  |  |  | ≤ 7 |  |

Proposal 1: Use allocations regions found in Table 1 and A-MPR proposed in Table 2 for 15MHz,20MHz, 25MHz, 30MHz and 40MHz CBW NS_50.

4 Simulations

4.1

5 References

[1] R4-2107738, “WF on A-MPR for PC2 n39”, Huawei, HiSilicon, Apple, Nokia, 3GPP TSG RAN4#99-e

[2] 3GPP TS 38.101-1 v16.6.0