3GPP TSG-RAN WG4 Meeting # 100-e                                                        R4-2114757

Electronic Meeting, Aug. 16-Aug. 27, 2021

Source:	ZTE Corporation

Title:	TP for 36.717-03-02: CA_1A-20A-38A

Agenda Item:	11.5.3

Document for:	Approval

- Introduction

This contribution provides a text proposal to add UL configuration CA_1A-20A to CA_1A-20A-38A for TR 36.717-03-02.

- Reference

- TR 36.717-03-02, LTE-A inter-band CA for x bands (x=3,4,5) DL with 2 bands UL in Rel-17, v0.3.0

- Text Proposal

----- Start of TP -----

  - 6.x		LTE-A inter-band CA: Band 1 and Band 20 and Band 38 DL with 2 bands UL

    - 6.x.1	List of specific combination issues

      - 6.x.1.1	Channel bandwidths per operating band for CA

Table 6.x.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-20A-38A | CA_1A-20A | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

      - 6.x.1.2	Co-existence studies for LTE-A UL CA_1A-20A and DL CA_1A-20A-38A

For 2UL / 3DL own receiver desensitization study 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.x.1.2-1

Table 6.x.1.2-1: Harmonic and IMD analysis for UL_CA_1A-20A

| UL frequency | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
|  | 1920 | 1980 | 832 | 862 |
| 2nd harmonic products | 3840 | 3960 | 1664 | 1724 |
| 3rd harmonic products | 5760 | 5940 | 2496 | 2586 |
| 4th harmonic products | 7680 | 7920 | 3328 | 3448 |
| 5th harmonic products | 9600 | 9900 | 4160 | 4310 |
| 2nd order IMD products | ¦fy_low-fx_high¦ | ¦fy_high-fx_low¦ | ¦fy_low+fx_low¦ | ¦fy_high+fx_high¦ |
| IMD frequency limits (MHz) | 1148 | 1058 | 2752 | 2842 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2978 | 3128 | 316 | 196 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4672 | 4822 | 3584 | 3704 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2116 | 2296 | 5504 | 5684 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4898 | 5108 | 516 | 666 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6592 | 6802 | 4416 | 4506 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1528 | 1348 | 7088 | 6818 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5248 | 5428 | 8512 | 8782 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1254 | 1464 | 4276 | 4036 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6336 | 6546 | 7424 | 7664 |

Based on Table 6.x.1.2-1:

- no IMD products by Band 1 and Band 20  falls into the own Rx frequency of Band 38.

- the 3rd harmonic of band 20 falls into the own Rx frequency of Band 38.

      - 6.x.1.3	MSD

According to the co-existence studies as above, there is no MSD requirements caused by IMD when uplink CA configurations CA_1A-20A is paired with downlink CA configuration CA_1A-20A-38A.

For the harmonic MSD requirements, it have already been solved in the fallback combination of CA_20A-38A. Thus there are no additional MSD requirement for the uplink CA configurations CA_1A-20A paired with downlink CA configuration CA_1A-20A-38A.

      - 6.x.1.4	∆TIB and ∆RIB values

The same requirements of lower order combination which are specified in TS36.101 can be applied.

----- End of TP -----