3GPP TSG-RAN WG4 Meeting # 101bis-e	R4-2200065

Electronic Meeting, January 17-25, 2022

Agenda Item: 6.22.2.3

Source: MediaTek Inc.

Title: Discussion on conditional PSCell addition

Document for: Discussion

- Introduction

In RAN4 #101-e meeting, a WF [1] was agreed on further discussions of conditional PSCell addition. There are still some remaining issues. In this paper, we provide some analysis on the remaining issues.

- Discussion

We will discuss the open issues following in the order of that captured in [1].

- Tprocessing in conditional PSCell addition delay

In RAN4#100-e meeting, the following agreements have been reached:

Issue 3-1-1: Scenarios for CPCA - CPAC related requirements are developed for: - ENDC: FR1+FR1, FR1+FR2 - NRDC: PCell on FR1+ PSCell FR2 Issue 3-1-4: Conditional PSCell addition delay Conditional PSCell addition delay can use the existing requirements of conditional PSCell change as a baseline - FFS: option 3: Be based on the existing requirements of conditional PSCell change but with Tprocessing = 40ms. - FFS: option 2: Be based on the existing requirements of conditional PSCell change but with removing the 2ms margin delay

Regarding the last FFS, it is agreed to keep the 2ms margin in last meeting. The first FFS is still open. The existing requirements in conditional PSCell change are specified as: Tprocessing = 20 ms when source and target cells are in the same FR, and Tprocessing = 40 ms when source and target cells are in different FRs. We prefer to follow the same principle. As per agreed in RAN4#100-e meeting that CPAC related requirements are developed for

- ENDC: FR1+FR1, FR1+FR2

- NRDC: PCell on FR1+ PSCell FR2

Therefore, for EN-DC FR1+FR1, Tprocessing = 20 ms. For EN-DC FR1+FR2 and NR-DC FR1+FR2, Tprocessing = 40 ms.

Proposal 1: In conditional PSCell addition delay requirement, Tprocessing = 20 ms when PSCell and PCell are in the same FR, and Tprocessing = 40 ms when PSCell and PCell are in different FRs.

- TUE_preparation in conditional PSCell addition delay

Certain company proposed to shorten TUE_preparation from 10ms to 8ms. We suggest to keep the legacy 10ms. For conditional PSCell change, TUE_preparation is the UE preparation time, and starts after UE realizes the condition of PSCell change is met and identity of new PSCell is determined. For conditional PSCell addition, similar UE preparation time is also needed.

Proposal 2: TUE_preparation shall be 10ms in conditional PSCell addition delay requirement.

- Summary

In this paper, the discussion on the remaining issues of conditional PSCell addition is provided. We have the following proposals:

Proposal 1: In conditional PSCell addition delay requirement, Tprocessing = 20 ms when PSCell and PCell are in the same FR, and Tprocessing = 40 ms when PSCell and PCell are in different FRs.

Proposal 2: TUE_preparation shall be 10ms in conditional PSCell addition delay requirement.

4 Reference

R4-2120334, WF on R17 further Multi-RAT Dual-Connectivity enhancements, Huawei, HiSilicon