3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2200079

Electronic Meeting, January 17-25, 2022

Title: 	Further discussion on measurement procedure requirements for NTN

Source: 	CATT

Agenda item:	6.13.5.5

Document for:	Discussion

- Introduction

In last RAN4 meeting, a way forward on NR NTN RRM requirements was approved [1]. About measurement procedure requirements, some agreements and conclusions have been reached as captured in section 5 and some issues  are still FFS and open. This document will further discuss these open issues and present our understanding and proposals.

- Discussion

1).  Issue 3-1: Multiple SMTCs and Measurement Gap

Issue 3-1-1: The maximum number of SMTCs per Frequency layer

Agreement (1st round GTW): - The maximum number of SMTCs configured per measurement object for the same ssbFrequency is 4. - FFS on the assumptions on the number of configured parallel SMTCs to be used for requirements definition - FFS if different UE capabilities to support measurements for multiple configured SMTCs are needed

It is agreed in WF [1] that the maximum number of SMTCs configured per measurement object for the same ssbFrequency is 4, and there are FFS for number of configured parallel SMTCs and UE capabilities for multiple configured SMTC. RAN4 has sent a reply LS to RAN2 [2] ask for these issues. It need not further discuss these FFS until received RAN2 reply LS.

Proposal 1: Wait RAN2 responding LS for discussion on issues for parallel SMTCs configured.

Issue 3-1-2: Capability on the number of Measurement Cell Groups

It has conclusion in WF [1] that revisit the issue in the next RAN4 meeting based on RAN2#116 agreements and/or a partial/full response to R4-2120308 “Reply LS on Multiple SMTCs for NR NTN” if made available by/during the next RAN4 meeting.

Until now RAN2 responding LS has not been received. RAN4 should wait the LS.

Proposal 2: Wait RAN2 responding LS for discussion on issues for UE capability on number of measurement cell groups.

Issue 3-1-3: Capability on the number of Measurement Carriers/Cells/SSBs

It is agreed that revisit the issue in this meeting and having 5 options in WF[1]. TN UE capability on the number of Cells/SSBs required for FR1 in current TN spec are as following:

For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

-	8 identified cells, and

-	14 SSBs with different SSB index and/or PCI on the intra-frequency layer, where the number of SSBs in the serving cell (except for the SCell) is not smaller than the number of configured RLM-RS SSB resources.

For each inter-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

-	4 identified cells, and

-	7 SSBs with different SSB index and/or PCI on the inter-frequency layer.

These requirements may be reused. But for NTN network, cell will be very larger and the signal’s level will be damping very more quickly than TN cell’s. The number of Cells/SSBs required for NTN UE could be reduced, such as 5/9 for intra-frequency and 3/5 for inter-frequency.

Proposal 3: The number of Cells/SSBs required for NTN FR1 UE can be reused TN requirements, or reduced suitably.

Issue 3-1-4: Measurement with multiple SMTCs

It is agreed in WF[1] following issues will be discussed in this meeting.

Issue 1: Determine in which way RAN4 should specify the requirements.

- Option 1: the UE is required to measure on the target neighbour cell with configured timing offsets and receive/transmit on the serving cell at the same time - Option 2: specify scheduling restrictions to avoid such complexity - Option 3: measurement gaps are used on the target neighbour cells

Three options are listed in WF [1]. We prefer option 1, i.e. the UE is required to measure on the target neighbour cell with configured timing offsets and receive/transmit on the serving cell at the same time. But some restriction may be needed such as: UE only measure on one SMTC window if multiple SMTC is overlapped.

Proposal 4: UE is required to measure on the target neighbour cell with configured timing offsets and receive/transmit on the serving cell at the same time. Some restriction may be needed such as: UE only measure on one SMTC window if multiple SMTC is overlapped.

Issue 2: FFS on scaling of the measurement period is needed if UE is configured to measure multiple SMTC windows per MO.

Issue 3: Scaling scheme on multi-SMTC for intra-frequency is needed when simultaneous measurement is invalid in case of partial overlapping. But simultaneous measurement in case of partial overlapping shall not be precluded, instead of optional support.

As proposed in issue 1, scaling of the measurement period is needed if UE is configured to measure multiple overlapped SMTC windows per MO. The scaling factor can be number of overlapped SMTC.

Proposal 5: The scaling of the measurement period can be number of overlapped SMTC.

Issue 4: Whether and how to define the applicability rule for requirements regarding the number for MOs

It is agreed that maximum 4 SMTCs configured per measurement object for the same ssbFrequency. The number for MO can be limited only one for same frequency and can be equal to carrier number of inter-frequency measurement plus intra-frequency carrier.

Proposal 6: The number for MOs can be limited one MO per carrier frequency.

Issue 5: Measurement with Gap-less vs. Gap-based

Proposal 7: The measurement requirements with Gap-less vs. Gap-based for TN UE can be reused. But number of Gap pattern configured parallel should be further discussed based GAP enhancement WI and RAN2 process.

Issue 6:

For intra-frequency MO without MG and inter-frequency MO without MG

Proposal 8: We prefer no scheduling restriction, and the maximum number of SMTCs per measurement object for the same ssbFrequency can be equal to 4.

For intra-frequency MO with MG and inter-frequency MO with MG

Proposal 9: Agree the conditions in WF [1] for intra-frequency MO with MG and inter-frequency MO with MG:

  - the maximum number of SMTCs per measurement object for the same ssbFrequency used by UE shall meet both of following conditions:

    - smaller than or equal to 4, and

    - guarantee these SMTCs can be contained in active measurement gaps. The concurrent MG number is up to the conclusions in MG enhancement WI.

3).  Issue 3-2: Measurement relaxation

Proposal 10: we prefer to reuse measurement relaxation for TN UE for NTN UE in GEO network, and no relaxation for NTN UE in LEO in rel-17.

- Conclusion

This document discussed measurement procedure requirements for NTN UE and presented following proposals.

Proposal 1: Wait RAN2 responding LS for discussion on issues for parallel SMTCs configured.

Proposal 2: Wait RAN2 responding LS for discussion on issues for UE capability on number of measurement cell groups.

Proposal 3: The number of Cells/SSBs required for NTN FR1 UE can be reused TN requirements, or reduced suitably.

Proposal 4: UE is required to measure on the target neighbour cell with configured timing offsets and receive/transmit on the serving cell at the same time. Some restriction may be needed such as: UE only measure on one SMTC window if multiple SMTC is overlapped.

Proposal 5: The scaling of the measurement period can be number of overlapped SMTC.

Proposal 6: The number for MOs can be limited one MO per carrier frequency.

Proposal 7: The measurement requirements with Gap-less vs. Gap-based for TN UE can be reused. But number of Gap pattern configured parallel should be further discussed based GAP enhancement WI and RAN2 process.

Proposal 8: We prefer no scheduling restriction, and the maximum number of SMTCs per measurement object for the same ssbFrequency can be equal to 4.

Proposal 9: Agree the conditions in WF [1] for intra-frequency MO with MG and inter-frequency MO with MG:

  - the maximum number of SMTCs per measurement object for the same ssbFrequency used by UE shall meet both of following conditions:

    - smaller than or equal to 4, and

    - guarantee these SMTCs can be contained in active measurement gaps. The concurrent MG number is up to the conclusions in MG enhancement WI.

Proposal 10: we prefer to reuse measurement relaxation for TN UE for NTN UE in GEO network, and no relaxation for NTN UE in LEO in rel-17.

- Reference

[1]	R4-2120307, WF on NR NTN RRM requirements, Qualcomm

[2]	R4-2120308, Reply LS on Multiple SMTCs for NR NTN, Qualcomm

- Annex: Agreement and conclusion for measurement procedure

Issue 3-1-5: Legacy smtc1 and smtc2 configuration

RAN4 to not consider legacy smtc2 configuration when multiple SMTCs with different offsets configured for NTN MO.

Issue 3-1-6: Measurement Gap

Conclusion: Needs further RAN2 progress. There are 5 options in WF [1].

Issue 3-1-7: Others (Related to RAN2 discussion)

Agreements: The following aspects can be further discussed.

- Question 1: Shall propagation delay estimation error, ephemeris error and UE position error be considered in SMTC/MG configuration? If yes, what is the level or order by estimation?

- Question 2: Mitigate the impact on SMTC/MG introduced by propagation delay estimation error, ephemeris error and UE position error, if applicable.

  - Option 1 Consider SMTC/MG window/length margin as [x]ms

  - Option 2 other methods are not precluded

3).  Issue 3-2: Measurement relaxation

Issue 3-2-1: Cell Service Time based Measurement Relaxation

Conclusion: RAN4 can discuss the following aspect further:

- Measurement can be relaxed based on cell service time information if exist and applicable. The detail scheme can be studied whereafter, e.g. no mandatory measurements before T2 or relaxed measurement required before T2. T2 is the starting time of [a new cell] measurement.

Issue 3-2-2: Location based Measurement Relaxation

Conclusion: RAN4 can discuss the following aspect further:

- Measurement period relaxation based on distance between UE and the [reference location].

Issue 3-2-3: TN Cell Measurement Relaxation in RRC Idle mode

Conclusion: RAN4 can discuss the following aspect further:

- For idle mode, the UE upon detecting any NTN cell, shall meet legacy (non-relaxed) measurement requirements (low mobility relaxation can refer to section 4.2.2.9 in TS38.133) for TN cells even if it is configured with and meets the relaxed measurement criterion for TN cells.