3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2200082

Electronic Meeting, January 17-25, 2022

Title: 	Discussion on ACIR requirement for 71 GHz

Source: 	CATT

Agenda item:	6.16.5

Document for:	Approval

- Introduction

The co-existence simulation requirements were provided in last meeting, but the ACIR requirements were not agreed. This contribution provides our proposal for the ACIR requirements for 71GHz.

- ACIR requirement

According to the simulation results summarized in the WF [1], the provided results from many companies show more stringent requirement than the requirement in TR 38.803. Some companies think the requirements should be derived from the simulation results in this WI. The maximum ACIR value from companies was used in R15, it’s 25.5 dB for DL, 22dB for UL. It’s very stringent compared with R15 even for 30GHz, especially for the UL ACIR, R15 requirement for 30 GHz is 15.2 dB.  If average is used, the requirement would be 20.3 dB for DL, 16.6 dB for UL. 20.3 dB DL ACIR is close to the 48GHz requirement 20.5 dB but 16.6 dB is still more stringent than 48GHz ACIR. The provided UL simulation results are very divergent, for example, the maximum is 22 dB the smallest is 12 dB. So in theory, more checking or alignment is needed. However, considering the limited time for the WI, some compromise can be considered. The solution suggested from our side is to reuse the current 48GHz ACLR/ACS requirements. Then the ACIR,ACLR,ACS is as follows are proposed for 71GHz.

| DL ACIR | BS ACLR | UE ACS |
| --- | --- | --- |
| 20.5 dB | 26 dB | 22 dB |

| UL ACIR | BS ACS | UE ACLR |
| --- | --- | --- |
| 15 dB | 23 dB | 16 dB |

Proposal: 48GHz ACIR/ACLR/ACS requirements are reused for 71GHz for both DL and UL.

- Conclusion

This contribution provides our proposal for 71GHz ACIR/ACLR/ACS according to the simulation results provided in last RAN4 meeting.

Proposal: 48GHz ACIR/ACLR/ACS requirements are reused for 71GHz for both DL and UL.

Reference

[1] R4-2120001, “WF on DL/UL ACIR and BS/UE ACLR/ACS for FR2-2”, CATT