3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2200095

Electronic Meeting, January 17-25, 2022

Agenda item:	6.8.2.2

Source: 	CATT

Title: 	Discussion on remaining issues for inter-frequency measurement for FR1 HST

Document for:	Discussion

Introduction

There were extensive discussions for FR1 HST during the past RAN4 meetings and a WF on RRM requirements for NR FR1 HST was approved [1] in RAN4#101-e meeting.

In this contribution, we provide our consideration of remaining open issue for inter-frequency measurement and give our proposals.

Discussion

- Inter-frequency measurement in RRC_Connected state

For with MG, the measurement delay requirement for inter-frequency measurement with MG in HST in connected state for HST has been agreed in last meeting as below:

Issue 2-2-1: PSS/SSS detection time requirement for inter-frequency measurement with MG in connected state for HST - Agreements in 2nd round: DRX cycle; T PSS/SSS_sync_inter for FR1 HST. No DRX; max(600ms, N1 Max(MGRP, SMTC period)) CSSFinter N1 = 7. DRX cycle ≤ 160ms; max(600ms, ceil(N2) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter N2 = 7 x M2. 160ms < DRX cycle ≤ 320ms; ceil(N3) x DRX cycle x CSSFinter N3 = 7 x M2. DRX cycle>320ms; FFS. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1.

In previous meetings, companies argued for the number of AGC samples. Finally, all companies compromised to align with the values using in measurement delay requirements.

The only “FFS” item is the PSS/SSS detection time when DRX cycle>320ms. In last meeting, the proposed compromised option is:

N4 x DRX cycle x CSSFinter N4 = 4 x M2 NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1

In our view, the basic principle is that the detection time for inter-frequency measurement should be larger than the detection time for intra-frequency measurement because there are additional processes for inter-frequency measurement compared to intra-frequency measurement. At least it cannot be less than that in intra-frequency measurement.

In current intra-frequency requirements when DRX cycle is larger than 320ms is shown as:

| DRX cycle>320ms | 5 x max(MGRP, DRX cycle) x CSSFintra |
| --- | --- |

For the proposed compromised option, N4= 4 if SMTC periodicity<=40ms or 6 if SMTC periodicity > 40ms. If SMTC periodicity<=40ms, the detection time is even smaller than intra-frequency measurement which is not aligned with basic rules. Therefore, our proposal is as below:

Proposal 2: PSS/SSS detection time requirement for inter-frequency measurement with MG in connected state for HST

| DRX cycle | T PSS/SSS_sync_inter for FR1 HST |
| --- | --- |
| No DRX | max(600ms, N1 Max(MGRP, SMTC period)) CSSFinter N1 = 7 |
| DRX cycle ≤ 160ms | max(600ms, ceil(N2) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter N2 = 7 x M2 |
| 160ms < DRX cycle ≤ 320ms | ceil(N3) x DRX cycle x CSSFinter N3 = 7 x M2 |
| DRX cycle>320ms | N4 DRX cycle CSSFinter N4 = 6 if SMTC periodicity>40ms N4 = 5 if SMTC periodicity≤40ms |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 |  |

Conclusion

This document presents our consideration of open issue for inter-frequency measurement. The following proposal is concluded as below:

Proposal 1: PSS/SSS detection time requirement for inter-frequency measurement with MG in connected state for HST

| DRX cycle | T PSS/SSS_sync_inter for FR1 HST |
| --- | --- |
| No DRX | max(600ms, N1 Max(MGRP, SMTC period)) CSSFinter N1 = 7 |
| DRX cycle ≤ 160ms | max(600ms, ceil(N2) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter N2 = 7 x M2 |
| 160ms < DRX cycle ≤ 320ms | ceil(N3) x DRX cycle x CSSFinter N3 = 7 x M2 |
| DRX cycle>320ms | N4 DRX cycle CSSFinter N4 = 6 if SMTC periodicity>40ms N4 = 5 if SMTC periodicity≤40ms |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 |  |

Reference

[1] R4-2120285, WF on RRM for FR1 HST, CMCC